3GPP TSG-RAN WG4 Meeting AH-1807                                                       R4-1808907

Montreal ,Canada, 2nd  – 6th July, 2018

Agenda Item:   5.4.1.5

Source:        Samsung

Title:          Views on CQI test cases

Document for:  Discussion

Introduction

A dedicated specification was assigned for NR UE performance requirements as 38.101-4. The target competition time for Rel-15 NR performance part is RAN#82 in Dec 2018. In this contribution, preliminary views for UE CQI requirements were provided.

Discussion

CQI Scope:

Generally, CQI test cases can be categorized as below test cases:

- Static CQI definition test: verify UE properly implementation of effective SNR calculation and SNR to CQI table mapping followed CQI definition

- Wideband CQI test: verify UE properly tracking channel/interference variation in time domain to avoid excessive averaging across sub-frames

- Sub-band CQI test: verify UE properly tracking channel/interference variation in frequency domain to avoid excessive averaging across frequency domain

Proposal 1: For CQI test, both static CQI definition test, wideband fading CQI test and sub-band frequency selective test need to be introduced.

Test Metric

In general, existing test metric can be reused for static CQI test at least in FR1. For FR2, FFS pending on further conclusion for achievable SNR accuracy and SNR range of TE implementation under ‘pure baseband’ test methodology.

Proposal 2: For static CQI test, test metrics (CQI distribution and BLER requirements) as existing LTE test can be reused as starting point at least for FR1 and for FR2, FFS pending on achievable SNR accuracy and SNR range of TE implementation under ‘pure baseband’ test methodology.

Test Set-up

For CQI tables, three types of CQI tables introduced in TS 38.214:

- Table 1 (without 256QAM), table 2 with 256QAM for target BLER 10%

- Table 3 with configurable target BLER

Followed by agreements from UE feature list, 256QAM supported in FR1 is mandatory, and for FR2, 256QAM not supported in Rel-15.

Proposal 3: In Rel-15, only focused on CQI table 1 and CQI table 2 (FR1 only) for introducing CQI test.

Channel bandwidth &SCS: In Rel-15, several combinations of channel Bandwidth and SCS supported, and UE processing of supporting flexible combinations of channel bandwidth and numerologies can be verified by demodulation test cases with several cases covering different combinations. For CSI test cases, a generic test set-up can be used:

Proposal 4: For CSI test cases, below channel bandwidth and numerology combination can be used:

- FR1 FDD: 10MHz & 15kHz

- FR1 TDD: 40MHz & 30kHz

- FR2: 100MHz &120kHz

Proposal 5: MIMO configuration: For static CQI test, similar as LTE, 2Tx2Rx, 2Tx4Rx for FR1 and 2Tx2Rx for FR2 with dual CWs enabled.

Proposal 6: Periodic CSI report and periodic CSI-RS resource used for static CQI test cases.

Based on above analysis, a detailed test set-up proposed in below table.

| Parameter | Unit | Static CQI test (FR1) | Static CQI test (FR2) |
| --- | --- | --- | --- |
| System parameter |  |  |  |
| Bandwidth/SCS | MHz/kHz | 40MHz/30KHz (FDD) 10MHz/15kHz (TDD) | 100MHz/120kHz |
| Transmission Scheme |  | 1 | 1 |
| Duplex Mode |  | FDD &TDD | TDD |
| DLUL configuration |  | FFS | FFS |
| Propagation channel |  | AWGN | AWGN |
| Correlation and antenna configuration |  | 2Rx 2*2 with 4Rx: 2*4 with | 2Rx 2*2 with |
| CSI-RS resource setting (periodic) |  |  |  |
| resourceConfigType |  | FFS | FFS |
| CSI-RS/CSI-IM periodicity and slot offset |  | FFS | FFS |
| NZP CSI-RS ports |  | 2 port {3000,3001} | 2 port {3000,3001} |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density |  | FD-CDM2/density 1 | FD-CDM2/density 1 |
| CSI-IM configuration |  | FFS | FFS |
| CSI report setting (periodic) |  |  |  |
| reportConfigType |  | periodic | periodic |
| Reporting periodicity and slot offset |  | FFS | FFS |
| Codebook Restriction |  | 010000 | 010000 |
| Reporting granularity |  | Single CQI reporting | Single CQI reporting |
| Physical channel for reporting |  | PUSCH | PUSCH |
| PUCCH reporting format |  | FFS | FFS |
| CQI table |  | Table 2 with 256QAM | Table 1 without 256QAM |

Conclusion

In this contribution, preliminary views for UE CQI requirements were provided.

CQI Test

Proposal 1: For CQI test, both static CQI definition test, wideband fading CQI test and sub-band frequency selective test need to be introduced.

Proposal 2: For static CQI test, test metrics (CQI distribution and BLER requirements) as existing LTE test can be reused as starting point at least for FR1 and for FR2, FFS pending on achievable SNR accuracy and SNR range of TE implementation under ‘pure baseband’ test methodology.

Proposal 3: In Rel-15, only focused on CQI table 1 and CQI table 2 (FR1 only) for introducing CQI test.

Proposal 4: For CSI test cases, below channel bandwidth and numerology combination can be used:

- FR1 FDD: 10MHz & 15kHz

- FR1 TDD: 40MHz & 30kHz

- FR2: 100MHz &120kHz

Proposal 5: MIMO configuration: For static CQI test, similar as LTE, 2Tx2Rx, 2Tx4Rx for FR1 and 2Tx2Rx for FR2 with dual CWs enabled.

Proposal 6: Periodic CSI report and periodic CSI-RS resource used for static CQI test cases.

A detailed test set-up also provided for static CQI test cases in this contribution.

Reference

[1] R4-1808027, “Way Forward on NR UE performance requirements”, Ericsson