3GPP TSG-RAN WG4 #8x	R4-1809115

Montreal, Canada, 2nd July – 6th July 2018

Source: 	Ericsson

Title:  	On synchronized LTE-NR general Test model parameters for TDD UL/DL configuration

Agenda Item:	5.1.2

Document for:	Approval

# Introduction

Discussions on NR test models for BS conformance is ongoing. Given the number of bandwidth, numerologies, and slot (subframe) configurations supported by NR, test models for TDD UL/DL configuration need to be synchronized both for LTE and NR. This contribution discusses NR uplink-downlink configurations for TDD test models to ensure that it fits the configurations specified for E-UTRA TDD test models.

# Discussion

## Background

Considering the number of different configurations NR supports, the number of BS conformance tests that need to be specified would increase significantly. Discussion in RAN4 include formulating procedures for suitable selection of test configurations such that the number of test permutations is reduced [1].

Moreover, the test configurations defined for NR TDD need ensure that the UL/DL configuration is aligned with LTE TDD UL/DL configuration in order to maintain test models specified E-UTRA TDD.

All E-UTRA test models use the following general parameters:

-	The test models are defined for a single antenna port (using p = 0); 1 code word (q = 0), 1 layer, precoding is not used; unless specified otherwise

-	Duration is 10 subframes (10 ms)

-	Normal CP

-	Virtual resource blocks of localized type, no intra-subframe hopping for PDSCH

-	UE-specific reference signals are not used

Power settings of physical channels are defined by physical channel EPRE relative to the EPRE of the RS. The relative accuracy of the physical channel EPRE as referred to the EPRE of the RS shall have a tolerance of ±0.5 dB.

TS36.141 specifies that for E-UTRA TDD test models are derived based on the UL/DL configuration 3 (Table 4.2-2) and special subframe configuration 8 (Table 4.2-1) defined in TS36.211, i.e. as shown in the table 6.1.1-1 (excluding Channel access procedure test for downlink operation in Band 46 where Frame structure Type 3 is defined in TS 36.211 clause 4.3 is used). Number of frames for the test models is 2. The tables mentioned above are re-written in Appendix 1.

## Adopting LTE TDD UL/DL configurations for NR

In adopting LTE TDD UL/DL configuration for NR, the objective is to ensure that NR frames used for TDD test models are aligned in time with LTE frames.  From the description of NR TDD UL/DL switching pattern configuration described in Appendix 2, it follows that LTE UL/DL configurations 0 and 3 can be configured for NR directly. That is, according to Table 2 Appendix 2, LTE UL/DL configuration 0 in Table 4.2-2 (TS36.211) can be configured as

- x = 5 ms with 1 DL slot and 3 UL slots

while UL/DL configuration 3 can be configured as

- x = 5 ms with 1 DL slot and 3 UL slots and

- y = 5 ms with 5 DL slots and 0 UL slot

where x and y refer to the period for TDD pattern 1 and TDD pattern 2 respectively, see: Appendix 2. The diagram below demonstrates the frame alignment of UL/DL configuration #0 and configuration #3 for 15kHz SCS. It should be noted that UL/DL configuration #3 is the configuration specified in TS36.141 for E-UTRA TDD TM.

LTE UL/DL configuration 0

LTE UL/DL configuration 3

Even though LTE UL/DL configurations #1, #2, #4 and #5 may still be configured for NR, there is the problem of misalignment in LTE-NR frame timings as demonstrated below for 15kHz SCS.

LTE UL/DL configuration 1

LTE UL/DL configuration 2

LTE UL/DL configuration 4

LTE UL/DL configuration 5

It is specified in TS 36.211 that LTE UL/DL configuration 6 cannot be configured for NR.

Proposal: For NR TDD test configurations, LTE UL/DL configuration #3 and special subframe configuration #8 can also be configured for NR as specified in TS36.141.

However, it should be noted here that the proposal stated above does not include UL/DL configuration for NB-IoT which cannot configured with UL/DL configuration #3. In its current form, UL/DL configuration for NB-IoT test models cannot be aligned with the test model specified E-UTRA TDD.  Due to time constraints, the work would need to happen in parallel to NR TM discussions, it is then proposed to use configuration #3 for NR and readdress this issue when the work has been completed regarding NB-IoT.

# Conclusion

This contribution looked at LTE and NR UL/DL configurations and showed that TDD UL/DL configuration #3 specified for E-UTRA test models in TS36.141 can also be configured for NR without any NR-LTE frame timing misalignment. The contribution then puts forward the proposal to re-use UL/DL configuration #3 for NR TDD test configurations.

Proposal: For NR TDD test configurations, LTE UL/DL configuration #3 and special subframe configuration #8 can also be configured for NR as specified in TS36.141.

# References

- R4-1802570, On NR conformance testing and excessive test permutations, Ericsson

- TS 36.141, Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing, v15.2.0

- TS 36.211, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing, v15.1.0

- TS 36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification, v15.1.0

# Appendix 1: Test configurations for TDD

The supported uplink-downlink configurations are listed in Table 4.2-2 of TS36.211 shown below, where, for each subframe in a radio frame, "D" denotes a downlink subframe reserved for downlink transmissions, "U" denotes an uplink subframe reserved for uplink transmissions and "S" denotes a special subframe with the three fields DwPTS, GP and UpPTS.

Table 4.2-2: Uplink-downlink configurations

| Uplink-downlink configuration | Downlink-to-Uplink Switch-point periodicity | Subframe number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 5 ms | D | S | U | U | U | D | S | U | U | U |
| 1 | 5 ms | D | S | U | U | D | D | S | U | U | D |
| 2 | 5 ms | D | S | U | D | D | D | S | U | D | D |
| 3 | 10 ms | D | S | U | U | U | D | D | D | D | D |
| 4 | 10 ms | D | S | U | U | D | D | D | D | D | D |
| 5 | 10 ms | D | S | U | D | D | D | D | D | D | D |
| 6 | 5 ms | D | S | U | U | U | D | S | U | U | D |

The length of DwPTS and UpPTS is then given by Table 4.2-1 of TS36.211, also shown below, subject to the total length of DwPTS, GP and UpPTS being equal to 30720∙Ts=1ms where X is the number of additional SC-FDMA symbols in UpPTS provided by the higher layer parameter srs-UpPtsAdd if configured otherwise X is equal to 0.

Table 4.2-1: Configuration of special subframe (lengths of DwPTS/GP/UpPTS)

| Special subframe configuration | Normal cyclic prefix in downlink |  |  | Extended cyclic prefix in downlink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DwPTS | UpPTS |  | DwPTS | UpPTS |  |
|  |  | Normal cyclic prefix in uplink | Extended cyclic prefix in uplink |  | Normal cyclic prefix in uplink | Extended cyclic prefix in uplink |
| 0 |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  | - | - | - |
| 9 |  |  |  | - | - | - |
| 10 |  |  |  | - | - | - |

Table 6.1.1-1: Configurations of TDD eNB test models

| Downlink-to-Uplink Switch-point periodicity | Number of UL/DL sub-frames per radio frame (10 ms) |  | DwPTS | GP | UpPTS |
| --- | --- | --- | --- | --- | --- |
|  | DL | UL |  |  |  |
| 10ms | 6 | 3 |  |  |  |

# Appendix 2: NR TDD pattern configuration

In NR, according to TS36.331 (6.3.2), cell-specific RRC signaling specify that a common TDD pattern can be semi-statically configured for all users in a cell. Additionally, individual users can be configured with a dedicated pattern overwriting individual slots of the configuration. The common TDD pattern can be a concatenation of a maximum of two TDD pattern (TDD-UL-DL-ConfigCommon) as shown in Figure 1.

TDD pattern 1

Must divide 20 ms evenly

TDD pattern 1

TDD pattern 2

Must divide 20 ms evenly

Figure 1: Common NR TDD patterns

Each of the two TDD pattern can be configured with a given TDD periodicity depending on the subcarrier spacing employed. Moreover, the combined periodicity for the cell-specific DL/UL assignments must divide 20ms evenly where the first slot of each even radio frame is taken to be a starting slot.

Table 1: UL-DL switching periodicity for different numerologies

| SCS [kHz] | Allowed UL-DL switching periodicity [ms] |
| --- | --- |
| 15 | 0.5, 1, 2, 5, 10 |
| 30 | 0.5, 1, 2, 2.5, 5, 10 |
| 60 | 0.5, 1, 1.25, 2, 2.5, 5, 10 |
| 120 | 0.5, 0.625, 1, 1.25, 2, 2.5, 5, 10 |

The downlink-uplink periodicity for each DL-unknown-UL includes the configurations stated in the table below.

Table 2: UL-DL switching configurations

| # of DL slots at the beginning of the period | 0,1,… |
| --- | --- |
| # of DL symbol(s) following the DL slots | 0,1,…,13 |
| # of UL symbols before full UL slots | 0,1,…,13 |
| # of full UL slots at the end of the period | 0,1,… |

Resource(s) between DL and UL segments are unknown resources. Figure 2 below is an example demonstration for D D D S U configuration

Figure 2: Example of D D D S U configuration