3GPP TSG-RAN WG4 Meeting AH-1807	R4-1809230

## Montreal, Canada, 2 – 6 July, 2018

Source: 	Huawei

Title:	TP to TS 38.141-2: manufacturer's declaration for OTA tests

Agenda Item:	5.1.4.3.6 (Declarations)

Document for:	Approval

# Introduction

In this TP to TS 38.141-2 it is proposed to follow the manufacturer declarations framework used in TS 38.141-1, where the applicability column was used for the BS type 1-C and BS type 1-H. In this TP to TS 38.141-2 the manufacturer declarations and their applicability table is proposed for radiated tests for BS type 1-H (for EIRP and EIS requirements, similar to the hybrid AAS BS), for BS type 1-O and BS type 2-O.

This TP was drafted on top of the latest version of the TS 38.141-2 [1]. Technical content of the OTA declarations for NR BS has been aligned with the manufacturer declaration descriptions proposed in DraftCR to TS 37.145-2 [2].

The following principles were considered while drafting the OTA declarations for NR BS:

- No OTA sensitivity for FR2, i.e. only OTA REFSENS for FR2

- No co-location requirements for FR2

- No MSR declarations considered

- Optional declaration for FR2 64QAM EVM test requirements was  introduced, based on [3]

NOTE: The below used declaration IDs are re-used from AAS BS and from TS 38.141-1 specifications – those ID numbers are used for the referencing purposes and are subject for the optimisation.

# Conclusions

Based on the above introduction, the following proposal is formulated:

Proposal 1:  approve the attached TP to TS 38.141-2 on manufacturer declarations for radiated testing.

# References

[1] 		Draft TS 38.141-2 v 0.2.0, shared after RAN4#87 on the RAN4 e-mail reflector

[2] 		R4-1809227	Draft CR to TS 37.145-2: Descriptions of the OTA declarations

[3]	 	R4-1809231	Optional declaration for 64QAM power backoff for BS type 2-O

# TP to TS 38.141-2

------------------------------ Modified section ------------------------------

## 4.6	Manufacturer’s declarations

The following BS declarations listed in table 4.6-1 are required to be provided by the manufacturer for radiated requirements testing for BS type 1-H, BS type 1-O and BS type 2-O.

For the BS type 1-H declarations required for the conducted requirements testing, refer to 3GPP TS 38.141-1 [3], subclause 4.6.

Table 4.6-1 Manufacturers declarations for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| D6.71 | BS class | BS class of the BS, declared as Wide Area BS, Medium Range BS, or Local Area BS. | c | x | x |
| D6.2 | BS requirements set | Declaration of one of the NR base station requirement’s set as defined for BS type 1-H, BS type 1-O, or BS type 2-O. | c | x | x |
| D9.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the BS enclosure. | x | x | x |
| D9.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the BS enclosure. | x | x | x |
| D9.3 | Beam identifier | A unique title to identify a beam, e.g. a, b, c or 1, 2, 3. The vendor may declare any number of beams, the minimum requirement to declare for conformance are the beams with the highest intend EIRP for each of the beams widths below: 1) Narrowest intended BeWθ, narrowest intended BeWϕ (possible when narrowest intended BeWθ) at the reference beam direction. 2) Narrowest intended BeWϕ, narrowest intended BeWθ (possible when narrowest intended BeWϕ) at the reference beam direction. 3) Widest intended BeWθ, widest intended BeWϕ (possible when widest intended BeWθ at the reference beam direction. 4) Widest intended BeWϕ, widest intended BeWθ (possible when widest intended BeWϕ) at the reference beam direction. 5) BeWθ and BeWϕ which provide highest intended EIRP of all possible beams at the reference beam direction. NOTE 1: Depending on the capability of the system some of these beams may be the same. When selecting the above five beam widths for declaration, all beams that the BS is intended to produce shall be considered, including beams that during operation may be identified by any kind of cell or UE specific reference signals, with the exception of any type of beam that is created from a group of transmitters that are not all phase synchronised. | x | x | x |
| D9.4 | Operating band support | NR operating band(s) supported by BS. Supported bands declared for every beam in D9.3. NOTE 2: these operating bands are related to their respective single-band RIBs. | c | x | x |
| D9.6 | NR supported channel bandwidths and SCS | NR channel bandwidth and SCS supported. Declared for each beam (D9.3) and each NR operating band (D9.4). | c | x | x |
| D9.7 | Reference beam direction pair | The beam direction pair, describing the reference beam peak direction and the reference beam centre direction. Declared for every beam (D9.3). | x | x | x |
| D9.8 | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam (D9.3). | x | x | x |
| D9.9 | Maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D9.3). | x | x | x |
| D9.10 | Rated beam EIRP | The rated EIRP level per carrier at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D9.9), as well as the reference beam direction pair (D9.7). Declared for every beam (D9.3). | x | x | x |
| D9.11 | Beamwidth | The beamwidth for the reference beam direction pair and the four maximum steering directions. Declared for every beam (D9.3). | x | x | x |
| D9.12 | Equivalent beams | List of beams which are declared to be equivalent. Equivalent beams imply that the beams are expected to have identical OTA peak directions sets and intended to have identical spatial properties at all steering directions within the OTA peak directions set when presented with identical signals. All declarations (D9.4-D9.11) made for the beams are identical. | x | x | x |
| D9.13 | Parallel beams | List of beams which have been declared equivalent (D9.12) and can be generated in parallel using independent RF power resources. Independent power resources means that the beams are transmitted from mutually exclusive transmitter units. | x | x | x |
| D9.14 | Number of carriers at maximum EIRP | The number of carriers per operating band the BS is capable of generating at maximum EIRP. Declared for every beam (D9.3). | x | x | x |
| D9.16 | Operating bands with multi-band dependencies | List of operating band combinations which are declared if those are generated using transceiver units supporting operation in multiple operating bands (i.e. multi-band transceiver units) through common active electronic component(s). NOTE 3: these operating bands are related to their respective multi-band RIBs. | c | x | n/a |
| D9.17 | Maximum radiated Base Station RF Bandwidth | Maximum Base Station RF Bandwidth in the operating band. Declared for each supported operating band (D9.4). | c | x | x |
| D9.18 | Maximum radiated Base Station RF Bandwidth for contiguous operation | Maximum Base Station RF Bandwidth for contiguous spectrum operation. Declared for each supported operating band (D9.4). | c | x | x |
| D9.19 | Maximum radiated Base Station RF Bandwidth for non- contiguous operation | Maximum Base Station RF Bandwidth for non-contiguous spectrum operation. Declared for each supported operating band (D9.4). | c | x | x |
| D9.26 | Maximum radio bandwidth with multi-band dependencies | Maximum radio bandwidth that can be supported by the operating bands with multi-band dependencies. Declared for each supported operating band which has multi-band dependencies (D9.16) | c | x | n/a |
| D9.20 | Inter-band CA bands | Declared inter-band CA bands supported per operating band (D9.4). | c | x | x |
| D9.21 | CA only operation | Declared per operating band (D9.4). | c | x | x |
| D9.27 | Total number of supported carriers for operating bands with multi-band dependencies | Total number of supported carriers for operating bands declared to have multi-band dependencies (D9.16). | c | x | x |
| D9.28 | Contiguous or non-contiguous spectrum | Ability to support contiguous or non-contiguous (or both) frequency distribution of carriers when operating multi-carrier. Declared per RIB. | c | x | x |
| D9.29 | Contiguous and non-contiguous parameters identical | If contiguous and non-contiguous operation is possible then parameters are the same. | c | x | x |
| D10.1 | OSDD identifier | A unique identifier for the OSDD. | x | x | n/a |
| D10.2 | OSDD operating band support | Operating band supported by the OSDD, declared for every OSDD (D10.1). NOTE 4: As each identified OSDD has a declared minimum EIS value (D10.6), multiple operating bands can be only be declared if they have the same minimum EIS declaration. | x | x | n/a |
| D10.4 | OTA sensitivity supported channel bandwidths and SCS | The NR channel bandwidths and SCS supported by each OSDD. | x | x | n/a |
| D10.5 | Redirection of receiver target support | Ability to redirect the receiver target related to the OSDD. | x | x | n/a |
| D10.6 | Minimum EIS | The minimum EIS requirement (i.e. maximum allowable EIS value) applicable to all sensitivity RoAoA per OSDD. Declared per supported channel BW for the OSDD (D10.4). Declared per OSDD identified in D10.1. The lowest EIS value for all the declared OSDD’s is called minSENS, while its related range of angles of arrival is called minSENS RoAoA. NOTE 5: If the BS is not capable of redirecting the receiver target related to the OSDD then there is only one RoAoA applicable to the OSDD. | x | x | n/a |
| D10.7 | Receiver target reference direction Sensitivity Range of Angle of Arrival | The sensitivity RoAoA associated with the receiver target reference direction (D10.9) for each OSDD. | x | x | n/a |
| D10.8 | Receiver target redirection range | For each OSDD the associated union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD. | x | x | n/a |
| D10.9 | Receiver target reference direction | For each OSDD an associated direction inside the receiver target redirection range (D10.8). NOTE 6: For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA. | x | x | n/a |
| D10.10 | Conformance test directions sensitivity RoAoA | For each OSDD that includes a receiver target redirection range, four sensitivity RoAoA comprising the conformance test directions (D10.11). | x | x | n/a |
| D10.11 | Conformance test directions | For each OSDD four conformance test directions. If the OSDD includes a receiver target redirection range the following four directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. If an OSDD does not include a receiver target redirection range the following 4 directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. | x | x | n/a |
| D11.2 | OTA coverage range | Declared as common range of directions within which selected TX OTA requirements are intended to be met. NOTE 7: OTA coverage range is used for conformance testing of such OTA Tx requirements as occupied bandwidth, frequency error, TAE or EVM. | x | x | x |
| D11.3 | The rated carrier OTA BS power, Pmax,c,TRP | Pmax,c,TRP is declared as TRP OTA power per carrier. Declared per supported operating band. | n/a | x | x |
| D11.4 | Equivalent passive BS gain | Declared value of the equivalent gain of the passive BS, used for translation of regional regulatory requirements for unwanted emissions (i.e. EIRP value using an assumed passive BS gain). | n/a | x | n/a |
| D6.3 | Spurious emission category | Declare the BS spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [5]. | c | x | x |
| D6.4 | Geographic area support | The manufacturer shall declare the regions the BS may operate in. e.g. CEPT. | c | x | x |
| [D6.5] | [Band n20 support, operating in geographical areas allocated to broadcasting (DTT)] | Editor’s note: this declaration is subject to technical discussion on the applicability of the DTT protection from NR BS. [If the BS supports Band n20, the manufacturer shall declare if the BS may operate in geographical areas allocated to broadcasting (DTT).] | [c] | [x] | n/a |
| [D6.6] | [Band n20 support, emission level for channel N (PEM,N)] | Editor’s note: this declaration is subject to technical discussion on the applicability of the DTT protection from NR BS. [If the BS supports Band n20 and has been declared to operate in geographical areas allocated to broadcasting (DTT), the emission level for channel N (as defined in annex G of 3GPP TS 36.104 []) shall be declared.] | [c] | [x] | n/a |
| [D6.7] | [Band n20 support, Maximum output Power in [10] MHz (P10MHz)] | Editor’s note: this declaration is subject to technical discussion on the applicability of the DTT protection from NR BS. [If the BS supports Band n20 and has been declared to operate in geographical areas allocated to broadcasting (DTT), the maximum output power in [10] MHz (as defined in Annex G of 3GPP TS 36.104 []) shall be declared.] | [c] | [x] | n/a |
| D11.11 | Co-existence with other systems | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA, PHS and/or NR operating in another band are deployed. | c | x | x |
| D11.12 | Co-location with other base stations | The manufacturer shall declare whether the BS under test is intended to operate co-located with Base Stations of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or NR operating in another band. | c | x | n/a |
| D11.14 | Single or multiple carrier | BS capability to operate with a single carrier (only) or multiple carriers. Declared per supported operating band (D9.4). | c | x | x |
| D11.15 | Maximum number of supported carriers per operating band | Maximum number of supported carriers. Declared per operating band (D9.4). | c | x | x |
| D11.16 | Other band combination multi-band restrictions | Declare any other limitation under simultaneous operation in the declared band combinations (D9.16), which have any impact on the test configuration generation. | c | x | n/a |
| D11.17 | Ncells | Number corresponding to the minimum number of cells that can be transmitted by a BS in a particular operating band. Declared per operating band (D9.4). | c | x | n/a |
| D11.18 | Maximum supported power difference between carriers | Maximum supported power difference between carriers in each supported operating band. Declared per operating band (D9.4). | c | x | x |
| D11.19 | Maximum supported power difference between carriers is different operating bands | Maximum supported power difference between any two carriers in any two different supported operating bands. Declared per operating bands combination (D9.16, D11.16). | c | x | n/a |
| D11.20 | Operating band combination support | List of operating bands combinations supported by BS. | c | x | n/a |
| D11.23 | Inter-band CA | Declaration of operating band(s) combinations supporting inter-band CA. Declared per operating band combination (D9.16, D11.16). | c | x | x |
| D11.24 | Intra-band contiguous CA | Declaration of operating band(s) supporting intra-band contiguous CA. Declared per operating band with CA support. | c | x | x |
| D11.25 | Intra-band non-contiguous CA | Declaration of operating band(s) supporting intra-band non-contiguous CA. Declared per operating band with CA support. | c | x | x |
| D11.26 | OTA REFSENS RoAoA | Range of angles of arrival associated with the OTA REFSENS. | x | x | x |
| D11.27 | OTA REFSENS receiver target reference direction | Reference direction inside the OTA REFSENS RoAoA (D11.26). | x | x | x |
| D11.28 | OTA REFSENS conformance test directions | The following four OTA REFSENS conformance test directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. | x | x | x |
| Dx.yy | 64QAM power back-off | Additional power back-off declared for 64QAM EVM test requirement for BS type 2-O. | n/a | n/a | x (Note 3) |
| NOTE 1: Manufacturer declarations applicable per BS requirement set were marked as “x”. Manufacturer declarations not applicable per BS requirement set were marked as “n/a”. NOTE 2: For BS type 1-H, the only radiated declarations are related to EIRP and EIS requirements. For BS type 1-H declarations required for the conducted requirements testing, refer to 3GPP TS 38.141-1 [3] – those declarations were marked in this table as “c”. NOTE 3: This manufacturer declaration is optional. |  |  |  |  |  |

------------------------------ End of modified section ------------------------------