3GPP TSG-RAN WG4 AH1807	R4- 1809421

Montreal, Canada 2nd –6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR |  | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.843 MU for EVM measurement in a Near Field Test Range |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MVG Industries |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-07-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding text for MU assessment of EVM measurement in a Near Field Test setup |  |  |  |  |  |  |  |  |
| Summary of change: |  | The addition of text for MU EVM measurement for directional requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The text is needed in order to clarify the MU associated with measuring EVM in a Near Field Test Range |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.5.4.1, 10.2.5.4.2, 10.2.5.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 10.2.5.4	Near Field

##### 10.2.5.4.1	General

The system is depicted in section 10.2.2.4.1. In case of OTA EVM type of measurements, NF to FF transform is not needed. EVM is measured in Near Field for the declared directions.

##### 10.2.5.4.2 	Test method limitations and scope

For AAS BS implementations that point a beam in the EVM conformance direction during testing, the near field technique may report too high, but will never report too low EVM. In case the BS does not point a beam centre in the test direction, there is a risk of the near field technique report too low EVM

##### 10.2.5.4.3 	Calibration

Stage 1 – Calibration:

Calibration shall be done with the procedure shown in 10.2.2.4.2. to ensure that the SNR at the measurement equipment input is appropriate and the reception signal level at the measurement equipment is within the dynamic range of measurement equipment.

##### 10.2.5.4.4 	Procedure

Stage 2 - Measurement:

The testing procedure consists of the following steps:

- Align the AAS BS with (Theta,Phi) angles corresponding to the declared beam direction to be measured

- Configure TX branch and carrier at a time according to the manufacturer's declared rated output power

- Set the AAS BS to transmit the test signal according to E-TM3.1 at 5 MHz bandwidth configuration.

- Measure EVM of each carrier arriving at the measurement equipment (such as a spectrum analyzer or equivalent instrument).

- Repeat steps 3-4 for all conformance test beam direction pairs as described in 3GPP TS 37.145-2 [24], subclause 6.2.

- Repeat steps 3-5 for E-TM3.2, E-TM3.3 and E-TM2.

The EVM of each E-UTRA carrier for different modulation schemes on PDSCH to be less than the limits defined in 3GPP TS 37.105 [3], subclause 9.6.

For conformance tests, EVM shall be measured at maximum and minimum power settings while frequency error, and occupied BW at only maximum power setting.

##### 10.2.5.4.5 	MU assessment

###### 10.2.5.4.5.1 	MU Budget

EVM is a relative measurement given that the wanted signal and noise signal are at the same frequency and measured at the same time therefore most of the OTA anechoic chamber errors will cancel out.

Nearly all of uncertainty terms for the EVM measurement and EVM calibration are the same and hence EVM is a differential or relative measurement.

Potentially, the EVM may vary in space due to different patterns of wanted signal and distortion. Thus for narrow beams, it may be possible that beam pointing and alignment errors could impact EVM results. The importance and impact of such effects  is likely to be even smaller than for far field based measurements.

As EVM is also dependent on the phase of the calibrated path it is possible that phase ripple in the quiet zone or elsewhere, which arises due to multipath reflections, may lead to frequency ripple and cause additional EVM errors which do not appear in a power accuracy analysis as done for EIRP accuracy. Analysis of the effect of such effects is that the effects will be even smaller than for far field based measurements.

###### 10.2.5.4.5.2 	MU Value

The Near Field budget is carried out without consideration of the measurement equipment therefore the MU is given in % and can be converted to dB , for example:

2% is equivalent to 20*log10(2/100) = -33.98dB

If the unwanted signal is 0.35dB higher than the wanted due to the test system then this will be degraded to -33.63dB, and

-33.63dB is equivalent to;	10(-33.63/20) *100 = 2.08%

Additional error due to potential phase error has not been considered however the potential increase due to then OTA test equipment is well within the contribution allowable with a 1% linear MU.

For Near Field Test Range a new measurement uncertainty term shall be added to the MU. This term will take into account the fact that in Near Field the phase pattern will sum up so that the signal level is increasing while the noise level is the same. This MU term will only cause an error in the direction of increasing the reported EVM value and not decreasing it, and will depend on the implementation of the DUT.

Thus for the near field test range the MU is decided to be 1% + increase in EVM due to different wanted signal/distortion beam patterns (dependent on DUT architecture).

Note: Analysis of the phase uncertainties indicates that the contributions are not significant enough to affect the final MU value, however if future work indicates that phase or any other errors not related to amplitude calibration may affect the EVM measurement uncertainty, the MU analysis may need to be re-examined.