3GPP TSG-WG SA2 Meeting #157e	S2-2307712

# May 22th – 26th, 2023, Berlin					                       (revision of S2-2306842, S2-2307652)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0292 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on QoS handling for 5G ProSe Layer-3 UE-to-UE Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As specified in clause 6.4.3.7.3 in 23.304, the U2U relay decides the QoS info of the second hop QoS to the target End UE. After QoS Info of the second hop QoS in Direct Communication Accept message is received, Relay UE will decide the QoS info of the first hop QoS considering the received second hop QoS. - The 5G ProSe Layer-3 UE-to-UE Relay decides the QoS Info of the first hop QoS with considering the received second hop QoS, the Direct Communication Accept message over the first hop PC5 reference point additionally includes IP Address Configuration, Link-Local IPv6 Address, QoS Info of the first hop QoS and may include IP address of the target 5G ProSe End UE. However, the description in clause 5.6.3.1 is not reflected explicitly. So we propose to clarify that relay provides the QoS Info to the source 5G ProSe Layer-3 End UE with considering the received second hop QoS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | We propose to clarify that relay provides the QoS Info to the source 5G ProSe Layer-3 End UE with considering the received second hop QoS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The QoS handling for 5G ProSe Layer-3 UE-to-UE Relay is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X |  |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.6.3.1	QoS handling for 5G ProSe Layer-3 UE-to-UE Relay

For a 5G ProSe Layer-3 End UE connecting with another 5G ProSe Layer-3 End UE(s) via 5G ProSe Layer-3 UE-to-UE Relay, the QoS requirement of the relay traffic between the peer 5G ProSe Layer-3 End UE(s) can be satisfied by the corresponding QoS control for the PC5 link between source 5G ProSe Layer-3 End UE and 5G ProSe Layer-3 UE-to-UE Relay (i.e. first hop PC5 QoS control) and the QoS control for the PC5 link between 5G ProSe Layer-3 UE-to-UE Relay and target 5G ProSe Layer-3 End UE (i.e. second hop PC5 QoS control). The first hop PC5 QoS and second hop PC5 QoS is controlled with PC5 QoS rules and PC5 QoS parameters (e.g. PQI, GFBR, MFBR, PC5 LINK-AMBR) as specified in clause 5.6.1.

As shown in figure 5.6.3.1-1 below, the end-to-end QoS is met only when the QoS requirements are properly translated and satisfied over the two legs respectively.

Figure 5.6.3.1-1: End-to-End QoS for 5G ProSe Layer-3 UE-to-UE Relay operation

To achieve this, the source 5G ProSe Layer-3 End UE initiates PC5 QoS Flows setup or modification during the Layer-2 link establishment or modification procedure, the source 5G ProSe Layer-3 End UE provides the QoS Info as described in clause 6.4.3.7.3 to the 5G ProSe Layer-3 UE-to-UE Relay. The received PC5 QoS parameters of the QoS Info (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.) are interpreted as the end-to-end QoS requirements by the 5G ProSe Layer-3 UE-to-UE Relay for the traffic transmission between source 5G ProSe Layer-3 End UE and target 5G ProSe Layer-3 End UE. The 5G ProSe Layer-3 UE-to-UE Relay, based on its implementation, decides the PQI for the first hop PC5 QoS control and the PQI for the second hop PC5 QoS control. The 5G ProSe Layer-3 UE-to-UE Relay provides the QoS Info (including PQI value chosen by the 5G ProSe Layer-3 UE-to-UE Relay for the second hop) to the target 5G ProSe Layer-3 End UE. After accepted QoS Info of the second hop QoS from the target 5G ProSe Layer-3 End UE is received, 5G ProSe Layer-3 UE-to-UE Relay provides the QoS Info (including PQI value chosen by the 5G ProSe Layer-3 UE-to-UE Relay for the first hop) to the source 5G ProSe Layer-3 End UE with considering the received second hop QoS. If the source 5G ProSe Layer-3 End UE performs the Layer-2 link modification procedure to add new PC5 QoS Flow(s) or modify the existing PC5 QoS Flow(s) for IP traffic or Ethernet traffic over PC5 reference point, the source 5G ProSe Layer-3 End UE may also provide the PC5 QoS Rule(s) for the PC5 QoS Flow(s) to be added or modified to the 5G ProSe Layer-3 UE-to-UE Relay. The 5G ProSe Layer-3 UE-to-UE Relay may generate the Packet Filters used over the second hop based on the received PC5 QoS Rule(s).

# * * * * End of changes * * * *