SA WG2 Meeting #157	S2-2307625

# Berlin, Germany, May 22 – 26, 2023	(was S2-2304208,S2-2307411, S2-2307612)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4240 | rev | 4 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Closing ENs for the procedures for discontinuous coverage reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Samsung, CMCC, Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-22 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Several open issues were documented and captured with related ENs. Editor's note: Whether the protocol used over the user plane between the UE and the external server to provide the satellite coverage availability information is standardised or not is FFS. Editor's note: Whether the format of the Satellite coverage availability information provisioned to UE will be standardised or not is FFS. It is proposed that an informative Annex will be define to provide some examples of SCAI provision to a UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Closes open issue regarding the information defined for SCAI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Open issues remain in the specification. ENs remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.13.2, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

#### 5.4.13.2	Coverage availability information provisioning to the UE

A UE may use satellite coverage availability information for satellite access to support discontinuous coverage operations. Satellite coverage availability information can be provided to a UE by an external server via a PDU Session or SMS. The protocol and format of satellite coverage availability information via PDU session or SMS is not defined in this release of the specification, but some examples on possible information that constitutes the satellite coverage availability information is defined in Annex X.

# >>>> NEXT CHANGES<<<<

Annex X (informative):
Satellite coverage availability information

The protocol and format of satellite coverage availability information to be provisioned to the UE via PDU session or SMS is not defined in this release of the specification, but this annex provides some examples on the information that constitutes input to the source of satellite coverage availability information e.g. external server and the output it provides to the UE.

Satellite coverage availability information can be indicated to the UE by indications corresponding to whether or not coverage is available for a specific NTN RAT Type for a particular location and time, where:

-	These indications can be Boolean "True" (e.g. coverage available) and "False" (coverage not available);

-	locations can correspond to grid points in a fixed array (e.g. rectangular, hexagonal);

-	Coverage availability times may occur at fixed periodic intervals; and

-	Coverage availability information is per RAT Type. The information provisioned to the UE can include coverage information on only one PLMN or multiple PLMNs.

If Satellite coverage availability information indicates coverage is available then additional information on whether PLMN is allowed to operate in that location can be provided to the UE.

In order for the source of satellite coverage availability information to provide accurate information to the UE, a UE might indicate for example the following information to a source of satellite coverage availability information (e.g. an external server):

-	Serving PLMN ID (if not already known or implied)

-	One or more satellite RAT Types (where satellite coverage availability information is then expected for these one or more RAT Types)

-	List of supported NTN frequency bands (if not implied by the particular RATs)

-	Present UE location (e.g. latitude and longitude) for a reference grid point (e.g. the most Southerly and then most Westerly grid point)

-	Type of Array (e.g. rectangular or hexagonal)

-	Minimum elevation angle

Based on the above information provided by the UE, satellite coverage availability information could be delivered to the UE as a sequence of time durations for each grid point where each time duration includes an indication of coverage availability or unavailability one example of many alternatives as illustrated below for a particular grid point with N different durations:

Satellite coverage availability information	at a given gridpoint = <N> <Binary 0 or 1><Duration 1> <Binary 0 or 1><Duration 2> . . . . <Binary 0 or 1><Duration N>

The above would be concatenated for all of the grid points to produce the satellite coverage availability information.

When SMS is used to deliver the satellite coverage availability information, the UE input and satellite coverage availability information output can be delivered in a series of concatenated SMS messages using possibly the same format.

# >>>>END OF CHANGES<<<<