3GPP TSG-WG SA2 Meeting #157	S2-2308152

# Berlin, Germany, May 22 – 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4572 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PIN traffic routing support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-05-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As specified in the TS, traffic routing between two PINEs via different PEGCs needs to be supported: Between two PINEs, which is via 5G core network when the two PINEs connect to different PEGCs; Each PEGC establishes the PDU session related to the PIN to route the traffic between PINEs connected via different PEGCs of the same PIN. In the case multiple PDU sessions for the same PIN connect to the same UPF, local switching can be supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describes that PINEs communication within a PIN via different PEGCs connected via different PDU sessions to the same UPF can be local switched. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to support the traffic routing between two PINEs connected to different PEGCs is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.44.1, 5.44.3.1, 5.8.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

5.44.1	General

Personal IoT Network (PIN) provides local connectivity between PIN elements i.e. UEs and/or non-3GPP devices. PIN elements communicate using PINE-to-PINE direct connection (the connection between two PIN elements) or PINE-to-PINE indirect connection (the connection between two PIN elements is relayed via one specific PIN element and/or 5GS). The management of the PINE-to-PINE direct connection is out of the scope of this specification. For the PINE-to PINE indirect connection, a UE acts as the specific PIN element with Gateway Capability (PEGC). With the support of the PEGC registered to 5G network, the PIN Elements have access to the 5G network services and may communicate with other PIN Elements via 5GC. A PEGC may support multiple PINs. For each PIN, a dedicated DNN/S-NSSAI shall be configured.

PIN and PIN elements are managed by specific PIN element with Management Capability (PEMC) and by an AF if AF deployed. A PIN includes at least one PEGC and at least one PEMC. The management of the PIN network and PIN Element (including the management role distribution between PEMC and AF) is out of the scope of this specification.

The PEGC is a UE with subscription data related to PIN within the 5GS and may register to 5GS. The PEMC does not have subscription data related to PIN within the 5GS and behaves as normal UE if it is registered in 5GS. See information in Annex O for the relation between PIN and 5GS.

An AF for PIN may be deployed to support the PIN service. The AF for PIN may communicate with PEMC and PEGC via application layer for management of the PIN which is transported as user plane data transparently to 5GS. The 5GC is enhanced to support the delivery of UE policy related to PIN service for UE acting as PEGC (as specified in clause 5.44.2) and to support the PDU session management for PIN service (as specified in clause 5.44.3).

The support of 5G-RG as a PEGC is not considered in this Release.

# * * * * Next change * * * *

5.44.3.1	PDU Session Establishment for PIN

When a PDU Session associated with a PIN is established by UE with PEGC, an SMF is selected according to clause 4.3.2.2.3 of TS 23.502 [3]. The UE with PEGC may use IP address allocation methods as specified in clause 5.8.2.2 (e.g. IPv6 Prefix Delegation feature).

One PEGC may serve more than one PINs and in this case the PEGC shall have at least one PDU Session for each PIN if the PIN traffic is via PEGC/5GC. One PIN may be served by more than one PDU sessions in the PEGC. If there are multiple PDU sessions for a PIN from different PEGCs connecting to the same UPF, the same mechanism for local switching in the UPF as defined for 5G VN group communication, as described in subclause 5.8.2.13, may be applied.

Editor's note:	One PIN served by more than one PDU sessions in PEGC is FFS.

Editor's note:	The handling of the PEMC in 5GC in relation with PIN is FFS.

# * * * * Next change * * * *

5.8.5.3	Packet Detection Rule

The following table describes the Packet Detection Rule (PDR) containing information required to classify a packet arriving at the UPF. Every PDR is used to detect packets in a certain transmission direction, e.g. UL direction or DL direction.

Table 5.8.5.3-1: Attributes within Packet Detection Rule

| Attribute |  | Description | Comment |
| --- | --- | --- | --- |
| N4 Session ID |  | Identifies the N4 session associated to this PDR. NOTE 5. |  |
| Rule ID |  | Unique identifier to identify this rule. |  |
| Precedence |  | Determines the order, in which the detection information of all rules is applied. |  |
| Packet | Source interface | Contains the values "access side", "core side", "SMF", "N6-LAN", "5G VN internal". | Combination of UE IP address (together with Network instance, if necessary), CN tunnel info, |
| Detection | UE IP address | One IPv4 address and/or one IPv6 prefix with prefix length (NOTE 3). | packet filter set, application identifier, Ethernet PDU Session |
| Information. NOTE 4. | Network instance (NOTE 1) | Identifies the Network instance associated with the incoming packet. | Information and QFI are used for traffic detection. Source interface identifies the |
|  | CN tunnel info | CN tunnel info on N3, N9 interfaces, i.e. F-TEID. | interface for incoming packets |
|  | Packet Filter Set | Details see clause 5.7.6. | where the PDR applies, e.g. from access side (i.e. up-link), |
|  | Application identifier |  | from core side (i.e. down-link), |
|  | QoS Flow ID | Contains the value of 5QI or non-standardized QFI. | from SMF, from N6-LAN (i.e. the |
|  | Ethernet PDU Session Information | Refers to all the (DL) Ethernet packets matching an Ethernet PDU session, as further described in clause 5.6.10.2 and in TS 29.244 [65]. | DN), or from "5G VN internal" (i.e. local switch). |
|  | Framed Route Information | Refers to Framed Routes defined in clause 5.6.14. | Details like all the combination possibilities on N3, N9 interfaces are left for stage 3 decision. |
| Packet replication and detection carry on information | Packet replication skip information NOTE 7 | Contains UE address indication or N19/N6 indication. If the packet matches the packet replication skip information, i.e. source address of the packet is the UE address or the packet has been received on the interface in the packet replication skip information, the UP function neither creates a copy of the packet nor applies the corresponding processing (i.e. FAR, QER, URR). Otherwise the UPF performs a copy and applies the corresponding processing (i.e. FAR, QER, URR). |  |
| NOTE 6 | Carry on indication | Instructs the UP function to continue the packet detection process, i.e. lookup of the other PDRs. |  |
| Outer header removal |  | Instructs the UP function to remove one or more outer header(s) (e.g. IP+UDP+GTP, IP + possibly UDP, VLAN tag), from the incoming packet. | Any extension header shall be stored for this packet. |
| Forwarding Action Rule ID (NOTE 2) |  | The Forwarding Action Rule ID identifies a forwarding action that has to be applied. |  |
| Multi-Access Rule ID (NOTE 2) |  | The Multi-Access Rule ID identifies an action to be applied for handling forwarding for a MA PDU Session. |  |
| List of Usage Reporting Rule ID(s) |  | Every Usage Reporting Rule ID identifies a measurement action that has to be applied. |  |
| List of QoS Enforcement Rule ID(s) |  | Every QoS Enforcement Rule ID identifies a QoS enforcement action that has to be applied. |  |
| Protocol Description |  | Indicates service protocol used by the flow e.g. H.264/RTP, SRTP (NOTE 8). |  |
| NOTE 1: Needed e.g. if: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or AN node in different IP domains. - UPF "local switch", N6-based forwarding and N19 forwarding is used for different 5G LAN groups. - UPF "local switch" may be used for DNN/S-NSSAI dedicated for PIN. NOTE 2: Either a FAR ID or a MAR ID is included, not both. NOTE 3: The SMF may provide an indication asking the UPF to allocate one IPv4 address and/or IPv6 prefix. When asking to provide an IPv6 Prefix the SMF provides also an IPv6 prefix length. NOTE 4: When in the architecture defined in clause 5.34, a PDR is sent over N16a from SMF to I-SMF, the Packet Detection Information may indicate that CN tunnel info is to be locally determined. This is further defined in clause 5.34.6. NOTE 5: In the architecture defined in clause 5.34, the rules exchanged between I-SMF and SMF are not associated with a N4 Session ID but are associated with a N16a association. NOTE 6: Needed in the case of support for broadcast/multicast traffic forwarding using packet replication with SMF-provided PDRs and FARs as described in clause 5.8.2.13.3.2. NOTE 7: Needed in the case of packet replication with SMF-provided PDRs and FARs as described in clause 5.8.2.13.3.2, to prevent UPF from sending the broadcast/multicast packets back to the source UE or source N19/N6. NOTE 8: May be provided when PDU Set Identification marking applies to the PDR. |  |  |  |

# * * * * End of changes * * * *