3GPP TSG-SA5 Meeting #149 	S5-234304

Berlin, Germany, 22 - 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0119 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-15 CR 28.554 Correction of accessibility and integrity KPI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2023-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The KPI for AMFMeanRegNbr and UDMRegNbr refer to the wrong measurement. The KPIs for upstream and downstream throughput for network and network slice instance includes more than IP packets. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change to use the “RM.” for the AMFMeanRegNbr and UDMRegNbr as well as the names for the KPIs in line with rel-16. Removing IP for the UTSNSI and DTSNSI KPIs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definitions of the KPIs will be unclear which may lead to interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.2, 6.3.2, 6.3.3,and 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 6.2.1	Registered Subscribers of Network and Network Slice Instance through AMF

a)	Registered Subscribers of Single Network Slice Instance through AMF.

b)	This KPI describe the total number of subscribers that are registered to a network slice instance.

c)	This KPI is obtained by counting the subscribers in AMF that are registered to a network slice instance.

d)	AMFMeanRegNbr=AMFRM.RegisteredSubNbrMean.SNSSAI

e)	RM.RegisteredAMFSubNbrMean

f)	5GS

g)	Accessibility

h)	Interger

i)	MEAN

Next change

### 6.2.2	Registered Subscribers of Network through UDM

a)	Registered Subscribers of Network through UDM

b)	This KPI describes the total number of subscribers that are registered to a network through UDM.

c)	This KPI is corresponding to the measurement RM.RegisteredSubUDMNbrMean that counts subscribers registered in UDM.

d)	UDMRegNbr=UDMRM.RegisteredSubUDMNbrMean

e)	RM.RegisteredSubUDMNbrMean

f)	5GS

g)	Accessibility

h)	Integer

i)	MEAN

Next change

### 6.3.2	Upstream throughput for network and Network Slice Instance

a)	Upstream throughput for network and network slice instance.

b)	This KPI describes the upstream throughput of one single network slice instance by computing the packet size for each successfully transmitted UL packet through the network slice instance during each observing granularity period and is used to evaluate integrity performance of the end-to-end network slice instance.

c)	This KPI is obtained by measuring the total number of upstream octets provided by N3 interface from NG-RAN to all UPFs, related to the single network slice instance, divided by the granularity period (in milliseconds).

d)	UTSNSI= UPFGTP.InDataOctN3UPF.SNSSAIGranularityPeriod×8

e)	GTP.InDataOctN3UPF.

f)	5GS.

g)	Integrity.

h)	kbit/s.

i)	MEAN.

Next change

### 6.3.3	Downstream Throughput for Single Network Slice Instance

a)	Downstream throughput for network and network slice instance.

b)	This KPI describes the downstream throughput of one single network slice instance by computing the packet size for each successfully transmitted DL packet through the network slice instance during each observing granularity period and is used to evaluate integrity performance of the end-to-end network slice instance.

c)	This KPI is obtained by downstream throughput provided by N3 interface from all UPFs to NG-RAN which are related to the single network slice instance.

d)	DTSNSI= UPFGTP.OutDataOctN3UPF.SNSSAIGranularityPeriod×8

e)	GTP.OutDataOctN3UPF.

f)	5GS.

g)	Integrity.

h)	kbit/s.

i)	CUM.

Next change

### 6.3.5	Downstream Throughput at N3 interface

a)	Downstream GTP data throughput at N3 interface.

b)	This KPI describes the total number of octets of all downstream GTP data packets on the N3 interface (transmitted downstream from UPF) which have been generated by the GTP-U protocol entity on the N3 interface, during a granularity period. This KPI is used to evaluate integrity performance at N3 interface.

c)	This KPI is obtained by measuring the total number of octets GTP data packets downstream provided by N3 interface from UPF to NG-RAN, divided by the granularity period (in milliseconds).

GTPTN=  GTP.InDataOctN3UPFGranularityPeriod×8

d)	GTPTN=  GTP.OutDataOctN3UPFGranularityPeriod×8

e)	GTP.OutDataOctN3UPF

f)	5GS

g)	Integrity

h)	kbit/s

i)	MEAN

End of changes