3GPP TSG-SA5 Meeting #149 	S5-234138

Berlin, Germany, 22nd – 26th May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0410 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Addition of “Available MIMO Layers Coverage Map per UE and per PRB” measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Available MIMO Layers Coverage Map per UE and per PRB should be monitored, as it reflects possible improvement brought by MIMO with the increased number of layers in the 5G business network. The operators can use this information to monitor what may be the limitations from the number of assigned layers per UE and PRB in the observed cell based on UE distance from the base station. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of “Available MIMO Layers Coverage Map per UE and per PRB in the DL/UL” measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operator cannot monitor what may be the limitations from the number of assigned layers per UE and PRB in the observed cell based on UE distance from the base station. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.30.x (new), 5.1.1.30.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-234138 is a revision of S5-234136. S5-234136 is a revision of S5-233930. |  |  |  |  |  |  |  |  |

1st Modified Section

##### 5.1.1.30.x	Available MIMO Layers Coverage Map per UE and per PRB on the DL

a)	This measurement provides the sub-cell radius within the observed cell up to where a UE with the given number of MIMO layers in DL per PRB can be localized.

b)	SI.

c)	This measurement is obtained by computing the sub-cell radius d’y within the observed cell up to where a UE with the given number of MIMO layers y in DL per PRB can be localized using the following formula:

dy'=i=1MdiyM

where diy denotes maximum distance of a UE from base station with the assigned MIMO layers number y per PRB in DL from all slots within the measurement period within the i-th AoA interval. The M denotes the number of AoA (sub-angle) intervals. The width of the AoA (sub-angle) intervals can be constant which may be obtained as total cell angle 360º divided by M or can be separately configured by operator. The UE distance from base station and angle of arrival are based on the last reported TA and AoA values. The measurement is provided for SU MIMO, MU MIMO and also for the number of layers equal to 1 which may be specific case with non MIMO applied.

d)	Each measurement is a real value in meters [m].

e) The measurement name has the form MIMOLayersDLy where y ranges from 1 to maximum possible MIMO layers that can be assigned per UE and PRB in DL

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i）One usage of this measurement is to monitor the cell capacity for MIMO scenario on the DL.

##### 5.1.1.30.y	Available MIMO Layers Coverage Map per UE and per PRB on the UL

a)	This measurement provides the sub-cell radius within the observed cell up to where an UE with the given number of MIMO layers in UL per PRB can be localized.

b)	SI.

c)	This measurement is obtained by computing the sub-cell radius d’y within the observed cell up to where an UE with the given number of MIMO layers y in UL per PRB can be localized using the following formula:

dy'=i=1MdiyM

where diy denotes maximum sample of distance of an UE from base station with the assigned MIMO layers number y per PRB in UL from all slots within the measurement period within the i-th AoA interval. The M denotes the number of AoA (sub-angle) intervals. The width of the AoA (sub-angle) intervals can be constant which may be obtained as total cell angle 360º divided by M or can be separately configured by operator. The UE distance from base station and angle of arrival are based on the last reported TA and AoA values. The measurement is provided for SU MIMO, MU MIMO and also for the number of layers equal to 1 which may be specific case with non MIMO applied.

d)	Each measurement is a real value in meters [m].

e) The measurement name has the form MIMOLayersULy where y ranges from 1 to maximum possible MIMO layers that can be assigned per UE and PRB in UL

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i）One usage of this measurement is to monitor the cell capacity for MIMO scenario on the UL.

End of Modified Section