3GPP TSG-SA5 Meeting #149	S5-234679

# Berlin, Germany,  22 - 26 May 2023 									    			 revision of S5-234145

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0422 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurements related to the coordination between multiple NWDAFs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MANWDAF |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently PM cannot support the performance measurement related to the coordination between multiple NWDAFs. The TR 28.864 proposed performance measurements for the NWDAF with aggregation capability. It is necessary to add the related measurements in TS 28.552 based on the recommendations of TR 28.864. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add measurements related to the coordination between multiple NWDAFs based on TR 28.864. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance of NWDAF on the coordination with other multiple NWDAFs cannot be evaluated efficiently. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[3]	3GPP TS 32.404: "Performance Management (PM); Performance measurements - Definitions and template".

[4]	3GPP TS 23.501: "System Architecture for the 5G System".

[5]	IETF RFC 5136: "Defining Network Capacity".

[6]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)".

[7]	3GPP TS 23.502: "Procedures for the 5G System".

[8]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[9]	3GPP TS 32.425: "Performance Management (PM); Performance measurements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[10]	3GPP TS 32.451: "Key Performance Indicators (KPI) for Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Requirements".

[11]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[12]	Void.

[13]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".[14]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[15]	Void.

[16]	3GPP TS 29.244: "Technical Specification Group Core Network and Terminals; Interface between the Control Plane and the User Plane Nodes; Stage 3".

[17]	ETSI GS NFV-IFA027 v2.4.1: "Network Functions Virtualisation (NFV); Management and Orchestration; Performance Measurements Specification".

[18]	Void.

[19]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[20]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[21]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[22]	3GPP TS 29.413: "Application of the NG Application Protocol (NGAP) to non-3GPP access".

[23]	3GPP TS 29.122: "Technical Specification Group Core Network and Terminals; T8 reference point for Northbound APIs".

[24]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[25]	ETSI ES 202 336-12 V1.2.1: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[26]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[27]	3GPP TS 29.274: "Evolved General Packet Radio Service (GPRS); Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[28]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[29]	3GPP TS 38.314: "NR; layer 2 measurements".

[30]	3GPP TS 38.313: "Self-Organizing Networks (SON) for 5G networks".

[31]	3GPP TS 38.415: "NG-RAN; PDU session user plane protocol".

[32]	3GPP TS 38.321: "NR MAC protocol specification".

[33]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[34]	3GPP TS 38.215: "NR; Physical layer measurements".

[35]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[36]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[37]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[38]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[39]	3GPP TS 29.507: "5G System; Access and Mobility Policy Control Service; Stage 3".

[40]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[41]	3GPP TS 29.531: "5G System; Network Slice Selection Services".

[42]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[43]	3GPP TS 29.540: "5G System; SMS Services; Stage 3".

[44]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[45]	3GPP TS 29.541: "5G System; Network Exposure FunctionServices for Non-IP Data Delivery (NIDD); Stage 3".

[46]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[47]	3GPP TS 29.504: "5G System; Unified Data Repository Services; Stage 3".

[48]	3GPP TS 29.554: "5G System; Background Data Transfer Policy Control Service; Stage 3".

[49]	3GPP TS 38.300: "NR and NG-RAN Overall description; Stage-2".

[50]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[51]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[52]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[53]	3GPP TS 23.273: "5G System (5GS); Location Services (LCS); Stage 2".

[54]	3GPP TS 29.572: "5G System (5GS); Location Management Services; Stage 3".

[55]	Void

[X]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

Next Modified Section

3.3	Measurement family

The measurement names defined in the present document are all beginning with a prefix containing the measurement family name. This family name identifies all measurements which relate to a given functionality and it may be used for measurement administration.

The list of families currently used in the present document is as follows:

-	DRB (measurements related to Data Radio Bearer).

-	RRC (measurements related to Radio Resource Control).

-	UECNTX (measurements related to UE Context).

-	RRU (measurements related to Radio Resource Utilization).

-	RM (measurements related to Registration Management).

-	SM (measurements related to Session Management).

-	GTP (measurements related to GTP Management).

-	IP (measurements related to IP Management).

-	PA (measurements related to Policy Association).

-	MM (measurements related to Mobility Management).

-	VR (measurements related to Virtualized Resource).

-	CARR (measurements related to Carrier).

-	QF (measurements related to QoS Flow).

-	AT (measurements related to Application Triggering).

-	SMS (measurements related to Short Message Service).

-	PEE (measurements related to Power, Energy and Environment).

-	NFS (measurements related to NF service).

-	PFD (measurements related to Packet Flow Description).

-	RACH (measurements related to Random Access Channel).

-	MR (measurements related to Measurement Report).

-	L1M (measurements related to Layer 1 Measurement).

-	NSS (measurements related to Network Slice Selection).

-	PAG (measurements related to Paging).

-	NIDD (measurements related to Non-IP Data Delivery).

-	EPP (measurements related to external parameter provisioning).

-	TI (measurements related to traffic influence).

-	CE (measurements related to Connection Establishment).

-	SPP (measurements related to Service Parameter Provisioning).

-	BDTP (measurements related to Background Data Transfer Policy).

-	DM (measurements related to Data Management).

-	AFQ (measurements related to AF session with QoS).

-	UCM (measurements related to UE radio Capability Management).

-	PAU (measurements related to Policy Authorization).

-	EEX (measurements related to Event Exposure).

-	SDM (measurements related to subscriber data management).

-	PPV (measurements related to parameter provisioning).

-	DIS (measurements related to discovery).

-	Location Management (measurements related to Location Management).

-	SP (measurement related to service provisioning).

-	DANS (measurements related to Data Analytics Service).

Next Modified Section

5.X	Performance measurements for NWDAF

5.X.B	Measurements related to the coordination between multiple NWDAFs

5.X.B.1	Number of analytics service subscription from Aggregator NWDAF

a)	This measurement provides the number of service subscription generated by the Aggregator NWDAF.

b)	CC

c)	On sending the service subscription by the Aggregator NWDAF to other NWDAFs to subscribe analytics information for analytics aggregation (See TS 23.288 [2]), each subscription is added to the corresponding counter. The measurement can be split into subcounters per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [X]) and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DANS.SerSubGenerated, and

DANS.SerSubGenerated.AnalyticsID, where AnalyticsID identifies different analytics, and

DANS.SerSubGenerated.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

5.X.B.2	Number of analytics service requests from Aggregator NWDAF

a)	This measurement provides the number of service requests generated by the Aggregator NWDAF.

b)	CC

c)	On sending the service request by the Aggregator NWDAF to other NWDAFs to request analytics information for analytics aggregation (See TS 23.288 [2]), each request is added to the corresponding counter. The measurement can be split into subcounters per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [X]) and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DANS.SerReqGenerated, and

DANS.SerReqGenerated.AnalyticsID, where AnalyticsID identifies different analytics, and

DANS.SerReqGenerated.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

5.X.B.3	Number of service notifications received by the Aggregator NWDAF

a)	This measurement provides the number of analytics service notifications received by the Aggregator NWDAF.

b)	CC

c)	On receipt by the Aggregator NWDAF of the notification corresponding to the analytics service subscription from Aggregator NWDAF (See TS 23.288 [2]), each transmitted notification is added to the corresponding counter. The measurement can be split into subcounters per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [X]) and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DANS.SerNotificationReceived, and

DANS.SerNotificationReceived.AnalyticsID, where AnalyticsID identifies different analytics, and

DANS.SerNotificationReceived.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

5.X.B.4	Number of service responses received by the Aggregator NWDAF

a)	This measurement provides the number of analytics service responses received by the Aggregator NWDAF.

b)	CC

c)	On receipt by the Aggregator NWDAFs of the response corresponding to the analytics service request from Aggregator NWDAF (See TS 23.288 [2]), each received response is added to the corresponding counter. The measurement can be split into subcounters per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [X]) and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DANS.SerResponseReceived, and

DANS.SerResponseReceived.AnalyticsID, where AnalyticsID identifies different analytics, and

DANS.SerResponseReceived.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

End of modification