3GPP TSG-SA5 Meeting #149	S5-234686

# Berlin, Germany,  22 - 26 May 2023 									    			 revision of S5-234147

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0424 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurements related to the NWDAF service provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MANWDAF |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently PM cannot support the measurements related to NWDAF service provisioning. The TR 28.864 proposed performance measurements of the NWDAF on providing different type of services, including the analytics service, ML model provisioning service and data management service, etc. It is necessary to add the related measurements in TS 28.552 based on the recommendations of TR 28.864. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added measurements related to NWDAF service provisioning based on the recommendations in TR 28.864. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance of NWDAF on service provisioning cannot be evaluated efficiently. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

5.X	Performance measurements for NWDAF

5.X.C	Measurements related to the NWDAF service provisioning

5.X.C.1	NWDAF service provisioning subscription

5.X.C.1.1	Number of service subscription

a)	This measurement provides the number of service subscriptions received by the NWDAF.

b)	CC

c)	On receipt by the NWDAF of the subscription to NWDAF service (See clause 7 in TS 23.288 [2]), each received subscription is added to the corresponding counter. The measurement can be split into subcounters per service name (see clause 4.1 in TS 29.520 [X]), subcounters per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [X]) and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DANS.SerSubReceived, and

DANS.SerSubReceived.ServiceName, where ServiceName identifies different services, and

DANS.SerSubReceived.AnalyticsID, where AnalyticsID identifies different analytics, and

DANS.SerSubReceived.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

5.X.C.1.2	Number of successful service subscription

a)	This measurement provides the number of service subscriptions received and accepted by the NWDAF.

b)	CC

c)	On receipt by the NWDAF of the subscription to NWDAF service (See clause 7 in TS 23.288 [2]), each accepted subscription is added to the corresponding counter. The measurement can be split into subcounters per service name (see clause 4.1 in TS 29.520 [X]), subcounters per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [X]) and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DANS.SerSubAccepted, and

DANS.SerSubAccepted.ServiceName, where ServiceName identifies different services, and

DANS.SerSubAccepted.AnalyticsID, where AnalyticsID identifies different analytics, and

DANS.SerSubAccepted.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

5.X.C.1.3	Number of failed service subscription

a)	This measurement provides the number of service subscriptions received but rejected by the NWDAF.

b)	CC

c)	On receipt by the NWDAF of the subscription to NWDAF service (See clause 7 in TS 23.288 [2]), each rejected subscription is added to the corresponding counter. The measurement can be split into subcounters per service name (see clause 4.1 in TS 29.520 [X]), subcounters per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [X]) and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DANS.SerSubRejected, and

DANS.SerSubRejected.ServiceName where ServiceName identifies different services, and

DANS.SerSubRejected.AnalyticsID, where AnalyticsID identifies different analytics, and

DANS.SerSubRejected.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

5.X.C.2	NWDAF service provisioning request

5.X.C.2.1	Number of service requests

a)	This measurement provides the number of service requests received by the NWDAF.

b)	CC

c)	On receipt by the NWDAF of the request to NWDAF service (See clause 7 in TS 23.288 [2]), each received request is added to the corresponding counter. The measurement can be split into subcounters per service name (see clause 4.1 in TS 29.520 [X]), subcounters per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [X]) and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DANS.SerReqReceived, and

DANS.SerReqReceived.ServiceName, where ServiceName identifies different services, and

DANS.SerReqReceived.AnalyticsID, where AnalyticsID identifies different analytics, and

DANS.SerReqReceived.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

5.X.C.2.2	Number of successful service requests

a)	This measurement provides the number of service requests received and accepted by the NWDAF.

b)	CC

c)	On receipt by the NWDAF of the request to NWDAF service (See clause 7 in TS 23.288 [2]), each accepted request is added to the corresponding counter. The measurement can be split into subcounters per service name (see clause 4.1 in TS 29.520 [X]), subcounters per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [X]) and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DANS.SerReqAccepted, and

DANS.SerReqAccepted.ServiceName, where ServiceName identifies the different services, and

DANS.SerReqAccepted.AnalyticsID, where AnalyticsID identifies different analytics, and

DANS.SerReqAccepted.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

5.X.C.2.3	Number of failed service requests

a)	This measurement provides the number of service requests received but rejected by the NWDAF.

b)	CC

c)	On receipt by the NWDAF of the request to NWDAF service (See clause 7 in TS 23.288 [2]), each rejected request is added to the corresponding counter. The measurement can be split into subcounters per service name (see clause 4.1 in TS 29.520 [X]), subcounters per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [X]) and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DANS.SerReqRejected, and

DANS.SerReqRejected.ServiceName, where ServiceName identifies different services, and

DANS.SerReqRejected.AnalyticsID, where AnalyticsID identifies different analytics, and

DANS.SerReqRejected.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

5.X.C.3	Successful NWDAF service provisioning

5.X.C.3.1	Number of service notifications generated by the NWDAF

a)	This measurement provides the number of service subscriptions notified by the NWDAF.

b)	CC

c)	On transmission by the NWDAF of the notification corresponding to the service subscription (See clause 7 in TS 23.288 [2]), each transmitted notification is added to the corresponding counter. The measurement can be split into subcounters per service name (see clause 4.1 in TS 29.520 [X]), subcounters per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [X]) and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DANS.SerNotificationGenerated, and

DANS.SerNotificationGenerated.ServiceName, where ServiceName identifies different services, and

DANS.SerNotificationGenerated.AnalyticsID, where AnalyticsID identifies different analytics, and

DANS.SerNotificationGenerated.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

5.X.C.3.2	Number of service responses generated by the NWDAF

a)	This measurement provides the number of service requests responsed by the NWDAF.

b)	CC

c)	On transmission by the NWDAF of the response corresponding to the service request (See clause 7 in TS 23.288 [2]), each transmitted response is added to the corresponding counter. The measurement can be split into subcounters per service name (see clause 4.1 in TS 29.520 [X]), subcounters per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [X]) and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DANS.SerResponseGenerated, and

DANS.SerResponseGenerated.ServiceName, where ServiceName identifies different services, and

DANS.SerResponseGenerated.AnalyticsID, where AnalyticsID identifies different analytics, and

DANS.SerResponseGenerated.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

End of modification