3GPP TSG-SA5 Meeting #149 	S5-234780

# Berlin, Germany, 22nd May 2023 - 26th May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0044 | rev | 1 | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR TS 28.312 Correct the supported qualifier for ExpectationObject and allowed value for contextCondition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei,Nokia,Deutsche Telekom, China Mobile Com. Corporation, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In clause 6.2.1.3.1.2, the Support Qualifier for attribute “expectationObject” is optional, which is incorrect. The MnS consumer(s) need to express what object they want to delivered or ensured, otherwise the expectation can not be accurately interpreted by the system. - In clause 6.2.1.4, the contextValueRange for “coverageTACContext” and “nRFqBandContext”, “rATContext” is a list, however, corresponding contextCondition is “IS_WITHIN_RANGE” represented by min and max value, which is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the the Support Qualifier for attribute “expectationObject” from “O” to “M”. - Add “IS_ALL_OF” as allowed value for targetCondition/ contextCondition and correct corresponding scenario specific context (“coverageTACContext” and “nRFqBandContext”, “rATContext”, etc.)which is a list. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some issues exist in the published TS 28.312 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.3.1.2, 6.2.1.4, 6.2.2.2, 7.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge link: https://forge.3gpp.org/rep/sa5/MnS/-/tree/TS28.312_Rel-17_CR0044_Correct_the_supported_qualifier_for_ExpectationObject_and_allowed_value_for_contextCondition |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - S5-234780 is the revision of S5-233992. |  |  |  |  |  |  |  |  |

1st Change

6.2.1.3.1	IntentExpectation <<dataType>>

6.2.1.3.1.1	Definition

IntentExpectation <<dataType>>represents MnS consumer's requirements, goals and contexts given to a 3GPP system.

6.2.1.3.1.2	Attributes

The IntentExpectation includes the following attributes.

Table 6.2.1.3.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| expectationId | M | T | T | T | T |
| expectationVerb | O | T | T | T | F |
| expectationObject | M | T | T | F | F |
| expectationTargets | M | T | T | F | F |
| expectationContexts | O | T | T | F | F |
| expectationfulfilmentInfo | O | T | F | F | T |
| NOTE: The scenario/requirements-specific IntentExpectations are defined utilizing the constructs of this generic IntentExpectation model. |  |  |  |  |  |

6.2.1.3.1.3	Attribute constraints

None.

6.2.1.3.1.4	Notifications

The notifications specified for the IOC using this <<dataType>> for its attribute(s), shall be applicable.

2nd Change

6.2.1.4	Attribute definition

Table 6.2.1.4-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| userLabel | A user-friendly (and user assignable) name of the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentExpectations | It describes the expectations including requirements, goals and contexts (including constraints and filter information) given to a 3GPP system. It states the list of specific outcomes desired to be realized for expectation object(s). allowedValues: Not Applicable | type: IntentExpectation multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentFulfilmentInfo | It describes status of fulfilment of an intent and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationFulfilmentInfo | It describes status of fulfilment of an intentExpectation and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| targetFulfilmentInfo | It describes status of fulfilment of an expectationTarget and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fulfilmentStatus | It describes the current status of the intent fulfilment result, which is configured by MnS producer and can be read by MnS consumer. allowedValues: "FULFILLED", "NOT_FULFILLED" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| notFulfilledState | It describes the current progress of or the reason for not achieving fulfilment for the intent, intentExpectation or expectationTarget. It is configured/written by MnS producer and can be read by MnS consumer. allowedValues: "ACKNOWLEDGED", "COMPLIANT", "DEGRADED", "SUSPENDED", "TERMINATED" "FULFILMENTFAILED" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| notFulfilledReasons | It describes the reasons/observations related to the specific noted notFulfilledState allowedValues: Not Applicable | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentContexts | It describes the list of IntentContext(s) which represents the constraints and conditions that should apply for the entire intent even if there may be specific contexts defined for specific parts of the intent. allowedValues: triple of (attribute, condition, value range) | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationId | A user-friendly (and user assignable) name of the intentExpectation. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationVerb | It describes the characteristic of the intentExpectation and is the property that describes the types of intentExpectations. Examples of verbs and their related types of expectation are Deliver: DeliveryIntentExpectation, e.g. Deliver a RAN network, Service, Slice, function Ensure: AssuranceintentExpectation, e.g. Ensure the performance targets values allowedValues: DELIVER, ENSURE | type: String multiplicity: 1 isOrdered:N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationObject | It describes the expectation objects to which the IntentExpectation should apply. allowedValues: Not Applicable | type: ExpectationObject multiplicity: 1 isOrdered:N/A isUnique: N/A defaultValue: None isNullable: False |
| objectType | It describes the type of expectation object of the IntentExpectation that are required to be applied on. It can be class name of the managed object. allowedValues: see scenario specific Intent Expectation | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectInstance | It describes a specific object instance (e.g. instance of managed object) to which the intentExpectation should apply. allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectContexts | It describes the list of ObjectContext(s) which represents the constraints and conditions to be used as filter information to identify the object(s) to which a given intentExpectation should apply. Note there may be other constraints and conditions defined either for the entire intent, for the specific intentExpectation or for the expectationTarget of the considered intentExpectation. The concrete ObjectContext depends on the ExpectationObject, which is defined in clause 6.2.2. All the concrete ObjectContexts follow the common structure of ObjectContext | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationTarget | It describes the list of ExpectationTarget(s) which represent specific outcomes on the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s) that are desired to be realized for a given intentExpectation. The concrete ExpectationTarget depends on the ExpectationObject, which is defined in clause 6.2.2. All the concrete ExpectationTargets follow the common structure of ExpectationTarget | type: ExpectationTarget multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationContexts | It describes the list of context(s) which represents the constraints and conditions that should apply for a specific intentExpectation. Note there may be other constraints and conditions defined for the entire intent or for specific parts of the intentExpectation. allowedValues: depends on Expectation Object in the IntentExpectation | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| targetName | It describes the name of the Expectation of the expectation target which represents specific outcomes on the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s) that are desired to be realized for a given intentExpectation.allowedValues: depends on ExpectationObject in the IntentExpectation | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| targetCondition | It expresses the limits within which the targetName is allowed/supposed to be. allowedValues: IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE", "IS_ONE_OF", "IS EQUAL TO OR LESS THAN”, "IS EQUAL TO OR GREATER THAN", "IS NOT ONE OF","IS_ALL_OF" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "is equal to" isNullable: False |
| targetValueRange | It describes the range of values that applicable to the targetName and the targetCondition. allowedValues: depends on the targetCondition. The value will be a single real number when the targetCondition is either "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS EQUAL TO OR LESS THAN", "IS EQUAL TO OR GREATER THAN” The value will be a pair of real numbers when the targetCondition is either "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE" The value will be a list when the targetCondition is "IS_ONE_OF", "IS NOT ONE OF","IS_ALL_OF" Note: For "IS_ALL_OF", the value shall be an match of the entire list. | type: Real multiplicity: 1..* isOrdered: False isUnique: TRUE defaultValue: Null isNullable: True |
| targetContexts | It describes the list of constraints and conditions that should apply for a specific expectationTarget. Note there may be other constraints and conditions defined for the entire intent or the intentExpectation. allowedValues: Not Applicable | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| contextAttribute | It describes a specific attribute of or related to the object or to characteristics thereof (e.g. its control parameter, gauge, counter, KPI, weighted metric, etc) to which the expectation should apply or an attribute related to the operating conditions of the object (such as weather conditions, load conditions, etc). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| contextCondition | It expresses the limits within which the ContextAttribute is allowed/supposed to be allowedValues: "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE, "IS_ONE_OF", "IS EQUAL TO OR LESS THAN", "IS EQUAL TO OR GREATER THAN", "IS NOT ONE OF","IS_ALL_OF" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "is equal to" isNullable: False |
| contextValueRange | It describes the range of values that applicable to the ContextAttribute and the ContextCondition. AllowedValue: depends on the contextAttribute The value will be a single real number when the targetCondition is either "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS EQUAL TO OR LESS THAN", "IS EQUAL TO OR GREATER THAN" The value will be a pair of real numbers when the targetCondition is either "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE" The value will be a list when the targetCondition is "IS_ONE_OF", "IS NOT ONE OF","IS_ALL_OF". Note: For "IS_ALL_OF", the value shall be an match of the entire list. | type: Real multiplicity: 1..* isOrdered: False isUnique: True defaultValue: Null isNullable: True |

3rd Change

6.2.2.2	Attribute definition

Table 6.2.2.2-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| coverageAreaPolygonContext | It describes the coverage areas for the RAN SubNetwork that the intent expectation is applied in the form of polygon. CoverageAreaPolygonContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "CoverageAreaPolygon" - contextCondition: "IS_ALL_OF" - contextValueRange: a list of CoverageArea defined in 3GPP TS 28.541 [5] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| coverageTACContext | It describes the coverage areas for the RAN SubNetwork that the intent expectation is applied in the form of TAC. CoverageTACContext is a Context including attributes: contextAttribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "CoverageAreaTAC" - contextCondition: "IS_ALL_OF" - contextValueRange: a list of nRTAC defined in 3GPP TS 28.541 [5] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| plMNContext | It describes the PLMN(s) supported by the RAN SubNetwork that the intent expectation is applied. PLMNContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "PLMN" - contextCondition: "IS_ALL_OF" - contextValueRange: a list of PLMNId defined in TS 28. 658 [10] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| nRFqBandContext | It describes the nRFqBands supported by the RAN SubNetwork that the intent expectation is applied. nRFqBandContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "NRFqBand" - contextCondition: "IS_ALL_OF" - contextValueRange: a list of NRFqBand expressed as string. Valid frequency band values are specified in clause 5.4.2 in 3GPP TS 38.104 [8] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| rATContext | It describes the RAT supported by the RAN SubNetwork that the intent expectation is applied. RATContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "RAT" - contextCondition: "IS_ALL_OF" - contextValueRange: a list of ENUM with allowed value: UTRAN, EUTRAN and NR | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| weakRSRPRatioTarget | It describes the downlink weak coverage ratio target for the RAN SubNetwork that the intent expectation is applied. WeakRSRPRatioTarget is an ExpectationTarget including attributes: targetName, targetCondition,targetValueRange and targetContext. Following are the allowed values: - targetName: "WeakRSRPRatio" - targetCondition: "IS_LESS_THAN" - targetValueRange: integer with allowed value [0,100] - targetContext: WeakRSRPContext | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| WeakRSRPRatioTarget.weakRSRPContext | It describes the threshold for downlink weak RSRP of the RAN SubNetwork that the intent expectation is applied. WeakRSRPContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "WeakRSRPThreshold" - contextCondition: "IS_LESS_THAN" - contextValueRange: Float | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| LowSINRRatioTarget | It describes the low SINR ratio target for the RAN SubNetwork that the intent expectation is applied. LowSINRRatioTarget is an ExpectationTarget including attributes: targetName, targetCondition,targetValueRange and targetContxt. Following are the allowed values: - targetName: "LowSINRRatio" - targetCondition: "IS_LESS_THAN" - targetValueRange: integer with allowed value [0,100] - targetContext: LowSINRContext | type:ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| LowSINRRatioTarget.lowSINRContext | It describes the threshold for low SINR for RAN SubNetwork that the intent expectation is applied. LowSINRContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "LowSINRThreshold" - contextCondition: "IS_LESS_THAN" - contextValueRange: integer | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| aveULRANUEThptTarget | It describes the average UL RAN UE throughput target for RAN SubNetwork that the intent expectation is applied. AveULRANUEThptTarget is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. Following are the allowed values: - targetName: "AveULRANUEThpt" - targetCondition: "IS_GREATER_THAN" - targetValueRange: integer | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| aveDLRANUEThptTarget | It describes the average DL RAN UE throughput target for RAN SubNetwork that the intent expectation is applied. AveDLRANUEThptTarget is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. Following are the allowed values: - targetName: "AveDLRANUEThpt" - targetCondition: "IS_GREATER_THAN" - targetValueRange: integer | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| lowULRANUEThptRatioTarget | It describes the low UL RAN UE throughput ratio target for the RAN SubNetwork that the intent expectation is applied. LowULRANUEThptRatioTarget is an ExpectationTarget including attributes: targetName, targetCondition,targetValueRange and targetContext. Following are the allowed values: - targetName: "LowULRANUEThptRatio" - targetCondition: "IS_LESS_THAN" - targetValueRange: integer with allowed value [0,100] - targetContext: LowULRANUEThptContext | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| LowULRANUEThptRatioTarget.lowULRANUEThptContext | It describes the threshold for the low UL RAN UE throughput of the RAN SubNetwork that the intent expectation is applied LowULRANUEThptContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "LowULRANUEThptThreshold" - contextCondition: "IS_LESS_THAN" - contextValueRange: Float | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| lowDLRANUEThptRatioTarget | It describes the low DL RAN UE throughput ratio target for the RAN SubNetwork that the intent expectation is applied. LowDLRANUEThptRatioTarget is an ExpectationTarget including attributes: targetName, targetCondition,targetValueRange and targetContext. Following are the allowed values: - targetName: "LowDLRANUEThptRatio" - targetCondition: "IS_LESS_THAN " - targetValueRange: integer with allowed value [0,100] - targetContext: LowDLRANUEThptContext | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| owDLRANUEThptRatioTarget.lowDLRANUEThptContext | It describes the threshold for the low DL RAN UE throughput of the RAN SubNetwork that the intent expectation is applied. LowDLRANUEThptContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "LowDLRANUEThptThreshold" - contextCondition: "IS_LESS_THAN" - contextValueRange: Float | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| serviceStartTimeContext | This describes the start time at which the service shall be available. This contributes to the selection of the appropriate edge data network to be used for service deployment. Following are the allowed values: - contextAttribute: "serviceStartTime" - contextCondition: "IS_EQUAL_TO" - contextValueRange: start time stamp | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| serviceEndTimeContext | This describes the end time after which the service shall not be available. This contributes to the selection of the appropriate edge data network to be used for service deployment. Following are the allowed values: - contextAttribute: "serviceEndTime" - contextCondition: "IS_EQUAL_TO" - contextValueRange: end time stamp | type:Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| edgeIdenfiticationIdContextt | This identifies the edge network where the service need to be deployed. This should be used when the edge identification is known to the consumer Following are the allowed values: - contextAttribute: "edgeIdentificationId" - contextCondition: "IS_EQUAL_TO" - contextValueRange: EDNidentifier as defined in 3GPP TS 28.538 [9] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| edgeIdenfiticationLocContext | This identifies the location where the service need to be deployed. This should be used when the edge identification is not known to the consumer Following are the allowed values: - contextAttribute: "edgeIdentificationTarget" - contextCondition: "IS_EQUAL_TO" - contextValueRange: geographical target location. This will take a form of either single latitude & longitude or a TAI | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| coverageAreaTAContext | It describes Tracking Coverage Areas for service supporting that the intent expectation is applied. coverageAreaTAContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "coverageAreaTA" - contextCondition: "IS_ALL_OF" - contextValueRange: a list of Tracking Coverage Areas, coverageAreaTAList in clause 6.3.1 in 3GPP TS 28.541[5] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| dlThptPerUETarget | It describes the DL throughput target by the per UE for the service Supporting that the intent expectation is applied. DLThptperUETarget is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange: - targetName: "DLThptperUE" - targetCondition: "IS_GREATER_THAN" - targetValueRange: dlThptPerUE defined in clause 6.3.1 of 3GPP TS 28.541 [5] | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| UlThptPerUETarget | It describes the UL throughput target by the per UE for the service Supporting that the intent expectation is applied. ULThptperUETarget is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. - targetName: "UlThptperUE" - targetCondition: "IS_GREATER_THAN" - targetValueRange: uLThptPerUE defined in clause 6.3.1 of 3GPP TS 28.541 [5] | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| dLLatencyTarget | It describes the DL latency target for the service Supporting that the intent expectation is applied. DLLatencyTarget is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. - targetName: "UlThptperUE" - targetCondition: "IS_LESS_THAN" - targetValueRange: dLLatency defined in clause 6.3.1 of 3GPP TS 28.541 [5] | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| uLLatencyTarget | It describes the UL latency target for the service Supporting that the intent expectation is applied. UlThptperUETarget is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. - targetName: "UlThptperUE" - targetCondition: "IS_LESS_THAN" - targetValueRange: ULLatency defined in clause 6.3.1 of 3GPP TS 28.541 [5] | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| maxNumberofUEsTarget | It describes the maximum number of UEs for service supporting that the intent expectation is applied. maxNumberofUEsContext is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. Following are the allowed values: - targetName: "maxNumberofUEs" - targetCondition: " IS_LESS_THAN" - targetValueRange: maxNumberofUEs in clause 6.3.1 in 3GPP TS 28.541 [5] | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| activityFactorTarget | It describes the percentage value of the amount of simultaneous active UEs to the total number of UEs where active means the UEs are exchanging data with the network for service supporting that the intent expectation is applied. activityFactorContext is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. Following are the allowed values: - targetName: " activityFactorContext " - targetCondition: " IS_EQUAL_TO" - targetValueRange: activityFactor in clause 6.3.1 in 3GPP TS 28.541 [5] | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| uESpeedTarget | It describes the maximum speed (in km/hour) supported for service supporting that the intent expectation is applied. uESpeedContext is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. Following are the allowed values: - targetName: "uESpeedContext" - targetCondition: " IS_LESS_THAN" - targetValueRange: uESpeedContext in clause 6.3.1 in 3GPP TS 28.541 [5] | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| uEMobilityLevelContext | It describes the mobility level of UE for service supporting that the intent expectation is applied. uEMobilityLevelContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: " uEMobilityLevel " - contextCondition: "IS_WITHIN_RANGE" - contextValueRange: uEMobilityLevel in clause 6.3.1 in 3GPP TS 28.541 [5] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| resourceSharingLevelContext | It describes the resource sharing level for service supporting that the intent expectation is applied. resourceSharingLevelContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: - contextAttribute: "resourceSharingLevel" - contextCondition: "IS_WITHIN_RANGE" - contextValueRange: resourceSharingLevel in clause 6.3.1 in 3GPP TS 28.541 [5] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |

4th Change

7.2.2	OpenAPI document "TS28312_IntentNrm.yaml"

openapi: 3.0.1

info:

title: Intent NRM

version: 17.3.0

description: >-

OAS 3.0.1 definition of the Intent NRM

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.312; Intent driven management services for mobile networks

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.312/

paths: {}

components:

schemas:

#-------Definition of concrete IOCs ----------#

SubNetwork-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/SubNetwork-Attr'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/SubNetwork-ncO'

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

Intent:

$ref: '#/components/schemas/Intent-Multiple'

Intent-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

userLabel:

type: string

intentExpectations:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/IntentExpectation"

- $ref: "#/components/schemas/RadioNetworkExpectation"

- $ref: "#/components/schemas/ServiceSupportExpectation"

intentContexts:

type: array

items:

$ref: "#/components/schemas/IntentContext"

intentFulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

#-------Definition of concrete IOCs ----------#

#-------Definition of the IntentExpectation dataType ----------#

IntentExpectation:

description: >-

This data type is the "IntentExpectation" data type without specialisations

type: object

properties:

expectationId:

type: string

expectationVerb:

$ref: "#/components/schemas/ExpectationVerb"

expectationObjects:

type: array

items:

$ref: "#/components/schemas/ExpectationObject"

expectationTargets:

type: array

items:

$ref: "#/components/schemas/ExpectationTarget"

expectationContexts:

type: array

items:

$ref: "#/components/schemas/ExpectationContext"

expectationfulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

RadioNetworkExpectation:

description: >-

This data type is the "IntentExpectation" data type with specialisations to represent MnS consumer's expectations for radio network  delivering and performance assurance

type: object

properties:

expectationId:

type: string

expectationVerb:

$ref: "#/components/schemas/ExpectationVerb"

expectationObjects:

type: array

items:

$ref: "#/components/schemas/RadioNetworkExpectationObject"

expectationTargets:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/WeakRSRPRatioTarget"

- $ref: "#/components/schemas/LowSINRRatioTarget"

- $ref: "#/components/schemas/AveULRANUEThptTarget"

- $ref: "#/components/schemas/AveDLRANUEThptTarget"

- $ref: "#/components/schemas/LowULRANUEThptRatioTarget"

- $ref: "#/components/schemas/LowDLRANUEThptRatioTarget"

- $ref: "#/components/schemas/ExpectationTarget"

expectationContexts:

type: array

items:

$ref: "#/components/schemas/ExpectationContext"

expectationfulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

ServiceSupportExpectation:

description: >-

This data type is the "IntentExpectation" data type with specialisations to represent MnS consumer's expectations for service deployment

type: object

properties:

expectationId:

type: string

expectationVerb:

$ref: "#/components/schemas/ExpectationVerb"

expectationObjects:

type: array

items:

$ref: "#/components/schemas/ServiceSupportExpectationObject"

expectationTargets:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/DLThptPerUETarget"

- $ref: "#/components/schemas/ULThptPerUETarget"

- $ref: "#/components/schemas/DLLatencyTarget"

- $ref: "#/components/schemas/ULLatencyTarget"

- $ref: "#/components/schemas/MaxNumberofUEsTarget"

- $ref: "#/components/schemas/ActivityFactorTarget"

- $ref: "#/components/schemas/UESpeedTarget"

- $ref: "#/components/schemas/ExpectationTarget"

expectationContexts:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/ServiceStartTimeContext"

- $ref: "#/components/schemas/ServiceEndTimeContext"

- $ref: "#/components/schemas/UEMobilityLevelContext"

- $ref: "#/components/schemas/ResourceSharingLevelContext"

- $ref: "#/components/schemas/ExpectationContext"

expectationfulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

#-------Definition of the IntentExpectation dataType ----------#

#-------Definition of the ExpectationObject dataType ----------#

ExpectationObject:

description: >-

This data type is the "ExpectationObject" data type without specialisations

type: object

properties:

objectType:

type: string

enum:

- RAN_SubNetwork  #value for Radio Network Expectation--#

- Service_Support  #value for Service Support Expectation--#

- TBD    #-This will be added based on defined scenario specfic intent expectation-#

objectInstance:

$ref: "TS28623_ComDefs.yaml#/components/schemas/Dn"

objectContexts:

type: array

items:

$ref: "#/components/schemas/ObjectContext"

RadioNetworkExpectationObject:

description: >-

This data type is the "ExpectationObject" data type with specialisations for RadioNetworkExpectation

type: object

properties:

objectType:

type: string

enum:

- RAN_SubNetwork  #value for Radio Network Expectation--#

objectInstance:

$ref: "TS28623_ComDefs.yaml#/components/schemas/Dn"

objectContexts:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/CoverageAreaPolygonContext"

- $ref: "#/components/schemas/CoverageTACContext"

- $ref: "#/components/schemas/PLMNContext"

- $ref: "#/components/schemas/NRFqBandContext"

- $ref: "#/components/schemas/RATContext"

- $ref: "#/components/schemas/ObjectContext"

ServiceSupportExpectationObject:

description: >-

This data type is the "ExpectationObject" data type with specialisations for ServiceSupportExpectation

type: object

properties:

objectType:

type: string

enum:

- Service_Support  #value for Radio Network Expectation--#

objectInstance:

$ref: "TS28623_ComDefs.yaml#/components/schemas/Dn"

objectContexts:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/EdgeIdenfiticationIdContext"

- $ref: "#/components/schemas/EdgeIdenfiticationLocContext"

- $ref: "#/components/schemas/CoverageAreaTAContext"

- $ref: "#/components/schemas/ObjectContext"

#-------Definition of the ExpectationObject dataType ----------#

#-------Definition of the generic dataType --------------#

Condition:

type: string

enum:

- IS_EQUAL_TO

- IS_LESS_THAN

- IS_GREATER_THAN

- IS_WITHIN_RANGE

- IS_OUTSIDE_RANGE

- IS_ONE_OF

- IS_NOT_ONE_OF

- IS_EQUAL_TO_OR_LESS_THAN

- IS_EQUAL_TO_OR_GREATER_THAN

- IS_ALL_OF

FulfilStatus:

type: string

enum:

- FULFILLED

- NOT_FULFILLED

NotFulfilledState:

type: string

enum:

- ACKNOWLEDGED

- COMPLIANT

- DEGRADED

- SUSPENDED

- TERMINATED

- FULFILMENTFAILED

FulfilmentInfo:

type: object

properties:

fulfilStatus:

$ref: "#/components/schemas/FulfilStatus"

notFullfilledState:

description: ->

An attribute which is used when FulfilmentInfo is implemented for IntentFulfilmentInfo

$ref: "#/components/schemas/NotFulfilledState"

notFulfilledReasons:

description: ->

An attribute which is used when FulfilmentInfo is implemented for IntentFulfilmentInfo

type: string

ExpectationVerb:

type: string

enum:

- DELIVER

- ENSURE

#-------Definition of the generic dataType --------------#

#-------Definition of the IntentContext dataType --------------#

IntentContext:

description: >-

This data type is the "IntentContext" data type without specialisations

type: object

properties:

contextAttribute:

type: string

contextCondition:

$ref: "#/components/schemas/Condition"

contextValueRange:

type: array

items:

type: number

#-------Definition of the IntentContext dataType --------------#

#-------Definition of the ExpectationTarget dataType----------#

ExpectationTarget:

description: >-

This data type is the "ExpectationTarget" data type without specialisations

type: object

properties:

targetName:

type: string

targetCondition:

$ref: "#/components/schemas/Condition"

targetValueRange:

type: number

targetContexts:

type: array

items:

$ref: "#/components/schemas/TargetContext"

TargetContext:

description: >-

This data type is the "TargetContext" data type without specialisations

type: object

properties:

contextAttribute:

type: string

contextCondition:

$ref: "#/components/schemas/Condition"

contextValueRange:

type: number

WeakRSRPRatioTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for WeakRSRPRatioTarget

type: object

properties:

targetName:

type: string

enum:

- WeakRSRPRatio

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: "#/components/schemas/WeakRSRPContext"

targetFulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

WeakRSRPContext:

description: >-

This data type is the "TargetContext" data type with specialisations for WeakRSRPContext

type: object

properties:

contextAttribute:

type: string

enum:

- WeakRSRPThreshold

contextCondition:

type: string

enum:

- IS_LESS_THAN

contextValueRange:

type: number

LowSINRRatioTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for LowSINRatioTarget

type: object

properties:

targetName:

type: string

enum:

- LowSINRRatio

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: "#/components/schemas/LowSINRContext"

targetFulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

LowSINRContext:

description: >-

This data type is the "TargetContext" data type with specialisations for LowSINRContext

type: object

properties:

contextAttribute:

type: string

enum:

- LowSINRThreshold

contextCondition:

type: string

enum:

- IS_LESS_THAN

contextValueRange:

type: integer

AveULRANUEThptTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for AveULRANUEThptTarget

type: object

properties:

targetName:

type: string

enum:

- AveULRANUEThpt

targetCondition:

type: string

enum:

- IS_GREATER_THAN

targetValueRange:

type: integer

targetFulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

AveDLRANUEThptTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for AveDLRANUEThptTarget

type: object

properties:

targetName:

type: string

enum:

- AveDLRANUEThpt

targetCondition:

type: string

enum:

- IS_GREATER_THAN

targetValueRange:

type: integer

targetFulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

LowULRANUEThptRatioTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for LowULRANUEThptRatioTarget

type: object

properties:

targetName:

type: string

enum:

- LowULRANUEThptRatio

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: "#/components/schemas/LowULRANUEThptContext"

targetFulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

LowULRANUEThptContext:

description: >-

This data type is the "TargetContext" data type with specialisations for LowULRANUEThptContext

type: object

properties:

contextAttribute:

type: string

enum:

- LowULRANUEThptThreshold

contextCondition:

type: string

enum:

- Is_less_than

contextValueRange:

type: number

LowDLRANUEThptRatioTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for LowDLRANUEThptRatioTarget

type: object

properties:

targetName:

type: string

enum:

- LowDLRANUEThptRatio

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: "#/components/schemas/LowDLRANUEThptContext"

targetFulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

LowDLRANUEThptContext:

description: >-

This data type is the "TargetContext" data type with specialisations for LowDLRANUEThptContext

type: object

properties:

contextAttribute:

type: string

enum:

- LowDLRANUEThptThreshold

contextCondition:

type: string

enum:

- IS_LESS_THAN

contextValueRange:

type: number

DLThptPerUETarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for DLThptPerUETarget

type: object

properties:

targetName:

type: string

enum:

- DlThptPerUE

targetCondition:

type: string

enum:

- IS_GREATER_THAN

targetValueRange:

$ref: "TS28541_SliceNrm.yaml#/components/schemas/XLThpt"

ULThptPerUETarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for ULThptPerUETarget

type: object

properties:

targetName:

type: string

enum:

- UlThptPerUE

targetCondition:

type: string

enum:

- IS_GREATER_THAN

targetValueRange:

$ref: "TS28541_SliceNrm.yaml#/components/schemas/XLThpt"

DLLatencyTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for DLLatencyTarget

type: object

properties:

targetName:

type: string

enum:

- DlLatency

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

ULLatencyTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for ULLatencyTarget

type: object

properties:

targetName:

type: string

enum:

- UlLatency

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

MaxNumberofUEsTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for MaxNumberofUEsTarget

type: object

properties:

targetName:

type: string

enum:

- maxNumberofUEs

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

ActivityFactorTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for ActivityFactorTarget

type: object

properties:

targetName:

type: string

enum:

- activityFactor

targetCondition:

type: string

enum:

- IS_EQUAL_TO

targetValueRange:

type: integer

UESpeedTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for UESpeedTarget

type: object

properties:

targetName:

type: string

enum:

- uESpeed

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

#-------Definition of the concrete ExpectationTarget  dataType----------#

#-------Definition of the concrete ObjectTarget dataType----------------#

ObjectContext:

description: >-

This data type is the "ObjectContext" data type without specialisations

type: object

properties:

contextAttribute:

type: string

contextCondition:

$ref: "#/components/schemas/Condition"

contextValueRange:

type: array

items:

type: number

CoverageAreaPolygonContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for CoverageAreaPolygonContext

type: object

properties:

contextAttribute:

type: string

enum:

- CoverageAreaPolygon

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

$ref: "#/components/schemas/CoverageArea"

CoverageArea:

type: string

CoverageTACContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for CoverageTACContext

type: object

properties:

contextAttribute:

type: string

enum:

- CoverageAreaTac

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

$ref: "TS28541_NrNrm.yaml#/components/schemas/NrTac"

PLMNContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for PLMNContext

type: object

properties:

contextAttribute:

type: string

enum:

- PLMN

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

$ref: "TS28623_ComDefs.yaml#/components/schemas/PlmnId"

NRFqBandContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for NRFqBandContext

type: object

properties:

contextAttribute:

type: string

enum:

- NRFqBand

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

type: string

RATContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for RATContext

type: object

properties:

contextAttribute:

type: string

enum:

- RAT

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

type: string

enum:

- UTRAN

- EUTRAN

- NR

EdgeIdenfiticationIdContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for EdgeIdenfiticationIdContext

type: object

properties:

contextAttribute:

type: string

enum:

- edgeIdentificationId

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

type: array

items:

type: string

EdgeIdenfiticationLocContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for EdgeIdenfiticationLocContext

type: object

properties:

contextAttribute:

type: string

enum:

- edgeIdentificationTarget

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

type: array

items:

type: string

CoverageAreaTAContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for CoverageAreaTAContext

type: object

properties:

contextAttribute:

type: string

enum:

- coverageAreaTA

contextCondition:

type: string

enum:

- IS_ALL_OF

contextValueRange:

type: array

items:

$ref: "#/components/schemas/CoverageAreaTAList"

CoverageAreaTAList:

type: integer

#-------Definition of the concrete ObjectTarget dataType----------------#

#-------Definition of the concrete ExpectionContext dataType----------------#

ExpectationContext:

description: >-

This data type is the "ExpectationContext" data type without specialisations

type: object

properties:

contextAttribute:

type: string

contextCondition:

$ref: "#/components/schemas/Condition"

contextValueRange:

type: array

items:

type: number

ServiceStartTimeContext:

description: >-

This data type is the "ExpectationContext" data type with specialisations for ServiceStartTimeContext

type: object

properties:

contextAttribute:

type: string

enum:

- ServiceStartTime

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

type: string

ServiceEndTimeContext:

description: >-

This data type is the "ExpectationContext" data type with specialisations for ServiceEndTimeContext

type: object

properties:

contextAttribute:

type: string

enum:

- ServiceEndTime

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

type: string

UEMobilityLevelContext:

description: >-

This data type is the "ExpectationContext" data type with specialisations for UEMobilityLevelContext

type: object

properties:

contextAttribute:

type: string

enum:

- UEMobilityLevel

contextCondition:

type: string

enum:

- IS_WITHIN_RANGE

contextValueRange:

type: array

items:

$ref: "TS28541_SliceNrm.yaml#/components/schemas/MobilityLevel"

ResourceSharingLevelContext:

description: >-

This data type is the "ExpectationContext" data type with specialisations for ResourceSharingLevelContext

type: object

properties:

contextAttribute:

type: string

enum:

- ResourceSharingLevel

contextCondition:

type: string

enum:

- IS_WITHIN_RANGE

contextValueRange:

type: array

items:

$ref: "TS28541_SliceNrm.yaml#/components/schemas/SharingLevel"

#-------Definition of the concrete ExpectionContext dataType----------------#

#------Definition of JSON arrays for name-contained IOCs ---------------#

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

Intent-Multiple:

type: array

items:

$ref: '#/components/schemas/Intent-Single'

#------Definition of JSON arrays for name-contained IOCs ---------------#

#----- Definitions in TS 28.312 for TS 28.532 --------------------------#

resources-intentNrm:

oneOf:

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/Intent-Single'

#----- Definitions in TS 28.312 for TS 28.532 --------------------------#

End of Changes