3GPP TSG-SA WG6 Meeting #55	S6-231775

Berlin, Germany 22nd – 26th May 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0368 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Information element alignment for ECS discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | "Service Provisioning filters" are used in several procedures, but this information element is unclear and not consistent with the information flows which SA6 has approved in S6-231625. |  |  |  |  |  |  |  |  |
| Summary of change: |  | "Service Provisioning filters" is replaced with "AC profile(s)," based on the information flows in S6-231625. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is inconsistency between the procedures and the information flows. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.17.2.3.2, 8.17.2.3.3.2, 8.17.2.3.3.4, 8.17.2.4.2.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.17.2.3.2	Request-response model

When required to find a suitable partner ECS, the ECS queries the ECS-ER by providing information such as the location of the UE, applications required by the UE etc. In response the ECS-ER provides ECS configuration information of partner ECS(s) providing the required application at the location indicated by the ECS. Figure 8.17.2.3.2-1 illustrates the procedure.

Pre-conditions:

1.	The ECS has the address (e.g. URI) of the ECS-ER.

Figure 8.17.2.3.2-1: ECS querying ECS-ER

1.	The ECS sends a ECS discovery request to the ECS-ER. The request contains security information of the ECS, and optionally AC profile(s) and UE’s current location to filter ECS. If the UE hosting the EEC is roaming in a V-PLMN (determined using the serving PLMN information in the received service provisioning request, or by interacting with the H-PLMN), then the ECS discovery request includes the information of the serving PLMN i.e. the V-PLMN.

2.	The ECS-ER authorizes the received request. If authorized, the ECS-ER processes the request and gathers the information of partner ECS(s) that can satisfy the query parameters received in step 1.

Editor's note: Whether and how ECS-ER uses a different procedure to collect information from another ECS-ER or ECS from a different domain is FFS.Related updates to EDGE-10 interface are FFS.

3.	The ECS-ER sends a ECS discovery response to the ECS. The response includes ECS configuration information of the Partner ECS(s) available in the H-PLMN and/or the V-PLMN, depending on the serving PLMN of the UE as indicated in step 1 and may include DNN and S-NSSAI information for roaming UEs to establish an LBO PDU session with the ECS as specified in 3GPP TS 23.548 [20]. The ECS caches the received information for further use.

NOTE:	The ECS sending the ECS discovery request can also be an ECS-ER.

* * * Next Change * * * *

8.17.2.3.3.2	Subscribe

Figure 8.17.2.3.3.2-1 illustrates the procedure.

Pre-conditions:

1.	The ECS has the address (e.g. URI) of the ECS-ER.

Figure 8.17.2.3.3.2-1: ECS discovery subscription procedure

1.	The ECS sends a ECS discovery subscription request to the ECS-ER. The request contains security information of the ECS. The request optionally contains AC profile(s),	 UE's location information and the PLMN information.

2.	The ECS-ER authorizes the received request. If authorized, the ECS-ER creates and stores the subscription.

3.	The ECS-ER sends a ECS discovery subscription response to the ECS which includes the subscription identifier and may include the expiration time, indicating when the subscription will automatically expire. To maintain the subscription, the EEC shall send a ECS discovery subscription update request prior to the expiration time. If a ECS discovery subscription update request is not received prior to the expiration time, the ECS-ER shall treat the ECS as implicitly unsubscribed.

* * * Next Change * * * *

8.17.2.3.3.4	Subscription update

Figure 8.17.2.3.3.4-1 illustrates the procedure.

Pre-conditions:

1.	The ECS has subscribed for ECS discovery.

Figure 8.17.2.3.3.4-1: ECS discovery subscription update procedure

1.	The ECS sends a ECS discovery subscription update request to the ECS-ER. The request contains security information of the ECS and may include an AC profile(s), UE's location information and the PLMN information.

2.	The ECS-ER authorizes the received request. If authorized, the ECS-ER updates the stored subscription.

3.	The ECS-ER sends an ECS discovery subscription update response indicating success or failure of the subscription update operation. The ECS-ER may provide an updated expiration time to indicate to the ECS when the updated subscription will automatically expire. To maintain the subscription, the ECS needs to send another subscription update request prior to the expiration time. If a subscription update request is not received prior to the expiration time, the ECS-ER treats the ECS as implicitly unsubscribed.

* * * Next Change * * * *

8.17.2.4.2.2	Request-response model

Figure 8.17.2.4.2.2-1 illustrates service provisioning procedure based on request/response model.

Pre-condition(s):

1.	ECS has received a sevice provisioning request from an EEC or T-EES discovery request from an EES.

2.	The ECS has the ECS configuration information of the partner ECS (e.g. through preconfiguration, OAM configuration or ECS discovery).

Figure 8.17.2.4.2.2-1: Service provisioning information retrieval – Request/Response

1.	The ECS sends a service provisioning information retrieval request to the partner ECS. The request contains security information of the ECS, and optionally AC profile(s) and UE’s current location. If the UE hosting the EEC is roaming in a V-PLMN (determined using the serving PLMN information in the received service provisioning request, or by interacting with the H-PLMN), then the service provisioning infromation retrieval request includes the information of the serving PLMN i.e. the V-PLMN.

2.	The partner ECS authorizes the received request. If authorized, the partner ECS processes the request and gathers the provisioning information of the EDNs that can satisfy the query parameters received in step 1.

3.	The partner ECS sends a service provisioning information retrieval response to the ECS. The response includes provisioning information of the EDNs that can satisfy the query parameters received in step 1 and may include lifetime information. The ECS caches the received information for further use. If the Lifetime IE is included, then the ECS may reuse the retrieved information only for the duration specified by the Lifetime IE, without the need to repeat step 1.

NOTE:	Partner ECS can be a V-ECS based on the roaming status of the UE for which the service provisioning information is being retrieved.

Editor's note:	Information flows for service provisioning information retrieval is FFS. It is FFS whether this procedure and its information flows can be merged to the existing service procedure and flows.

Editor's note: Applicability of this procedure for federation is FFS.