3GPP TSG-WG SA2 Meeting #157	S2-2308112

# Berlin, May 22 – 26, 2023		(revision of S2-2307726)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0369 | rev | 4 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Further clarification on the support of PRUs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | ON/OFF indicates activation/deactivation of a PRU when the PRU is associated with its serving LMF. It could be used for different purposes: such as PRU temporarily with a bad network connection, or temporary have other high-priority service to serve than PRU functionality. ON/OFF status does not mean Power ON/OFF for the PRU. Using PRU ON/OFF state allows for flexible management of PRU and resuming the usage of a PRU fast whenever the PRU is available again.It can not be replaced by de-association and re-association. Because de-association would mean the PRU information of the UE (including the location information, location capability), the information whether a UE can serve as a PRU as well as the allocated network resource for the PRU is completely removed from its associated LMF. This introduces unecessary signaling overhead and latency when the PRU is re-associated with the same LMF. Meanwhile, de-association is also used in case a PRU moves to another LMF. Distinguish the PRU on/off from PRU mobility case can avoid the signaling to track the PRU. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add texts for PRU support with ON/OFF state. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN in PRU clasue is not resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.3.8	Location Management Function, LMF

The LMF manages the overall co-ordination and scheduling of resources required for the location of a UE that is registered with or accessing 5GCN. It also calculates or verifies a final location and any velocity estimate and may estimate the achieved accuracy. The LMF receives location requests for a target UE from the serving AMF using the Nlmf interface. The LMF interacts with the UE in order to exchange location information applicable to UE assisted and UE based position methods and interacts with the NG-RAN, N3IWF or TNAN in order to obtain location information.

The LMF shall determine the result of the positioning in geographical co-ordinates as defined in TS 23.032 [8] and/or in local co-ordinates as defined in TS 23.032 [8]. If requested and if available, the positioning result may also include the velocity of the UE. The coordinate type(s) is determined by LMF when receiving a location request, based on LCS Client type and supported GAD shapes. If the location request indicates regulatory LCS Client type the LMF shall determine a geographical location and optionally a location in local coordinates. For location request indicates a value added LCS Client type, the LMF may determine the UE location in local coordinates or geographical co-ordinates or both. If the supported GAD shapes is not received or Local Co-ordinates is not included in the supported GAD shapes, the LMF shall determine a geographical location.

NOTE 1:	Some RAT independent position methods (e.g. GNSS based position methods) can only determine a UE location in geographical co-ordinates. In such a case, the LMF may translate a UE location in geographical co-ordinates into a location in local co-ordinates when an origin for the local co-ordinates has known global coordinates. When an origin for the local co-ordinates does not have known global coordinates, position methods that can only determine a UE location in geographical co-ordinates cannot be used to determine a UE location in local co-ordinates.

Additional functions which may be performed by an LMF to support location services include the following.

-	Support a request for a single location received from a serving AMF for a target UE.

-	Support a request for periodic or triggered location received from a serving AMF for a target UE.

-	Determine type and number of position methods and procedures based on UE and PLMN capabilities, QoS, UE connectivity state per access type, LCS Client type, co-ordinate type and optionally service type and indication of requiring reliable UE location information.

-	Report UE location estimates directly to a GMLC for periodic or triggered location of a target UE.

-	Support cancelation of periodic or triggered location for a target UE.

-	Support the provision of broadcast assistance data to UEs via NG-RAN in ciphered or unciphered form and forward any ciphering keys to subscribed UEs via the AMF.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	Support of receiving stored UE Positioning Capability from AMF and support of providing updated UE Positioning Capability to AMF.

-	Map the UE location to a geographical area where the PLMN is or is not allowed to operate based on the request from AMF.

-	Support determination of a UE location at a scheduled location time.

-	Determine whether to use user plane or control plane for positioning.

-	Support handling of 5GC-MT-LR, 5GC-NI-LR, 5GC-MO-LR and deferred 5GC-MT-LR for periodic or triggered location over a user plane connection between UE and LMF.

Editor's note:	LMF functionality of user plan connection handling will be updated according to the protocol stack decision made by CT WG1 and SA WG3 groups.

-	Support collection of GNSS assistance data from AFs.

-	Support service level PRU Association, PRU Association update or PRU Disassociation.

-	LMF supports verification of a PRU initiated Association or Disassociation by checking whether there is an PRU verified indication from AMF.

-	LMF stores the received PRU information contained in service level PRU Association message.

-	LMF may indicate support of PRU function to NRF via NF profile and may further send the PRU information to NRF via NF profile update.

-	LMF may request a PRU to associate to a new LMF by returning a Routing ID of the new LMF.

PRU ON/OFF states indicate temporarily availability of the PRU functionality of a UE at the serving LMF (e.g., PRU OFF due to other high priority tasks/energy saving at the UE, or the UE temporarily loses network coverage).

-	Support selection of a PRU based on stored PRU information if the LMF needs to obtain the location measurements from the PRU to assist positioning of a target UE.

-	Support to obtain PRU location measurements as described in clause 5.4.5 of TS 38.305 [9] by triggering the procedure in clause 6.11.

-	Support to obtain PRU location measurements from other PRU serving LMF(s).

-	As a serving LMF of target UE(s), support discovery and selection of other PRU serving LMF(s) by querying the NRF and support to request PRU location measurements from the selected LMF(s).

-	As a serving LMF of PRU(s), support to provide PRU location measurements to other LMF(s) after receiving a request from other LMF(s).

-	Support to determine UE location by considering obtained PRU location measurements.

Editor's note:	Whether and how AF provides PRU information (e.g. known location) on behalf of a PRU to network is FFS.

NOTE 2:	Country, area within a country, or an international area can be supported as different types of geographical area.

-	Support a request for user plane reporting from a UE to an LCS Client or AF for a periodic or triggered 5GC-MT-LR. Subsequently, support the transfer of cumulative event reports from the target UE via control plane back to the H-GMLC and LCS Client or AF. Also support any request for assistance data received in a cumulative event report.

-	Determine UE location for a UE connecting to a MBSR based on location and velocity of the MBSR and the timing of the location estimations for the target UE and MBSR.

**** End of Changes ****