3GPP TSG-SA3 Meeting #111 	S3-233238

# Berlin, Germany, 22 -26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |
|  | 33.216 | CR | 0025 | rev | - | Current version: | 17.0.0 |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to eNB SCAS for the support of UP IP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The support of User Plane integrity protection has been introduced for eNBs in the previous release. Therefore, for the completion of the SCAS work, it is important to introduce the relevant tests for eNBs similarly to how it was done for the gNB in the past in order to cover the integrity protection of the user plane and the use of policies to actiavte it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a new requirement and test for user plane integrity protection. Addition of new requirements and corresponding tests related to the use of the UP IP policy. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete SCAS work for the UP IP feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.x(new), 4.2.2.1.y(new), 4.2.2.1.z(new), 4.2.2.1.w(new), 4.2.2.1.v(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | SA3#108e: S3-222381, S3-222382, S3-2223813 SA3#109: S3-223493, S3-223495 |  |  |  |  |  |  |  |  |

********* Begin 1st change*********

##### 4.2.2.1.x	Integrity protection of user data between the UE and the eNB

Requirement Name: Integrity protection of user data between the UE and the eNB.

Requirement Reference: TS 33.401 [3], clause 5.1.4.

Requirement Description: "User plane packets between the eNB and the UE may be integrity protected on the Uu interface. " in clause 5.1.4

Threat References: TBD

Test Case:

Test Name: TC-UP-DATA-INT_eNB

Purpose: To verify that the user data packets are integrity protected over the Uu interface.

Pre-Condition:

-	The eNB network product shall be connected in emulated/real network environments. UE may be simulated.

-	Tester shall have knowledge of integrity algorithm and integrity protection keys.

-	The tester can capture the message via the Uu interface, or can capture the message at the UE.

Execution Steps:

1. eNB sends RRCConnectionReconfiguration with integrity protection indication "on".

2. Check any User data sent by eNB after sending RRCConnectionReconfiguration and while the UE is in active state is integrity protected.

Expected Results:

Any user plane packets sent between UE and eNB over the Uu interface after eNB sending RRCConnectionReconfiguration is integrity protected.

Expected format of evidence:

Evidence suitable for the interface e.g. Screenshot containing the operational results.

********* End of 1st change*********

********* Begin 2nd change*********

##### 4.2.2.1.y	Local UP integrity protection configuration

Requirement Name: Select the right UP integrity protection policy.

Requirement Reference: TS 33.401 [2] clause 7.3.3

Requirement Description: " The eNB shall be locally configured with UP integrity protection policy. " in clause 7.3.3

Threat References: TBD

Test Case:

Test Name: TC_LOCAL_UP_INTEGRITY_PROTECTION_CONFIGURATION

Purpose: To verify that the eNB is locally configured with a UP integrity protection policy

Pre-Condition:

-	The eNB network product shall be connected in emulated/real network environments. UE and MME may be simulated.

-	The eNB locally configured to activate UP integrity protection by default if no UP integrity protection policy is received from MME.

-	Tester shall have knowledge of integrity algorithm and integrity protection keys.

-	The tester can capture the message via the Uu interface, or can capture the message at the UE.

Execution Steps:

1. MME sends EPS security capability with EIA7 indicating the UP integrity protection is supported by the UE. But the MME does not send a UP integrity protection policy to the eNB.

2. eNB sends RRCConnectionReconfiguration with integrity protection indication "on".

3. Check any User data sent by eNB after sending RRCConnectionReconfiguration and while the UE is in active state is integrity protected.

Expected Results:

Any user plane packets sent between UE and eNB over the Uu interface after eNB sending RRCConnectionReconfiguration is integrity protected.

Expected format of evidence:

Evidence suitable for the interface e.g. Screenshot containing the operational results.

********* End of 2nd change*********

********* Begin 3rd change*********

##### 4.2.2.1.z	UP IP policy selection

Requirement Name: Select the right UP IP policy.

Requirement Reference: TS 33.401 [2] clause 7.3.3

Requirement Description: " If the eNB receives UP integrity protection policy from the MME, the eNB shall use the received UP integrity protection policy, otherwise, the eNB shall use the locally configured UP integrity protection policy if EIA7 in the EPS security capability indicates that the UE supports user plane integrity protection with EPC. " in clause 7.3.3

Threat References: TBD

Test Case:

Test Name: TC_ UP_IP_POLICY_Selection

Purpose: To verify that the eNB has a locally configured UP IP policy

Pre-Condition:

-	The eNB network product shall be connected in emulated/real network environments. UE and MME may be simulated.

-	The eNB locally UP IP is set to NOT NEEDED.

-	Tester shall have knowledge of integrity algorithm and integrity protection keys.

-	The tester can capture the message via the Uu interface, or can capture the message at the UE.

Execution Steps:

1. MME sends EPS security capability with EIA7 indicating the UP IP is supported by the UE. But the MME does sends a UP IP policy with REQUIRED to the eNB.

2. eNB sends RRCConnectionReconfiguration with integrity protection indication "on".

3. Check any User data sent by eNB after sending RRCConnectionReconfiguration and while the UE is in active state is integrity protected.

Expected Results:

Any user plane packets sent between UE and eNB over the Uu interface after eNB sending RRCConnectionReconfiguration is integrity protected.

Expected format of evidence:

Evidence suitable for the interface e.g. Screenshot containing the operational results.

********* End of 3rd change*********

********* Begin 4th change*********

##### 4.2.2.1.w	UP IP policy selection in S1 Handover

Requirement Name: Select the right UP IP policy in S1 handover.

Requirement Reference: TS 33.401 [2] clause 7.3.3

Requirement Description: " At an S1-handover, the source MME shall send the UE's UP integrity protection policy and the UE EPS security capability to the target eNB via the target MME. Besides, the source eNB shall also send the UE's UP integrity protection policy if received from the source MME to the target eNB in a source-to-target container. The target eNB shall use the UE capability indicating support of UP IP in EPS together with the UP integrity protection policy received from the MME and ignore the UP integrity protection received in the source-to-target container. If the target eNB does not receive the UP integrity protection policy from the MME, the target eNB shall use the UE capability indicating support of UP IP in EPS together with the UP integrity protection policy received from the source eNB. If both policies from MME and source eNB are absent, but EIA7 in the EPS security capability indicates that the UE supports use of user plane protection with EPC, the eNB shall use locally configured UP integrity protection policy." in clause 7.3.3

Threat References: TR 33.926 [4], clause C.2.2.a, UP integrity protection policy selection

Test Case:

Test Name: TC_ UP_IP_POLICY_Selection_S1_Handover

Purpose: To verify that the eNB has correct selection on UP IP policy in S1 handover

Pre-Condition:

-	The target eNB network product shall be connected in emulated/real network environments. UE, source eNB and MME may be simulated.

-	The target eNB locally UP IP is set to NOT NEEDED.

-	Tester shall have knowledge of integrity algorithm and integrity protection keys.

-	The tester can capture the message via the Uu interface, or can capture the message at the UE.

Execution Steps:

Test Case 1:

- MME sends EPS security capability with EIA7 indicating the UP IP is supported by the UE. And the MME sends a UP IP policy with REQUIRED to the target eNB.

- Source eNB sends UP IP policy with NOT NEEDED in the source-to-target container to the target eNB.

- eNB sends RRCConnectionReconfiguration with integrity protection indication "on".

- Check any User data sent by eNB after sending RRCConnectionReconfiguration and before UE enters CM-Idle state is integrity protected.

Test Case 2:

- MME sends EPS security capability with EIA7 indicating the UP IP is supported by the UE. And the MME does not send a UP IP policy to the target eNB.

- Source eNB sends UP IP policy with REQUIRED in the source-to-target container to the target eNB.

- eNB sends RRCConnectionReconfiguration with integrity protection indication "on".

- Check any User data sent by eNB after sending RRCConnectionReconfiguration and before UE enters CM-Idle state is integrity protected.

Test Case 3:

- MME sends EPS security capability with EIA7 indicating the UP IP is supported by the UE. And the MME does not send a UP IP policy to the target eNB.

- Source eNB does not send UP IP policy in the source-to-target container to the target eNB.

- eNB sends RRCConnectionReconfiguration with integrity protection indication "off".

- Check any User data sent by eNB after sending RRCConnectionReconfiguration and before UE enters CM-Idle state is not integrity protected.

Expected Results:

For test case 1 and 2, any user plane packets sent between UE and eNB over the Uu interface after eNB sending RRCConnectionReconfiguration is integrity protected.

For test case 3, any user plane packets sent between UE and eNB over the Uu interface after eNB sending RRCConnectionReconfiguration is not integrity protected.

Expected format of evidence:

Evidence suitable for the interface e.g. Screenshot containing the operational results.

********* End of 4th change*********

********* Begin 5th change*********

##### 4.2.2.1.v	Bidding down prevention for UP IP Policy

Requirement Reference: TS 33.401 [3], clause 7.3.3

Requirement Description: "Further, in the Path-Switch message, the target eNB shall send the UE's UP integrity protection policy and corresponding E-RAB ID to the MME. The sent UP integrity protection policy can either be the one received from source eNB or the locally configured one if the target eNB does not receive it from the source eNB, but the EIA7 in the EPS security capability indicates that the UE supports user plane integrity protection with EPC. " as specified in TS 33.401 [3], clause 7.3.3.

Threat References: TR 33.926 [4], clause C.2.2.a, bidding down for UP IP Policy

Test Case:

Purpose:

Verify that bidding down for UP IP policy is prevented in X2-handovers.

Pre-Conditions:

-	The target eNB network product shall be connected in emulated/real network environments. UE, source eNB and MME may be simulated.

-	The target eNB locally UP IP is set to NOT NEEDED.

Execution Steps:

Test Case 1:

- Source eNB sends EPS security capability with EIA7 indicating the UP IP is supported and UP IP policy with REQUIRED in Handover Request message to the target eNB.

- eNB sends path-switch request message with UP IP policy with REQUIRED to the MME.

Test Case 2:

- Source eNB sends EPS security capability with EIA7 indicating the UP IP is supported in Handover Request message to the target eNB. The source eNB does not send UP IP policy in the Handover Request message.

- eNB sends path-switch request message with UP IP policy with NOT NEEDED to the MME.

Expected Results:

For test case 1, the UP IP policy with REQUIRED is in the path-switch request message.

For test case 2, the UP IP policy with NOT NEEDED is in the path-switch request message.

Expected format of evidence:

Snapshots containing the result.

********* End of change*********