3GPP TSG-SA5 Meeting #149 	S5-233999

# Berlin, Germany, 22nd May 2023 - 26th May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0912 | rev | - | Current version: | 18.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.541 Correct issues for NR NRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | adNRM |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The label <<names>> for name-contain relationship is missing in Figure 4.2.1.1-7(NRM fragment to support RIM) and Figure 4.2.1.1-20 (NRM fragment for CCO Management). - Both “NG-RAN MOCN network sharing with multiple cell identity”, “MOCN with multiple cell identity broadcast feature” and “NG-RAN MOCN network sharing with multiple cell identity broadcast” is used, which is inconsistent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add missing label <<names>> for name-contain relationship is missing in Figure 4.2.1.1-7(NRM fragment to support RIM) and Figure 4.2.1.1-20 (NRM fragment for CCO Management). - Change the “NG-RAN MOCN network sharing with multiple cell identity” and “MOCN with multiple cell identity broadcast feature” to “NG-RAN MOCN network sharing with multiple cell identity broadcast” to keep consistence. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Above issues observed in published TS 28.541. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1, 4.3.67.1, 4.3.68.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

4.2	Class diagram

4.2.1	Class diagram for gNB and en-gNB

4.2.1.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for this gNB and en-gNB. For the UML semantics, see 3GPP TS 32.156 [43]. Subsequent clauses provide more detailed specification of various aspects of these classes.

The model fragments are for management representation of gNB and en-gNB for all NG-RAN deployment scenario as listed below.

-	Non-split NG-RAN deployment scenario, represents the gNB defined in TS 38.401[4]. In this scenario, a gNB is represented by a combination of a GNBCUCPFunction, one or more GNBCUUPFunctions and one or more GNBDUFunctions.

-	2-split NG-RAN deployment scenario, represents the gNB consist of gNB-CU and gNB-DU defined in TS 38.401[4] clause 6.1.1. In this scenario, a gNB-CU is represented by a combination of a GNBCUCPFunction and one or more GNBCUUPFunctions, whereas a gNB-DU is represented by a GNBDUFunction.

-	3-split NG-RAN deployment scenario, represents the gNB consist of gNB-CU-CP, gNB-CU-UP and gNB-DU defined in TS 38.401[4] clause 6.1.2. In this scenario, a gNB-CU-CP is represented by a GNBCUCPFunction, a gNB-CU-UP is represented by a GNBCUUPFunction, and a gNB-DU is represented by a GNBDUFunction.

Figure 4.2.1.1-1: NRM for all deployment scenarios

Figure 4.2.1.1-2: NRM for EPs for all deployment scenarios

Figure 4.2.1.1-3: NRM for <<IOC>>NRSectorCarrier and <<IOC>>BWP for all deployment scenarios

Figure 4.2.1.1-4: Cell Relation view for all deployment scenarios

NOTE 1:	The above NRM fragment uses SubNetwork to hold both NR and LTE external entities and frequencies.

Figure 4.2.1.1-5: Cell Relation view for all deployment scenarios

NOTE 2:	The above NRM fragment uses NRNetwork to hold NR external entities and frequency and using EUtraNetwork to hold LTE external entities and frequency. The NRNetwork and EUtraNetwork are subclasses of SubNetwork (defined in TS 28.622 [30]) with no additional attributes. The reason using NRNetwork and EUtraNetwork is for a clean separation of NR external entities and frequency and LTE external entities and frequency.

Figure 4.2.1.1-6: NRM fragment for abstract RRM Policies

Figure 4.2.1.1-6a: NRM fragment for RRMPolicyRatio

Figure 4.2.1.1-7: NRM fragment to support RIM

The Figure 4.2.1.1-8 shows the NRM fragment for pre-configured 5QIs in NG-RAN.

Figure 4.2.1.1-8: NRM fragment for pre-configured 5QIs in NG-RAN

NOTE 3: Void

Figure 4.2.1.1-9: NRM fragment for DANR Management

Figure 4.2.1.1-10: NRM fragment for DES Management

Figure 4.2.1.1-11: NRM fragment for DRACH Management

Figure 4.2.1.1-12: NRM fragment for DMRO Management

Figure 4.2.1.1-13: NRM fragment for DPCI Management

Figure 4.2.1.1-14: NRM fragment for CES Management

Figure 4.2.1.1-15: NRM fragment for CPCI Management

The Figure 4.2.1.1-16 shows the NRM fragment for dynamic 5QIs in NG-RAN.

Figure 4.2.1.1-16: NRM fragment for dynamically assigned 5QIs in NG-RAN

Figure 4.2.1.1-17: NRM fragment for NG-RAN MOCN network sharing with multiple cell identity broadcast feature

Figure 4.2.1.1-18: NRM fragment for F1 related EPs to support individual F1 interface for NG-RAN MOCN network sharing with multiple cell identity broadcast feature

Figure 4.2.1.1-19: NRM fragment for DLBO Management

Figure 4.2.1.1-20: NRM fragment for CCO Management

2nd Change

4.3.67	OperatorDU

4.3.67.1	Definition

This IOC contains attributes to support the NG-RAN Multi-Operator Core Network (NG-RAN MOCN) network sharing with multiple Cell Identity broadcast feature. An instance of OperatorDU <<IOC>> should be created and configured for each POP. When configured the attributes override those in parent GNBDUFunction instance.

The OperatorDU <<IOC>> is only used to support NG-RAN MOCN with multiple cell identity broadcast feature. If NG-RAN MOCN with multiple cell identity broadcast feature is not supported, is not used.

The following table identifies the necessary end points required for the representation of shared gNB and shared en-gNB, of all deployment scenarios.

| Req Role | End point requirement for 3-split deployment scenario | End point requirement for 2-split deployment scenario | End point requirement for Non-split deployment scenario |
| --- | --- | --- | --- |
| Shared gNB | <<IOC>>EP_F1C, <<IOC>>EP_F1U | <<IOC>>EP_F1C, <<IOC>>EP_F1U | None. |
| Shared en-gNB | <<IOC>>EP_F1C, <<IOC>>EP_F1U | <<IOC>>EP_F1C, <<IOC>>EP_F1U | None. |

For scenarios with an F1 interface supporting multiple PLMN broadcast, the values of the EP_F1C and EP_F1U attributes contained by different OperatorDU of the same GNBDUFunction should be same.

4.3.67.2	Attributes

The OperatorDU IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |

4.3.67.3	Attribute Constraints

None

4.3.67.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

4.3.68	NROperatorCellDU

4.3.68.1	Definition

The NROperatorCellDU <<IOC>> contains attributes to support operator specific cell level information (including cellLocalId, pLMNInfoList, nRTAC) to support NG-RAN Multi-Operator Core Network (NG-RAN MOCN) network sharing with multiple Cell Identity broadcast feature. An instance of NROperatorCellDU <<IOC>> should be created and configured for each POP. When configured the attributes override those in the associated NRCellDU instance.

The NROperatorCellDU <<IOC>> is only used to support NG-RAN MOCN with multiple cell identity broadcast feature. If the NG-RAN MOCN with multiple cell identity broadcast feature is not supported, this IOC is not used.

4.3.68.2	Attributes

The NROperatorCellDU IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| administrativeState | M | T | T | F | T |
| pLMNInfoList | M | T | T | F | T |
| nRTAC | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| nRCellDURef | M | T | T | F | T |

4.3.68.3	Attribute Constraints

None

4.3.68.4	Notifications

The common notifications defined in clause 5.5 are valid for this IOC, without exceptions or additions.

End of Changes