3GPP TSG-SA5 Meeting #149	S5-234171

# Berlin, Germany, 22nd May 2023 - 26th May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.557 | CR | 0003 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add use case and requirements for management of NPN service customer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM_NPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The key issue of management of NPN service customer has been studied in clause 5.2 TR 28.907, and is recommended to be introduced in the normative work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add use case and requirements for management of NPN service customer |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NPN service customer management will be missing as one of the Rel-18 NPN features. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 5.1.0.X, 5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 5.1.0	Generic use cases

#### 5.1.0.X	Management of NPN service customer

As described in clause 4.2, an NPN service customer is used to represent the role of communication service customer in NPN environment. NPN service customer can request and consume the management capabilities exposed by the mobile network operator. The mobile network operator would restrict the types (e.g. provisioning, fault supervision, performance assurance) of management capabilities and corresponding managed network resource exposed to an NPN service customer. The restriction of MnS consumption can be achieved through the granular access control on NPN service customer and the filtering of MnS.

Next change

### 5.2.1	Generic requirements for management of NPN

REQ-NPN-FUN-01 The 3GPP management system shall have the capability to monitor the performance measurements and KPIs associated with an NPN.

REQ-NPN-FUN-02 The 3GPP management system shall have the capability to provide the performance measurements and KPIs associated with an NPN to authorized entity, either NPN-SC (when NPN-SP and NPN-OP are both played by the same actor) or NPN-SP (when NPN-SP and NPN-OP are played by different actors).

REQ-NPN-FUN-03 The 3GPP management system shall have the capability to receive SLA requirements from authorized NPN-SC and then translating the SLA requirements into service and network resources related requirements.

REQ-NPN-FUN-04 The 3GPP management system shall have the capability to evaluate SLS assurance related to an NPN.

REQ-NPN-FUN-05 The 3GPP management system shall have the capability to restrict the exposure of management capabilities and corresponding managed resources to NPN-SC.

REQ-NPN-FUN-06 The 3GPP management system shall have the capability to support management capabilities exposure, which includes management capabilities of network provisioning, fault supervision and performance assurance to the authorized NPN-SC.

REQ-NPN-FUN-07 The 3GPP management system shall have the capability to provision both physical and virtual NPNs.

REQ-NPN-FUN-08 The 3GPP management system shall have the capability to provision different NPNs intended to different NPN-SCs.

REQ-NPN-FUN-09 The 3GPP management system shall have the capability to provision an NPN which serves different NPN-SCs.

REQ-NPN-FUN-10 The 3GPP management system shall offer the NPN-SC the ability to manage its own NPN(s) and its private slice(s) in the PLMN in a combined manner.

REQ-NPN-FUN-n The 3GPP management system shall have the capability to maintain the NPN service customer information, e.g. authorized management capabilities.

End of changes