3GPP TSG-SA3 Meeting #111-bis-e 	S3-233244

# Berlin, Germany, 22 - 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.517 | CR | 0010 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New SCAS test on trust anchoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Federal Office for Information Security (BSI), Deutsche Telekom AG |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Trust anchoring as specified in clause 13.1.2 of TS 33.501 requires a test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case to verify the trust anchoring works as intended. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test case for the proper functioning of trust anchoring. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TR 33.926 CR 0065.. |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

4.2.5		Web Servers

Requirement Name: Correct Handling of HTTPS Trust Anchoring

Requirement Reference: TS 33.501 [3], clause 13.1.2

Requirement Description: The SEPP maintains a set of trust anchors, each consisting of a list of trusted root certificates and a list of corresponding PLMN-IDs. Any given PLMN-ID appears in at most one trust anchor. During N32-c connection setup, the SEPP maps the PLMN-ID of the remote SEPP leaf (server or client) certificate to the associated trust anchor for the purposes of certificate chain verification. Only the root certificates in the associated list is treated as trusted during certificate chain verification. If the remote SEPP certificate contains multiple PLMN-IDs that are mapped to different trust anchors, then that certificate is rejected.

Threat References: TR 33.926 [4], clause 5.3.3.7, Eavesdropping, clause 5.3.6.10, Insecure Network Services

Test Case:

Test Name: TC_CORRECT_TRUST_ANCHORING

Purpose:

Verify that the SEPP under test correctly implements trust anchoring when setting up HTTPS (TLS) connections.

Procedure and execution steps:

Pre-Conditions:

- 	System documentation of the SEPP under test, which details the methods supported for TLS protection on N32 and how internal log files can be accessed.

- 	Two simulated, independent root CAs, denoted RCA1 and RCA2

-	A peer SEPP instance of a remote PLMN for N32 communication with the SEPP under test, which may be simulated.

Execution Steps

1.	The tester selects two different PLMN IDs, denoted ID1 and ID2, e.g. ID1={MCC=262, MNC=01} and ID2={MCC=262, MNC=02}.

2.	The tester configures two trust anchors in the SEPP under test, as follows:

- 	ID1 gets associated with RCA1 and

-	ID2 gets associated with RCA2.

3. The tester generates a server certificate for the SEPP under test with which it authenticates itself towards a peer SEPP. The peer SEPP is configured to accept these certificates.

4.	The tester generates four TLS client certificates C1, C2, C3, C4 for the peer SEPP, as follows:

-	C1 contains ID1 and is signed by RCA1 (or a subCA which is signed only by RCA1)

-	C2 contains ID2 and is signed by RCA2 (or a subCA which is signed only by RCA2)

-	C3 contains ID1 and is signed by RCA2 (or a subCA which is signed only by RCA2)and

-	C4 contains both ID1 and ID2 and is signed by RCA1 or RCA2 (or a subCA which is signed by RCA1 or RCA2).

NOTE 1: 	The expression "contains IDX" means that there exists a Subject Alternative Name (SAN) field in the certificate that contains the value chosen in step 1. Examples of the SAN field contents are example.sepp.5gc.mnc02mcc262.3gppnetwork.org and example.sepp.5gc.mnc01mcc262.exampleipx.ipxnetwork.org

5.	The peer SEPP is configured to authenticate itself using C1. If C1 was issued by a SubCA, then the SubCA certificate is included in the certificate chain which the peer SEPP uses to authenticate itself. The tester initiates an N32c connection from the SEPP under test towards the peer SEPP and observes whether the HTTPS connection succeeds, and, if not, documents the failure reasons as shown in the log files of the SEPP under test.

6.	The tester repeats step 5,  replacing C1 with C2, C3, and C4 iteratively.

7.	Steps 3 to 6 are repeated in order to test for reversed client/server roles. Specifically, in step 3 the tester generates a client certificate (instead of a server certificate), in step 4 the tester generates four server certificates (instead of four client certificates), and in step 5 the tester initiates the connection from the peer SEPP (instead of initiating the connection from the SEPP under test).

Expected Results:

In step 5, the TLS (HTTPS) connection setup succeeds for the iterations with C1 and C2 and fails for the iterations with C3 and C4.

NOTE 2:	The iteration with C3 fails because the PLMN-ID indicated in the client certificate does not match any of the trusted certificates in the corresponding trust anchor. The iteration with C4 fails because the PLMN-IDs indicated in the client certificate are not associated with the same trust anchor.

Expected format of evidence:

The evidence is in the form of log file entries associated with eight TLS connection establishment attempts, where entries for the cases with C1 and C2 indicate success, and entries for the cases with C3 and C4 indicate failure and show the corresponding failure reasons, including TLS Error Alert 48 (unknown_ca).

********** END OF CHANGE **********