3GPP TSG-SA3 Meeting #111 	S3-233211

# Berlin, Germany, 22 -26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.926 | CR | 0074 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SCAS updates to threats and assets for Release 17 features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Many features introduced in the previous release require updates to the existing threats: - For SNPN support, a similar authorization mechanism (as for raoming) was adopted for service authorization during primary authentication. This mechanism relies on the inclusion of the SNPN ID similarly to PLMN ID in the token. The threats must be updated accordingly to take into account this to guarantee that services are only consumed by authorized SNPNs - For the UP IP support in the eNB, similar threats related to the use of UP policy in the gNB must be introduced in order to complete the SCAS work for such eNB products |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition to the token verification threat to cover the SNPN case. Addition to the PLMN ID mismatch threat to cover the SNPN case. Addition of new threats to cover the use of the UP IP policy in eNBs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete SCAS work for the previous release features. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.1, C.2.2.x(new), C.2.2.y(new), C.2.2.z(new), G.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | SA3#107e: S3-221230, S3-221231 SA3#108e: S3-222384, S3-222385 SA3#109: S3-223494 |  |  |  |  |  |  |  |  |

********* Begin 1st change*********

### 6.3.3	Threats related to service access

#### 6.3.3.1	Elevation of privilege via incorrect verification of access tokens

-	Threat name: Incorrect Verification of Access Tokens.

-	Threat category: Elevation of Privilege, Information Disclosure, Denial of Service.

-	Threat Description: there are following threats if the generic NF cannot correctly verify the access tokens:

-	An access token may be tampered so that an attacker can arbitrarily access any services from any NF service providers within the same PLMN or in different PLMNs or SNPNs, which leads to elevation of privilege and consequently information disclosure.

-	An access token may be tampered so that an attacker can arbitrarily access the services of any slices provided by the NF producer instances (excluded from the list of NSSAIs or the list NSI IDs) within the same PLMN or in different PLMNs or SNPNs, which leads to elevation of privilege and consequently information disclosure.

-	An access token may be tampered so that an attacker can arbitrarily access the services provided by the NF producer instances outside the NF Set which it is allowed to access within the same PLMN or in different PLMNs or SNPNs, which leads to elevation of privilege and consequently information disclosure.

-	An access token may be tampered so that an attacker can arbitrarily access the disallowed resources or conduct disallowed actions on the resources for the services provided by a NF service provider within the same PLMN or in different PLMNs or SNPNs, which leads to elevation of privilege and consequently information disclosure.

-	An access token may be tampered so that an attacker can block service access by replacing the granted services/NF service providers with unavailable services/NF service providers, which leads to denial of service.

-	An expired access token can be replayed so that an attack can access the services which may no longer be allowed by the NF service provider, which leads to elevation of privilege and consequently information disclosure.

-	Threatened Asset: NF API data, NF Application, Sufficient processing capacity

NOTE x: This SNPNs authorization aspects only apply to UDMs, NRFs and AUSFs in Credentials Holders  Credentials Holder using AUSF and UDM for primary authentication.

********* End of 1st change*********

********* Begin 2nd change*********

##### C.2.2.x Local user plane integrity protection configuration

- Threat name: Local user plane integrity protection configuration

- Threat Category: Tampering data

- Threat Description: When an eNB supports user plane integrity protection, and if the eNB is not preconfigured with a user plane integrity protection policy, and if the UE supports user plane integrity protection, then the protection might be disabled exposing the user plane data to tampering attacks when the MME does not send a user plane integrity protection policy to the eNB.

- Threatened Asset: user plane data.

********* End of 2nd change*********

********* Begin 3rd change*********

##### C.2.2.y UP integrity protection policy selection

- Threat name: UP integrity protection policy selection

- Threat Category: Tampering data

- Threat Description: When a eNB supports user plane integrity protection, and if the UP integrity protection policy sent by MME does not take precedence over the locally configured one, then the user plane data protection may be disabled which does no longer comply with the service session requirements and exposes the user plane data to tampering attacks.

- Threatened Asset: user plane data.

********* End of 3rd change*********

********* Begin 4th change*********

### C.2.2.z	Bidding down on UP IP policy

- Threat name: Bidding down on UP IP policy.

- Threat Category: Tampering Data, Information Disclosure, Denial of Service.

- Threat Description: If the eNB does not send the UP IP policy, the MME cannot verify current UP IP policy is the same as the UP IP policy that the MME has stored, the attacker (e.g. source eNB) may force the system to accept a weaker UP IP policy (e.g. REQUIRED -> NOT NEEDED or none) than the system is allowed, or there may be case that source eNB is not upgraded to support UP IP feature thus UP IP policy from MME is lost, it will force the system into a lowered security level making the system easily attacked and/or compromised.

- Threatened Asset: Sufficient processing capability, Mobility Management data.

********* End of 4th change*********

********* Begin 5th change*********

## G.2.3	Threats related to error handling in the SEPP

### G.2.3.1		Incorrect handling for PLMN ID or SNPN ID mismatch

-	Threat name: Incorrect handling for PLMN ID or SNPN ID mismatch

-	Threat Category: Denial of Service, Information Disclosure, Spoofing Identity

-	Threat Description: there are following threats if the SEPP does not make correct handling when detecting that the PLMN-ID or SNPN ID contained in the incoming N32-f message does not match the PLMN-ID or SNPN ID in the related N32-f context:

-	Without receiving error signalling message from the SEPP which detected the mismatch, the peer SEPP is not aware of such error and will continue to send the messages with errors. This can result in waste of system resources.

-	If the SEPP sends an error signalling message without indicating the error cause and the corresponding N32-f message ID, the peer SEPP is not able to identify what error occurs and what is the source message on which the error occurs. Hence the peer SEPP is not able to take actions accordingly. This can result in service interruption as well as waste of system resources.

-	The serving PLMN ID or SNPN ID appended in the subject claim of the access token sent by a NF service consumer in the serving PLMN will not be checked by the NF service producer in the home PLMN. If the SEPP in the HPLMN detected the mismatch of serving PLMN ID or SNPN ID in the access token but still forwards the NF Service Request to the NF service producer, the serving PLMN ID or SNPN ID mismatch will not be detected by the NF service producer and the request will be wrongly accepted if all the other checks on the access token get passed. This can result in unauthorized service access by NF service consumer as well as waste of system resources.

-	Threatened Asset: Application layer security data, Sufficient Processing Capacity

NOTE x: SNPN ID mismatch only applies to SEPPs in a Credentials Holder using AUSF and UDM for primary authentication.

********* End of change*********