3GPP TSG-SA5 Meeting #149	S5-234809

# Berlin, Germany, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0255 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS28.532 Rel17 correction the Information Type for objectClass and objectInstance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, REST_SS |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Information Type for objectClass, objectInstance are not correct. They are either not defined by using “--”, or not use exact information type. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the Information Type for objectClass, objectInstance. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification leads to incorrect implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-234809 is revision of S5-233984 |  |  |  |  |  |  |  |  |

Start of Modified Section

# 11	Management services – Stage 2

## 11.1	Generic provisioning management service

### 11.1.0	Introduction

This clause provides the stage 2 definitions of Create, Read, Update and Delete (CRUD) operations for managing managed objects. According to clause 4.2.2 of TS 28.533 [13], these CRUD operations are the MnS component type A. The operations specified in this clause in combination with a NRM (MnS component type B) constitute a MnS, as defined in clause 4.3 of TS 28.533 [13].

In addition, notifications to report changes related to managed objects and their attributes are specified.

### 11.1.1	Operations and notifications

#### 11.1.1.1	createMOI operation

##### 11.1.1.1.1	Description

This operation is invoked by MnS consumer to request the MnS producer to create a Managed Object instance in the MIB maintained by MnS producer. This operation will create only one Managed Object instance.

The MnS consumer supplies the values of all attributes that are supported, i.e. a) attributes whose Support Qualifier is M and b) attributes whose Support Qualifier is O. The special cases are:

1)	If the attribute has a default value specified. In such case, if the MnS consumer supplies a value, the supplied value is used; otherwise, the default value is used.

2)	If the attribute is specified as capable of carrying a null value or carrying no information. In such case, if the Generic Provisioning MnS consumer supplies a (non-null) value, the supplied value is used; otherwise, the null value is used.

3)	If the attribute does not have a default value specified and is specified as incapable of carrying null value and incapable of carrying no information, if there is a MnS producer defined default value, then that value will be used.

##### 11.1.1.1.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| managedObjectClass | M | class | This parameter specifies the class of the new managed object instance. |
| managedObjectInstance | M | DN | This parameter specifies the instance of the managed object that is to be created and registered. This is a full DN according to 3GPP TS 32.300 [5]. |
| attributeListIn | M | LIST OF SEQUENCE< attribute name, attribute value> | This parameter may have a null value. When this parameter is supplied, it contains a list of name/value pairs specifying attribute identifiers and their values to be assigned to the new managed object. These values override the values for the corresponding attributes derived from the default value set specified in the definition of the managed object's class. |

##### 11.1.1.1.3	Output parameters

| Parameter name | S | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListOut | M | LIST OF SEQUENCE< attribute name, attribute value> | This list of name/value pairs contains the attributes of the new managed object and the actual value assigned to each. |
| status | M | ENUM (OperationSucceeded, OperationFailed) |  |

##### 11.1.1.1.4	Results

In case of success, the ManagedEntity instance has been created with the supplied DN. In case of failure, indication of the failure is provided in the Output parameters.

#### 11.1.1.2	getMOIAttributes operation

##### 11.1.1.2.1	Definition

This operation is invoked by MnS consumer to request the retrieval of management information (Managed Object attribute names and values) from the MIB maintained by MnS producer. One or several Managed Objects may be retrieved - based on the containment hierarchy.

A SS may choose to split this operation in several operations (e.g. operations to get "handlers" or "iterators" to Managed Objects fulfilling the scope/filter criteria and other operations to retrieve attribute names/values from these "handlers").

##### 11.1.1.2.2	Input Parameters

| Name | S | Information Type | Comment |
| --- | --- | --- | --- |
| baseObjectInstance | M | DN | This parameter specifies the base object instance. If the "scope" parameter is absent, then either only the base object or the complete subtree below and including the base object shall be selected. The default behaviour is protocol specific. |
| scope | M | n/a | This parameter specifies the scope. It is a structured parameter and consists of the sub-parameters "scopeType" and "scopeLevel". The scope describes which object instances are selected with respect to a base object instance. The base object instance needs to be specified using a dedicated attribute. |
| > scopeType | M | ENUM { BASE_ONLY, BASE_ALL } | If the optional "scopeLevel" parameter is not supported or absent, allowed values of "scopeType" are "BASE_ONLY" and "BASE_ALL". The value "BASE_ONLY" indicates only the base object is selected. The value "BASE_ALL" indicates the base object and all of its subordinate objects (incl. the leaf objects) are selected. This parameter is redundant and can be omitted when confirming only the protocol specific default behaviour. |
|  |  | ENUM { BASE_NTH_LEVEL, BASE_SUBTREE } | If the "scopeLevel" parameter is supported and present, allowed values of "scopeType" are "BASE_NTH_LEVEL" and "BASE_SUBTREE". The value "BASE_NTH_LEVEL" indicates all objects on the level, which is specified by the "scopeLevel" parameter, below the base object are selected. The base object is at "scopeLevel" zero. The value "BASE_SUBTREE" indicates the base object and all of its subordinate objects down to and including the objects on the level, which is specified by the "scopeLevel" parameter, are selected. The base object is at "scopeLevel" zero. |
| > scopeLevel | O | Integer | See definition of "scopeType" parameter. |
| filter | O | See Comment. | This parameter defines filter criteria to be applied to the objects selected by the "baseObjectInstance", "scope" and "scopeLevel" parameters. The actual syntax and capabilities of the filter is SS specific. However, each SS should support a filter consisting of one or several assertions that may be grouped using the logical operators AND, OR and NOT. Each assertion is a logical expression of attribute existence, attribute value comparison ("equal to X, less than Y" etc.) and MO Class. |
| attributeListIn | O | LIST OF attribute name. | This parameter identifies the attributes to be returned by this operation. If the parmeter is absent or empty all attributes shall be returned. |

##### 11.1.1.2.3	Output Parameters

| Name | S | Matching Information | Comment |
| --- | --- | --- | --- |
| managedObjectClass | M | ManagedEntity class | For each returned MO: The class of the MO. |
| managedObjectInstance | M | ManagedEntity DN | For each returned MO: The name of the MO. This is a full DN according to 3GPP TS 32.300 [5]. |
| attributeListOut | M | LIST OF SEQUENCE< attribute name, attribute value > | For each returned MO: A list of name/value pairs for MO. |
| status | M | ENUM (OperationSucceeded, OperationFailed) | An operation may fail because of a specified or unspecified reason. |

##### 11.1.1.2.4	Results

In case of success, all of the ManagedEntity instances selected for retrieval are returned. In case of failure, a specified or unspecified reason may be provided in the Output parameters.

#### 11.1.1.3	modifyMOIAttributes operation

##### 11.1.1.3.1	Description

This operation is invoked by MnS consumer to request the modification of one or more Managed Object instances from MnS producer. Attributes of one or several Managed Objects may be modified.

##### 11.1.1.3.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| baseObjectInstance | M | DN | The MO instance that is to be used as the starting point for the selection of managed objects to which the filter (when supplied) is to be applied. This is a full DN according to 3GPP TS 32.300 [5]. |
| scopeType | M | See corresponding parameter in getMOIAttributes. | See corresponding parameter in getMOIAttributes. |
| scopeLevel | O | See corresponding parameter in getMOIAttributes. | See corresponding parameter in getMOIAttributes. |
| filter | O | See comment. | See corresponding parameter in getMOIAttributes. |
| modificationList | M | LIST OF SEQUENCE <attribute identifier, [attribute values], ENUM( replace, add values, remove values, set to default)> See Comment for when attribute values are require and when they are optional. | This parameter contains a set of attribute modification specifications, each of which contains: 1). attribute identifier: the identifier of the attribute whose value(s) is (are) to be modified. 2). attribute value: the value(s) to be used in the modification of the attribute. The use of this parameter is defined by the modify operator. This parameter is optional when the set to default modify operator is specified and if supplied, shall be ignored. 3). modify operator: the way in which the attribute values(s) (if supplied) is(are) to be applied to the attribute. The possible operators are: a) replace: the attribute value(s) specified shall be used to replace the current values(s) of the attribute; b) add values: the attribute values(s) specified shall be added to the current value(s) of the attribute. This operator shall only be applied to a set-valued attribute and shall perform a set union (in the mathematical sense) between the current values(s) of the attribute and the attribute value(s) specified. Value(s) specified in the attribute value parameter which is(are) already in the current values of the attribute shall not cause an error to be returned. c) remove values: the attribute value(s) specified shall be removed from the current values(s) of the attribute. This operator shall only be applied to a set-valued attribute and shall perform a set difference (in the mathematical sense) between the current value(s) of the attribute and the attribute values(s) specified. Value(s) specified in the attribute value parameter which is(are) not in the current value(s) of the attribute shall not cause an error to be returned; d) set to default: when this operator is applied to a single-valued attribute, the value of the attribute shall be set to its default value. When this operator is applied to a set–valued attribute, the value(s) of the attribute shall be set to their default value(s) and only as many values as defined by the default shall be assigned. If there is no default value defined, an error shall be returned. Note: Set is used here in the mathematical sense so that a set-valued attribute is an unordered set of unique values. The modify operator is optional, and if it is not specified, the replace operator shall be assumed. The modificationList parameter contains a single set of attribute modification specifications and this same set is applied to each MO instance to be modified. |

##### 11.1.1.3.3	Output parameters

| Parameter name | S | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| modificationListOut | M | LIST OF SEQUENCE< ManagedEntity DN, ManagedEntity class, LIST OF SEQUENCE< attribute name, attribute value >> | This parameter will provide for each managed object instance the full DN of the managed object instance, the managedObjectClass, and a list of name/value pairs with the values of all the attributes of the modified managed object instance after modification. The form of this information is SS dependant and may be provided in one or many data structures. |
| status | M | ENUM (OperationSucceeded, OperationFailed, OperationPartiallySucceeded) | An operation may fail because of a specified or unspecified reason and no attributes have been updated. The operation is only successful if all specified attributes of all selected objects are actually modified. Otherwise, the operation is partially successful. |

In lieu of a synchronization parameter, best effort synchronization will apply; that is, all managed objects selected for this operation will perform the operation if possible regardless of whether some managed objects fail to perform it.

##### 11.1.1.3.4	Results

In case of success, all of the ManagedEntity instances selected for modification are modified. In case of failure, a specified or unspecified reason may be provided in the Output parameters.

#### 11.1.1.4	deleteMOI operation

##### 11.1.1.4.1	Description

This operation is invoked by MnS consumer to request the deletion of one or more Managed Object instances in the MIB maintained by the MnS producer.

##### 11.1.1.4.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| baseObjectInstance | M | DN | The MO instance that is to be used as the starting point for the selection of managed objects to which the filter (when supplied) is to be applied. This is a full DN according to 3GPP TS 32.300 [5]. |
| scopeType | O | See corresponding parameter in getMOIAttributes. | See corresponding parameter in getMOIAttributes. |
| scopeLevel | O | See corresponding parameter in getMOIAttributes. | See corresponding parameter in getMOIAttributes. |
| filter | O | See comment. | See corresponding parameter in getMOIAttributes. |

##### 11.1.1.4.3	Output parameters

| Parameter name | S | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| deletionList | M | LIST OF SEQUENCE< ManagedEntity DN, ManagedEntity class name> | If the base object alone is specified, then this parameter is optional; otherwise it contains a list of managedObjectInstance/managedObjectClass pairs identifying the managed objects deleted. |
| status | M | ENUM (OperationSucceeded, OperationFailed, OperationPartiallySucceeded) | An operation may fail because of a specified or unspecified reason. The operation is partially successful if some, but not all, objects selected to be deleted are actually deleted. |

In lieu of a synchronization parameter, best effort synchronization will apply; that is, all managed objects selected for this operation will perform the operation if possible regardless of whether some managed objects fail to perform it.

##### 11.1.1.4.4	Results

In case of success, all of the ManagedEntity instances selected for deletion are deleted. In case of failure, a specified or unspecified reason may be provided in the Output parameters.

#### 11.1.1.5	Void

#### 11.1.1.6	Void

#### 11.1.1.7	Notification notifyMOICreation

##### 11.1.1.7.1	Definition

This notification notifies the subscribed consumers that a new Managed Object Instance has been created.

##### 11.1.1.7.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | ManagedEntity.objectClass | It specifies the class name of the IOC. A network event has occurred in an instance of this class. |
| objectInstance | M | ManagedEntity.objectInstance | It specifies a new instance of the above IOC in which the network event related to by carrying the Distinguished Name (DN) for the instance. |
| notificationId | M | This is an identifier for the notification, which may be used to correlate notifications. | The identifier of the notification shall be chosen to be unique across all notifications of a particular managed object instance throughout the time that correlation is significant, it uniquely identifies the notification from other notifications generated by the subject MOI. |
| notificationType | M | It specifies the type of provisioning management services related notifications. The value “notifyMOICreation” shall be carried. | It specifies the type of notification. |
| eventTime | M | It indicates the MOICreation event time. | The semantics of Generalised Time specified by ITU-T[17] shall be used here. |
| systemDN | M | It shall carry the DN of management service providers. | - |
| correlatedNotifications | CM | It specifies a set of notifications that are correlated to the subject notification. | The condition is that the MnS producer support the correlation of notifications |
| additionalText | O | It can contain further information in text on the event of the ManagedEntity(s). | - |
| sourceIndicator | O | ENUM( Resource_operation, Management_operation, SON_operation,Unknown) | This parameter, when present, indicates the source of the operation that led to the generation of this notification. It can have one of the following values: 1. resource operation: The notification was generated in response to an internal operation of the resource; 2. management operation: The notification was generated in response to a management operation applied across the managed object boundary external to the managed object; 3. SON operation: The notification was generated as result of a SON (Self Organising Network) process like self-configuration, self-optimization, self-healing etc. . 4. unknown: It is not possible to determine the source of the operation. Remark: A provisioning MnS provider may not in any case be aware that SON operation lead to the generation of this generation. In this case another value than SON_operation for sourceIndicator might be sent. |
| attributeList | O | LIST OF SEQUENCE <AttributeName, AttributeValue> | The attributes (name/value pairs) of the created MOI. |

##### 11.1.1.7.3	Triggering event

###### 11.1.1.7.3.1	From-state

stateBeforeObjectCreation.

| Assertion Name | Definition |
| --- | --- |
| stateBeforeObjectCreation | The number of instances of the IOC ManagedEntity is equal to N. |

###### 11.1.1.7.3.2	To-state

stateAfterObjectCreation.

| Assertion Name | Definition |
| --- | --- |
| stateAfterObjectCreation | The number of instances of the IOC ManagedEntity is equal to N + 1. |

#### 11.1.1.8	Notification notifyMOIDeletion

##### 11.1.1.8.1	Definition

This notification notifies the subscribed consumers that an existing Managed Object Instance has been deleted.

##### 11.1.1.8.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | ManagedEntity.objectClass | It specifies the class name of the IOC. A network event has occurred in an instance of this class. |
| objectInstance | M | ManagedEntity.objectInstance | It specifies an existing instance of the above IOC in which the network event related to by carrying the Distinguished Name (DN) for the instance. |
| notificationId | M | This is an identifier for the notification, which may be used to correlate notifications. | The identifier of the notification shall be chosen to be unique across all notifications of a particular managed object throughout the time that correlation is significant, it uniquely identifies the notification from other notifications generated by the subject MOI. |
| notificationType | M | It specifies the type of provisioning management services related notifications. The value “notifyMOIDeletion” shall be carried. | It specifies the type of notification. |
| eventTime | M | It indicates the MOIDeletion event time. | The semantics of Generalised Time specified by ITU-T[17] shall be used here. |
| systemDN | M | It shall carry the DN of management service providers. | - |
| correlatedNotifications | CM | It specifies a set of notifications that are correlated to the subject notification. | The condition is that the MnS producer support the correlation of notifications |
| additionalText | O | It can contain further information in text on the event of the ManagedEntity(s). | - |
| sourceIndicator | O | ENUM( Resource_operation, Management_operation, SON_operation,Unknown) | This parameter, when present, indicates the source of the operation that led to the generation of this notification. It can have one of the following values: 1. resource operation: The notification was generated in response to an internal operation of the resource; 2. management operation: The notification was generated in response to a management operation applied across the managed object boundary external to the managed object; 3. SON operation: The notification was generated as result of a SON (Self Organising Network) process like self-configuration, self-optimization, self-healing etc. . 4. unknown: It is not possible to determine the source of the operation. Remark: A provisioning MnS provider may not in any case be aware that SON operation lead to the generation of this generation. In this case another value than SON_operation for sourceIndicator might be sent. |
| attributeList | O | LIST OF SEQUENCE <AttributeName, AttributeValue> | The attributes (name/value pairs) of the deleted MOI. |

##### 11.1.1.8.3	Triggering event

###### 11.1.1.8.3.1	From-state

stateBeforeObjectDeletion.

| Assertion Name | Definition |
| --- | --- |
| stateBeforeObjectDeletion | The number of instances of the IOC ManagedEntity is equal to N. |

###### 11.1.1.8.3.2	To-state

stateAfterObjectDeletion.

| Assertion Name | Definition |
| --- | --- |
| stateAfterObjectDeletion | The number of instances of the IOC ManagedEntity is equal to N - 1. |

#### 11.1.1.9	Notification notifyMOIAttributeValueChanges

##### 11.1.1.9.1	Definition

This notification notifies the subscribed MnS consumers that changes of one or several attributes of a Managed Object Instance in the NRM.

##### 11.1.1.9.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | ManagedEntity.objectClass | It specifies the class name of the IOC. A network event has occurred in an instance of this class. |
| objectInstance | M | ManagedEntity.objectInstance | It specifies the existing instance of the above IOC in which the network event related to by carrying the Distinguished Name (DN) for the instance. |
| notificationId | M | This is an identifier for the notification, which may be used to correlate notifications. | The identifier of the notification shall be chosen to be unique across all notifications of a particular managed object throughout the time that correlation is significant, it uniquely identifies the notification from other notifications generated by the subject Information Object. |
| notificationType | M | It specifies the type of provisioning management services related notifications. The value “notifyMOIAttributeValueChange” shall be carried. | It specifies the type of notification. |
| eventTime | M | It indicates the MOIAttributeValueChange event time. | The semantics of Generalised Time specified by ITU-T[17] shall be used here. |
| systemDN | M | It shall carry the DN of management service providers. | - |
| correlatedNotifications | CM | It specifies a set of notifications that are correlated to the subject notification. | The condition is that the MnS producer support the correlation of notifications |
| additionalText | O | It can contain further information in text on the event of the ManagedEntity(s). | - |
| sourceIndicator | O | ENUM( Resource_operation, Management_operation, SON_operation,Unknown) | This parameter, when present, indicates the source of the operation that led to the generation of this notification. It can have one of the following values: 1. resource operation: The notification was generated in response to an internal operation of the resource; 2. management operation: The notification was generated in response to a management operation applied across the managed object boundary external to the managed object; 3. SON operation: The notification was generated as result of a SON (Self Organising Network) process like self-configuration, self-optimization, self-healing etc. . 4. unknown: It is not possible to determine the source of the operation. Remark: A provisioning MnS provider may not in any case be aware that SON operation lead to the generation of this generation. In this case another value than SON_operation for sourceIndicator might be sent. |
| attributeValueChange | M | LIST OF SEQUENCE <AttributeName, NewAttributeValue, CHOICE [NULL, OldAttributeValue]> | The changed attributes (name/value pairs) of the MOI (with both new and, optionally, old values). |

##### 11.1.1.9.3	Triggering event

###### 11.1.1.9.3.1	From-state

stateBeforeAttributeValueChange.

| Assertion Name | Definition |
| --- | --- |
| stateBeforeAttributeValueChange | The subject attribute has a value at time T1. |

###### 11.1.1.9.3.2	To-state

stateAfterAttributeValueChange.

| Assertion Name | Definition |  |
| --- | --- | --- |
| stateAfterAttributeValueChange |  | The subject attribute has been changed to a value other than the value at time T1. |

#### 11.1.1.10	Notification notifyEvent

##### 11.1.1.10.1	Definition

This notification notifies the MnS consumer, who has a subscription receiving this type of notification, that certain network events has occurred with potential service impact, for example, system restart and system redundancy shift (backup).

This notification definition is generic in the sense that the specific types of network event are not defined.

##### 11.1.1.10.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | ManagedEntity.objectClass | -- |
| objectInstance | M | ManagedEntity.objectInstance | -- |
| notificationId | M | It carries the identifier for the subject notification. | See Note 1. |
| eventTime | M | It indicates the time of the event. | The semantics of Generalised Time specified by ITU-T [17] shall be used here. |
| systemDN | M | It carries the DN of producer of the notification. | -- |
| notificationType | M | "notifyEvent" | -- |
| specificProblem | M | It indicates a problem detected. | -- |
| additionalText | O | It carries additional information. | -- |
| additionalInformation | O | It carries additional information. | -- |
| NOTE 1: If consumer receives notifications from one producer, consumer can use the notificationId and the objectInstance to uniquely identify all received notifications. If consumer receives notifications from multiple producers and notifications of each objectInstance are reported to at most by one producer, consumer can use the notificationId and objectInstance to uniquely identify all received notifications. If consumer receives notifications from multiple producers and notifications of one or more objectInstance(s) are reported byo two or more producers, consumer can use the notificationId together with objectInstance and the identity of producer (systemDN), to uniquely identify all received notifications. If the information systemDN is absent, consumer needs other means, which are outside the scope of this TS, to determine the identity of producer. How notificationId of notifications are re-used to correlate notifications is outside of the scope of this specification. |  |  |  |

#### 11.1.1.11	Notification notifyMOIChanges

##### 11.1.1.11.1	Definition

This notification reports NRM updates to subscribed MnS consumers. It can report multiple NRM updates that happen at the same time. All possible NRM updates can be reported:

-	Creation and deletion of an object.

-	Creation and deletion of an attribute, attribute field, attribute element and attribute field element.

-	Replacement of an attribute value, attribute field value, attribute element and attribute field element.

The MnS producer decides whether to send notifications of type notifyMOICreation, notifyMOIDeletion or notifyMOIAttributesValueChange, or a single notifyMOIChanges reporting all changes in a single notification. The MnS producer should take subscription information into account when deciding the notification types to be sent, and not try to send notifications that the MnS consumer did not subscribe to.

The notification header includes a notificationId. This identifier shall not be used in the parameter correlatedNotifications potentially carried in other notifications. The notificationId in mOIChanges shall be used instead. This is because the latter notification id is associated to a single MOI only, whereas the former notification id can be associated to changes of multiple MOIs. The correlatedNotifications associates to a single MOI one or more notification ids identifying notifications reporting events for that MOI.

In this clause the following definitions apply.

data type: Constraint on an attribute value.

simple type: Data type constraining an attribute value to a scalar.

complex type: Data type of a structured and/or multi-valued attribute.

attribute: Information element of an object composed of an attribute name and an attribute value.

attribute name: Name of an attribute.

attribute value: Value of an attribute. The value is defined by a simple type or a complex type.

attribute field: Attribute contained in an attribute. Attribute fields can contain attribute fields.

attribute field name: Name of an attribute field.

attribute field value: Value of an attribute field. The value is defined by a simple type or a complex type.

simple attribute: Attribute whose value is a simple type.

complex attribute: Attribute whose value is a complex type.

structured attribute: Attribute whose value contains one or more attribute fields. A structured attribute is a kind of a complex attribute.

multi-valued attribute: Attribute with multiplicity > 1. A multi-valued attribute is a kind of a complex attribute.

attribute element: A single value of a multi-valued attribute.

attribute field element: A single value of a multi-valued attribute field.

##### 11.1.1.11.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | See clause 11.1.1.7.2 | Identifies the classe name of a common ancestor object of the objects for which changes are reported. A MnS producer may set this parameter always to the class name of the parent of the local root object in the MIB. |
| objectInstance | M | See clause 11.1.1.7.2 | Identifies the instance of a common ancestor object of the objects for which changes are reported. A MnS producer may set this parameter always to the instance of the parent of the local root object in the MIB. |
| notificationId | M | See clause 11.1.1.7.2 | See clause 11.1.1.7.2 |
| notificationType | M | const string “notifyMOIChanges” | See clause 11.1.1.7.2 |
| eventTime | M | See clause 11.1.1.7.2 | See clause 11.1.1.7.2 |
| systemDN | M | See clause 11.1.1.7.2 | See clause 11.1.1.7.2 |
| moiChanges | M | SEQUENCE OF SET { notificationId (M), correlatedNotifications (O), additionalText (O), sourceIndicator (O), op (M), path (M), value (M) , oldValue (O) } | This parameter describes the reported NRM updates. It is a list of items; each item reports a single NRM update. The "notificationId" identifies an item. The NRM update itself is described by the parameters "op", "path", "value" and "oldValue". The parameters "correlatedNotifications", "additionalText" and "sourceIndicator " provide context information. The parameter "op" specifies the type of operation reporting the NRM update. Valid values are "add", "remove" and "replace". The operation describes what has conceptually happened to the NRM on the MnS producer. The operation applied to the NRM by the MnS producer and causing the reported NRM update can be different. "add" shall be used for reporting the creation of an object, attribute, attribute field or multi-value attribute element. "remove" shall be used for reporting the deletion of an object, attribute, attribute field or multi-value attribute element. "replace" shall be used for reporting the replacement of an existing attribute value, attribute field value or multi-value attribute element. The "path" and "objectInstance" identify the object, attribute, attribute field or multi-value attribute element, that was created, deleted or replaced. If an object creation is reported with "add", the "value" shall carry a complete representation of the created object. If an object deletion is reported with "remove", the "value" shall be absent. It may optionally carry a complete representation of the deleted object. If an attribute, attribute field or multi-value attribute element creation is reported with "add", the "value" shall carry the value of the created attribute, attribute field or multi-value attribute element. If an attribute, attribute field or multi-value attribute element deletion is reported with "remove", the "value" shall be absent. It may optionally carry the old value of the deleted attribute, attribute field or multi-value attribute element. If the replacement of an attribute, attribute field or multi-value attribute element value is reported with "replace", the "value" shall carry the new value of the attribute, attribute field or multi-value attribute element. The "oldValue" may optionally carry the old value of the attribute, attribute field or multi-value attribute element before the replacement. If multiple objects are created, the creation of parent objects shall be reported before the creation of the child objects. Vice versa, when the deletion of multiple objects is reported, the deletion of child objects shall be reported before the deletion of the parent objects. |

### 11.1.2	Managed Information

#### 11.1.2.1	ManagedEntity << ProxyClass>>

##### 11.1.2.1.1	Definition

The ProxyClass ManagedEntity represents the role that can be played by an instance of an IOC defined in NRMs, e.g. Generic NRM, NR and NG-RAN NRM, or 5GC NRM. ManagedEntity is used in the specification of provisioning operations and notifications to represent an instance of an IOC defined in these NRMs.

## 11.2	Generic fault supervision management service

### 11.2.1	Operations and notifications

#### 11.2.1.1	Fault supervision data report

##### 11.2.1.1.1	subscribe

###### 11.2.1.1.1.1	Definition

A MnS consumer invokes this operation to establish subscription to receive network events via notifications, under the filter constraint specified in this operation.

###### 11.2.1.1.1.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| consumerReference | M | NtfSubscriber.ntfManagerReference | It specifies the reference of the authorized MnS consumer to which notifications shall be sent. |
| timeTick | O | NtfSubscription.ntfTimeTick | It specifies the value of a timer held for the subject management service consumer. The value is in unit of whole minute. A special infinite value is assumed when parameter is absent or present but equal to zero. |
| filter | O | This attribute represents the filter of a subscription. | It specifies a filter constraint that MnS producer shall use to filter notification of the alarms. If this parameter is absent, then no filter constraint shall be applied. |

###### 11.2.1.1.1.3	Output parameters

| Parameter Name | S | Matching Information / Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| subscriptionId | M | NtfSubscription.ntfSubscriptionId. | It holds an unambiguous identity of this subscription. |
| status | M | ENUM (OperationSucceeded, OperationFailedExistingSubscription, OperationFailed) | If subscriptionCreated is true, status = OperationSuceeded. If operation_failed_existing_subscription is true, status = OperationFailedExistingSubscription If operation_failed is true, status = OperationFailed. |

###### 11.2.1.1.1.4	Pre-condition

notificationCategoriesNotAllSubscribed OR notificationCategoriesParameterAbsentAndNotAllSubscribed.

| Assertion Name | Definition |
| --- | --- |
| notificationCategoriesNotAllSubscribed | At least one notificationCategory identified in the notificationCategories input parameter is supported by management service producer and is not a member of the ntfNotificationCategorySet attribute of an NtfSubscription which is involved in a subscription relationship with the NtfSubscriber identified by the managerReference input parameter. |
| notificationCategoriesParameterAbsentAndNotAllSubscribed | The notificationCategories input parameter is absent and at least one notificationCategory supported by management service producer is not a member of the ntfNotificationCategorySet attribute of an ntfSsubscription which is involved in a subscription relationship with the NtfSubscriber identified by the managerReference input parameter. |

###### 11.2.1.1.1.5	Post-condition

subscriberPossiblyCreated AND subscriptionCreated.

| Assertion Name | Definition |
| --- | --- |
| subscriberPossiblyCreated | An NtfSubscriber with an ntfManagerReference attribute equal to the value of the managerReference input parameter is involved in a subscriptionRegistration relationship. |
| subscriptionCreated | An NtfSubscription has been created according to the following rules: - ntfSubscriptionState attribute value has been set to "notSuspended"; - ntfTimeTick attribute value has been set to the value of the timeTick input parameter if This value was higher or equal to 15, or set to 15 if this parameter value was between 1 and 15, or set to a special infinite value if the parameter value was lower or equal to 0 or if parameter was absent; - ntfTimeTickTimer has been reset with the value of timeTick attribute; - ntfFilter attribute value has been set to the value of the filter input parameter if present; - NtfSubscription is involved in a subscription relationship with the NtfSubscriber identified by the managerReference input parameter; - attribute ntfNotificationCategorySet of NtfSubscription contains EITHER the notification categories identified by the notificationCategories input parameter that were not already contained in the ntfNotificationCategorySet attribute of other NtfSubscription of the same NtfSubscriber identified by the managerReference input parameter OR if notificationCategories input parameter is absent, all notification categories supported by management service producer that were not already contained in the ntfNotificationCategorySet attribute of other subscriptions of the same NtfSubscriber identified by the managerReference input parameter. |

###### 11.2.1.1.1.6	Exceptions

| Name | Definition |
| --- | --- |
| operation_failed_existing_subscription | Condition: (notificationCategoriesNotAllSubscribed OR notificationCategoriesParameterAbsentAndNotAllSubscribed) not true Returned Information: The output parameter status Exit state: Entry State |
| operation_failed | Condition: Post-condition is false Returned Information: The output parameter status Exit state: Entry State |

##### 11.2.1.1.2	unsubscribe

###### 11.2.1.1.2.1	Definition

A MnS consumer invokes this operation to cancel subscriptions. The MnS consumer can cancel one subscription made with a consumerReference by providing the corresponding subscriptionId or all subscriptions made with the same consumerReference by leaving the subscriptionId parameter absent.

###### 11.2.1.1.2.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| consumerReference | M | DN | It specifies the reference of the MnS consumer to which notifications shall be sent. |
| subscriptionId | O | A unique identifier that is SS dependent. | It holds a subscriptionId carried as the output parameter in the subscribe operation. |

###### 11.2.1.1.2.3	Output parameters

| Parameter Name | S | Matching Information / Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| status | M | ENUM (OperationSucceeded, OperationFailed) | If (subscriptionDeleted OR allSubscriptionDeleted) is true, status = OperationSucceeded. If operation_failed is true, status = OperationFailed. |

###### 11.2.1.1.2.4	Pre-condition

validSubscriptionId&ManagerReference OR SubscriptionIdAbsent&ValidManagerReference.

| Assertion Name | Definition |
| --- | --- |
| validSubscriptionId&ManagerReference | The NtfSubscription identified by subscriptionId input parameter is involved in a subscription relationship with the NtfSubscriber identified by the managerReference input parameter. |
| subscriptionIdAbsent&ValidManagerReference | The subscriptionId input parameter is absent and the NtfSubscriber identified by the managerReference input parameter exists. |

###### 11.2.1.1.2.5	Post-condition

subscriptionDeleted OR allSubscriptionDeleted.

| Assertion Name | Definition |
| --- | --- |
| subscriptionDeleted | The NtfSubscription identified by subscriptionId input parameter is no more involved in a subscription relationship with the NtfSubscriber identified by the managerReference input parameter and has been deleted. If this NtfSubscriber has no more NtfSubscription, it is deleted as well. |
| allSubscriptionDeleted | "In the case subscriptionId input parameter was absent, the NtfSubscriber identified by the managerReference input parameter is no more involved in any subscription relationship and is deleted, the corresponding NtfSubscription have been deleted as well. |

###### 11.2.1.1.2.6	Exceptions

| Name | Definition |
| --- | --- |
| operation_failed | Condition: Pre-condition is false or post-condition is false Returned Information: The output parameter status Exit state: Entry State |

##### 11.2.1.1.3	getAlarmList

###### 11.2.1.1.3.1	Definition

A MnS consumer invokes this operation to request the MnS producer to provide either the complete list of AlarmInformation instances in the AlarmList or only a part of this list (partial alarm alignment).

The parameters baseObjectClass and baseObjectInstance are used to identify the part of the alarm list to be returned. If they are absent, then the complete alarm list shall be provided (full alarm alignment). If they identify a particular class instance, then only a) the AlarmInformation instances related to this class instance and b) the AlarmInformation instances related to the subordinate class instances of this class instance shall be provided (partial alarm alignment). An instance-a is said to be subordinate to instance-b if the DN of the latter is part of the DN of the former.

There are two modes of operation. One mode is synchronous. In this mode, the list of AlarmInformation instances in AlarmList is returned synchronously with the operation. The other mode is asynchronous. In this mode, the list of AlarmInformation instances is returned via alarm notifications. In asynchronous mode of operation, the only information returned synchronously is the status of the operation. A method allowing to abort an ongoing alarm alignment process shall be available in the asynchronous mode. The mode of operation to be used is determined by means outside the scope of specification. To use asynchronous mode, the authorized consumer needs to have established a subscription via the subscribe operation.

###### 11.2.1.1.3.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| alarmAckState | O | ENUM (all alarms, all active alarms, all active and acknowledged alarms, all active and unacknowledged, all Cleared and unacknowledged alarms, all unacknowledged) | It carries a constraint. The FaultSupervision MnS producer shall apply it on AlarmInformation instances in AlarmList when constructing its output parameter AlarmInformationList. |
| baseObjectClass | O, see note 1 | This parameter is either absent or carries the object class of a certain class. | See how this attribute is used to support full alarm alignment and partial alarm alignment in 11.2.1.1.3.1. See note 2. |
| baseObjectInstance | O, see note 1 | This parameter is either absent or carries the DN of a certain class instance. | See how this attribute is used to support full alarm alignment and partial alarm alignment in 11.2.1.1.3.1. See note 2. |
| filter | O | N/A | It carries a filter constraint. If the filter is present, the MnS producer shall apply it on AlarmInformation instances in AlarmList when constructing its output parameter AlarmInformationList. If the filter is not present, all of the AlarmInformation instances included by the scope are selected. |
| NOTE 1: If the notification notifyAlarmListRebuilt supports indicating that only a part of the alarm list has been rebuilt then the operation getAlarmList shall support partial alarm alignment. NOTE 2: The legal values of the parameters baseObjectClass and baseObjectInstance are restricted to those carried by the parameters baseObjectClass and baseObjectInstance in the recent notifyAlarmListRebuilt notifications. The timeline for "recent" is vendor-specific. |  |  |  |

###### 11.2.1.1.3.3	Output parameters

Table 11.2.1.1.3.3-1: Output parameters for the operation getAlarmList

| Parameter Name | S | Matching Information / Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| alarmInformationList | M | List of AlarmInformation. | It carries the requested AlarmInformation instances. Case when synchronous mode of operation is used: (a) The MnS producer shall apply the constraints expressed in alarmAckState and filter to AlarmInformation instances when constructing this output parameter. Case when asynchronous mode of operation is used (i.e. this output parameter is conveyed via notifications): (a) If the filter parameter is present, theMnS producer shall apply the constraint when constructing this output parameter. Furthermore, if the alarmAckState constraint is present, the MnS producer shall apply that constraint as well. The filter constraint, if any, that is currently active in the notification channel is not used for the construction of this output parameter. (b) If the filter parameter is absent, the MnS producer shall apply the filter constraint currently active in the notification channel when constructing this output parameter. If the alarmAckState constraint is present, the MnS producer shall apply that constraint as well. |
| status | M | ENUM (OperationSucceeded, OperationFailed) | If all the AlarmInformation are returned, status = OperationSucceeded. If operation is failed, status = OperationFailed. |

The following table defines an item of alarmInformationList.

Table 11.2.1.1.3.3-2: Definition of an item of alarmInformationList

| Parameter name | S | Matching information | Comment |
| --- | --- | --- | --- |
| objectClass, objectInstance | M | MonitoredEntity.objectClass, MonitoredEntity.objectInstance | The MonitoredEntity is identified by the relation-AlarmedObject-AlarmInformation. |
| notificationId | M | AlarmInformation.notificationId |  |
| notificationType | M | "notifyNewAlarm" or "notifyChangedAlarm" or "notifyClearedAlarm" | The parameter carries - notifyNewAlarm in case the alarm has not yet changed and has not yet been cleared. - notifyChangedAlarm in case the alarm has changed but has not yet been cleared. - notifyClearedAlarm in case the alarm has been cleared but not yet acknowledged. |
| eventTime | O | AlarmInformation.alarmRaisedTime or AlarmInformation.alarmChangedTime or AlarmInformation.alarmClearedTime | The parameter carries the - alarmRaisedTime in case notificationType carries notifyNewAlarm - alarmChangedTime in case notificationType carries notifyChangedAlarm - alarmClearedTime in case notificationType carries notifyClearedAlarm |
| systemDN | M | -- |  |
| alarmId | M | AlarmInformation.alarmId |  |
| [objectClass], [objectInstance] | n/a | MonitoredEntity.objectClass, MonitoredEntity.objectInstance | Parmeter identical to the first parameter in this list, shown here to clarify all elements of AlarmInformation are present |
| [notificationId] | n/a | AlarmInformation.notificationId | Parmeter identical to the second parameter in this list, shown here to clarify all elements of AlarmInformation are present |
| alarmRaisedTime | M | AlarmInformation.alarmRaisedTime |  |
| alarmChangedTime | O | AlarmInformation.alarmChangedTime | not applicable if the severity of related alarm was not changed |
| alarmClearedTime | M | AlarmInformation.alarmClearedTime | not applicable if related alarm was not cleared |
| alarmType | M | AlarmInformation.alarmType |  |
| probableCause | M | AlarmInformation.probableCause |  |
| specificProblem | O | AlarmInformation.specificProblem |  |
| perceivedSeverity | M | AlarmInformation.perceivedSeverity |  |
| backedUpStatus | O | AlarmInformation.backedUpStatus | not applicable if related alarm is a security alarm |
| backUpObject | O | MonitoredEntity.objectInstance | The MonitoredEntity is identified by relation-BackUpObject-AlarmInformation. Not applicable if related alarm is a security alarm |
| trendIndication | O | AlarmInformation.trendIndication | not applicable if related alarm is a security alarm |
| thresholdInfo | O | AlarmInformation.thresholdInfo | not applicable if related alarm is a security alarm |
| correlatedNotifications | O | The set of CorrelatedNotification instances related to this AlarmInformation. |  |
| stateChangeDefinition | O | AlarmInformation.stateChange | not applicable if related alarm is a security alarm |
| monitoredAttributes | O | AlarmInformation.monitoredAttributes | not applicable if related alarm is a security alarm |
| proposedRepairActions | O | AlarmInformation.proposedRepairActions | not applicable if related alarm is a security alarm |
| additionalText | O | AlarmInformation.additionalText |  |
| additionalInformation | O | AlarmInformation.additionalInformation |  |
| rootCauseIndicator | O | AlarmInformation.rootCauseIndicator |  |
| ackTime | M | AlarmInformation.ackTime | not applicable if related alarm was not acknowledged nor unacknowledged The availability and accuracy of time carried by the time parameters in individual entries of the list (i.e. eventTime, alarmRaisedTime, alarmClearedTime and ackTime) shall be "best effort". Reason: A Management System is not required to persistently store these times or other alarm information (as in case of synchronization information may be provided by the NE), while also some NE's do not keep these times (and a later attempt to retrieve the alarm data from the NEs will not deliver these time data). |
| ackUserId | M | AlarmInformation.ackUserId | not applicable if related alarm was not acknowledged nor unacknowledged |
| ackSystemId | O | AlarmInformation.ackSystemId | not applicable if related alarm was not acknowledged nor unacknowledged |
| ackState | M | AlarmInformation.ackState | not applicable if related alarm was not acknowledged nor unacknowledged |
| clearUserId | O | AlarmInformation.clearUserId | not applicable if related alarm was not cleared |
| clearSystemId | O | AlarmInformation.clearSystemId | not applicable if related alarm was not cleared |
| serviceUser | M | AlarmInformation.serviceUser | not applicable if related alarm is not a security alarm |
| serviceProvider | M | AlarmInformation.serviceProvider | not applicable if related alarm is not a security alarm |
| securityAlarmDetector | M | AlarmInformation.securityAlarmDetector | not applicable if related alarm is not a security alarm |
| comments | M | The set of Comment instances related to this AlarmInformation. | Not applicable if the related alarm has no related comments |

###### 11.2.1.1.3.4	Exceptions and constraints

| Exception Name | Definition |
| --- | --- |
| operation_failed | Condition: Operation is failed Returned Information: The output parameter status Exit state: Entry State |

##### 11.2.1.1.4	notifyNewAlarm

###### 11.2.1.1.4.1	Definition

This notification is generated by the MnS producer when a new AlarmInformation is added to the AlarmList. The notification parameters depend on the alarmType and are different for non-security and security alarms.

###### 11.2.1.1.4.2	Input parameters

The notifyNewAlarm notification is defined by Table 11.2.1.1.4.2-1, if the alarmType is equal to  "Communications Alarm", "Processing Error Alarm", "Environmental Alarm". "Quality Of Service Alarm" or "Equipment Alarm".

Table 11.2.1.1.4.2-1: Input parameters for notifications related to non-security alarms

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | MonitoredEntity.objectClass | The MonitoredEntity is identified by the relation-AlarmedObject-AlarmInformation of the new AlarmInformation. |
| objectInstance | M | MonitoredEntity.objectInstance | The MonitoredEntity is identified by the relation-AlarmedObject-AlarmInformation of the new AlarmInformation. |
| notificationId | M | -- |  |
| notificationType | M | "notifyNewAlarm" |  |
| eventTime | M | AlarmInformation.alarmRaisedTime |  |
| systemDN | M | -- |  |
| alarmId | M | AlarmInformation.alarmId |  |
| alarmType | M | AlarmInformation.alarmType |  |
| probableCause | M | AlarmInformation.probableCause |  |
| perceivedSeverity | M | AlarmInformation.perceivedSeverity |  |
| specificProblem | O | AlarmInformation.specificProblem |  |
| backedUpStatus | O | AlarmInformation.backedUpStatus |  |
| backUpObject | O | MonitoredEntity.objectInstance It carries the DN of the back up object. | The object is identified by relation-BackUpObject-AlarmInformation of the new AlarmInformation. |
| trendIndication | O | AlarmInformation.trendIndication |  |
| thresholdInfo | O | AlarmInformation.thresholdInfo |  |
| correlatedNotifications | O | The CorrelatedNotification instances related to this AlarmInformation. |  |
| stateChangeDefinition | O | AlarmInformation.stateChangeDefinition |  |
| monitoredAttributes | O | AlarmInformation.monitoredAttributes |  |
| proposedRepairActions | O | AlarmInformaton.proposedRepairActions |  |
| additionalText | O | AlarmInformation.additionalText |  |
| additionalInformation | O | AlarmInformation.additionalInformation |  |
| rootCauseIndicator | O | AlarmInformation.rootCauseIndicator |  |

###### 11.2.1.1.4.2a	Input parameters for notifications related to non-security alarms

The notifyNewAlarm notification is defined by Table 11.2.1.1.4.2a-1, if the alarmType is equal to "Integrity Violation", "Operational Violation", "Physical Violation", "Security Service or Mechanism Violation" or "Time Domain Violation".

Table 11.2.1.1.4.2a-1: Input parameters for notifications related to security alarms

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | MonitoredEntity.objectClass | The MonitoredEntity is identified by the relation-AlarmedObject-AlarmInformation of the new AlarmInformation. |
| objectInstance | M | MonitoredEntity.objectInstance | The MonitoredEntity is identified by the relation-AlarmedObject-AlarmInformation of the new AlarmInformation. |
| notificationId | M | -- |  |
| notificationType | M | "notifyNewAlarm" |  |
| eventTime | M | AlarmInformation.alarmRaisedTime |  |
| systemDN | M | -- |  |
| alarmId | M | AlarmInformation.alarmId |  |
| alarmType | M | AlarmInformation.alarmType |  |
| probableCause | M | AlarmInformation.probableCause |  |
| perceivedSeverity | M | AlarmInformation.perceivedSeverity |  |
| correlatedNotifications | O | The set of CorrelatedNotification related to this AlarmInformation. |  |
| additionalText | O | AlarmInformation.additionalText |  |
| additionalInformation | O | AlarmInformation.additionalInformation |  |
| rootCauseIndicator | O | AlarmIngormation.rootCauseIndicator |  |
| serviceUser | M | AlarmInformation.securityServiceUser | This may contain no information if the identify of the service-user (requesting the service) is not known. |
| serviceProvider | M | AlarmInformation.securityServiceProvider | This shall always identify the service-provider receiving a service request, from serviceUser, that provokes the security alarm. |
| securityAlarmDetector | M | AlarmInformation.securityAlarmDetector | This may contain no information if the detector of the security alarm is the serviceProvider. |

###### 11.2.1.1.4.3	Triggering event

11.2.1.1.4.3.1	From-state

noMatchedAlarm.

| Assertion Name | Definition |
| --- | --- |
| noMatchedAlarm | AlarmList does not contain an AlarmInformation that has the following properties: Its matching-criteria-attributes values are identical to that of the newly generated network alarm and it is involved in relation-AlarmObject-AlarmInformation with the same MonitoredEntity as the one identified by the newly generated network alarm. |

11.2.1.1.4.3.2	To-state

newAlarmInAlarmList.

| Assertion Name | Definition |
| --- | --- |
| newAlarmInAlarmList | AlarmList contains an AlarmInformation holding information conveyed by the newly generated network alarm. This AlarmInformation is involved in relation-AlarmObject-AlarmInformation with the same MonitoredEntity as the one identified by the newly generated network alarm. The following attributes of the AlarmInformation shall be populated with information in the newly generated alarm: notificationId, alarmRaisedTime, alarmId, alarmType, , probableCause, perceivedSeverity. The following attributes of the same AlarmInformation shall be populated with information of the newly generated alarm if the information is present (in the newly generated alarm) and if the attribute is supported: specificProblem, backedUpStatus, trendIndication, thresholdInfo, stateChangeDefinition, monitoredAttributes, proposedRepairActions, additionalText, additionalInformation. |

##### 11.2.1.1.5	notifyChangedAlarm

###### 11.2.1.1.5.1	Definition

This notification is generated by the MnS producer when the perceivedSeverity of an existing AlarmInformation changes (except to the value "CLEARED").

11.2.1.1.5.2	Input parameters

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | MonitoredEntity.objectClass | The MonitoredEntity is identified by the relation-AlarmedObject-AlarmInformation. |
| objectInstance | M | MonitoredEntity.objectInstance | The MonitoredEntity is identified by the relation-AlarmedObject-AlarmInformation. |
| notificationId | M | -- |  |
| notificationType | M | "notifyChangedAlarm" |  |
| eventTime | M | AlarmInformation.alarmChangedTime |  |
| systemDN | M | -- |  |
| alarmId | M | AlarmInformation.alarmId |  |
| alarmType | M | AlarmInformation.alarmType |  |
| probableCause | M | AlarmInformation.probableCause |  |
| perceivedSeverity | M | AlarmInformation.perceivedSeverity |  |

###### 11.2.1.1.5.3	Triggering event

11.2.1.1.5.3.1	From-state

alarmMatched AND alarmNotCleared AND alarmChanged.

| Assertion Name | Definition |
| --- | --- |
| alarmMatched | The matching-criteria-attributes of the newly generated network alarm has values that are identical (matches) with ones in one AlarmInformation in AlarmList. |
| alarmNotCleared | The perceivedSeverity of the newly generated network alarm is not Cleared. |
| alarmChanged | The perceivedSeverity of the newly generated network alarm and of the matched AlarmInformation are different. |

11.2.1.1.5.3.2	To-state

informationUpdate.

| Assertion Name | Definition |
| --- | --- |
| informationUpdate | The AlarmInformation identified in alarmMatched in from-state has been updated according to the following rules: - notificationId is updated; - alarmChangedTime is updated; - perceivedSeverity is updated; - ackTime, ackUserId and ackSystemId are updated to contain no information; - ackState is updated to "unacknowledged"; |

##### 11.2.1.1.6	notifyAlarmListRebuilt

###### 11.2.1.1.6.1	Definition

This notification is generated by the MnS producer when the AlarmList has been completely or partially rebuilt.

###### 11.2.1.1.6.2	Input parameters

| Parameter Name | S | Legal type | Comment |
| --- | --- | --- | --- |
| objectClass | M | -- | Identifies, together with the objectInstance parameter, the part of the alarm list that has been rebuilt. If this paramter specifies the class of the instance carried in systemDN, then all AlarmInformation instances in the AlarmList may have been rebuilt. If this parameter specifies some class represented by MonitoredEntity, then a subset of the AlarmInformation instances in the AlarmList may have been rebuilt. |
| objectInstance | M | -- | Identifies, together with the objetClass parameter, the part of the alarm list that has been rebuilt. If this parameter is equal to the instance carried in systemDN, then all AlarmInformation instances in the AlarmList may have been rebuilt. If this parameter is equal to some instance represented by MonitoredEntity, then only AlarmInformation related to this instance and its descendants may have been rebuilt.. |
| notificationId | M | -- | -- |
| notificationType | M | "notifyAlarmListRebuilt" |  |
| eventTime | M | -- | The time when the alarm list has been rebuilt. |
| systemDN | M | -- | It identifies the DN of MnS producer. |
| reason | M | "System-NE communication error", "System restarts", "indeterminate". Other values can be added. | The reason why the system has rebuilt the AlarmList. This may carry different reasons than that carried by the immediate previous notifyPotentialFaultyAlarmList. |
| alarmListAlignmentRequirement | O | "alignmentRequired", "alignmentNotRequired". | It carries an enumeration of "alignmentRequired" and "alignmentNotRequired". |

###### 11.2.1.1.6.3	Triggering event

11.2.1.1.6.3.1	From-state

alarmListRebuilt_0 OR alarmListRebuilt_1.

| Assertion Name | Definition |
| --- | --- |
| alarmListRebuilt_0 | MnS producer has cold-started, initialized, re-initialized or rebooted and it has initiated procedure to rebuild its AlarmList. |
| alarmListRebuilt_1 | MnS producer loses confidence in part or whole of its AlarmList. MnS producer has initiated procedure to repair its AlarmList. |

11.2.1.1.6.3.2	To-state

alarmListRebuilt_2.

| Assertion Name | Definition |
| --- | --- |
| alarmListRebuilt_2 | MnS producer rebuilds the whole or part of AlarmList. |

##### 11.2.1.1.7	notifyCorrelatedNotificationChanged

###### 11.2.1.1.7.1	Definition

This notification is generated by the MnS producer when the set of CorrelatedNotification is created, updated or deleted.

###### 11.2.1.1.7.2	Input parameters

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | MonitoredEntity.objectClass | The MonitoredEntity is identified by the relation-AlarmedObject-AlarmInformation. |
| objectInstance | M | MonitoredEntity.objectInstance | The MonitoredEntity is identified by the relation-AlarmedObject-AlarmInformation. |
| notificationId | M | -- |  |
| notificationType | M | "notifyCorrelatedNotificationChanged" |  |
| eventTime | M | It carries the time when the CorrelatedNotification is created, updated or deleted. |  |
| systemDN | M | -- |  |
| alarmId | M | AlarmInformation.alarmId |  |
| correlatedNotifications | M | The CorrelatedNotification instances related to this AlarmInformation. |  |
| rootCauseIndicator | O | AlarmInformation.rootCauseIndicator |  |

###### 11.2.1.1.7.3	Triggering event

11.2.1.1.7.3.1	From-state

newAlarmCorrelationInfoIsAvailable AND alarmInformationExists.

| Assertion Name | Definition |
| --- | --- |
| newAlarmCorrelationInfoIsAvailable | New alarm correlation information is available but not yet conveyed to any consumer. |
| alarmInformationExists | The AlarmInformation is in AlarmList. |

11.2.1.1.7.3.2	To-state

alarmCorrelatedInfoUpdated.

| Assertion Name | Definition |
| --- | --- |
| alarmCorrelatedInfoUpdated | The set of CorrelatedNotification network slice instances is created, updated or deleted. |

##### 11.2.1.1.8	getAlarmCount

###### 11.2.1.1.8.1	Definition

A MnS consumer invokes this operation to get the number of alarms in the alarm list. The alarms are counted separately for each perceived severity level. An input parameter allows to control which alarms are counted.

###### 11.2.1.1.8.2	Input parameters

| Name | S | Information Type | Comment |
| --- | --- | --- | --- |
| filter | O | N/A | It carries a filter constraint. The operation shall apply it when counting the AlarmInformation instances in AlarmList. Case when synchronous mode of operation is used for getAlarmList: (a) If this parameter is present, the operation shall count the AlarmInformation instances which satisfy both (a) this filter constraint and (b) the condition set by input parameter alarmAckState. (b) If this parameter is absent, the operation shall count all AlarmInformation instances that satisfy the condition set by input parameter alarmAckState. Case when asynchronous mode of operation is used for getAlarmList: (a) If this parameter is present, the operation shall count all AlarmInformation instances that satisfy this filter constraint and the condition set by input parameter alarmAckState. (b) If this parameter is absent, the operation shall count AlarmInformation instances that satisfy (a) the filter constraint currently active in the notification channel established between the authorized MnS consumer and the MnS produce and (b) the condition set by input parameter alarmAckState. |
| alarmAckState | O | ENUM (all alarms, all active alarms, all active and acknowledged alarms, all active and unacknowledged, all cleared and unacknowledged alarms, all unacknowledged) | It carries a constraint. The operation shall apply it on AlarmInformation instances in AlarmList when counting. |

###### 11.2.1.1.8.3	Output parameters

| Name | S | Matching Information | Comment |
| --- | --- | --- | --- |
| criticalCount, majorCount, minorCount, warningCount, indeterminateCount, clearedCount | M | N/A | They carry the number of AlarmInformation in AlarmList that has the following properties. Case when synchronous mode of operation is used: (a) The operation shall apply the constraints expressed in alarmAckState and filter to AlarmInformation instances when counting. Case when asynchronous mode of operation is used (i.e. this output parameter is conveyed via notifications): (a) If the filter parameter is present, the operation shall apply the constraint when counting. Furthermore, if the alarmAckState constraint is present, the operation shall apply that constraint as well. The filter constraint, if any, that is currently active in the notification channel is not used for the counting. (b) If the filter parameter is absent, the operation shall apply the filter constraint currently active in the notification channel when counting. If the alarmAckState constraint is present, the operation shall apply that constraint as well. |
| status | M | ENUM (OperationSucceeded, OperationFailed) | If allAlarmInformationCounted is true, status = OperationSucceeded. If operation_failed is true, status = OperationFailed. |

###### 11.2.1.1.8.4	Pre-condition

There are no pre-conditions.

###### 11.2.1.1.8.5	Post-condition

allAlarmInformationCounted.

| Assertion Name | Definition |
| --- | --- |
| allAlarmInformationCounted | All AlarmInformation that satisfy the constraints expressed in input parameters filter and alarmAckState and are present in the AlarmList at the moment of this operation invocation are counted and the result returned. All AlarmInformation in AlarmList remains unchanged as the result of this operation. |

###### 11.2.1.1.8.6	Exceptions

| Name | Definition |
| --- | --- |
| operation_failed | Condition: the pre-condition is false or the post-condition is true. Returned Information: The output parameter status. Exit state: Entry state. |
| filter_complexity_limit | Condition: Operation not performed because the filter parameter is too complex. Returned Information: The output parameter status. Exit state: Entry state. |

##### 11.2.1.1.9	setComment

###### 11.2.1.1.9.1	Definition

A MnS consumer invokes this operation to set a comment in one or more AlarmInformation instances in AlarmList.

###### 11.2.1.1.9.2	Input parameters

| Name | S | Information Type | Comment |
| --- | --- | --- | --- |
| alarmInformationReferenceList | M | List of AlarmInformation.alarmId | It carries one or more identifiers identifying AlarmInformation instances in the AlarmList. |
| commentUserId | M | Comment.commentUserId | The Comment is identified by the relation-AlarmInformation-Comment. |
| commentSystemId | O | Comment.commentSystemId | The Comment is identified by the relation-AlarmInformation-Comment. |
| commentText | M | Comment.commentText | The Comment is identified by the relation-AlarmInformation-Comment. |

###### 11.2.1.1.9.3	Output Parameters

| Name | S | Matching Information | Comment |
| --- | --- | --- | --- |
| badAlarmInformationReferenceList | M | List of pair of AlarmInformation.alarmId and the failure reason. | If allUpdated is true, it contains no information. If someUpdated is true, then it contains identifications of AlarmInformation that are not present in AlarmList or that they are present, but AlarmInformation.comments has not changed, in contrast to authorized consumer's request. |
| status | M | ENUM( Operation succeeded, Operation failed, Operation partially failed) | If allUpdated is true, then status = OperationSucceeded. If someUpdated is true, then status = OperationPartiallyFailed. If exception operationFailed is raised, then status = OperationFailed. |

#### 11.2.1.2	Fault supervision data control

##### 11.2.1.2.1	acknowledgeAlarms

###### 11.2.1.2.1.1	Definition

The MnS consumer invokes this operation to acknowledge one or more alarms.

When this operation is not supported, the MnS producer shall support acknowledging alarms.

###### 11.2.1.2.1.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| alarmInformationAnd SeverityReferenceList | M | SET OF SEQUENCE { AlarmInformation.alarmId (M) AlarmInformation.perceivedSeverity (O) } | It identifies the alarms to be acknowledged. If an alarm id is qualified with an optional perceived severity, the alarm shall be acknowledged only when the perceived severity in the alarm list matches the perceived severity provided in the operation request. |
| ackUserId | M | AlarmInformation.ackUserId | The identifier of the user acknowledgeding the alarm. |
| ackSystemId | O | AlarmInformation.ackSystemId | The identifier of the system where the acknowledgement request was originated. |

###### 11.2.1.2.1.3	Output parameters

| Parameter Name | S | Matching Information / Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| badAlarmInformationReferenceList | M | SET OF SEQUENCE { AlarmInformation.alarmId (M) errorReason (M) } errorReason ::= ENUM { UnknownAlarmId, AcknowledgmentFailed, WrongPerceivedSeverity } | If all alarms are acknowledged, it contains no information. If only some alarms are acknowledged, then it contains identifications of AlarmInformation that are (a) present in input parameter AlarmInformationReferenceList but absent in the AlarmList (errorReason = UnknownAlarmId; or (b) present in input parameter AlarmInformationReferenceList and present in the AlarmList but the Acknowledgement Information (see note below table) has not changed despite the consumer's request (errorReason = AcknowledgmentFailed); or (c) present in input parameter AlarmInformationReferenceList and present in the AlarmList but the perceivedSeverity to be acknowledged has changed and/or is different in the Alarm List (ErrorReason = WrongPerceivedSeverity), applicable only if perceivedSeverity is provided. |
| status | M | ENUM { OperationSucceeded, OperationPartiallySucceeded, OperationFailed } | If all alarms acknowledged, then status = OperationSucceeded. If some alarms are acknowledged, then status = OperationPartiallySuceeded. If operation failed is true, then status = OperationFailed. |

NOTE:	Acknowledgement Information is defined as the information contained in AlarmInformation.ackTime, AlarmInformation.ackUserId, AlarmInformaton.ackSystemId, AlarmInformation.ackState.

###### 11.2.1.2.1.4	Exceptions and constraints

| Exception Name | Definition |
| --- | --- |
| operation_failed | Condition: Operation is failed Returned Information: The output parameter status Exit state: Entry State |

##### 11.2.1.2.2	unacknowledgeAlarms

###### 11.2.1.2.2.1	Definition

The MnS consumer invokes this operation to remove acknowledgement information kept in one or more AlarmInformation instances.

###### 11.2.1.2.2.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| alarmInformationReferenceList | M | List of AlarmInformation.alarmId | It carries one or more identifiers identifying AlarmInformation in AlarmList. |
| ackUserId | M | AlarmInformation.ackUserId | The identifier of the user unacknowledgeding the alarm. |
| ackSystemId | O | AlarmInformation.ackSystemId | The identifier of the system where the acknowledgement request was originated. |

###### 11.2.1.2.2.3	Output parameters

| Parameter Name | S | Matching Information / Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| badAlarmInformationReferenceList | M | SET OF SEQUENCE { AlarmInformation.alarmId (M) errorReason (M) } errorReason ::= ENUM { UnknownAlarmId, AcknowledgmentFailed, WrongPerceivedSeverity, } | If all alarms are acknowledged, it contains no information. If only some alarms are acknowledged, then it contains identifications of AlarmInformation that are (a) present in input parameter AlarmInformationReferenceList but absent in the AlarmList (errorReason = UnknownAlarmId; or (b) present in input parameter AlarmInformationReferenceList and present in the AlarmList but the Acknowledgement Information (see note below table) has not changed despite the consumer's request (errorReason = AcknowledgmentFailed); or (c) present in input parameter AlarmInformationReferenceList and present in the AlarmList but the perceivedSeverity to be acknowledged has changed and/or is different in the Alarm List (ErrorReason = WrongPerceivedSeverity), applicable only if perceivedSeverity is provided. |
| status | M | ENUM { OperationSucceeded, OperationPartiallySucceeded, OperationFailed | If all alarms acknowledged, then status = OperationSucceeded. If some alarms are acknowledged, then status = OperationPartiallySuceeded. If operation failed is true, then status = OperationFailed. |

NOTE:	Acknowledgement Information is defined as the information contained in AlarmInformation.ackTime, AlarmInformation.ackUserId, AlarmInformaton.ackSystemId, AlarmInformation.ackState.

###### 11.2.1.2.2.4	Exceptions and constraints

| Exception Name | Definition |
| --- | --- |
| Operation_failed | Condition: Operation is failed Returned Information: The output parameter status Exit state: Entry State |

##### 11.2.1.2.3	clearAlarms

###### 11.2.1.2.3.1	Definition

The authorized consumer invokes this operation to clear one or more AlarmInformation instances in AlarmList. For example, this operation can be used to support the manual clearing of the ADMC (automatic detection and manual clearing, see also 3GPP TS 32.111-1 [3]) alarms.

###### 11.2.1.2.3.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| alarmInformationReferenceList | M | List of AlarmInformation.alarmId | It carries one or more identifiers identifying AlarmInformation instances in the AlarmList. |
| clearUserId | M | AlarmInformation.clearUserId | It identities the user clearing the alarm. |
| clearSystemId | O | AlarmInformation.clearSystemId | It identifies the authorized consumer. It may be absent implying that consumer does not wish this information be known to the MnS producer. |

###### 11.2.1.2.3.3	Output parameters

| Parameter Name | S | Matching Information / Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| badAlarmInformationReferenceList | M | List of pair of AlarmInformation.alarmId and the failure reason. | If all alarms are cleared, it contains no information. If some alarms are cleared, then it contains identifications of AlarmInformation that are not present in AlarmList or that are present in AlarmList but remain unchanged, in contrast to consumer's request. |
| status | M | ENUM( OperationSucceeded, OperationFailed, OperationPartiallySucceeded) | If all alarms are cleared, then status = OperationSucceeded. If some alarms are cleared, then status = OperationPartiallySucceeded. If operation is failed, then status = OperationFailed. |

###### 11.2.1.2.3.4	Exceptions and constraints

| Exception Name | Definition |
| --- | --- |
| operation_failed | Condition: Operation is failed Returned Information: The output parameter status Exit state: Entry State |

##### 11.2.1.2.4	notifyClearedAlarm

###### 11.2.1.2.4.1	Definition

This notification is generated by the MnS producer when the perceivedSeverity of an existing AlarmInformation changes to "CLEARED".

###### 11.2.1.2.4.2	Input parameters

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | MonitoredEntity.objectClass |  |
| objectInstance | M | MonitoredEntity.objectInstance |  |
| notificationId | M | -- |  |
| notificationType | M | "notifyClearedAlarm" |  |
| eventTime | M | AlarmInformation.alarmClearedTime |  |
| systemDN | M | -- |  |
| alarmId | M | AlarmInformation.alarmId |  |
| alarmType | M | AlarmInformation.alarmType |  |
| probableCause | M | AlarmInformation.probablaCause |  |
| perceivedSeverity | M | AlarmInformation.perceivedSeverity | Value shall be "CLEARED" |
| correlatedNotifications | O | The CorrelatedNotification instances related to this AlarmInformation. | This parameter contains references to other AlarmInformation instances whose perceivedSeverity levels are cleared as well. In this way, the perceivedSeverity level of multiple AlarmInformation instances can be cleared by one notification. |
| clearUserId | O | AlarmInformation.clearUserId | This parameter shall be present and contain valid information if the AlarmInformation is cleared by a clearAlarms operation request. |
| clearSystemId | O | AlarmInformation.clearSystemId | This parameter is present if clearUserId is present and if AlarmInformation.clearSystemId contains valid information. |

###### 11.2.1.2.4.3	Triggering event

11.2.1.2.4.3.1	From-state

alarmMatchedAndCleared OR clearedByProvider.

| Assertion Name | Definition |
| --- | --- |
| alarmMatchedAndCleared | The matching-criteria-attributes of the newly generated network alarm have values that are identical (matched) with ones in one AlarmInformation in AlarmList and the perceivedSeverity of the matched AlarmInformation is not Cleared AND The perceivedSeverity of the newly generated network alarm is cleared. |
| clearedByProvider | Reception of a valid clearAlarms operation that identifies the subject AlarmInformation instances. This triggering event shall occur regardless of the perceivedSeverity state of the identified AlarmInformation instances. |

11.2.1.2.4.3.2	To-state

alarmInformationCleared_1 OR alarmInformationCleared_2.

| Assertion Name | Definition |
| --- | --- |
| alarmInformationCleared_1 | Case if From-state is alarmMatchedAndCleared: The following attributes of the subject AlarmInformation are updated: notificationId, perceivedSeverity (updated to Cleared), alarmClearedTime. |
| alarmInformationCleared_2 | Case if From-state is clearedByProvider: The following attributes of the subject AlarmInformation are updated: notificationId, alarmClearedTime, perceivedSeverity (updated to CLEARED), alarmClearedUserId, alarmClearedSystemId. |

##### 11.2.1.2.5	notifyAckStateChanged

###### 11.2.1.2.5.1	Definition

This notification is generated by the MnS producer when a the acknowledgement state of an alarm changes from "UNACKNOWLEDGED" to "ACKNOWLEDGED" or back from "ACKNOWLEDGED" to "UNACKNOWLEDGED".

###### 11.2.1.2.5.2	Input parameters

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | MonitoredEntity.objectClass |  |
| objectInstance | M | MonitoredEntity.objectInstance |  |
| notificationId | M | -- |  |
| notificationType | M | "notifyAckStateChanged" |  |
| eventTime | M | AlarmInformation.ackTime |  |
| systemDN | M | -- |  |
| alarmId | M | AlarmInformation.alarmId |  |
| alarmType | M | AlarmInformation.alarmType |  |
| probableCause | M | AlarmInformation.probableCause |  |
| perceivedSeverity | M | AlarmInformation.perceivedSeverity |  |
| ackState | M | AlarmInformation.ackState |  |
| ackUserId | M | AlarmInformation.ackUserId | The identifier of the user who acknowledged or unacknowledged the alarm. |
| ackSystemId | O | AlarmInformation.ackSystemId | The identifier of the system where the acknowledgement or unacknowledgement request was originated. |

###### 11.2.1.2.5.3	Triggering event

11.2.1.2.5.3.1	From-state

ackedByConsumer OR ackedByProvider AND alarmInformationExists.

| Assertion Name | Definition |
| --- | --- |
| ackedByConsumer | Reception of an acknowledgeAlarms operation and a subsequent operation success return. |
| ackedByProvider | Reception of a local (non-standard) acknowlegeAlarms equivalent operation and a subsequent operation success return. |
| alarmInformationExists | The AlarmInformation exists in AlarmList. |

11.2.1.2.5.3.2	To-state

alarmAckStateHasChanged.

| Assertion Name | Definition |
| --- | --- |
| alarmAckStateHasChanged | The AlarmInformation.ackState of the AlarmInformation identified by from-state assertion alarmInformationExists have been updated. Specifically, the following attributes of the subject AlarmInformation are updated: -- notificationId, ackTime, ackUserId, ackState, ackSystemId. |

##### 11.2.1.2.6	notifyComments

###### 11.2.1.2.6.1	Definition

This notification is generated by the MnS producer when a Comment instance is added to an AlarmInformation instance in the AlarmList.

A MnS producer shall support this notification if it supports the operation setComment.

###### 11.2.1.2.6.2	Input parameters

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | MonitoredEntity.objectClass |  |
| objectInstance | M | MonitoredEntity.objectInstance |  |
| notificationId | M | -- |  |
| notificationType | M | "notifyComments" |  |
| eventTime | M | Comment.commentTime |  |
| systemDN | M | -- |  |
| alarmId | M | AlarmInformation.alarmId |  |
| alarmType | M | AlarmInformation.alarmType |  |
| probableCause | M | AlarmInformation.probableCause |  |
| perceived Severity | M | AlarmInformation.perceivedSeverity |  |
| comments | M | The Comment instances related to this AlarmInformation. |  |

###### 11.2.1.2.6.3	Trigger event

11.2.1.2.6.3.1	From-state

commentedByServiceprovider OR commentedByServiceprovider AND alarmInformationExists.

| Assertion Name | Definition |
| --- | --- |
| commentedByServiceprovider | Reception of a setComment operation and a subsequent operation success return. |
| commentedByServiceprovider | Reception of a local (non-standard) setComment equivalent operation and a subsequent operation success return. |
| alarmInformationExists | The AlarmInformation is in AlarmList. |

11.2.1.2.6.3.2	To-state

commentInserted.

| Assertion Name | Definition |
| --- | --- |
| commentInserted | One Comment has been created and it is involved in a relationship with the AlarmInformation identified by from-state assertion alarmInformationExists. The following attributes of the newly created Comment instance shall be populated: commentTime, commentText, commentUserId and commentSystemId. |

##### 11.2.1.2.7	notifyPotentialFaultyAlarmList

###### 11.2.1.2.7.1	Definition

This notification is generated by the MnS producer when the MnS producer looses confidence in the integrity of its alarm list.

The MnS producer may then rebuilt the faulty alarm list. When the alarm List is rebuilt or confidence in the existing alarm list is re-established the MnS producer may generate a notifyAlarmListRebuilt notification.

The parameters objectClass and objectInstance are used to specify if the complete alarm list is unreliable or only parts thereof.

The MnS consumer behaviour, on reception of this notifyPotentialFaultyAlarmList notification, is not specified. The authorized consumer behaviour is considered not essential for the specification of the interface itself. However, the following are recommended actions the uthorized consumer should take, in case it receives this notification.

1)	The uthorized consumer should not perform any task requiring the integrity of the AlarmInformation identified as faulty or unreliable by the subject notification.

2)	The uthorized consumer should not invoke operations that require integrity of the AlarmList such as getAlarmList., acknolwedgeAlarms operations.

###### 11.2.1.2.7.2	Input parameters

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | It identifies the class of the instance identified by systemDN or the class of MonitoredEntity. | Identifies, together with the objectInstance parameter, the part of the alarm list that is not reliable. If this paramter specifies the class of the instance carried in systemDN, then all AlarmInformation instances in the AlarmList may not be reliable. If this parameter specifies some class represented by MonitoredEntity, then a subset of the AlarmInformation instances in the AlarmList is not reliable. |
| objectInstance | M | It identifies the instance identified by systemDN or an instance of MonitoredEntity. | Identifies, together with the objetClass parameter, the part of the alarm list that may not be reliable. If this parameter is equal to the instance carried in systemDN, then all AlarmInformation instances in the AlarmList may not be reliable. If this parameter is equal to some instance represented by MonitoredEntity, then only AlarmInformation related to this instance and its descendants may not be reliable. |
| notificationId | M | -- |  |
| notificationType | M | "notifyPotentialFaultyAlarmList" |  |
| eventTime | M | -- | Time when the MnS producer lost confidence in the integrity of the alarm list |
| systemDN | M | -- |  |
| reason | M | "serviceprovider-NE communication error", " serviceprovider restarts", "indeterminate". Other values can be added. | Reason why the MnS producer has to rebuild its AlarmList. |

###### 11.2.1.2.7.3	Trigger event

11.2.1.2.7.3.1	From-state

faultyAlarmListDetected.

| Assertion Name | Definition |
| --- | --- |
| faultyAlarmListDetected | MnS producer detects faults in part or whole of its AlarmList. |

11.2.1.2.7.3.2	To-state

faultyAlarmList

| Assertion Name | Definition |
| --- | --- |
| faultyAlarmList | MnS producer initiates the AlarmList rebuild process. |

##### 11.2.1.2.8	notifyChangedAlarmGeneral

###### 11.2.1.2.8.1	Definition

This notification is generated by the MnS producer when one or more of the following attributes of an AlarmInformation instance in the AlarmList changes its value: perceivedSeverity, backedUpStatus, backUpObject, trendIndication, thresholdInfo, stateChangeDefinition, monitoredAttributes, proposedRepairActions, additionalText, additionalInformation, serviceUser, serviceProvider or securityAlarmDetector. From the attributes listed above, only those that changed value shall be included in the notification.

The notification parameters depend on the alarmType and are different for non-security and security alarms.

###### 11.2.1.2.8.2	Input parameters for notifications related to non-security alarms

The notifyChangedAlarmGeneral notification is defined by Table 11.2.1.2.8.2-1, if the alarmType is equal to "Communications Alarm", "Processing Error Alarm", "Environmental Alarm",, "Quality Of Service Alarm" or "Equipment Alarm".

Table 11.2.1.2.8.2-1: Input parameters for notifications related to non-security alarms

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | MonitoredEntity.objectClass |  |
| objectInstance | M | MonitoredEntity.objectInstance |  |
| notificationId | M | -- |  |
| notificationType | M | "notifyChangedAlarmGeneral" |  |
| eventTime | M | AlarmInformation.alarmChangedTime |  |
| systemDN | M | -- |  |
| alarmId | M | AlarmInformation.alarmId |  |
| alarmType | M | AlarmInformation.alarmType |  |
| probableCause | O | AlarmInformation.probableCause |  |
| specificProblem | O | AlarmInformation.specificProblem |  |
| perceivedSeverity | O | AlarmInformation.perceivedSeverity |  |
| backedUpStatus | O | AlarmInformation.backedUpStatus |  |
| backUpObject | O | MonitoredEntity.objectInstance | The DN of the back up object. The object is identified by relation-BackUpObject-AlarmInformation of the new AlarmInformation. |
| trendIndication | O | AlarmInformation.trendIndication |  |
| thresholdInfo | O | AlarmInformation.thresholdInfo |  |
| correlatedNotifications | O | Set of CorrelatedNotification related to this AlarmInformation. |  |
| stateChangeDefinition | O | AlarmInformation.stateChange |  |
| monitoredAttributes | O | AlarmInformation.monitoredAttributes |  |
| proposedRepairActions | O | AlarmInformaton.proposedRepairActions |  |
| additionalText | O | AlarmInformation.additionalText |  |
| additionalInformation | O | AlarmInformation.additionalInformation |  |
| rootCauseIndicator | O | alarmInformation.rootCauseIndicator |  |
| changedAlarmAttributes | O | LIST OF SEQUENCE <AttributeName, OldAttributeValue> | The changed alarm attributes (name/value pairs) (with old values). |

###### 11.2.1.2.8.3	Input parameters for notifications related to security alarm

The notifyChangedAlarmGeneral notification is defined by Table 11.2.1.1.4.2a-1, if the alarmType is equal to "Integrity Violation", "Operational Violation", "Physical Violation", "Security Service or Mechanism Violation" or "Time Domain Violation".

Table 11.2.1.2.8.3-1: Input parameters for notifications related to security alarms

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | MonitoredEntity.objectClass |  |
| objectInstance | M | MonitoredEntity.objectInstance |  |
| notificationId | M | -- |  |
| notificationType | M | "notifyChangedAlarmGeneral". |  |
| eventTime | M | AlarmInformation.alarmChangedTime |  |
| systemDN | M | -- |  |
| alarmId | M | AlarmInformation.alarmId |  |
| alarmType | M | AlarmInformation.alarmType |  |
| probableCause | O | AlarmInformation.probableCause |  |
| perceivedSeverity | O | AlarmInformation.perceivedSeverity |  |
| correlatedNotifications | O | Set of CorrelatedNotification related to this AlarmInformation. |  |
| additionalText | O | AlarmInformation.additionalText |  |
| additionalInformation | O | AlarmInformation.additionalInformation |  |
| rootCauseIndicator | O | alarmInformation.rootCauseIndicator |  |
| serviceUser | M | AlarmInformation.serviceUser | This may contain no information if the identify of the service-user (requesting the service) is not known. |
| serviceProvider | M | AlarmInformation.serviceProvider | This shall always identify the service-provider receiving a service request, from serviceUser, that provokes the security alarm. |
| securityAlarmDetector | M | AlarmInformation.securityAlarmDetector | This may contain no information if the detector of the security alarm is the serviceProvider. |
| changedAlarmAttributes | O | LIST OF SEQUENCE <AttributeName, OldAttributeValue> | The changed alarm attributes (name/value pairs) (with old values). |

###### 11.2.1.2.8.4	Trigger event

11.2.1.2.8.4.1	From-state

alarmMatched AND alarmNotCleared AND alarmChanged.

| Assertion Name | Definition |
| --- | --- |
| alarmMatched | The matching-criteria-attributes of the newly generated network alarm has values that are identical (matches) with ones in one AlarmInformation in AlarmList. |
| alarmChanged | One or more of perceivedSeverity, backedUpStatus, backUpObject, trendIndication, thresholdInfo, stateChangeDefinition, monitoredAttributes, proposedRepairActions, additionalText, additionalInformation, serviceUser, serviceProvider or securityAlarmDetector of the newly generated network alarm and of the matched AlarmInformation are different. |

11.2.1.2.8.4.2	To-state

informationUpdate.

| Assertion Name | Definition |
| --- | --- |
| informationUpdate | The AlarmInformation identified in alarmMatched in from-state has been updated according to the following rules: perceivedSeverity, backedUpStatus, backUpObject, trendIndication, thresholdInfo, stateChangeDefinition, monitoredAttributes, proposedRepairActions, additionalText, additionalInformation, serviceUser, serviceProvider or securityAlarmDetector is updated; notificationId is updated; alarmChangedTime is updated; ackTime, ackUserId and ackSystemId are updated to contain no information; ackState is updated to "unacknowledged"; |

### 11.2.2	Managed information

#### 11.2.2.1	Alarm information, alarm state change and Information Object Classes

##### 11.2.2.1.1	Imported information entities and local labels

None.

##### 11.2.2.1.2	Class diagram

###### 11.2.2.1.2.1	Introduction

This clause introduces the fault supervision related classes (i.e. IOCs, SupportIOCs). The intent is to identify the information required for the Fault management service implementation of its operations and notification emission. This clause provides the overview of all support object classes in UML. Subsequent clauses provide more detailed specification of various aspects of these support object classes.

###### 11.2.2.1.2.2	Attributes and relationships

##### 11.2.2.1.3	Information Object Class Definitions

###### 11.2.2.1.3.1	AlarmInformation

11.2.2.1.3.1.1	Definition

AlarmInformation contains information about alarm conditions of an alarmed MonitoredEntity.

A MnS producer is related to at most one AlarmList. The MnS producer assigns an identifier, called alarmId, to each AlarmInformation in the AlarmList. An alarmId unambiguously identifies one AlarmInformation in the AlarmList.

11.2.2.1.3.1.2	Attribute

| Attribute name | S |
| --- | --- |
| alarmId | M |
| objectClass/objectInstance (attribute related to role) | M |
| notificationId | M |
| alarmRaisedTime | M |
| alarmChangedTime | O |
| alarmClearedTime | M |
| alarmType | M |
| probableCause | M |
| specificProblem | O |
| perceivedSeverity | M |
| backedUpStatus | O |
| backUpObject (attribute related to role) | O |
| trendIndication | O |
| thresholdInfo | O |
| correlatedNotifications (attribute related to role) | O |
| stateChangeDefinition | O |
| monitoredAttributes | O |
| proposedRepairActions | O |
| additionalText | O |
| additionalInformation | O(see note 3) |
| rootCauseIndicator | O |
| ackTime | M |
| ackUserId | M |
| ackSystemId | O |
| ackState | M |
| clearUserId | O (see note 1) |
| clearSystemId | O (see note 1) |
| serviceUser | O (see note 2) |
| serviceProvider | O (see note 2) |
| securityAlarmDetector | O (see note 2) |
| NOTE 1: These attributes and qualifiers are applicable only if the management service producer supports clearAlarms() (they are absent if clearAlarms() is not supported). NOTE 2: These attributes are supported if the management service producer emits notifyNewAlarm that carries security alarm information. NOTE 3: This attribute is optionally populated whenever vendor specific attributes are needed. |  |

11.2.2.1.3.1.3	State diagram

Alarms have states. The alarm state information is captured in AlarmInformation in AlarmList.

The solid circle icon represents the Start State. The double circle icon represents the End State. In this state, the alarm is Cleared and acknowledged. The AlarmInformation shall not be accessible via the Service interface and is removed from the AlarmList.

Note the state diagram uses " X / Y ^ Z " to label the arc that indicates state transition. The meanings of X, Y and Z are:

-	X identifies the triggering event;

-	Y identifies the action of FaultSupervision MnS producer because of the triggering event;

-	Z is the notification to be emitted by FaultSupervision MnS producer because of the triggering event.

Note that acknowledgeAlarm^notifyAckStateChanged and the unacknowledgeAlarm^notifyAckStateChanged refer to cases when the request of the management service consumer is successful for the AlarmInformation concerned. They do not refer to the cases when the request is a failure since in the failure cases, no state transition would occur.

Note that, to reduce cluttering to the diagram, the setComment^notifyComment is not included in the figure . One transition should be applied from unack&unclear to itself. Similarly, another transition should be applied from ack&unclear to itself. Another one is from unack&clear to itself.

"PS" used in the state diagram stands for "perceived severity".

Figure 11.2.2.1.3.1.3-1 is used if it supports ^notifyChangedAlarm and Figure 11.2.2.1.3.1.3-2 is used if it does not support ^notifyChangedAlarm.

Figure 11.2.2.1.3.1.3-1 notifyChangedAlarm supported

Figure 11.2.2.1.3.1.3-2 notifyChangedAlarm not supported

###### 11.2.2.1.3.2	AlarmList

11.2.2.1.3.2.1	Definition

The MnS producer maintains an AlarmList that contains currently active alarms (i.e. AlarmInformation whose perceivedSeverity is not Cleared) and alarms that are Cleared but not yet acknowledged.

11.2.2.1.3.2.2	Attribute

There is no additional attribute defined for this class besides those inherited.

###### 11.2.2.1.3.3	FSMnSProducer

11.2.2.1.3.3.1	Definition

FSMnSProducer is the representation of the entity who provides the fault supervision management service(s) and contains the AlarmList.

11.2.2.1.3.3.2	Attribute

There is no additional attribute defined for this class besides those inherited.

11.2.2.1.3.3.3	Notification Table

| Name | S | Notes |
| --- | --- | --- |
| notifyAlarmListRebuilt | M |  |
| notifyPotentialFaultyAlarmList | O | . |

###### 11.2.2.1.3.4	Comment

11.2.2.1.3.4.1	Definition

Comment contains commentary and associated information such as the time when the commentary is made.

11.2.2.1.3.4.2	Attribute

| Attribute Name | S |
| --- | --- |
| commentTime | M |
| commentUserId | M |
| commentSystemId | O |
| commentText | M |

###### 11.2.2.1.3.5	CorrelatedNotification

11.2.2.1.3.5.1	Definition

The sourceObjectInstance attribute of CorrelatedNotification identifies one MonitoredEntity. For the MonitoredEntity identified, a set of notification identifiers is also identified. One or more CorrelatedNotification instances can be related to an AlarmInformation. In this case, the information of the AlarmInformation is said to be correlated to information carried in the notifications identified by the CorrelatedNotification instances. See further definition of correlated notification in ITU-T Recommendation X.733 [4], clause 8.1.2.9.

The notification identified by the CorrelatedNotification, as defined in ITU-T and used here, can carry all types of information and is not restricted to carrying alarm information only. For example, a notification, identified by the CorrelatedNotification, can indicate a managed instance attribute value change. In this case, the information of the AlarmInformation is said to be correlated to the managed instance attribute value change event.

The meaning of correlation is dependent on the type of notification itself. See the comment column of the correlatedNotification input parameter for each type of notification, such as notifyNewAlarm.

Notification carries AlarmInformation. The AlarmInformation instances referred to by the correlatedNotification may or may not exist in the AlarmList. For example, the AlarmInformation carried by the identified notification may have been acknowledged and Cleared and therefore, no longer exist in the AlarmList.

11.2.2.1.3.5.2	Attribute

| Attribute Name | S |
| --- | --- |
| sourceObjectInstance | M |
| notificationIdSet | M |

###### 11.2.2.1.3.6	MonitoredEntity

11.2.2.1.3.6.1	Definition

It represents classes that can have an alarmed state. The types of classes that can have alarmed state are:

a) 	All classes whose Notification Tables include alarm notifications.

b) 	VSE subclass of 3GPP defined classes and VSE defined classes that can have alarmed state.

The objectClass and objectInstance of this class identifies an instance of this class. The AlarmInformation uses this information in two places. In one place, the information is used to identify the instance that is in alarmed state. In another place, the information is used to identify an instance that can be used as the back up network resource for the instance that is in alarmed state.

11.2.2.1.3.6.2	Attribute

There is no attribute for this class.

##### 11.2.2.1.4	Information relationships definition

###### 11.2.2.1.4.1	relation-FSMnSProducer-AlarmList (M)

11.2.2.1.4.1.1	Definition

This represents the relationship between FSMnSProducer and AlarmList.

11.2.2.1.4.1.2	Role

There is no role defined for this relationship.

11.2.2.1.4.1.3	Constraint

There is no constraint for this relationship.

###### 11.2.2.1.4.2	relation-AlarmList-AlarmInformation (M)

11.2.2.1.4.2.1	Definition

This represents the relationship between AlarmList and AlarmInformation.

11.2.2.1.4.2.2	Role

| Name | Definition |
| --- | --- |
| identifyAlarmInformation | It represents a capability to obtain the information contained in AlarmInformation. |

11.2.2.1.4.2.3	Constraint

| Name | Definition |
| --- | --- |
| inv_ hasAlarmInformation1 | No AlarmInformation playing the role of theAlarmInformation shall have its perceivedSeverity = "cleared" and its ackState = "acknowledged". |
| inv_ hasAlarmInformation2 | The alarmId of all AlarmInformation instances playing the role of theAlarmInformation are distinct. |

###### 11.2.2.1.4.3	relation-AlarmInformation-Comment (M)

11.2.2.1.4.3.1	Definition

This represents the relationship between AlarmInformation and Comment.

11.2.2.1.4.3.2	Role

| Name | Definition |
| --- | --- |
| comment | It represents a capability to obtain the information contained in Comment. |

11.2.2.1.4.3.3	Constraint

There is no constraint.

###### 11.2.2.1.4.4	relation-AlarmInformation-CorrelatedNotification (M)

11.2.2.1.4.4.1	Definition

This represents the relationship between AlarmInformation and CorrelatedNotification.

11.2.2.1.4.4.2	Role

| Name | Definition |
| --- | --- |
| correlatedNotification | It represents a capability to obtain the information contained in CorrelatedNotification. |

11.2.1.4.4.3	Constraint

There is no constraint.

###### 11.2.2.1.4.5	relation-AlarmedObject-AlarmInformation (M)

11.2.2.1.4.5.1	Definition

This represents the relationship between MonitoredEntity and AlarmInformation.

11.2.2.1.4.5.2	Role

| Name | Definition |
| --- | --- |
| objectClass/objectInstance | It represents the capability to obtain the identification, in terms of objectClass and objectInstance, of alarmed network resource. |

11.2.2.1.4.5.3	Constraint

| Name | Definition |
| --- | --- |
| inv_relation-AI-ME | All AlarmInformation involved in this relationship with the same MonitoredEntity shall have at least one different value in the following attributes: alarmType, probableCause and specificProblem. |

###### 11.2.2.1.4.6	relation-backUpObject-AlarmInformation (O)

11.2.2.1.4.6.1	Definition

The relationship represents the relationship between AlarmInformation and the backUpObject.

11.2.2.1.4.6.2	Role

| Name | Definition |
| --- | --- |
| backUpObject | It represents a capability to obtain the identification, in terms of objectClass and objectInstance, of the backUpObject. |

11.2.2.1.4.6.3	Constraint

| Name | Definition |
| --- | --- |
| inv_identifyBackUpObject | This relationship is present if and only if the AlarmInformation.backedUpStatus attribute is present and is indicating true. |

##### 11.2.2.1.5	Information attribute definition

###### 11.2.2.1.5.1	Definition and legal values

| Name | Definition | Legal Values |
| --- | --- | --- |
| alarmId | It identifies one AlarmInformation in the AlarmList. |  |
| notificationId | It identifies the notification that carries the AlarmInformation. |  |
| alarmRaisedTime | It indicates the date and time when the alarm is first raised by the alarmed resource. | All values indicating valid date and time. |
| alarmChangedTime | It indicates the last date and time when the AlarmInformation is changed by the alarmed resource. Changes to AlarmInformation caused by invocations of the management service consumer would not change this date and time. | All values indicating valid date and time. |
| alarmClearedTime | It indicates the date and time when the alarm is cleared. | All values indicating valid date and time. |
| alarmType | It indicates the type of alarm. Communications Alarm: An alarm of this type is associated with the procedure and/or process required conveying information from one point to another (ITU-T Recommendation X.733 [4]). Processing Error Alarm: An alarm of this type is associated with a software or processing fault (ITU T Recommendation X.733 [4]). Environmental Alarm: An alarm of this type is associated with a condition related to an enclosure in which the equipment resides (ITU-T Recommendation X.733 [4]). Quality of Service Alarm: An alarm of this type is associated with degradation in the quality of a service (ITU T Recommendation X.733 [4]). Equipment Alarm: An alarm of this type is associated with an equipment fault (ITU-T Recommendation X.733 [4]). Integrity Violation: An indication that information may have been illegally modified, inserted or deleted. Operational Violation: An indication that the provision of the requested service was not possible due to the unavailability, malfunction or incorrect invocation of the service. Physical Violation: An indication that a physical resource has been violated in a way that suggests a security attack. Security Service or Mechanism Violation: An indication that a security attack has been detected by a security service or mechanism. Time Domain Violation: An indication that an event has occurred at an unexpected or prohibited time. |  |
| probableCause | It qualifies alarm and provides further information than alarmType. Probable causes are ouside the scope of the present document. |  |
| specificProblem | It provides further refinement to the probableCause. This attribute value shall be single-valued and of simple type such as integer or string. See definition in ITU-T Recommendation X.733 [4] clause 8.1.2.2. | Provided by vendor. |
| perceivedSeverity | It indicates the relative level of urgency for operator attention. | Critical, Major, Minor, Warning, Indeterminate, Cleared: see ITU-T Recommendation X.733 [4]. The present document does not recommend the use of indeterminate. |
| backedUpStatus | It indicates if an object (the MonitoredEntity) has a back up. See definition in ITU-T Recommendation X.733 [4] clause 8.1.2.4. | All values that carry the semantics of backedUpStatus defined by ITU-T X.733 [4] clause 8.1.2.4. |
| trendIndication | It indicates if some observed condition is getting better, worse, or not changing. | "Less severe", "no change", "more severe": see definition in ITU-T Recommendation X.733 [4] clause 8.1.2.6. |
| thresholdInfo | It indicates the crossed threshold information such as: - The identifier of the monitored attribute whose value has crossed a threshold, - The threshold settings, - The observed value that have crossed a threshold, etc. See definition in ITU-T Recommendation X.733 [4] clause 8.1.2.7. See also for information in TS 32.401 [19] clause 5.6. |  |
| stateChangeDefinition | It indicates attribute value changes associated with the alarm for state attributes of the monitored entity (state transitions). The change is reported with the name of the state attribute, the new value and an optional old value. See definition in ITU-T Recommendation X.733 [4] clause 8.1.2.11. |  |
| monitoredAttributes | It indicates attributes of the monitored entity and their values at the time the alarm occurred that are of interest for the alarm report. How these attributes are chosen is outside of the scope of the present document. See definition in ITU-T Recommendation X.733 [4] clause 8.1.2.11. |  |
| proposedRepairActions | Used if the cause is known and the system being managed can suggest one or more solutions to fix the problem causing the alarm as defined in ITU-T Rec. X. 733 [4] |  |
| additionalText | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4]. | N/A |
| additionalInformation | This attribute when present allows the inclusion of a set of vendor specific alarm information in the alarm. A specific condition for this optional population is when an alarm presented by the Management System (e.g. via the user interface) has different values of perceived severity, and / or alarm type, compared with the values presented to the Itf-N. Any other uses of additional information on the alarm and its semantics is outside the scope of the present document | The additional information field is a list of one or more information parts. The present document allows the support of two such information parts to carry - vendor defined perceived severity - vendor defined alarm type using defined identification. Other vendor specific information parts are allowed by using vendor specific identifications. |
| rootCauseIndicator | It indicates that this AlarmInformation is the root cause of the events captured by the notifications whose identifiers are in the related CorrelatedNotification instances. | boolean |
| ackTime | It identifies the time when the alarm has been acknowledged or unacknowledged the last time, i.e. it registers the time when ackState changes. | All values that indicate valid time that are later than that carried in alarmRaisedTime. |
| ackUserId | It identifies the last user who has changed the acknowledgement state. | It can be used to identify the human operator such as "John Smith" or it can identify a group, such as "Team Six", or it can contain no information such as "". |
| ackSystemId | It identifies the system that last changed the ackState of an alarm, i.e. acknowledged or unacknowledged the alarm. | It can be used to identify the system, such as "system 6" or it can contain no information such as "". |
| ackState | It identifies the acknowledgement state of an alarm. | Acknowledged: the alarm has been acknowledged. Unacknowledged: the alarm has been unacknowledged or the alarm has never been acknowledged. |
| commentTime | It carries the time when the comment has been added to the alarm. |  |
| commentText | It carries the textual comment. |  |
| commentUserId | It carries the identification of the user who made the comment. |  |
| commentSystemId | It carries the identification of the system (Management System) from which the comment is made. That system supports the user that made the comment. |  |
| clearUserId | It carries the identity of the user who invokes the clearAlarms operation. | It can be used to identify the human operator such as "John Smith" or it can identify a group, such as "Team Six", or it can contain no information such as "". |
| clearSystemId | It carries the identity of the system in consuming the fault management service. That management service consumer supports the user who invokes the clearAlarms(). | It can be used to identify the system, such as "system 6" or it can contain no information such as "". |
| serviceUser | It identifies the service-user whose request for service provided by the serviceProvider led to the generation of the security alarm. | This attribute may carry no information if the server user is not identifiable. |
| serviceProvider | It identifies the service-provider whose service is requested by the serviceUser and the service request provokes the generation of the security alarm. |  |
| securityAlarmDetector | It carries the identity of the detector of the security alarm. | This attribute may carry no information if the security alarm detector is not identifiable. |
| sourceObjectInstance | It identifies one MonitoredEntity. | All values that carry the semantics of DN. |
| notificationIdSet | It carries one or more notification identifiers. |  |

###### 11.2.2.1.5.2	Constraints

| Name | Definition |
| --- | --- |
| inv_alarmChangedTime | Time indicated shall be later than that carried in alarmRaisedTime. |
| inv_alarmClearedTime | Time indicated shall be later than that carried in alarmRaisedTime. |
| inv_ackTime | Time indicated shall be later than that carried in alarmRaisedTime. |
| inv_notificationId | NotificationIds shall be chosen to be unique across all notifications of a particular Managed Object throughout the time that alarm correlation is significant. The algorithm by which alarm correlation is accomplished is outside the scope of the present document. |

#### 11.2.2.2	Subscription information, subscription state and Information Object Classes

##### 11.2.2.2.1	Imported information entities and local labels

None.

##### 11.2.2.2.2	Class Diagram

###### 11.2.2.2.2.1	Attributes and relationships

This clause depicts the set of Support IOCs that encapsulate information within the notification IRP. The intent is to identify the information required for the notification IRP implementation of its operations and notification emission. This clause provides the overview of all Support IOCs in UML. Subsequent clauses provide more detailed specification of various aspects of these Support IOCs.

###### 11.2.2.2.2.2	Inheritance

This clause depicts the inheritance relationships that exist between Support IOCs.

##### 11.2.2.2.3	Information object classes definition

###### 11.2.2.2.3.1	NtfSubscriber

11.2.2.2.3.1.1	Definition

This Support IOC represents a Subscriber from a notification perspective: a subscriber is fully identified by a management service consumer reference. A management service consumer using multiple management service consumer reference attributes to subscribe will result in multiple NtfSubscriber instances.

11.2.2.2.3.1.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| ntfConsumerReference | M | M | M |

###### 11.2.2.2.3.2	NtfSubscription

11.2.2.2.3.2.1	Definition

This Support IOC represents a subscription that has been requested by a management service consumer and created.

11.2.2.2.3.2.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| ntfSubscriptionId | M | M | - |
| ntfSubscriptionState | M | M | M |
| ntfTimeTick | M | M | M |
| ntfTimeTickTimer | M | - | - |
| ntfNotificationCategorySet | M | M | M |
| ntfFilter | M | M | M |

11.2.2.2.3.2.3	Void

###### 11.2.2.2.3.3	NotificationIRP

11.2.2.2.3.3.1	Definition

This Support IOC represents a notification IRP. It inherits from Support IOC ManagedGenericIRP.

##### 11.2.2.2.4	Information relationship definitions

###### 11.2.2.2.4.1	relation-ntfSubscriber-ntfSubscription (M)

11.2.2.2.4.1.1	Definition

This relationship defines the relationship between a NtfSubscriber and its current subscriptions.

11.2.2.2.4.1.2	Roles

| Name | Definition |
| --- | --- |
| theNtfSubscriber | This role represents the one who has subscribed. It can be played by instances of Support IOC NtfSubscriber |
| theNtfSubscription | This role represents the subscriptions which were made and not unsubscribed. It can be played by instances of Support IOC NtfSubscription |

11.2.2.2.4.1.3	Constraints

| Name | Definition |
| --- | --- |
| inv_notificationCategoriesAllDistinct | The notification categories contained in the ntfNotificationCategorySet attribute of NtfSubscription playing the role theNtfSubscription are all distinct from each other. |

###### 11.2.2.2.4.2	relation-ntfIRP-ntfSubscriber (M)

11.2.2.2.4.2.1	Definition

This relationship defines the relationship between the NotificationIRP and the current subscribers of notifications.

11.2.2.2.4.2.2	Roles

| Name | Definition |
| --- | --- |
| theNtfSubscriber | This role represents the entities to which IRPAgent will notify events. It is played by instances of Support IOC NtfSubscriber |
| theNotificationIRP | This role represents the NotificationIRP to which an IRPManager has subscribed. It is played by instances of Support IOC NotificationIRP |

11.2.2.2.4.2.3	Constraints

| Name | Definition |
| --- | --- |
| inv_uniqueManagerReference | All NtfSubscriber involved in the subscriptionRegistration relationship are distinguished from each other by their ntfManagerReference Attribute. |

##### 11.2.2.2.5	Information attribute definitions

###### 11.2.2.2.5.0	Introduction

This clause defines the semantics of the Attributes used in Support IOCs.

###### 11.2.2.2.5.1	Definitions and legal values

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| ntfSubscriptionId | It identifies uniquely a subscription | N/A |
| ntfSubscriptionState | It indicates the activation state of a subscription | "suspended": the subscription is suspended "notSuspended": the subscription is active |
| ntfTimeTick | This attribute represents the initial value of ntfTimeTickTimer. It is in unit of whole minute. This value defines a time window within which management service consumer intends to invoke getSubscriptionStatus (or subscribe) operation to confirm its subscription. A special value indicates infinity which is such that timer will never expire and management service producer needs other means to decide when to delete resources allocated to the management service consumer | Integer greater or equal to 15, OR special infinite value |
| ntfTimeTickTimer | This attribute represents the current value of a timer | integer greater or equal to zero |
| ntfNotificationCategorySet | This attribute represents a set of notification categories (see also Definition of notification category in clause 3.1) |  |
| ntfFilter | This attribute represents the filter of a subscription. The filter can be applied to parameters of notification header (see Notificationmaanagement service producer interface) and to parameters of notifications defined as filterable to IManagement service producer shall notifymanagement service consumer if the event satisfies the filter constraint. |  |
| ntfConsumerReference | This attribute contains the reference of a consumer. It uniquely identifies a subscriber |  |

###### 11.2.2.2.5.2	Constraints

-	"ntfTimeTickTimer is lower.

## 11.3	Performance assurance

### 11.3.1	Operations and notifications

#### 11.3.1.1	Void

#### 11.3.1.2	Void

#### 11.3.1.3	Notification notifyThresholdCrossing

##### 11.3.1.3.1	Definition

A MnS producer sends this notification to subscribed MnS consumers when a "ThresholdMonitor" (3GPP TS 28.622 [11]) on that MnS producer detects the threshold crossing of a monitored performance metric.

##### 11.3.1.3.2	Notification information

| Parameter Name | S | Information Type | Comment |
| --- | --- | --- | --- |
| objectClass | M | ManagedEntity.objectClass | Class of the managed object, where the threshold crossing occurred. |
| objectInstance | M | ManagedEntity.objectInstance | Instance of the managed object, where the threshold crossing occurred. |
| notificationId | M | -- |  |
| notificationType | M | "notifyThresholdCrossing" |  |
| eventTime | M | -- | Time when the threshold crossing occurred. |
| systemDN | M | MnSAgent.objectInstance |  |
| observedPerfMetricName | M | ThresholdMonitor.thresholdInfoList[11].\ performanceMetrics[y] | Name of the performance metric that has crossed the threshold. |
| observedPerfMetricValue | M | -- | Value of the performance metric, that has crossed the threshold, when the threshold crossing was observed |
| observedPerfMetricDirection | M | -- | Direction ("UP" or "DOWN") of the performance metric, when the threshold crossing was observed |
| thresholdValue | M | ThresholdMonitor.thresholdInfoList[11].\ thresholdvalue | Threshold value of the triggered threshold |
| hysteresis | O | ThresholdMonitor.thresholdInfoList[11].\ hysteresis | Hysteresis of the triggered threshold |
| monitorGranularityPeriod | M | ThresholdMonitor.monitorGranularityPeriod | Granularity period of the threshold monitor |
| additionalText | O | -- | Vendor specific information |

### 11.3.2	Managed information

#### 11.3.2.1	Performance data file

##### 11.3.2.1.1	Void

##### 11.3.2.1.2	Performance data file content description

Table 11.3.2.1.2-1 provides the content definition of a performance data file.

Table 11.3.2.1.2-1: Performance data file content description

| File content item | Description |
| --- | --- |
| measDataFile | Top-level tag indicating the file contains performance metrics. Each file includes a header ("measFileHeader"), a collection of information elements with produced performance metrics and associated meta data ("measData") and a footer ("measFileFooter"). |
| measFileHeader | File header including the file format version, information about the sending node (DN, type and vendor) and a time stamp indicating the begin of the first granularity period contained in the file ("collectionBeginTime"). |
| measData | Information element containing the DN of the common root of the measured object instances ("measObjRootDn ") included in that information element, followed by a list of information elements containing the produced performance metrics and associated meta data ("measInfo"). A "MeasDataFile" contains zero, one or more "measData" elements. |
| measFileFooter | File footer with a time stamp indicating the end of the last granularity period contained in the file ("collectionEndTime"). |
| fileFormatVersion | File format version applied by the sender as indicated by the specific format version identifier provided for each version. |
| senderName | DN of the entity, that generated and sent the file. The entity is either a managed element represented by a "ManagedElement" or a management node represented by a "ManagementNode" |
| senderType | Type of the entity, that generated and sent the file, as defined in 3GPP TS 28.620 [y]. The type of a management node is "MANAGEMENT_NODE". |
| vendorName | Vendor of the the entity, that generated and sent the file. |
| collectionBeginTime | Time stamp indicating the begin of the first granularity period for which performance metrics are stored in the file. |
| measObjRootDn | DN of the measured object root. The measured object root is the first common object name-containing all objects that the metrics in one "measData" element are related to. When the metrics are produced by a managed element, the root object is the "ManagedElement" representing this managed element. When (aggregated) metrics are produced by a management node (based on input metrics from managed elements), such as metrics for sub-networks or network slices, the root object is the root "SubNetwork" of this management node. |
| measObjRootUserLabel | User label of the measured object root. |
| measObjRootSwVersion | Software version of the measured object root, allowing post-processing systems to take care of vendor specific performance metrics. It is either the software version of a managed element or of a management node. |
| measInfo | Information element added to "measData" for each expired granularity period, containing information on the produced performance metrics, starting with a time stamp ("measTimeStamp"), the granularity period ("granularityPeriod") and reporting period ("reportingPeriod") that are associated to the following performance metrics ("measValues"), for which is indicated the performance metric name, the measured or computed performance metric value and the object instance to which the performance metric is related to. |
| measInfoId | Identifier of a "measInfo". |
| jobId | Job identifier of the related "PerfMetricJob" in this "measInfo". |
| reportingPeriod | Period used for performance metric reporting in this "measInfo". Unit is seconds |
| granularityPeriod | Period used for performance metric production in a "measInfo". Unit is seconds. |
| measTimeStamp | End time of the granularity period in a "measInfo". |
| measTypes | Performance metric names in a "measInfo" |
| measValues | Performance metric values in a "measInfo". Each item in this list includes the LDN of the object the metrics are related to ("measObjLdn"), the measured or computed values of the metrics ("measResults") and a flag that indicates whether the metrics are reliable ("suspectFlag"). |
| measObjLdn | Local distinguished name (LDN) of the object the performance metrics are related to (measured object) within the scope defined by the "measObjRootDn". The concatenation of the "measObjRootDn" and the "measObjLdn" is the DN of the measured object. The "measObjLdn" is therefore empty if the "measObjRootDn" already specifies completely the DN of the measured object, which is the case for metrics associated to "ManagedElement" or the root "SubNetwork". For example, if the measured object is a "ManagedElement" representing RNC "RNC-Gbg-1", then the "measObjRootDn" may look like "DC=a1.operatorNN.com,SubNetwork=CountryNN,ManagedElement=RNC-Gbg-1" and the "measObjLdn" is empty. However, if the measured object is an "UtranCell" representing cell "Gbg-997" managed by that RNC, then the "measObjRootDn" is the same as above, i.e. "DC=a1.companyNN.com,SubNetwork=CountryNN,ManagedElement=RNC-Gbg-1" and the "measObjLdn" is "RncFunction=RF-1,UtranCell=Gbg-997". The class of the measured object is defined in item f) of measurement definitions (3GPP TS 32.404 [47], TS 28.552 [18]) and in item d) of KPI definitions (TS 28.554 [6]). |
| measResults | List of result values for the observed or computed performance metrics. The "measResults" sequence shall have the same number of elements and follow the same order as the "measTypes" sequence. The NULL value is reserved to indicate that the performance metric is not applicable or could not be produced for the object instance. |
| suspectFlag | Reliability of the performance metrics. FALSE means the metrics are reliable, TRUE means they are not reliable. The default value is "FALSE". |
| collectionEndTime | Time stamp indicating the end of the last granularity period for which performance metrics are stored in the file. |

The representation of all timestamps in PM files shall follow the representations allowed by the ISO 8601 [20]. 
The precise format for timestamp representation shall be determined by the technology used for encoding the PM file (e.g. ASN.1, XML DTD, and XML Schema). The choice of technology should ensure that this representation is derived from ISO 8601 [20]. Based on the representation used, the timestamp shall refer to either UTC time or local time or local time with offset from UTC.

##### 11.3.2.1.3	Void

###### 11.3.2.1.3.1	Void

###### 11.3.2.1.3.2	Void

##### 11.3.2.1.4	Performance data file naming convention

This clause defines a rule that shall be applied for constructing names for files containing performance data.

<Type><Startdate>.<Starttime>-[<Enddate>.]<Endtime>[_-<jobIdList>][_<UniqueIdList>][_-_<RC>]

1)	The "Type" field indicates if the file contains measurement results for single or multiple measured objects and/or granularity periods where:

-	"A" means single measured object, single granularity period (this is used when granularity period is equal to reporting period);

-	"B" indicates multiple measured objects, single granularity period (this is used when granularity period is equal to reporting period);

-	"C" signifies single measured object, multiple granularity periods (this is used when reporting period is multiples of the granularity period and will contain multiple measurement reports);

-	"D" stands for multiple measured objects, multiple granularity periods (this is used when reporting period is multiples of the granularity period and will contain multiple measurement reports).

2)	The "Startdate" field indicates the date when the granularity period began if the "Type" field is set to A or B. If the "Type" field is either "C" or "D" then "Startdate" contains the date when the first granularity period of the measurement results contained in the file started. The "Startdate" field is of the form YYYYMMDD, where:

-	YYYY is the year in four-digit notation;

-	MM is the month in two digit notation (01 - 12);

-	DD is the day in two-digit notation (01 - 31).

3)	The "Starttime" field indicates the time when the granularity period began if the "Type" field is set to A or B. If the "Type" field is either "C" or "D" then "Starttime" contains the time when the first granularity period of the measurement results contained in the file began. The "Starttime" field is of the form HHMMshhmm, where:

-	HH is the two-digit hour of the day (local time), based on 24-hour clock (00 - 23);

-	MM is the two digit minute of the hour (local time), based on 60-minutes clock (00 - 59);

-	s is the sign of the local time differential from UTC (+ or -), in case the time differential to UTC is 0 then the sign may be arbitrarily set to "+" or "-";

-	hh is the two-digit number of hours of the local time differential from UTC (00-23);

-	mm is the two digit number of minutes of the local time differential from UTC (00-59).

4)	The "Enddate" field shall only be included if the "Type" field is set to "C" or "D", i.e. measurement results for multiple granularity periods are contained in the file. It identifies the date when the last granularity period of these measurements ended, and its structure corresponds to the "Startdate" field.

5)	The "Endtime" field indicates the time when the granularity period ended if the "Type" field is set to A or B. If the "Type" field is either "C" or "D" then "Endtime" contains the time when the last granularity period of the measurement results contained in the file ended. Its structure corresponds to the "Starttime" field.

6)	The "UniqueIdList" field indicates the DNs of the measured objects.

7)	The "RC" field is a running count, starting with the value of "1", and shall be appended only if the filename is otherwise not unique, i.e. more than one file is generated and all other parameters of the file name are identical. Therefore it may only be used by the EM, since the described situation cannot occur with NE generated files. Note that the delimiter for this field, _-_, is an underscore character (_), followed by a minus character (-), followed by an underscore character (_).

8)	The "jobIdList" indicates the measurement job id(s) that the performance data file is associated with.

Some examples describing file-naming convention:

1)	file name:	A20000626.2315+0200-2330+0200_gNBId, 
meaning:	file produced for gNB <gNBId> on June 26, 2000, granularity period 15 minutes from 23:15 local to 23:30 local, with a time differential of +2 hours against UTC.

2)	file name:	B20021224.1700-1130-1705-1130_-job10_S-NSSAI,
meaning:	file containing results for multiple measured objects, generated for measurement job job10, produced for NSI <S-NSSAI> on December 24, 2002, granularity period 5 minutes from 17:00 local to 17:05 local, with a time differential of –11:30 hours against UTC.

3)	file name:	D20050907.1030+0000-20050909.1500+0000_SubnetworkId_-_2,
meaning:	file containing results subnetwork <SubnetworkId>, start of first granularity period 07 September 2005, 10:30 local, end of last granularity period 09 September 2005, 15:00 local, with a time differential of 0 against UTC. This is the second file for this subnetwork/granularity period combination.

4)	file name:	C20050907.1030+0000-20050909.1500+0000_gNBId,
meaning:	file produced for the gNB <gNBId>, start of first granularity period 07 September 2005, 10:30 local, end of last granularity period 09 September 2005, 15:00 local, with a time differential of 0 against UTC.

#### 11.3.2.1.4	Void

## 11.4	Heartbeat

### 11.4.1	Operations and notifications

#### 11.4.1.1	Notification notifyHeartbeat

##### 11.4.1.1.1	Definition

This notification notifies the subscribed consumer(s) that the MnS producer heartbeat period has expired or that a MnS consumer requested the emission of an immediate heartbeat notification.

The emission of heartbeat notifications is controlled by the HeartbeatControl IOC (3GPP TS 28.622 [11]).

##### 11.4.1.1.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | HeartbeatControl.objectClass |  |
| objectInstance | M | HeartbeatControl.objectInstance | Instance controlling the emission of this notifyHeartbeat notification. |
| notificationId | M | -- |  |
| notificationType | M | "notifyHeartbeat" |  |
| eventTime | M | -- | Time at which the notification is emitted. The semantics of Generalised Time specified by ITU-T [17] shall be used here. |
| systemDN | M | -- |  |
| heartbeatNtfPeriod | M | HeartbeatControl.heartbeatNtfPeriod |  |

##### 11.4.1.1.3	Triggering event

###### 11.4.1.1.3.1	From-state

stateBeforeHeartbeatNotification1 OR stateBeforeHeartbeatNotification2.

| Assertion Name | Definition |
| --- | --- |
| stateBeforeHeartbeatNotification1 | The internal countdown timer of the MOI emitting the notifyHeartbeat notification has reached the value ‘0’ (zero). |
| stateBeforeHeartbeatNotification2 | The value of the attribute triggerHeartbeatNtf of the MOI emitting the notifyHeartbeat notification is TRUE. |

###### 11.4.1.1.3.2	To-state

stateAfterOHeartbeatNotification1 OR stateAfterOHeartbeatNotification2.

| Assertion Name | Definition |
| --- | --- |
| stateAfterHeartbeatNotification1 | If From-state is stateBeforeHeartbeatNotification1 then: the internal countdown timer of the MOI is reset to the value of its heartbeatNtfPeriod attribute. |
| stateAfterHeartbeatNotification2 | If From-state is stateBeforeHeartbeatNotification2 then: the value of the internal countdown timer of the MOI is not affected. |

## 11.5	Streaming data reporting service

### 11.5.1	Operations and notifications

#### 11.5.1.1	establishStreamingConnection operation (M)

##### 11.5.1.1.1	Definition

This operation enables the MnS producer to establish a connection to the MnS consumer (i.e. streaming target). The connection establishement includes the exchange of meta-data (producer informs consumer about its own identity and the nature of the data to be reported via streaming) phase and the actual connection (a data pipe for streaming) establishment.

Established connection supports stream multiplexing (one connection supports one or more reporting streams simultaneously).

Upon successful connection establishment, the MnS consumer is aware of the MnS producer's identity, the list of reporting streams and the nature of data being reported on each of the streams.

The established connection may be kept "alive" either by built-in functionality of the solution set or by periodic reporting of empty stream data.

##### 11.5.1.1.2	Input parameters

| Parameter Name | S | Information type | Comment |
| --- | --- | --- | --- |
| producerId | M | The identity of the producer requesting the connection establishment. | DN of the MnS producer. If the MnS producer is not modeled as 3GPP NRM MOI, an alternative identifer other than DN may be used. |
| streamInfoList | M | List of StreamInfo | This parameter contains the list of meta-data about each reporting stream. For streaming trace reporting each StreamInfo includes: - StreamType carrying the value "TRACE"; - SerializationFormat carrying the value "GPB" or "ASN1"; - Trace Reference (see clause 5.6 of TS 32.422 [38]) as stream identifier; - TraceJob (see clause 4.3.30 of TS 28.622 [11]) providing the details about the configuration of the trace job for which the data is being reported. For streaming performance data reporting each StreamInfo includes: - StreamType carrying the value "PERFORMANCE"; - SerializationFormat carrying the value "GPB" or "ASN1"; - streamId globally unique stream identifier; - measObjDn: the DN of the measured object instance; - performanceMetrics: a list of performance metric names whose values are to be reported by the Performance Data Stream Units (see Annex C of TS 28.550 [42]) via this stream. Performance metrics include measurement and KPI; - either: - jobId defined in the PerfMetricJob MOI (see clause 4.3.31 of TS 28.622 [11]) for which the data is being reported; - or: - jobId globally unique identifier of a measurement job (see TS 28.550 [42]). For streaming analytics reporting each StreamInfo includes: - StreamType carrying the value "ANALYTICS"; - SerializationFormat carrying the value "GPB" or "ASN1"; - streamId globally unique stream identifier; - AnalyticsInfo providing the details about the analytics activity for which the data is being reported. For proprietary data streaming reporting each StreamInfo includes: - StreamType carrying the value "PROPRIETARY"; - streamId globally unique stream identifier; - VsDataContainer (see clause 4.3.9 of TS 28.622 [11]) providing the details about the data being reported. |

##### 11.5.1.1.3	Output parameters

| Parameter Name | S | Matching Information | Comment |
| --- | --- | --- | --- |
| connectionId | M | Identifier of the established streaming connection. | It identifies the established streaming connection. The format may have dependency on the solution set. |
| status | M | ENUM (Success, Failure) | An operation may fail because of a specified or unspecified reason. |

##### 11.5.1.1.4	Exceptions

| Exception Name | Definition |
| --- | --- |
| unexpectedStreams | Condition: Some information in the list of streamInfo was unexpected by the MnS consumer. Returned Information: Name of the exception; status is set to "Failure". |

#### 11.5.1.2	terminateStreamingConnection operation (M)

##### 11.5.1.2.1	Definition

This operation enables the MnS producer to terminate the connection to theMnS consumer (i.e. streaming target).

Upon successful termination of the streaming connection, the MnS producer stops reporting data to the MnS consumer on this connection.

##### 11.5.1.2.2	Input parameters

| Parameter Name | S | Information type | Comment |
| --- | --- | --- | --- |
| connectionId | M | See clause 11.5.1.1.3 | It identifies the streaming connection being terminated. The format may have dependency on the solution set. |

##### 11.5.1.2.3	Output parameters

| Parameter Name | S | Matching Information | Comment |
| --- | --- | --- | --- |
| status | M | ENUM (Success, Failure) | An operation may fail because of a specified or unspecified reason. |

##### 11.5.1.2.4	Exceptions

| Exception Name | Definition |
| --- | --- |
| unknownConnection | Condition: the connectionId is invalid. Returned Information: Name of the exception; status is set to "Failure". |

#### 11.5.1.3	reportStreamData operation (M)

##### 11.5.1.3.1	Definition

This operation enables the MnS producer to send a unit of streaming data to the MnS consumer.

##### 11.5.1.3.2	Input parameters

| Parameter Name | S | Information type | Comment |
| --- | --- | --- | --- |
| connectionId | M | See clause 11.5.1.1.3 | It identifies the streaming connection on which the reported data are being sent. The format may have dependency on the solution set. |
| streamingData | M | Unit of streaming data | This parameter contains the actual data (payload) being reported via stream. For streaming trace reporting each streamingData is encoded according to the format specified in the clause 5 of 3GPP TS 32.423 [39]. For streaming performance data reporting each streamingData is encoded according to the format specified in the Annex C of 3GPP TS 28.550 [42]. For proprietary data streaming reporting each streamingData is encoded according to the format specified in the product documentation. |

##### 11.5.1.3.3	Output parameters

| Parameter Name | S | Matching Information | Comment |
| --- | --- | --- | --- |
| status | M | ENUM (Success, Failure) | An operation may fail because of a specified or unspecified reason. |

##### 11.5.1.3.4	Exceptions

| Exception Name | Definition |
| --- | --- |

#### 11.5.1.4	addStream operation (M)

##### 11.5.1.4.1	Definition

This operation allows the MnS producer to add one or more reporting streams to an already established streaming connection.

##### 11.5.1.4.2	Input parameters

| Parameter Name | S | Information type | Comment |
| --- | --- | --- | --- |
| connectionId | M | See clause 11.5.1.1.3 | It identifies the streaming connection to which new reporting streams are being added. The format may have dependency on the solution set. |
| streamInfoList | M | List of StreamInfo | This parameter contains the list of meta-data about each reporting stream being added to the already established connection. For streaming trace reporting each StreamInfo includes: - StreamType carrying the value "TRACE"; - SerializationFormat carrying the value "GPB" or "ASN1"; - Trace Reference (see clause 5.6 of TS 32.422 [38]) as stream identifier; - TraceJob (see clause 4.3.30 of TS 28.622 [11]) providing the details about the configuration of the trace job for which the data is being reported. For streaming performance data reporting each StreamInfo includes: - StreamType carrying the value "PERFORMANCE"; - SerializationFormat carrying the value "GPB" or "ASN1"; - streamId globally unique stream identifier; - measObjDn: the DN of the measured object instance; - performanceMetrics: a list of performance metric (i.e. measurement or KPI) names whose values are to be reported by the Performance Data Stream Units (see Annex C of TS 28.550 [42]) via this stream; - either: - jobId defined in the PerfMetricJob MOI (see clause 4.3.31 of TS 28.622 [11]) for which the data is being reported; - or: - jobId globally unique identifier of a measurement job (see TS 28.550 [42]). For streaming analytics reporting each StreamInfo includes: - StreamType carrying the value "ANALYTICS"; - SerializationFormat carrying the value "GPB" or "ASN1"; - streamId globally unique stream identifier; - AnalyticsInfo providing the details about the analytics activity for which the data is being reported. For proprietary data streaming reporting each StreamInfo includes: - StreamType carrying the value "PROPRIETARY"; - streamId globally unique stream identifier; - VsDataContainer (see clause 4.3.9 of TS 28.622 [11]) providing the details about the data being reported. |

##### 11.5.1.4.3	Output parameters

| Parameter Name | S | Matching Information | Comment |
| --- | --- | --- | --- |
| streamInfoList | M | List of StreamInfo | This parameter contains the list of meta-data about each reporting stream that has been successfully added as a result of this operation. For streaming trace reporting each StreamInfo includes: - StreamType carrying the value "TRACE"; - SerializationFormat carrying the value "GPB" or "ASN1"; - Trace Reference (see clause 5.6 of 3GPP TS 32.422 [38]) as stream identifier; - TraceJob (see clause 4.3.30 of 3GPP TS 28.622 [11]) providing the details about the configuration of the trace job for which the data is being reported. For streaming performance data reporting each StreamInfo includes: - StreamType carrying the value "PERFORMANCE"; - SerializationFormat carrying the value "GPB" or "ASN1"; - streamId globally unique stream identifier; - measObjDn: the DN of the measured object instance; - performanceMetrics: a list of performance metric names whose values are to be reported by the Performance Data Stream Units (see Annex C of TS 28.550 [42]) via this stream. Performance metrics include measurement and KPI; - either: - jobId defined in the PerfMetricJob MOI (see clause 4.3.31 of 3GPP TS 28.622 [11]) for which the data is being reported; - or: - jobId globally unique identifier of a measurement job (see TS 28.550 [42]). For streaming analytics reporting each StreamInfo includes: - StreamType carrying the value "ANALYTICS"; - SerializationFormat carrying the value "GPB" or "ASN1"; - streamId globally unique stream identifier; - AnalyticsInfo providing the details about the analytics activity for which the data is being reported. For proprietary data streaming reporting each StreamInfo includes: - StreamType carrying the value "PROPRIETARY"; - streamId globally unique stream identifier; - VsDataContainer (see clause 4.3.9 of 3GPP TS 28.622 [11]) providing the details about the data being reported. |
| status | M | ENUM (Success, Failure, PartialSuccess) | An operation may fail because of a specified or unspecified reason. |

##### 11.5.1.4.4	Exceptions

| Exception Name | Definition |
| --- | --- |
| duplicateStream | Condition: One or more of stream identifiers in the streamInfoList already exist on this connection. Returned Information: Name of the exception; status is set to "Failure" or "PartialSuccess". |
| unexpectedStreams | Condition: Some information in the list of streamInfo was unexpected by the MnS consumer. Returned Information: Name of the exception; status is set to "Failure". |
| unknownConnection | Condition: the connectionId is invalid. Returned Information: Name of the exception; status is set to "Failure". |

#### 11.5.1.5	deleteStream operation (M)

##### 11.5.1.5.1	Definition

This operation allows the MnS producer to remove one or more reporting streams from an already established streaming connection.

##### 11.5.1.5.2	Input parameters

| Parameter Name | S | Information type | Comment |
| --- | --- | --- | --- |
| connectionId | M | See clause 11.5.1.1.3 | It identifies the streaming connection from which the reporting streams are being removed. The format may have dependency on the solution set. |
| streamIdList | M | List of stream identifiers | This parameter contains the list of identifiers for streams being removed from the already established connection. For streaming trace reporting Trace Reference (see clause 5.6 of 3GPP TS 32.422 [38]) is used as stream identifier. For streaming performance data reporting streamId globally unique stream identifier. For streaming analytics reporting streamId globally unique stream identifier. For proprietary data streaming reporting streamId globally unique stream identifier. |

##### 11.5.1.5.3	Output parameters

| Parameter Name | S | Matching Information | Comment |
| --- | --- | --- | --- |
| status | M | ENUM (Success, Failure, PartialSuccess) | An operation may fail because of a specified or unspecified reason. |

##### 11.5.1.5.4	Exceptions

| Exception Name | Definition |
| --- | --- |
| unknownStreamId | Condition: One or more of stream identifiers in the streamIdList does not exist on this connection. Returned Information: Name of the exception; status is set to "Failure" or "PartialSuccess". |
| unknownConnection | Condition: the connectionId is invalid. Returned Information: Name of the exception; status is set to "Failure". |

#### 11.5.1.6	getConnectionInfo operation (M)

##### 11.5.1.6.1	Definition

This operation enables the MnS producer to obtain information about one or more streaming connections from the MnS consumer.

##### 11.5.1.6.2	Input parameters

| Parameter Name | S | Information type | Comment |
| --- | --- | --- | --- |
| connectionIdList | M | List of streaming connection identifiers | This parameter contains the list of streaming connection identifiers for which the stream information is to be returned. The empty list indicates the stream information for all connections are to be returned. |

##### 11.5.1.6.3	Output parameters

| Parameter Name | S | Matching Information | Comment |
| --- | --- | --- | --- |
| connectionInfoList | M | List of <connectionId, streamReporter, streamIdList> tuples | This parameter contains the list of meta-data about each streaming connection requested by this operation. Each entry in this list is a tuple of connectionId, streamReporter and streamIdList. For streaming trace reporting: - streamReporter is the identity of the streaming data reporting MnS producer reporting data for this connectionId; - streamIdList is the list of Trace References (see clause 5.6 of 3GPP TS 32.422 [38]) used as stream identifiers. For streaming performance data reporting: - streamReporter is the identity of the streaming data reporting MnS producer reporting data for this connectionId; - streamIdList is the list of streamId globally unique stream identifiers. For streaming analytics reporting: - streamReporter is the identity of the streaming data reporting MnS producer reporting data for this connectionId; - streamIdList is the list of streamId globally unique stream identifiers. For streaming proprietary data reporting: - streamReporter is the identity of the streaming data reporting MnS producer reporting data for this connectionId; - streamIdList is the list of streamId globally unique stream identifiers. |
| status | M | ENUM (Success, Failure, PartialSuccess) | An operation may fail because of a specified or unspecified reason. |

##### 11.5.1.6.4	Exceptions

| Exception Name | Definition |
| --- | --- |
| unknownConnectionId | Condition: One or more of connection identifiers in the connectionIdList is not known to this MnS consumer. Returned Information: Name of the exception; status is set to "Failure" or "PartialSuccess". |

#### 11.5.1.7	getStreamInfo operation (M)

##### 11.5.1.7.1	Definition

This operation enables theMnS producer to obtain information about one or more reporting streams the MnS consumer.

##### 11.5.1.7.2	Input parameters

| Parameter Name | S | Information type | Comment |
| --- | --- | --- | --- |
| streamIdList | M | List of stream identifiers | This parameter contains the list of stream identifiers for which the stream information is to be returned. The empty list indicates the stream information for all streams are to be returned. For streaming trace reporting Trace Reference (see clause 5.6 of 3GPP TS 32.422 [38]) is used as stream identifier. For streaming performance data reporting streamId globally unique stream identifier. For streaming analytics reporting streamId globally unique stream identifier. For proprietary data streaming reporting streamId globally unique stream identifier. |

##### 11.5.1.7.3	Output parameters

| Parameter Name | S | Matching Information | Comment |
| --- | --- | --- | --- |
| streamInfoSumList | M | List of <StreamInfo, StreamReporters> tuples | This parameter contains the list of meta-data about each reporting stream requested by this operation. Each entry in this list is a tuple of StreamInfo and StreamReporters. For streaming trace reporting each StreamInfo includes: - StreamType carrying the value "TRACE"; - SerializationFormat carrying the value "GPB" or "ASN1"; - Trace Reference (see clause 5.6 of TS 32.422 [38]) as stream identifier; - TraceJob (see clause 4.3.30 of TS 28.622 [11]) providing the details about the configuration of the trace job for which the data is being reported. For streaming trace the StreamReporters is a list of the identities of the streaming data reporting MnS producer(s) reporting data for this Trace Reference to this MnS consumer. For streaming PM reporting each StreamInfo includes: - StreamType carrying the value "PERFORMANCE"; - SerializationFormat carrying the value "GPB" or "ASN1"; - streamId globally unique stream identifier; - measObjDn: the DN of the measured object instance; - performanceMetrics: a list of performance metric names whose values are to be reported by the Performance Data Stream Units (see Annex C of TS 28.550 [42]) via this stream. Performance metrics include measurement and KPI; - either: - jobId defined in the PerfMetricJob MOI (see clause 4.3.31 of TS 28.622 [11]) for which the data is being reported; - or: - jobId globally unique identifier of a measurement job (see TS 28.550 [42]). For streaming performance data the StreamReporters is a list of the identities of the streaming data reporting MnS producer(s) reporting data for this streamId to this MnS consumer. For streaming analytics reporting each StreamInfo includes: - StreamType carrying the value "ANALYTICS"; - SerializationFormat carrying the value "GPB" or "ASN1"; - streamId globally unique stream identifier; - AnalyticsInfo providing the details about the analytics activity for which the data is being reported. For streaming analytics the StreamReporters is a list of the identities of the streaming data reporting MnS producer(s) reporting data for this streamId to this MnS consumer. For proprietary data streaming reporting each StreamInfo includes: - StreamType carrying the value "PROPRIETARY"; - streamId globally unique stream identifier; - VsDataContainer (see clause 4.3.9 of TS 28.622 [11]) providing the details about the data being reported. For proprietary data streaming the StreamReporters is a list of the identities of the streaming data reporting MnS producer(s) reporting data for this streamId to this MnS consumer. |
| status | M | ENUM (Success, Failure, PartialSuccess) | An operation may fail because of a specified or unspecified reason. |

##### 11.5.1.7.4	Exceptions

| Exception Name | Definition |
| --- | --- |
| unknownStreamId | Condition: One or more of stream identifiers in the streamIdList is not known to this MnS consumer. Returned Information: Name of the exception; status is set to "Failure" or "PartialSuccess". |

## 11.6	File data reporting service

### 11.6.1	Operations and notifications

#### 11.6.1.1	Notification notifyFileReady

##### 11.6.1.1.1	Definition

A MnS producer sends this notification to subscribed MnS consumers when a new file becomes ready (available) on the MnS producer for upload by MnS consumers. The "fileInfoList" parameter provides information (meta data) about the new file and optionally, in addition to that, information about all other files, which became ready for upload earlier and are still available for upload when the notification is sent.

The "objectClass" and "objectInstance" parameters of the notification header identify the object representing the function (process) making the file available for retrieval, such as the "PerfMetricJob" or the "TraceJob" defined in TS 28.622 [11]. When no dedicated object is standardized or instantiated, the "ManagedElement", where the file is processed, shall be used. For the case that the file is processed on a mangement node, the "ManagementNode", where the file is processed, shall be used instead.

##### 11.6.1.1.2	Input parameters

| Parameter Name | S | Information Type | Comment |
| --- | --- | --- | --- |
| objectClass | M | Entity.objectClass | See clause 11.6.1.1.1 for the definition of Entity |
| objectInstance | M | Entity.objectInstance | See clause 11.6.1.1.1 for the definition of Entity |
| notificationId | M | -- |  |
| notificationType | M | "notifyFileReady" |  |
| eventTime | M | -- | Time when the file, that triggered this notification, was ready for upload. |
| systemDN | M |  |  |
| fileInfoList | M | List of struct < fileLocation (M), fileCompression (M), fileSize (O), fileDataType (M), fileFormat (M), fileReadyTime (O), fileExpirationTime (O), …jobId (CO) > Each element is defined as following: - "fileLocation": Location of the file. The location may be a directory path or a URL, for example "\\202.112.101.1\D:\user\Files\<xxx>", or "ftp://nms.telecom_org.com/datastore/<xxx>, where <xxx> is the filename. - "fileCompression": Name of the algorithm used for compressing the file. An empty or absent "fileCompression" parameter indicates the file is not compressed. The MnS producer selects the compression algorithm. It is encouraged to use popular algorithms such as GZIP. - "fileSize": Size of the file. Its value is a non negative integer. The unit is byte. - "fileDataType": Type of the management data stored in the file. Allowed values are: - "PERFORMANCE" - "TRACE" - "ANALYTICS" - "PROPRIETARY" The value "PERFORMANCE" refers to measurements and KPIs. - "fileFormat": Identifier of the XML or ASN.1 schema (incl. its version) used to produce the file content. - "fileReadyTime": Date and time when the file was closed (the last time) and made available on the MnS producer. The file content will not be changed anymore. - "fileExpirationTime": Date and time after which the file may be deleted. It shall not be empty and shall be later than "fileReadyTime". - "jobId": Job identifier of the "PerfMetricJob" (TS 28.622 [11]) or "TraceJob" (TS 28.622 [11]) that produced the file. This parameter should be present, when the file is related to a job and that job is represented by a "PerfMetricJob" or "TraceJob". Multiple jobs may share the same job identifier. This may for example be the case for jobs collecting measurements to compuate a KPI or for jobs related to a specific task in some analytics application. Note that a specific job is identified by the objectClass/objectInstance parameters of the notification header. | Information (meta data) about the new file, that became ready for upload and triggered this notification, and information about files, which became ready for upload earlier and are still available for upload when the notification is sent. |
| additionalText | O | -- | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] |

#### 11.6.1.2	Notification notifyFilePreparationError

##### 11.6.1.2.1	Definition

A MnS producer sends this notification to subscribed MnS consumers when an error occurs while preparing a file. For many error reasons, such as low memory or hard disk full, it is very likely that all ongoing file preparation processes fail at the same time. For that reason, it is possible to report with this notification that multiple file preparation processes failed.

In case the MnS producer keeps the file, where an error occurred during preparation, the "fileInfoList" parameter contains a list item with information about that file, otherwise, if the file is deleted or not created at all, the "fileInfoList" parameter has no list item related to that file.

##### 11.6.1.2.2	Input parameters

| Parameter Name | S | Information Type | Comment |
| --- | --- | --- | --- |
| objectClass | M | Entity.objectClass. | See clause 11.6.1.1.1 for the definition of Entity |
| objectInstance | M | Entity.objectInstance | See clause 11.6.1.1.1 for the definition of Entity. |
| notificationId | M | -- | See Table 11.6.1.1.2-1. |
| notificationType | M | "notifyFilePreparationError" |  |
| eventTime | M | -- | Time when the file preparation error occured |
| systemDN | M |  |  |
| fileInfoList | M | See Table 11.6.1.1.2-1. | Each list item contains information about a file where a file preparation error occurred and that is kept on the MnS producer. Files, that are deleting or not created at all, have no list item. |
| reason | M | -- | Detailed error reason, including - errorInPreparation - hardDiskFull - hardDiskFailure - tooManyFiles - collectionTimeOut - incompleteTruncatedFile - corruptedFile - lowMemory - dataNotAvailable |
| additionalText | O | -- | Allows a free form text description to be reported as defined in ITU-T Rec. X. 733 [4] |

#### 11.6.1.3	Operation subscribe

##### 11.6.1.3.1	Definition

This operation allows a MnS consumer to subscribe to the notifications of the file data reporting service producer.

##### 11.6.1.3.2	Input parameters

| Parameter Name | S | Information Type | Comment |
| --- | --- | --- | --- |
| consumerReference | M | Reference (address) of the MnS consumer to which the notifications shall be sent. |  |
| timeTick | O | Initial value of a timer held by the MnS producer. This value defines the time window within which the MnS consumer intends to invoke the "subscribe" operation again to confirm its subscription. The value "0" shall indicate infinity. In this case the subscription is not terminated by the MnS producer. Unit is minutes |  |
| filter | O | Filter constraint that the MnS producer shall use to filter notifications. The filter can be applied to all parameters of a notification The filter constraint grammar is solution set dependent |  |

##### 11.6.1.3.3	Output parameters

| Parameter Name | S | Matching Information | Comment |
| --- | --- | --- | --- |
| subscriptionId | M | Unambiguous identity of this subscription. |  |
| status | M | ENUM (OperationSucceeded, OperationFailedExistingSubscription, OperationFailed) | If subscription is successfully created, status = OperationSuceeded. If subscription is not created because it is duplicated or conflict with existing subscription(s), status = OperationFailedExistingSubscription If the operation is failed for any other reason than being duplicated or conflict with existing subscription(s), status = OperationFailed. |

##### 11.6.1.3.4	Exceptions

| Name | Definition |
| --- | --- |
| operation_failed_existing_subscription | Condition: The subscription is duplicated or conflict with existing subscription(s) Returned Information: The output parameter status |
| operation_failed | Condition: The operation failed for any other reason than being duplicated or conflict with subscription(s) Returned Information: The output parameter status |

#### 11.6.1.4	Operation unsubscribe

##### 11.6.1.4.1	Definition

This operation allows a MnS consumer to cancel subscription(s) at a MnS producer.

A MnS consumer can cancel one subscription made with a "consumerReference" by providing the corresponding "subscriptionId" or all subscriptions made with the same "consumerReference" by leaving the "subscriptionId" parameter absent.

##### 11.6.1.4.2	Input parameters

| Parameter Name | S | Information Type | Comment |
| --- | --- | --- | --- |
| consumerReference | M | Reference of the MnS consumer whose subscriptions are to be cancelled. | The format of the reference may have dependency on the solution set. |
| subscriptionId | O | Subscription id returned in the subscribe operation response | If this parameter is absent, all subscriptions made with the same "consumerReference" shall be cancelled. |

##### 11.6.1.4.3	Output parameters

| Parameter Name | S | Matching Information | Comment |
| --- | --- | --- | --- |
| status | M | ENUM (OperationSucceeded, OperationFailed) | If subscription(s) as identified in the input parameter are cancelled, status = OperationSucceeded. If the operation is failed, status = OperationFailed. |

##### 11.6.1.4.4	Exceptions

| Name | Definition |
| --- | --- |
| operation_failed | Condition: the operation is failed Returned Information: The output parameter status |

#### 11.6.1.5	Operation listAvailableFiles

##### 11.6.1.5.1	Definition

This operation allows a MnS consumer to retrieve a list of files available for upload on a MnS producer. The request message contains the file data type of the files, that shall be listed in the response. In addition to that it is possible to specify that only files shall be included in the response whose file ready time falls into a specific time window defined by the "beginTime" and "endTime" input parameters.

##### 11.6.1.5.2	Input parameters

| Parameter Name | S | Information type | Comment |
| --- | --- | --- | --- |
| fileDataType | M | It specifies the type of the management data stored in the file. | For performance data (including measurement data and KPI) files, the value is assigned to "PERFORMANCE". For trace data files, the value is assigned to "TRACE". For analytic data files, the value is assigned to "ANALYTICS". For proprietary data files, the value is assigned to "PROPRIETARY". |
| beginTime | M | The consumer requests to list information about the available file(s) whose ready time(s) are later or equal to this time. This parameter is expressed in UTC time. | This parameter indicates date and time. If this parameter is empty or absent, no restriction on begin time is applied on the file ready time. |
| endTime | M | The consumer requests to list information about the available file(s) whose ready time(s) are earlier than this time. This parameter is expressed in UTC time. | This parameter indicates date and time. If this parameter is empty or absent, no restriction on end time is applied on the file ready time. |

##### 11.6.1.5.3	Output parameters

| Parameter Name | S | Matching Information | Comment |
| --- | --- | --- | --- |
| fileInfoList | M | See "fileInfoList" defined in notifyFileReady notification (clause 11.6.1.1.1) |  |
| status | M | ENUM (Success, Failure) |  |

##### 11.6.1.5.4	Exceptions

| Exception Name | Definition |
| --- | --- |
| invalidTimes | Condition: Either "beginTime" or "endTime" is invalid. Returned information: output parameter status is set to Failure. |

### 11.6.2	File transfer protocols

The MnS producer shall support at least one of the following file transfer protocols:

-	SFTP;

-	FTPES,

-	HTTPS.

The MnS producer shall always act as the server while the MnS consumer shall always act as the initiator (client) of file transfer actions.

End of Modified Section