3GPP TSG-SA Meeting #100	SP-230694

Taipei, 12th – 16th June 2023

3GPP TSG-SA WG6 Meeting #55	S6-232215

Berlin, Germany 22nd – 26th May 2023	(revision of S6-232070)

# Source:	SA6

# Title:	New WID on Railways specific Enhancements to Mission Critical Services

# Document for:	Approval

# Agenda Item:	6.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Railways specific Enhancements to Mission Critical Services

Acronym :	enh4FRMCS

Unique identifier :	1000038

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-19

# Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  |  |
| Don’t know |  |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 950025 | Interconnection and Migration Aspects for Railways | SA6 work on interconnection and migration in Rel-18 |
| 930017 | Gateway UE function for Mission Critical Communication | SA6 work on the gateway UE in Rel-18 |
| 980051 | Mission Critical ad hoc group Communications | SA6 work on ad hoc group communications in Rel-18 |

# 3	Justification

The present level on Mission Critical interconnection, migration, interworking, ad hoc group communication and gateway UE functionality has not fully addressed the needs for railway communications. Trains are crossing the border much more frequent than in public safety scenarios and MC services should not be impacted. Interworking with GSM-R is important during the transitions phase towards mission critical communication. For emergency situations additional ad hoc group communication functionalities are required, e.g., multi-talker support. The use of MC gateway UEs is needed for connecting existing railway equipment to the mission critical system and interconnection and migration aspects were not fully addressed.

The work is built on Rel-18 achievements and aims to complete and enhance several aspects which could not be finished within Rel-18 timeframe.

# 4	Objective

Considering the use cases and the requirements from 3GPP SA1 (and possibly from other 3GPP groups and different SDOs) and the results achieved in Rel-18, the objectives of the work include:

- Specify several enhancements on ad hoc group communications, new procedures for all MC services to cover the gaps if any identified and ad hoc group emergency alerts (e.g., enhanced location information, migration, multi-talker feature, change criteria during an active communication, addition information IE, recording, merging of ad hoc groups)

- Specify the architecture and procedures to support interworking with GSM-R for MCPTT private calls, MCPTT ad hoc group calls, MCData SDS and automatic subsequent MCPTT ad hoc group calls following an ad hoc group emergency alert.

- Several enhancements and corrections to existing procedures to support Interconnection and Migration for Railways (e.g., functional alias, multi-talker feature)

- Further enhancements to the existing architecture and procedures to support MC gateway Ues, e.g., migration, interconnection, support of multiple gateway Ues per service.

- Specify the architecture and procedures to support the separation of the signalling and the media paths within an MC system.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.280 | Enhancements on ad hoc group communication, MC gateway UE and media path optimisation. | SA#102 (December 2024) |  |
| TS 23.281 | Enhancements for ad hoc group communication, interconnection, and migration | SA#102 (December 2024) |  |
| TS 23.282 | Enhancements for ad hoc group communication, interconnection, and migration | SA#102 (December 2024) |  |
| TS 23.379 | Enhancements for ad hoc group communication, interconnection, and migration | SA#102 (December 2024) |  |
| TS 23.283 | Interworking with GSM-R | SA#102 (December 2024) |  |
| TS 23.289 | Mission Critical services over 5G System | SA#102 (December 2024) |  |

# Work item Rapporteur(s)

Oettl, Martin, Nokia, martin.oettl@nokia.com

# Work item leadership

SA6

# 8	Aspects that involve other WGs

EPS/5GS transport aspects will involve SA2. Security aspects will involve SA3 and SA5 for management aspects.

# 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell

UIC

Kontron Transportation France

Ericsson

Samsung

Motorola Solutions

BDBOS

Airbus

Netherlands Police

AT&T

Huawei