3GPP TSG-SA6 Meeting #55	S6-231919

# Berlin, Germany, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0402 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EEC subscribing to SEAL notification service and requesting UE ID in ACR to CAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-16 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To facilitate future ACRs from the CAS back to EAS, Enabling the EEC to request an AF-specific UE ID or Edge UE ID from the S-EES when there is an ACR to a CAS to facilitate the ACR in the way back. This ID can be used afterwards by the CAS to perform EAS discovery on the S-EES when there is an ACR from CAS to EDN. The EEC should also subscribe to notifications from the S-EES via the EEL to receive ACR related notifications in case the EEC is not reachable by the S-EES anymore. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is specified that for all scenarios of EAS to CAS ACR, the EEC may request an AF-specific ID or Edge UE ID from the S-EES and subscribe to notifications from the S-EES to receive ACR related notifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CAS does not have a UE ID to use for EAS discovery in an ACR from CAS to EAS, and the EEC may not be able to receive ACR related notifications from the S-EES. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.2A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * Change 1 * * * *

### 8.8.2A	Scenarios for ACR between EAS and CAS

#### 8.8.2A.1	General

Service continuity between CAS and EAS is supported without CES, corresponding to the architecture options described in clause 6.DD.

To facilitate future ACRs from the CAS back to EAS, the EEC may request and receive an AF-specific UE ID or Edge UE ID from the S-EES as described in clause 8.6.5, which can be used by the CAS to perform EAS discovery on the S-EES when there is an ACR from CAS back to an EAS. The EEC may also request to subscribe to notifications from the S-EES via the SEAL notification management service as described in clause 9.1 to receive ACR notifications if the EEC is no longer within reach from the S-EES.

# * * * End of Change * * * *