3GPP TSG-SA WG6 Meeting #55	S6-232100

Berlin, Germany 22nd – 26th May 2023	(revision of S6-231922)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0404 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving the ENs related to retrieve EES procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-16 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Removing the EN introduced by S6-231648 as it is already resolved by S6-231649: Editor’s Note: It is FFS whether clause 8.8.3.3 of TS 23.558 can be used to determine target EESs for Common EAS announcement if the message does not include a list of EESs for a group. Resolving the EN introduced by S6-231649: Editor's Note: The wording "with same service" is to be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the EN introduced by S6-231648 and resolving the EN introduced by S6-231649. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification retains the EN and is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.15.2.2, 8.8.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is dependant on SA6#54-e agreed CRs in S6-231649 and S6-231648. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.15.2.2	EAS Information provisioning

Pre-conditions:

1.	The EEC has performed service provisioning procedure

2.	The EEC has performed the EAS discovery procedure

Figure 8.15.2.2-1: EAS information provisioning

1.	The EEC sends the EAS information provisioning request to the EES:

a-	"ACR scenario selection announcement". The request may include ACR scenario list selected by the EEC, EEC security credentials, selected EASID, selected EAS endpoint, EECID and ACID. For the EAS bundles scenario, the request may include the ACR scenario list for EAS bundles selected by the EEC.

b-	"ACR scenario selection request". The request may include AC profile, EEC service continuity support, EEC security credentials, EECID and ACID.

c- " EAS selection request". Informs the EES of EAS selection for Application Group. The request message may contain the list of EESs for a certain Application Group ID, received from the ECS in the service provisioning response message. This application group related information may be used for further common EAS announcement(s) between EES(s). In the case of no Central Repository, if the request message does not contain the list of EESs for a certain Application Group ID, then the EES determines the other EESs to which announce common EAS request needs to be sent as described in clause 8.8.3.3. If there is Central Repository, the EEC selected EAS is used in interaction between the EES and Central Repository, and EEC receives common EAS in the EAS information provisioning response.

Editor’s Note: It is up to implementation how it is determined that EES no longer serves the application group.

Editor’s Note: For the “EAS selection” option, how the described functionality can be better captured in steps 1 and 2 is FFS.

If the EEC has selected an uninstantiated EAS, the EEC includes the selected EASID without including the selected EAS endpoint in the request.

2.	Upon receiving the request from the EEC, the EES validates the EEC information request and verifies if the EEC is authorized for this operation.

a-	"ACR scenario selection announcement". The EES may send the ACR Selection notification to the selected EAS if the EAS has subscribed and if EES allows EEC based ACR scenario selection. Otherwise, EES may respond with status failure and include appropriate reason. For the EAS bundles scenario, the EES may send the ACR selection notification to the bundled EAS(s).

b-	"ACR scenario selection request". The EES selects the ACR scenario list and may send the ACR Selection notification to the selected EAS if the EAS has subscribed. The EES may include the ACR scenario list in the EAS information provisioning response. For the EAS bundles scenario, the EES selects the ACR scenario list for EAS bundles based on the AC/EEC/EES/EAS(s) service continuity support, and sends the ACR scenario list to the bundled EAS(s).

If the request contains selected EAS ID and selected EAS Endpoint, the EES may apply the EAS traffic influence with the N6 routing information of the EAS in the 3GPP Core Network, based on application KPIs and if the EAS traffic influence was not done before (e.g. neither in EAS discovery procedure nor the EAS perform traffic influence).

If the request contains the selected EASID and the selected EAS endpoint is not included, the EES verifies if instantiation of EAS is needed and may trigger the EAS management system to instantiate the EAS as in clause 8.12.

When the request contains EEC Service Continuity Support IE and the EEC context has been established, the EES includes the IE into the EEC context described in Table 8.2.8-1.

NOTE 1:	EES can also influence the EAS traffic in advance.

NOTE 2:	It is up to the AC to decide when to connect to the selected EAS (either immediately or wait for a while) once the AC knows the selected EAS.

3.	If the processing of the request was successful, the EES sends an EAS information provisioning response to the EEC indicating a successful status. If an EEC context has been established, the response also includes the list of selected ACR scenario(s) into the session context IE within EEC context as described in Table 8.2.8-2; otherwise, the EES shall indicate a failure status and include appropriate reasons. If the EES has triggered EAS instantiation based on the EAS information provisioning request and obtained the newly instantiated EAS information, the response contains information about the newly instantiated EAS, including the EAS endpoint information.

The EEC, EES and EAS (or the bundled EAS(s)) use the selected ACR scenario list to determine if they should perform ACR detection and/or ACR decision.

Upon receiving the EAS information provisioning response, if the response includes instantiated EAS information, the EEC uses the endpoint information to subscribe to ACR event notification, as needed, and provides necessary notifications to the AC.

NOTE 3:	Other ACR selection criteria are out of scope of the current specification.

NOTE 4:	The common supported ACR scenarios is decided as part of the EAS discovery and selection procedure.

For the "EAS selection" case, in the case of no Central Repository, the selected common EAS shall be announced to other relevant EES(s) as per procedure in clause 8.X.

Editor’s Note: How the EES informs ECS that it serves an Application group ID is FFS

* * * Next Change * * * *

#### 8.8.3.3	Retrieve T-EES procedure

Figure 8.8.3.3-1 illustrates the procedure for the S-EES to retrieve the T-EES information from the ECS.

Pre-condition:

1.	The S-EES has been pre-configured with the address of the ECS; and

2.	The AC at the UE already has on-going application traffic with the S-EAS.

Figure 8.8.3.3-1: Retrieve T-EES procedure

1.	The S-EES sends the Retrieve EES request (UE location information or UE identity, EASID of the S-EAS, target DNAI and UE connectivity information) to the ECS in order to identify the T-EES which has an EAS available to serve the given AC in the UE. For obtaining announcement EES list, the Retrieve EES request includes application group id.

2.	If the request contains the UE identity (e.g. GPSI) but the UE location is not known to the ECS, then the ECS interacts with 3GPP core network to retrieve the UE location. The ECS determines T-EES(s) as per the parameters (e.g. EASID, target DNAI) in the request and the UE location information.

If no central repository is available and when the Retrieve EES request includes application group id to the ECS then the request EES list retrieval is for the announcement of common EAS, ECS determines the list of EESs serving the EASs (with same EASID) for the Group ID included in the Retrieve EES request.

If the ECS does not identify any suitable EES(s) based on EDN configuration available at the ECS and UE’s location, the ECS determines a partner ECS that may satisfy the requirements. Based on ECSP policy, the ECS may use preconfigured or OAM configured information about the partner ECSs or ECS discovery via ECS-ER as specified in clause 8.x.2.3 or both.

If required by the ECSP policies, the ECS may use service provisioning information retrieval procedure as specified in clause 8.X.2.4 to obtain seivice provisioning information from the partner ECS.

3.	The ECS sends the Retrieve EES response. If the ECS has determined the EDN configuration information, the retrieve EES response includes the list of EDN configuration information to the S-EES. The list of EDN configuration information includes the EDN details with the endpoint information of T-EES(s) as described in table 8.3.3.3.3-2. If the ECS has determined suitable partner ECS(s) instead, the retrieve EES response includes a list of ECS configuration information.

If the retrieve EES response contains a list of ECS configuration information, the S-EES may initiate retrieve T-EES procedure with one or more ECS(s) listed in the retrieve EES response.

NOTE:	The Retrieve EES request initiated by the S-EES can be restricted only to its registered ECS.

* * * End of Changes * * * *