3GPP TSG-SA WG6 Meeting #55	S6-231985

Berlin, Germany 22nd – 26th May 2023	(revision of S6-231937)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0309 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Example of MCData services which are not handled by SIP core |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | 2023-05-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The example provided in clause 7.12.2 regarding the MCData services which are not handled by the SIP core may not be an accurate one. The clause itself may lead to misunderstanding. Therefore, the CR proposes to remove this clause, and rather focus on the procedures themselves. The clauses title of 7.5.2.2.2 and 7.5.2.3.2 includes the word “without QoS” to indicate the procedures that does not require QoS hence SIP core. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clause 7.12.2 is removed - The title of clauses 7.5.2.2.2 and 7.5.2.3.2 are updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion may occur that IP connectivity service does not require SIP core. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.12.2, 7.5.2.2.2 and 7.5.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 7.12.2	Void

# * * * * Second change* * * *

##### 7.5.2.2.2	Procedure for uploading the file residing in the local storage of the MCData UE without QoS

The procedure in figure 7.5.2.2.2-1 describes the case where an MCData user is uploading a file to media storage function on the MCData content server.

Pre-conditions:

1.	The MCData user on the media storage client is registered for receiving MCData service.

2.	The MCData content server has the ability to verify if the requesting MCData user is authorised to upload.

Figure 7.5.2.2.2-1: Uploading of the file residing in MCData UE using HTTP

1.	The user at the media storage client initiates a file upload request of the chosen file. If MCData emergency state is already set for the media storage client (due to previously triggered MCData emergency alert), the media storage client sets emergency indicator in the request. The media storage client verifies that the size of the file is within the maximum data size for FD for the intended MCData FD request (by checking the group configuration for a group FD request and by checking the service configuration for a one-to-one FD request).

2.	The file to be uploaded is received by the media storage client and sent to the media storage function on the MCData content server for storing using the MCData upload data request.

3.	The MCData content server stores the file and provides a MCData upload data response indicating success (along with file URL to the media storage client) or failure.

# * * * * Third change* * * *

##### 7.5.2.3.2	Procedure for file download from the MCData content server without QoS

The procedure in figure 7.5.2.3.2-1 describes the case where an MCData user is downloading a file from the media storage function of the MCData content server.

Pre-conditions:

1.	The MCData user on the media storage client is registered for receiving MCData service.

Figure 7.5.2.3.2-1: File download using HTTP

1.	The user at the media storage client initiates a file download request available at the indicated URL.

2.	The file available at the URL (received in MCData FD request or MCData group standalone FD request) is requested to be downloaded by the media storage client from the media storage function on the MCData content server using a MCData download data request. If emergency indicator is set in received in MCData FD request or MCData group standalone FD request, the media storage client sets emergency indicator in MCData download data request.

NOTE:	The media storage client can perform partial download requests to complete the missing parts after an incomplete file transfer.

3.	The media storage function on the MCData content server may apply reception control policy and provides a MCData download data response including the file to the media storage client.

# * * * * End of changes* * * *