3GPP TSG-SA WG6 Meeting #55	S6-231992

Berlin, Germany 22nd – 26th May 2023	(revision of S6-231714, S6-231982)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0349 | rev | 2 | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct MCPTT ID usage in some information flow tables |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | at&t |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC |  |  |  |  |  | Date: |  |  | 2023-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MCPTT ID is used in many information flow tables. What the MCPTT ID represents in these information tables depends on what the message is defined. Many of the descriptions are not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the descriptions in the relevant information tables. When a request message is sent from MCPTT server to MCPTT client, it is sent to the target user of the group, not the call initiator. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Will give the wrong guidance to stage 3 development. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.2.2.9, 10.6.2.2.13, 10.6.2.2.27, 10.19.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 10.6.2.2.9	Group call request (MCPTT server – MCPTT client)

Table 10.6.2.2.9-1 describes the information flow group call request from the MCPTT server to the MCPTT client.

Table 10.6.2.2.9-1 Group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| MCPTT ID | M | The identity of the MCPTT user towards which the request is sent |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is initiated |
| SDP offer | M | Media parameters of MCPTT server |
| Broadcast indicator | O | Indicates that the group call request is for a broadcast group call |
| Remotely initiated call request indicator | O | Indicates that the MCPTT group call request is a result of receiving of a remotely initiated call request and may be included only for remotely initiated call |

* * * Next Change * * * *

##### 10.6.2.2.13	Group call notify (MCPTT server – MCPTT client)

Table 10.6.2.2.13-1 describes the information flow group call notify from the MCPTT server to the MCPTT client.

Table 10.6.2.2.13-1 Group call notify information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| MCPTT ID | M | The identity of the MCPTT user towards which the notify is sent |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is requested |
| MCPTT ID list (see NOTE) | O | The list of the affiliated MCPTT group members who did not acknowledge the group call request |
| NOTE: Only applicable to acknowledged group calls. |  |  |

* * * Next Change * * * *

##### 10.6.2.2.27	Group-broadcast group call request (MCPTT server – MCPTT client)

Table 10.6.2.2.27-1 describes the information flow group-broadcast group call request from the MCPTT server to the MCPTT client.

Table 10.6.2.2.27-1 Group-broadcast group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| MCPTT ID | M | The identity of the MCPTT user towards which the request is sent |
| Functional alias | O | The functional alias of the calling party |
| MCPTT group ID | M | The MCPTT group ID of the group on which the call is requested |
| SDP offer | M | Media parameters of MCPTT clients |
| Broadcast indicator | M | Indicates that the group call request is for a broadcast group call |

* * * Next Change * * * *

#### 10.19.2.4	Ad hoc group call request (MCPTT server – MCPTT client)

Table 10.19.2.4-1 describes the information flow ad hoc group call request from the MCPTT server to the MCPTT client.

Table 10.19.2.4-1 Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| MCPTT ID | M | The identity of the MCPTT user towards which the request is sent |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID | M | The MCPTT group ID to be associated with the ad hoc group call |
| SDP offer | M | Media parameters of MCPTT server |
| Broadcast indicator (see NOTE ) | O | Indicates that the ad hoc group call request is for a broadcast ad hoc group call |
| Imminent peril indicator (see NOTE ) | O | Indicates that the ad hoc group call request is an MC service imminent peril call |
| Emergency Indicator (see NOTE ) | O | Indicates that the ad hoc group call request is an MC service emergency call |
| Preconfigured ad hoc group identity | O | Group identity whose configuration is to be applied for this ad hoc group call. |
| NOTE: If used, only one of these information elements is present. |  |  |