3GPP TSG-SA WG6 Meeting #55	S6-231954

Berlin, Germany 22nd – 26th May 2023	(revision of S6-231772)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0353 | rev | 1 | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | User regroup MCPTT ID list update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Motorola Solutions |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | 2023-05-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The CR introduces a new procedure to update the MCPTT ID list within an ongoing preconfigured user regroup. The introduced procedure includes the case when a new user matches the predefined MCPTT user criteria, or another no longer matches the criteria. The MCPTT user criteria is known at the MCPTT server as described in clause 10.6.2.2.34. The introduced update matches stage 1 requirement [R-6.6.4.2-002b] in 3GPP TS 22.280. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the precondition to allow adding and removing MCPTT clients based on MCPTT user criteria known at the MCPTT server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 1 requirements related to new MCPTT clients which meet/no longer meet the criteria of the ongoing preconfigured group regroup are not met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | The following new clauses: 10.6.2.12.x, 10.6.2.2.x and 10.6.2.2.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - updating the content to be MCPTT serive specific. - adding Motorola Solutions as cosigner |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 10.6.2.12.x	User regroup MCPTT ID list update

Figure 10.6.2.12.x-1 illustrates the procedure to add or remove an MCPTT user based on the MCPTT user criteria known at the MC service server.

NOTE 1: 	This procedure applies to MCPTT users that have been added and are to be added to the preconfigured regroup group based on MCPTT user criteria known at the MCPTT server. If only the criteria are to be applied, then only the criteria are included in the request and both the group and user lists are not included

Preconditions:

- A preconfigured regroup group has been established.

-	The MCPTT server has selected MCPTT user 2 to participate in the preconfigured regroup group based on MCPTT user criteria known at the MCPTT server.

NOTE 2:	Other participants of the preconfigured regroup group are not shown in figure 10.6.2.12.x-1 for simplicity.

Figure 10.6.2.12.x-1: Procedure to update the MCPTT ID list

1.	The MCPTT server detects that MCPTT client 2, which has been earlier included in the preconfigured regroup group, no longer meets the MCPTT user criteria known at the MCPTT server.

2.	The MCPTT server sends MCPTT client 2 a preconfigured user regroup remove request.

3.	The MCPTT client 2 notifies the MCPTT user of the preconfigured user regroup removal.

4.	The MCPTT client 2 may send the MCPTT server a preconfigured user regroup remove response to acknowledge the preconfigured user regroup removal.

5.	The MCPTT server updates the preconfigured regroup group (including the MCPTT user ID list) and potential group communication.

6.	The MCPTT server detects that the MCPTT client 3 meets the MCPTT user criteria.

7.	The MCPTT server sends the MCPTT client 3 a preconfigured regroup request.

8.	The MCPTT client 3 notifies the MCPTT user of the preconfigured regroup request.

9.	The MCPTT client 3 may send the MCPTT server a preconfigured regroup response.

10.	The MCPTT server updates the preconfigured regroup group (including the MCPTT ID list), and potential group communication.

# * * * * Second change* * * *

#### 10.6.2.2.x	Preconfigured user regroup remove request (MCPTT server – MCPTT client)

Table 10.6.2.2.x-1 describes the information flow preconfigured user regroup remove request from the MCPTT server to MCPTT client.

Table 10.6.2.2.x-1 Preconfigured user regroup remove request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the MCPTT user to be removed from the regroup group |
| MCPTT group ID | M | MCPTT group ID of the regroup group from which the MCPTT user is to be removed |

# * * * * Third change* * * *

#### 10.6.2.2.y	Preconfigured user regroup remove response (MCPTT client – MCPTT server)

Table 10.6.2.2.y -1 describes the information flow preconfigured user regroup remove response from the MCPTT client to the MCPTT server.

Table 10.6.2.2.y -1 Preconfigured user regroup remove response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the MCPTT user to be removed from the preconfigured regroup |
| Result | M | Result of the preconfigured user regroup removal operation |

# * * * * End of changes* * * *