3GPP TSG-SA WG6 Meeting #55	S6-231993

Berlin, Germany 22nd – 26th May 2023	(revision of S6-231716, S6-231984)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0351 | rev | 2 | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove unnecessary information flow tables |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | at&t, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC |  |  |  |  |  | Date: |  |  | 2023-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Remove redundant information flow tables that are not used. A MCPTT group call request and response is always between the hosting MCPTT server and targeting MCPTT client(s) in the application layer; there is no need to communicate between MCPTT servers that may be on the path to the destination. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add ENs to related MCPTT server to MCPTT server information flow tables as for further study. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Will give the wrong guidance to stage 3 development. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.2.3, 10.19.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.19.2.3	Ad hoc group call request (MCPTT server – MCPTT server)

Table 10.19.2.3-1 describes the information flow ad hoc group call request between the MCPTT servers.

Table 10.19.2.3-1 Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID | M | The MCPTT group ID to be associated with the ad hoc group call |
| MCPTT ID list (see NOTE 1, NOTE 3) | O | MCPTT IDs of the participants being invited for the ad hoc group call |
| SDP offer | M | Media parameters of MCPTT server |
| Broadcast indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is for a broadcast ad hoc group call |
| Imminent peril indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is an MC service imminent peril call |
| Emergency Indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is an MC service emergency call |
| Preconfigured ad hoc group identity | O | Group identity whose configuration is to be applied for this ad hoc group call. |
| Criteria for determining the participants (see NOTE 3) | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCPTT server for determining the participants e.g., it can be a location based criteria to invite particpants in a particular area |
| Requested priority | O | Application priority level requested for this group call |
| NOTE 1: This element is included only when the originating client sends the list of participants. NOTE 2: If used, only one of these information elements is present. NOTE 3: Only one of these information elements is present. |  |  |

Editor’s Note: It is FFS if the server to server message is needed in a call request or response message.

* * * Next Change * * * *

#### 10.19.2.6	Ad hoc group call response (MCPTT server – MCPTT server)

Table 10.19.2.6-1 describes the information flow ad hoc group call response between the MCPTT servers.

Table 10.19.2.6-1 Ad hoc group call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the target MC service user |
| Functional alias | O | The functional alias of the target MC service user |
| MCPTT ad hoc group ID | M | The MCPTT group ID to be associated with the ad hoc group call |
| SDP answer | O | Media parameters selected and present if the Result is success. |
| Result | M | Result of the ad hoc group call request (success or failure) |

Editor’s Note: It is FFS if the server to server message is needed in a call request or response message.