3GPP TSG-SA Meeting #100	SP-230778

Taipei, 12th – 16th June 2023	Revision of SP-230691

3GPP TSG-SA WG6 Meeting #55	S6-232224

Berlin, Germany 22nd – 26st May 2023

Source:	SA6

Title:	Study on enhanced application layer support for location services

Document for:	Approval

Agenda Item:	6.4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on enhanced application layer support for location services

Acronym:	FS_eLSAPP

Unique identifier:	1000035

Potential target Release:	Rel-19

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No |  |  | X |  |  |
| Don't know | X |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| x | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 830102 | Enhancement to the 5GC LoCation Services | Stage 2 Normative work for 5GC LoCation Services in Rel-16 |
| 910052 | Enhancement to the 5GC LoCation Services-Phase 2 | Stage 2 Normative work for 5GC LoCation Services in Rel-17 |
| 980111 | 5GC LoCation Services - Phase 3 | Stage 2 Normative work for 5GC LoCation Services in Rel-18 |
| 980003 | CT aspects of 5G_eLCS_Ph3 | Stage 3 Normative work for 5GC LoCation Services in Rel-18 |
| 830077 | NR Positioning Support | Stage 2 Normative work for NR positioning in Rel-16 |
| 900060 | NR Positioning Enhancements | Stage 2 Normative work for NR positioning in Rel-17 |
| 981038 | Expanded and improved NR positioning | Stage 2 Normative work for NR positioning in Rel-18 |
| 960018 | Study on Upper layer traffic steering, switching and split over dual 3GPP access | Stage 1 study work for dual-access based Services in Rel-19 |
| 32079 | Location Services (LCS); Service description; Stage 1 | Stage 1 study work for location Services |

Dependency on non-3GPP (draft) specification:

# 3	Justification

In Rel-16 and Rel-17, the SEAL (Service Enabler Architecture Layer for Verticals) work item in SA6 has introduced the basic location management architecture serving the different vertical applications, including the generic on-network/off-network function model, procedure, information flows, reference points, and APIs for location management.

In Rel-18, the 5G-enabled Fused Location Service (5GFLS) is studied as part of SEAL-LM(Location Management) architecture and agreed to obtain the UE location information from more sources to further improve the LCS QoS (e.g. accuracy, latency) to meet the requirements from vertical applications.

However, there are still many value-added location service requirements from vertical applications that cannot be fulfilled based on the current SEAL-LM architecture. E.g. Location Alerting, History Trace request or playback, Location format mapping, Location information analysis, etc. And such requests have been identified as a key issue in the 5GFLS study item, but postponed to be studied in future releases due to time limitations.

Besides, lots of AF (Application Function) requests/functionalities related to location services which is specified in SA2 (e.g. TS 23.273, TS 23.586) should be utilized and performed by SEAL-LM architecture in SA6. The functionalities of the Location Management Server or VAL (Vertical Application Layer) server should consider aligning with the AF requests/functionalities as defined in TS 23.273 and TS 23.586. Such as the Location service exposure service requests from AF, GNSS assistance data collection from AF, Ranging/Sidelink Positioning service requests from AF and so on.

What’s more, SA1 and SA2 have also specified some location-related requirements for multi-SIM and multi-USIM users. E.g. if one person takes the mobile phone and wears the watch at the same time, both of them could report the person’s location information to the LM Server separately. Then the LM Server could estimate this information to get an accurate location for the person. Further, there is no need for the LM Server to obtain the position for the watch separately, as the position for the watch can be replaced by the position of the mobile phone to reduce the signaling message and save power consumption, and vice versa.

Therefore, it’s very necessary to further study how to support the above location services based on the SEAL-LM application layer.

# 4	Objective

Based on the above analysis, the objectives of this study will include:

- Identify the gaps between the service requirements for AF functionalities specified by SA2, new SA1 requirements, an enabler for the value-added location service, and the existing SEAL-LM architecture in the application layer.

- Study SEAL-LM architectural enhancements mainly to support the following items based on the above gap analysis:

  - Further improve the LCS QoS, especially for location accuracy and latency, to meet the strict location requirements for different vertical applications.

  - Enabler for Value-Added Location Services, includes(not limited to):

    - Location information analysis

    - Location Alerting

    - History Trace request or playback

    - Location format mapping

- Identify key issues based on objective 2 and develop the corresponding functional model and potential solutions.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.xxx | Study on enhanced application layer support for location services | SA#103 (Mar. 2024) | SA#104 (Jun. 2024) | Liping Wu, CATT, wuliping@cictmobile.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Liping Wu, CATT, wuliping@cictmobile.com

# 7	Work item leadership

SA6

# 8	Aspects that involve other WGs

SA2 for core network architecture aspects, SA3 for security aspects, SA5 for charging aspects.

# 9	Supporting Individual Members

Supporting IM name

CATT

Huawei

ZTE

CMCC

Lenovo

China Telecom

Vivo