3GPP TSG-SA Meeting #102 	SP-231732

Edinburgh, UNITED KINGDOM, 11th Dec 2023 - 15th Dec 2023

# Source:	SA WG5

# Title:	New SID: Study on data management regarding subscriptions and reporting

# Document for:	Approval

3GPP TSG-SA5 Meeting #152 	S5-238201

Chicago,US, 13-17 November 2023	(revision of S5-237885)

# Source:	Ericsson

# Title:	New SID on data management regarding subscriptions and reporting

# Document for:	Approval

# Agenda Item:	6.2.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on data management regarding subscriptions and reporting

Acronym: FS_Data_SREP

Unique identifier:	1020012

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X | X |  |
| No | X | X |  |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 970029 | Study on Data management phase 2 | Study in Rel-18 |

# 3	Justification

The number of automation functions are increasing in the 3GPP system. There are domain specific and interdomain entities that are performing automated functionality. All of these require data from the NFs and/or 3GPP mgmt. system. Several subscription functions are remaining from the time when it was few consumers. With many different consumers the uniqueness of the trace and QoE identities cannot be guaranteed.

Example:

Consumer A (e.g. NWDAF) request MDT data “1”, “2”, “5”, and “22” in Cell “a” and want the collected data to be sent to itself.

Consumer B (e.g. a customer care central) request MDT data “1”, “5” and “22” in Cell “a” and want the collected data to be sent to second line support.

The second request comes 1 minute after the first request.

As the two consumers are different and do not have any possibility to correlate the value for the Trace Reference, both use the value “123”, which will lead to that the base station supporting Cell “a” cannot differ on which collected data shall be sent where.

Also how to avoid that the same data from different consumers does not lead to redundant subscriptions for the same data in the NFs and/or 3GPP mgmt. system and that the reporting is not multiplied should be studied.
Example:

Consumer A (e.g. NWDAF) request MDT data “1”, “2”, “5”, and “22” in Cell “a” and want the collected data to be sent to itself.

Consumer B (e.g. a customer care central) request MDT data “1”, “5” and “22” in Cell “a” and want the collected data to be sent to second line support.

These two request comes within 1 minute difference.

What is to be studied is whether there can be an entity outside the traffic node that can combine the two requests into on towards the traffic node, in order to minimize the administration and reporting in the traffic node. The new entity should be able to change trace jobs without ending and starting a new job (i.e. keep the same Trace Reference when new measurements are added or removed).

# 4	Objective

To study:

- WT-1 Study how the load on the NFs and/or 3GPP mgmt. system can be decreased by extracting some of the required functionality to be deployed outside the NFs implementing management capabilities defined in SA5.

- WT-2 Study how to solve to get unique job identifiers for performance measurement, Trace/MDT and QoE subscriptions/requests within a PLMN from different consumers.
Propose potential requirements, use cases and solutions for this issue.

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | SA Dependency (Yes/No/Maybe) |
| --- | --- | --- | --- | --- |
| WT-1 | 1 | 1 | Maybe | Maybe |
| WT-2 | 1 | 1 | Maybe | Maybe |

Total TU estimates for the study phase: 2

Total TU estimates for the normative phase: 2

Total TU estimates: 4

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 28.XXX | Study on data management, subscriptions and reporting | SA#104 (Jun. 2024) | SA#105 (Sep. 2024) | {<FamilyName>, <GivenName>, <Company>, <email address>. See Note 2} |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

{Mandatory: <FamilyName>, <GivenName>, <Company>, <email address>}

{Optional: <FamilyName>, <GivenName>, <Company>, <email address>: Secondary task(s)}

{The first listed Rapporteur is the work item primary Rapporteur. The role of a Rapporteur is further described in www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec. By default, the primary Rapporteur shall ensure the production of the post-completion summary. 
Secondary Rapporteur(s) are possible for specific secondary task(s), such as: "Write the post-completion summary"; "In charge of a specific aspect of the work item (specify which)"; "Rapporteur for a secondary responsible WG (specify which)"}

# 7	Work item leadership

SA5

# 8	Aspects that involve other WGs

Potential collaborations:

SA2, CT WGs and RAN WGs.

# 9	Supporting Individual Members

Supporting IM name

Ericsson

Nokia

Microsoft

Verizon

Huawei