3GPP TSG-SA/WG4 Meeting #127bis-e	S4-240755

# Online, 8 – 12 April, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR | 0001 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ABNF Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson France S.A.S |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP |  |  |  |  |  | Date: |  |  | 2024-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some ABNF definitions are not providing correct separation between defined parameters, also not matching the provided SDP examples |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating ABNF definitions to separate parameters and align with examples. Changing xr-pose media:<value> parameter syntax to media=<value>. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ABNF parameters will not be separated and be hard or not even possible to distinguish from other parameters or parameter values, likely causing incorrect or at least mismatching configuration for RTP senders and receivers, which could cause failure to communicate |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.5, 4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-240660 Initial revision S4-240755 Changing xr-pose parameter media: to media= |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 4.2.5	SDP Signaling

An RTP sender capable of sending RTP HE for PDU Set marking shall use the SDP extmap attribute for RTP HE for PDU Set marking in the media description of the RTP stream(s) carrying the RTP HE for PDU Set marking. An RTP receiver that does not support RTP HE for PDU Set marking can ignore that RTP HE when included. The signaling of the PDU Set marking RTP HE shall follow the SDP signaling design and the syntax and semantics of the "extmap" attribute as outlined in RFC8285.The URN for the PDU Set marking shall be set to "urn:3gpp:pdu-set-marking:rel-18".

The ABNF syntax for the extmap attribute for the signaling of RTP HE for PDU Set marking is defined as follows, extending the ABNF in RFC 8285:

extensionname = "urn:3gpp:pdu-set-marking:rel-18"

extensionattributes = [pdu-set-attribute *2(SP pdu-set-attribute)]

pdu-set-attribute = format / "pdu-set-size" / "num-pdus-in-pdu-set"

format = "short" / "long"

The extension attributes have the following semantics:

-	format: indicates if the RTP HE for PDU Set marking uses the 1-byte (short) or the 2-byte (long) format. This extension attribute shall not be included more than once.

NOTE:	Regardless if this extension attribute is present or not, the use of long or short format is determined as described by section 4.1.2 of RFC 8285, i.e., based on what format other RTP HEs use in the same RTP session, unless both endpoints announced support for handling mixed format with "a=extmap-allow-mixed" as described by section 6 of RFC 8285.

-	pdu-set-size: if present, this extension attribute indicates that the RTP sender will provide the PDU Set size in bytes in the RTP HE with every RTP packet. This results in an additional 3 bytes of length for the RTP HE. This extension attribute shall not be included more than once.

-	num-pdus-in-pdu-set: if present, this extension attribute indicates that the RTP sender will provide the number of PDUs in the PDU Set (NPDS) in the RTP HE with every RTP packet. This results in an additional 2 bytes of length for the RTP HE. This extension attribute shall not be included more than once.

Below is an example:

a=extmap:7 urn:3gpp:pdu-set-marking:rel-18 short pdu-set-size

Second CHANGE

### 4.3.2	SDP Signaling

An RTP client that supports the RTP HE for XR pose shall negotiate the use of the extension using SDP. The signaling of the RTP HE for XR pose shall follow the SDP signaling design, the syntax, and semantics of the "extmap" attribute as outlined in RFC8285 [11].

For IANA registration, the "reference" field in the registry is 3GPP TS 26.522.

The ABNF syntax for this RTP HE extends the "extmap" attribute as follows:

extensionname = "urn:3gpp:xr-pose"

extensionattributes = xr-pose-dof [SP xr-pose-media]

xr-pose-dof = "3DOF" / "6DOF"

xr-pose-media = "media=" xr-pose-media-id *("," xr-pose-media-id)

xr-pose-media-id = token

The extension attribute "3DOF" indicates that the sender uses the RTP HE to signal a 3DoF XR pose, i.e., an XR pose that does not include the position fields x, y, z.

An RTP client that supports the RTP HE for XR pose and receives an SDP offer with "a=extmap" attribute with the URN "urn:3gpp:xr-pose" and the extension attribute "3DOF", shall include the extension attribute "3DOF" in the SDP answer, if the SDP offer is accepted.

The extension attribute "6DOF" indicates that the sender uses the RTP HE to signal a 6DoF XR pose, i.e., an XR pose that includes both the position fields x, y, z and the orientation fields rx, ry, rz, rw.

An RTP client that supports the RTP HE for XR pose and receives an SDP offer with "a=extmap" attribute with the URN "urn:3gpp:xr-pose" and the extension attribute "6DOF", shall include the extension attribute "6DOF" in the SDP answer, if the SDP offer is accepted.

The extension attribute "media" is followed by a list of tokens for "mid" (as defined in RFC 5888 [19]) for media streams that can reuse the pose included in the RTP HE. Further details on reuse are provided later in the section.

An RTP client that supports the RTP HE for XR pose and receives an SDP offer with "a=extmap" attribute with the URN "urn:3gpp:xr-pose" shall remove the attribute from the answer for any media that will not use the extension, and retain it for any media that will use it.

End of changes