3GPP TSG-SA4 Meeting #129-e	S4-241632

# Online, 19th Aug 2024 - 23rd Aug 2024										Revision of S4-241583

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.253 | CR | 0004 | rev | 1 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to TS 26.253 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby Sweden AB, Ericsson LM, Fraunhofer IIS, Huawei Technologies Co Ltd., Nokia, NTT, Orange, Panasonic Holdings Corporation, Philips International B.V., Qualcomm Incorporated, VoiceAge Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IVAS_Codec |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Errors and ambiguities have been identified in the detailed algorithmic description. Following inclusion of split rendering functionality, several ambiguities are also present in the general description of the IVAS codec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | General description of the IVAS codec is updated to include external rendering and split rendering as necessary. Errors corrected in detailed algorithmic description of the encoder, decoder, renderer, and bitstream. Clarifications on ambiguous statements. Further improvements for consistency and readability of the text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be algorithmic errors or ambiguities in the IVAS standard leading to inconsistencies of implementation and use. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 4.2.10, 4.3.1, 4.3.2, 4.4, 5.1, 5.2.2.2.2, 5.3.2.3.5, 5.3.2.4.4.1, 5.3.2.4.4.2, 5.3.2.4.4.3, 5.3.2.4.4.4, 5.3.2.4.8, 5.3.2.4.13, 5.3.3.8.1, 5.3.5.1.4, 5.6.2.3.2, 5.7.4.6, 5.7.5.1, 5.8.1, 5.9.4.2, 5.9.5, 5.11.3.2, 5.11.3.3, 5.11.3.4, 5.11.3.5, 5.11.3.5.1, 5.11.3.5.2, 5.11.3.5.3, 5.11.3.5.4, 5.11.3.6, 6.2.3.3, 6.2.7.3.2, 6.3.4.5.1, 6.3.4.5.2, 6.3.5.1.1, 6.3.6.1, 6.4.1.4, 6.4.1.5, 6.4.9, 6.4.10, 6.7.1.7.3.7, 6.7.6, 6.7.7.1, 6.8.4, 6.8.5, 6.8.6, 6.9.7.5 (new), 6.9.11, 7.1, 7.5.3.1, 7.6.1, 7.6.4.2.3.3, 7.6.4.2.6, 7.6.4.2.7.5, 7.6.4.2.9.3, 7.6.4.2.9.4, 8.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Correction of work item code |  |  |  |  |  |  |  |  |

CHANGE 1

# 2	References

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.441: "Codec for Enhanced Voice Services (EVS); General Overview".

[3]	3GPP TS 26.445: "Codec for Enhanced Voice Services (EVS); Detailed Algorithmic Description".

[4]	3GPP TS 26.447: "Codec for Enhanced Voice Services (EVS); Error concealment of lost packets".

[5]	3GPP TS 26.448: "Codec for Enhanced Voice Services (EVS); Jitter Buffer Management"

[6]	3GPP TS 26.250: "Codec for Immersive Voice and Audio Services (IVAS); General overview".

[7]	3GPP TS 26.251: "Codec for Immersive Voice and Audio Services (IVAS); C code (fixed-point)".

[8]	3GPP TS 26.252: "Codec for Immersive Voice and Audio Services (IVAS); Test Sequences".

[9]	3GPP TS 26.254: "Codec for Immersive Voice and Audio Services (IVAS); Rendering".

[10]	3GPP TS 26.255: "Codec for Immersive Voice and Audio Services (IVAS); Error concealment of lost packets".

[11]	3GPP TS 26.256: "Codec for Immersive Voice and Audio Services (IVAS); Jitter Buffer Management".

[12]	3GPP TS 26.258: "Codec for Immersive Voice and Audio Services (IVAS); C code (floating point)".

[13]	C. de Boor and K. Höllig (1987), B-splines without divided differences, in Geometric Modeling, G. Farin ed., SIAM, 21–27.

[14]	Borß, C. A Polygon-Based Panning Method for 3D Loudspeaker Setups. In Audio Engineering Society Convention 137, Los Angeles, USA, Oct. 2014.

[15]	Zotter, F. and Frank, M., All-Round Ambisonic Panning and Decoding, J. Audio Eng. Soc., vol. 60, no. 10, pp. 807-820 (Oct. 2012).

[16]	Allen, J. B., & Berkley, D. A., Image method for efficiently simulating small room acoustics. Journal of the Acoustical Society of America, 65(4), 943-950, (1979).

[17]	Box, G. E. P. and Cox, D. R. (1964). An analysis of transformations, Journal of the Royal Statistical Society, Series B, 26, 211-252.

[18]	3GPP TS 26.118: “Virtual Reality (VR) profiles for streaming applications”.

[19]	Ivanic, J., & Ruedenberg, K., Rotation matrices for real spherical harmonics. Direct determination by recursion. The Journal of Physical Chemistry, 100(15), 6342-6347, 1996.

[20]	C. R. Helmrich and B. Edler, “Signal-Adaptive Transform Kernel Switching for Stereo Audio Coding,” in Proc. IEEE WASPAA, New Paltz, NY, USA, Oct. 2015.

[21]	AES, “AES69-2022: AES standard for file exchange - Spatial acoustic data”, Audio Engineering Society, 2022.

[22]	F. Thomas, "Approaching Dual Quaternions From Matrix Algebra," in IEEE Transactions on Robotics, vol. 30, no. 5, pp. 1037-1048, Oct. 2014.

[23]	J. Daniel, “Représentation de champs acoustiques, application à la transmission et à la reproduction de scènes sonores complexes dans un contexte multimedia”, Thèse de doctorat de l’Université Paris 6, 2001

CHANGE 2

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ACELP	Algebraic Code-Excited Linear Prediction

AGC		Automatic Gain Control

AllRAD	All-Round Ambisonics Decoding

BPF	Bass PostFilter

BRIR	Binaural Room Impulse Response

BWE	BandWidth Extension

DirAC	Directional Audio Coding

CBR	Constant Bit Rate

CICP	Coding Independent Code Points

CLDFB	Complex Low-Delay FilterBank

CMR	Codec Mode Request

CNA	Comfort Noise Addition

CNG	Comfort Noise Generation

CPE	Channel Pair Element

DFT	Discrete Fourier Transform

DoA	Direction of Arrival

DoF	Degree of Freedom

DTX	Discontinuous Transmission

EC	Entropy Coding

EFAP	Edge Fading Amplitude Panning

EVS	(Codec for) Enhanced Voice Services

FEC	Frame Erasure Concealment

FOA	First-Order Ambisonics

GCC-PHAT	Generalized Cross-Correlation PHAse Transform

HQ	High-Quality

HOA	Higher-Order Ambisonics

HRIR	Head-Related Room Impulse Response

HRTF	Head-Related Transfer Function

IC-BWE	Inter-Channel BandWidth Extension

ICA	Inter-Channel Alignment

ICC	Inter-Channel Coherence

IGF	Intelligent Gap Filling

ILD	Inter-Channel Level Difference

IO	InterOperable

IR	Impulse Response

ITD	Inter-Channel Time Difference

ISM	Independent Streams with Metadata

JBM	Jitter Buffer Management

LCLD (codec)	Low-Complexity Low-Delay (codec)

LC3plus	Low Complexity Communication Codec plus

LFE	Low-Frequency Effects

LP	Linear Prediction

LR, L/R	Left-Right

LS	LoudSpeaker

MASA	Metadata-Assisted Spatial Audio

MASA1	MASA with 1 TC (mono-MASA)

MASA2	MASA with 2 TC (stereo-MASA)

McMASA		Multi-Channel MASA

MC	Multi-Channel

MCT	Multi-channel Coding Tool

MD	MetaData

MDCT	Modified Discrete Cosine Transform

MDFT	Modified Discrete Fourier Transform

MDST	Modified Discrete Sine Transform

MS, M/S	Mid-Side

OLA	OverLap-Add

OMASA	Objects with MASA

OSBA	Objects with SBA

PCA	Principal Component Analysis

PI	Processing Information

PLC	Packet Loss Concealment

SAD	Sound Activity Detection

SBA	Scene-Based Audio

SCE	Single Channel Element

SDP	Session Description Protocol

SH	Spherical Harmonics

SNS	Spectral Noise Shaping

SPAR	Spatial Reconstruction

STFT	Short-Term Fourier Transform

TC	Transport Channels

TCX	Transform-Coded eXcitation

TD	Time-Domain

TF	Time-Frequency

TNS	Temporal Noise Shaping

TSM	Time Scale Modification

VAD	Voice Activity Detection

VBAP	Vector Base Amplitude Panning

VBR	Variable Bit Rate

CHANGE 3

### 4.2.10 	Discontinuous Transmission (DTX) Operation

DTX is a functionality of operation where the encoder encodes frames containing only background noise with a lower bit rate and lower packet frequency than normally used for encoding speech. A terminal and the network may adapt their transmission scheme to take advantage of the smaller frames and longer transmission interval to reduce power consumption, average bit rate and network activity. The discontinuous transmission (DTX) functionality of the IVAS codec includes voice activity detection (VAD) and comfort noise generation (CNG). DTX functionality is supported for IVAS operation points, i.e., audio formats and bitrates, that are especially optimized for efficient stereo and immersive conversational voice transmissions.

The size of the SID frames for EVS interoperable modes is unchanged relative to EVS, 48 bits for EVS primary modes. For IVAS modes, the SID frame size is 104 bits. The default SID frame interval is once per 8 frames, but other update intervals are also supported.

DTX is supported in IVAS formats and bitrates as summarized in Table 4.2-3. The algorithmic description of DTX/CNG is then discussed in related IVAS format specific clauses.

Table 4.2-3: DTX support overview in the IVAS codec

| Input audio format | DTX support |
| --- | --- |
| Stereo | Yes, up to 256 kbps |
| Scene-based audio (SBA) | Yes, up to 80 kbps |
| Metadata assisted spatial audio (MASA) | Yes, up to 512 kbps |
| Object-based audio (ISM) | Yes, up to 512 kbps |
| Multi-channel audio (MC) | No |
| Combined ISM and SBA (OSBA) | No |
| Combined ISM and MASA (OMASA) | No |

CHANGE 4

### 4.3.1 	Input/output audio sampling rate

The IVAS coder is capable of processing input audio signals sampled at 16, 32 and 48 kHz. The sampling frequency is denoted as Fs. The same set of sample rates is also supported by the IVAS decoder and the IVAS renderer except for the split rendering for which only 48 kHz sampling rate is supported. In case of EVS-compatible mono operation the IVAS codec supports also 8 kHz sample rate both for input and output signals. The input audio is processed in frames of 20 ms. The external renderer can then process the audio in frames of 5, 10, or 20 ms.

CHANGE 5

### 4.3.2 	Input/output audio formats

The IVAS coder accepts the following input audio formats:

-	single-channel mono audio format denoted as sn

-	two-channel stereo and binaural input audio format, where the left channel is denoted as s1n and the right channel is denoted as s2n

-	scene-based (ambisonic) input audio format, where the ambisonic order is denoted as l and the number of individual input channels is  l+12 . The individual input channels are stored in the ACN component ordering, denoted as smln, where m ∈-l,…,+l is the ambisonic degree. In case of the first-order ambisonic format (FOA), the individual input channels may also be denoted as:

s00n=Wns-11n=Xns01n=Yns11n=Zn

For both, first-order ambisonic signals and higher-order ambisonic signals (HOA), the notation smln may be simplified to:

skn=smln,  k=l2+l+m

	object-based input audio format (ISM), where the number of objects is denoted as NISM and the individual “streams” related to the objects are denoted as skn where k∈1,…, NISM. Each individual stream is associated with its input metadata signal, denoted as Ikm where m is the frame index.

	multi-channel input audio format (MC), where the individual channels are denoted as skn where k∈1,…, Nchan. The input channels correspond to one of the following loudspeaker layouts, where the loudspeaker positions are assumed to have azimuth and elevation as per ISO/IEC 23091-3:2018 Table 3 [24], unless otherwise noted below. The channel order is as per ISO/IEC 23008-3:2015 Table 95 [25]:

Table 4.2-4: Ordering of input channels in MC format

| loudspeaker layout | CICP layout |
| --- | --- |
| 5.1 | CICP6 |
| 5.1+2 | CICP14, 35°elevation |
| 5.1+4 | CICP16, 35°elevation |
| 7.1 | CICP12 |
| 7.1+4 | CICP19, 35°elevation |

	metadata-assisted spatial audio (MASA) format, where for mono-MASA (MASA1), the mono channel is denoted as sn and, for stereo-MASA (MASA2), the left channel is denoted as s1n and the right channel is denoted as s2n. The MASA audio channel(s) is/are associated with the MASA metadata that is denoted as Im where m is the frame index

	objects with metadata-assisted spatial audio (OMASA), which is a combination of MASA and object-based input audio format (ISM)

	objects with scene-based (ambisonic) input audio format (OSBA), which is a combination of SBA and object-based input audio format (ISM)

The IVAS codec is capable to output the audio into the following output formats:

- single-channel mono audio format

- two-channel stereo audio format

- two-channel binaural audio format (Binaural output without room acoustic synthesis, Binaural output with room acoustics synthesized using impulse responses, Binaural output with room acoustics synthesized using parametric reverberator)

- scene-based (ambisonic) audio format (ambisonics order l = 1, 2, 3)

- multi-channel audio format (loudspeaker layouts listed in Table 4.2-4 + custom layouts)

- split-rendering intermediate bitstream (supported for all IVAS formats except of stereo and EVS mono)

Note that some output audio formats are supported only for certain input audio formats. The overview of supported combination of input/output formats is provided in Table 4.4-1.

In addition, a pass-through operation is supported at which the codec outputs the audio in an output format corresponding to the input format including a separate metadata (when available). This operation primarily intends to skip the internal renderering and use of an external or a custom renderer. A pass-through operation is supported for all input audio formats and it is also referred as an external (EXT) processing output.

CHANGE 6

## 4.4 	Algorithmic delay

The input signals (audio, or audio and metadata) are processed using 20-ms frames. The codec algorithmic delay depends on the input/output audio formats as described in Table 4.4-1.

Table 4.4-1: IVAS algorithmic delay for different input/output format combinations (rounded to integer milliseconds; in case multiple values are provided they depend on the bitrate)

|  |  | Decoder output format |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Mono | Stereo | Multi-Channel | Binaural audio | Scene-based audio | External (EXT) output |
| Encoder input format | Mono | 32 |  |  |  |  |  |
|  | Stereo (NOTE1 | 32 | 32 | 32 |  |  | 32 (stereo) |
|  | Multi-channel audio | 32 | 32 | 32 / 37 | 32 / 37 | 32 / 37 | 32 / 37 (MC audio) |
|  | Scene-based audio | 33 | 33 | 38 | 38 | 38 | 38 (SBA audio) |
|  | Object-based audio | 32 | 32 | 32 / 37 | 32 / 37 | 32 / 37 | 32 / 37 (object-based audio) |
|  | Metadata-assisted spatial audio (NOTE2 | 32 / 37 | 37 | 37 | 37 | 37 | 32 (MASA audio) |
|  | OSBA | 33 | 33 | 38 | 38 | 38 | 38 (object-based + SBA audio) |
|  | OMASA | 32 / 37 | 37 | 37 | 37 | 37 | 32 (object-based + MASA audio) |

NOTE1:	Stereo input format also includes binaural input format.

NOTE2:	Metadata-assisted spatial audio (MASA) decoder output allows also for mono or stereo decoder output at 32 ms algorithmic delay by stripping the metadata file.

CHANGE 7

## 5.1	Encoder overview

The IVAS codec encoder expects mono, stereo, objects, multichannel, ambisonics, MASA, combination of objects and MASA, or combination of objects and SBA as input audio channels. In case of objects or MASA, also input metadata are expected. The encoder analyzes the scene, derives the spatial audio parameters and downmixes the input channels to the so-called transport channels which are subsequently processed by the encoding tools. These tools comprise Single Channel Elements (SCE comprising one core-coder, see clause 5.2.3.2), Channel Pair Elements (CPE comprising two core-coders, see clause 5.2.3.3), and Multichannel Coding Tool (MCT comprising a joint coding of multiple core-coders, see clause 5.2.3.4) while the core-coder is inherited from the EVS codec with additional flexibility and variable bitrate (clause 5.2.2).

Figure 5.1-1: Encoder Data Flow from input data to IVAS bitstream

The IVAS format is signalled in every active frame at the beginning of the IVAS bitstream. The number of signalling bits in active frames and their values corresponding to individual formats is summarized in Table 5.1-1.

Table 5.1-1: IVAS format signalling, active frames

| IVAS format | configuration | number of bits | Value |
| --- | --- | --- | --- |
| STEREO | - | 2 | 0 |
| ISM | < 24.4 kbps | 2 | 2 |
|  | ≥ 24.4 kbps | 3 | 4 |
| SBA | - | 3 | 6 |
| MASA | - | 3 | 7 |
| MC | - | 2 | 1 |
| OSBA | < 24.4 kbps | 5 | 14 |
|  | ≥ 24.4 kbps | 4 | 11 |
| OMASA | Rend mode | 3 | 7 |
|  | other modes | 4 | 10 |

In SID frames of the DTX operation, the IVAS format is signalled at the beginning of the IVAS bitstream. The number of signalling bits in SID frames and their values corresponding to individual formats is summarized in Table 5.1-2.

Table 5.1-2: IVAS format signalling, SID frames

| IVAS format | configuration | number of bits | Value |
| --- | --- | --- | --- |
| STEREO | Unified stereo | 2 | 0x0 |
|  | MDCT stereo | 2 | 0x1 |
| ISM | - | 2 | 0x2 |
| SBA | 1 TC | 2 | 0x5 |
|  | 2 TCs | 2 | 0x6 |
|  | 3+ TCs | N/A | N/A |
| MASA | MASA1 | 2 | 0x3 |
|  | MASA2 | 2 | 0x7 |
| MC | - | N/A | N/A |
| OSBA | - | N/A | N/A |
| OMASA | - | N/A | N/A |

CHANGE 8

##### 5.2.2.2.2	Sample rate conversion

The shorter look-ahead in IVAS wrt. EVS has an impact on the procedure of resampling the down-processed signal (down-mixed signal for TD and DFT stereo modes) in every frame. Two methods are used:

- SCE mono, TD stereo, MDCT stereo: time-domain FIR resampling (decimation, see clause 5.1.3 in [3]) is performed using the delay of 0.9375 ms. As this resampling delay is not available in IVAS, the resampling delay is compensated by adding zeroes at the end of the input signal. Consequently, the 0.9375 ms long compensated part of the down-mixed signal needs to be recomputed (resampled again) at the next frame;

- DFT stereo: Resampling is performed in the DFT domain and, therefore, introduces no additional delay.

The resampling in the SCE coder, the DFT stereo coder, the TD stereo coder and the MDCT stereo coder, is done from the input sampling rate (16, 32, or 48 kHz) to the internal sampling rate (12.8, 16, 25.6, or 32 kHz). The resampled signal is then used in the pre-processing and the core-coding.

The redressed/extrapolated signal part in the look-ahead is not subject to actual coding but used for analysis and classification. Consequently, the redressed/extrapolated, signal part in the look-ahead is re-computed in the next frame and the resulting mono or down-processed signal (down-mixed signal for TD and DFT stereo modes) is then used for the actual core-coding. The length of the re-computed signal depends on the stereo mode and coding processing:

- DFT stereo: The 8.75 ms long signal is subject to re-computation both at the input stereo signal sampling rate

and internal sampling rate;

- TD stereo: The 7.5 ms long signal is subject to re-computation at the input stereo signal sampling rate while the

7.5 + 0.9375 = 8.4375 ms long signal is subject to re-computation at the internal sampling rate.

- MDCT stereo: Re-computation is usually not needed at the input stereo signal sampling rate while the 0.9375 ms

long signal is subject to re-computation at the internal sampling rate. However, in practice, some signals with

length of 7.5 + 0.9375 = 8.4375 ms are recomputed similarly as in the TD stereo.

- SCE mono: Re-computation is usually not needed at the input stereo signal sampling rate while the 0.9375 ms

long signal is subject to re-computation at the internal sampling rate.

The length of recomputated portions of different signals plays a substantial role to ensure a seamless switching between different stereo modes. In case of the stereo switching, the length of recomputed portions of different signals can thus slightly differ from the values listed above. The details are described in clause 5.3.4.

CHANGE 9

##### 5.3.2.3.5	LP filter coherence

The ACELP encoder in the secondary channel is optimized in such a way that it requires only a minimal amount of bits. To reduce the required amount of bits in the secondary channel certain parameters from the primary channel may be reused such as the LP filter or the OL pitch lag. First, a low-complexity pre-processing is performed on the signal in the secondary channel including the VAD, described in clause 5.2.2.2.5, LP analysis, described in clause 5.2.2.2.8, and OL pitch analysis, described in clause 5.2.2.2.9. Then, the characteristics of the secondary channel are analyzed and the signal is classified as GENERIC, UNOVICED or INACTIVE. The signal classification is done using the mechanism described in clause 5.2.2.3.1.2.

An important part of the bitrate consumption in the secondary channel is the quantization of LP filter coefficients. Given that the spectral content of the secondary channel is often close to that of the primary channel the TD stereo coder may decide on reusing the LP filter coefficients of the primary channel in the secondary channel. The TD stereo coder performs LP analysis as described in clause 5.2.2.2.8 on both the primary channel Y(n) and the seconadry channel X(n). Let’s denote the LP filter coefficients of the primary channel as AYi, where i=0,…,M and M is the order of the LP filter. Using the same symbols, let’s denote the LP filter coefficients of the secondary channel as AXi. The TD stereo coder calculates two versions of LP residuals for the secondary channel. The first version of the LP residual is calculated by filtering the signal in the secondary channel with the LP filter of the primary channel. That is

rYn=Xn+i=1MAY(i)∙Xn-i,        n=0,…,N-1	(5.3-78)

The second version of the of the LP residual is calculated by filtering the signal in the secondary channel with the LP filter of the secondary channel. That is

rXn=Xn+i=1MAX(i)∙Xn-i,        n=0,…,N-1	(5.3-79)

The absolute energy of the secondary channel is calculated using the following relation

EX=10∙log10n=0N-1X(n)2	(5.3-80)

The TD stereo coder calculates the energy of the first version of the LP residual signal rYn as

ErY=10∙log10n=0N-1rY(n)2	(5.3-81)

and the energy of the second version of the LP residual signal rXn as

ErX=10∙log10n=0N-1rX(n)2	(5.3-82)

The TD stereo coder then calculates the LP prediction gains for both the first and the second version of the LP residual signal in the secondary channel by subtracting the energies of LP residuals from the absolute energy of the secondary channel. That is

GY=EX-ErYGX=EX-ErX	(5.3-83)

Finally, the TD stereo coder calculates a ratio of gains GY|X using the following relation

GY|X=GYGX	(5.3-84)

The decision on reusing the LP filter coefficients from the primary channel is also based on LP filter similarity between the primary and the secondary channel. The TD stereo coder calculates the similarity of LP filters by measuring the Euclidean distance between the line spectral pairs (LSPs) of the primary and the secondary channel. The LSPs of the primary and the secondary channel are converted from the respective LP filter coefficients using the procedure described in clause 5.1.9.5 of [3]. Let’s denote the LSPs of the primary channel as lspYi where i=0,…,M and M is the order of the LP filter. Similarly, let’s denote the LSPs of the secondary channel as lspXi. The LSPs lspYi and lspXi are weighted in such a way that emphasis is put on certain critical parts of the frequency spectrum. Then, the Eucidean distance between the LSPs is calculated with

dist=i=0M-1lspYi-lspXi2	(5.3-85)

Then, a series of conditions is performed to decide if the LP filter of the primary channel is reused during the encoding of the secondary channel. This is illustrated in Figure 5.3-21. In all cases, the transition frame counter, TCcnt, defined as iTC in clause 5.1.13.4 of [3], must be equal to 0 as it is better to use the LP filter of the secondary channel in case when signal transition is encountered.

The LP filter of the primary channel is reused when:

-	the energy of the secondary channel EX is below or equal to 30.0, or

-	the ratio of gains GY|X  is higher than the pre-defined threshold τ, typically 0.92, and the LP filter similarity measured by the Euclidean distance dist is lower than the threshold σ, typically 0.04, or

-	the prediction gain of the secondary channel GX  is below the threshold of 3.0 and the Euclidean distance dist is lower than 0.08, or

-	the down-mixing ratio β[-1] in the previous frame is between 0.4 and 0.6 and the prediction ratio  GY|X is higher or equal to 0.86 and either the Euclidean distance dist is below 0.08 or the prediction gain of the secondary channel GX  is below 3.0.

Furthermore, the LP filter of the primary channel is also reused in situations when the bitrate of the encoder is sufficiently high or if the input signal is considered as uncorrelated. The decision is based on the following set of, more restrictive, conditions:

GY|X distσ

-	when LRTD sub-mode has been selected and the available bit rate is lower than 16400 bps and the ratio of gains GY|X  is higher than 0.94, and the LP filter similarity measured by the Euclidean distance dist is lower than 0.02, or

-	when LRTD sub-mode has been selected and the available bit rate is higher or equal to 16400 bps and the ratio GY|X  is higher than 0.96, and the LP filter similarity measured by the Euclidean distance dist is lower than 0.01.

Figure 5.3-21: Re-using LP filter from the primary channel

In case the decision about re-using the LP filter from the primary channel is negative, then the encoder of the secondary channel will use the LP filter AX(i) for quantization and in further processing. As a consequence, the secondary channel will need to allocate more bits to transmit the information about the LP filter to the decoder. The decision logic on re-using the LP filter from the primary channel is depiced in the schematic diagram in Figure 5.3-21.

CHANGE 10  (style for readability and ToC)

###### 5.3.2.4.4.1	Description of GCC-PHAT implementation

CHANGE 11  (style for readability and ToC)

###### 5.3.2.4.4.2	DTX hangover adaptation

CHANGE 12  (style for readability and ToC)

###### 5.3.2.4.4.3	Peak picking algorithm for ITD estimation

CHANGE 13  (style for readability and ToC)

###### 5.3.2.4.4.4	ITD stabilization hangover

CHANGE 14

##### 5.3.2.4.8	Calculation of the side and residual prediction gains

In case of the residual prediction gain is transmitted or if the residual is directly coded in the frequency band b currently considered, the side gain stereo parameter is computed as the optimal gain for predicting the side signal Sik=Lik-Ri[k] by the mid signal Mk=L[k]+R[k], such that the energy of the remainder

p[k]=S[k]-g[b]M[k]

(5.3-135)

is minimal. The optimal prediction gain can be calculated from the energies in the subbands

EL[b]=k∈lb⎸Lb⎸2     and     ER[b]=k∈lb⎹R[b]⎸2

(5.3-136)

and the absolute value of the inner product of L and R

XL/R[b]=⎸k∈lbLkR*[k]⎹

(5.3-137)

as

g[b]=EL[b]-ER[b]EL[b]+ER[b]+2XL/R[b]

(5.3-138)

From this it follows that g[b] lies in [-1,1].

In bands, where the residual gain is transmitted, the side gain is calculated similarly as previously defined. The residual gain is then computed form the side gain, the channel energies and the inner product of the channels as:

r[b]=1-g[b]EL[b]+1+gbER[b]-2XL/R[b]reg[b]+ EL[b]+ER[b]+2XL/R[b]

(5.3-139)

where regb is regularization factor obtained by adding a coherent low energy contribution to avoid singularity for very low energy signals:

regb=(nIb*NFFT32)2

(5.3-140)

where the nIb gives the cardinality of Ib, the set of frequency indexes laying in the frequency band b, and NFFT32 is the size of the analysis DFT at 32 kHz.

It implies that

0 ≤ r[b]≤ 1-g[b]2.

(5.3-141)

In particular, this shows that r[b] ∈ [0,1]. This way, the stereo parameters can be calculated independently from the downmix by calculating the corresponding energies and the inner product.

In the case of non-zero ∆ITD and time alignment in the frequency domain, there is a mismatch in the DFT analysis windows between the two channels, which biases the calculation of the residual prediction gain. An offset is therefore calculated, involving a normalized autocorrelation of the DFT analysis windows, modeling the mismatch of the two windows after a circular shift of ∆ITD samples. The normalized autocorrelation function is precalculated and stored in the table given in Table 5.3-17, for a resolution of 8 samples at 32 kHz. The delta ITD is then first converted to 32 kHz, and the corresponding offset is found by linear interpolation of the two Wn_table entries.

Based on this normalized autocorrelation function WX(n), the offset parameter rt is calculated as:

rt=2 cc+121-WX(∆ITD)1+c2+2 c WX(∆ITD),

(5.3-142)

where

c=EL[b]ER[b].

(5.3-143)

The residual gains r[b] is then corrected as given in the equation:

r[b]←max{0,r[b]-rt}.

(5.3-144)

CHANGE 15

##### 5.3.2.4.13 	Reverberation gain parameter determination

CHANGE 16

##### 5.3.3.8.1	Spectrum quantization and coding

In general, quantization and coding of the MDCT coefficients follows the procedure described for EVS in clause 5.3.3.2.7 to 5.3.3.2.13 of [3]. For changes with respect to clause 5.3.3.2.11 (Intelligent Gap Filling) see clause 5.2.2.3.3.4 above. The Arithmetic coder described in [3], clause 5.3.3.2.8, has been replaced by a range coder for all MDCT-based encoding modes, see clause 5.2.2.3.3.3. The remaining changes to the MDCT spectrum quantization are described in the following.

Generally, in MDCT-based stereo coding, the context based arithmetic coder with no Harmonic Model is used. Residual quantization as described in clause 5.3.3.2.9.3 of [3] is used only for bitrates <= 64 kbps. Also, no Adaptive LowFrequency Emphasis (ALFE) is applied to the spectrum before quantization, hence clause 5.3.3.2.4 of [3] is not applied.

CHANGE 17

##### 5.3.5.1.4	Stereo CNG ITD and IPD encoding

The ITD value ITD(m) is obtained as in 5.3.2.4.4 and the IPD value gIPD(m) is obtained as in clause 5.3.2.4.7 and applied in the down-mix creation as described in clause 5.3.2.4.10 even before the decision is made to activate the Unified stereo CNG. In SID frames a coarser quantization step and narrower range is applied on the ITD in comparison to the active frames. The SID ITD values are calculated as follows,

ITDSIDm=max15,  ITDm-2562, ITDm>255max15,  ITDm2, ITDm≤255

(5.3-309)

CHANGE 18

##### 5.6.2.3.2	Bitrate adaptation based on ISM importance

In the ISM format, there is used a classification of ISM importance based on several core-coder parameters, namely local VAD (VADlocal, see clause 5.1.12 in [3]), core-encoder type (coder_type, see clause 5.1.13 in [3]), and core-coder mode (flagtcxonly). The ISM importance classification is done as part of the bit-budget allocator for rating the importance of a particular ISM stream based on a metric how critical coding of an audio stream to obtain a given quality of a decoded synthesis is. As a result, four (4) distinct ISM importance classes, classISM, are defined:

-	No metadata class, ISM_NO_META: frames without metadata coding, i.e. inactive frames with VADlocal=0;

-	Low importance class, ISM_LOW_IMP: frames where coder_type is UNVOICED or INACTIVE;

-	Medium importance class, ISM_MEDIUM_IMP: frames where coder_type is VOICED;

-	High importance class ISM_HIGH_IMP: frames where coder_type is GENERIC.

In case of core-coder employing only the TCX core, i.e. flagtcxonly=1, inactive frames with VADlocal=0 are classified as ISM_LOW_IMP and frames with coder_type being UNVOICED are classified as ISM_HIGH_IMP.

The ISM importance class is then used by the bit-budget allocator, in the bitrate adaptation algorithm (see clause 5.6.2.3, step 6) to assign a higher bit-budget to streams with a higher ISM importance and a lower bit-budget to streams with a lower ISM importance. Thus, for every audio stream n,  n=0,…,NISM-1, the following bitrate adaptation algorithm is used by the bit-budget allocator:

Step 1: In frames classified as classISM = ISM_NO_META, a constant low bitrate BVAD0=2450 bps is assigned.

Step 2: In frames classified as classISM = ISM_LOW_IMP, the core-coder total bitrate, bratetotal, is lowered as:

brate'total[n]=maxαlow∙bratetotaln,Blow	(5.6-10)

where the constant αlow=0.6. Then the constant Blow represents a minimum bitrate threshold supported by the core-coder for a particular configuration, which is dependent on the internal sampling rate of the codec and the coded audio bandwidth.

Step 3: In frames classified as classISM = ISM_MEDIUM_IMP, the core-coder total bitrate, total_brate, is lowered as

brate'total[n]=maxαmed∙bratetotaln,Blow  	(5.6-11)

where the constant αmed=0.8.

Step 4: In frames classified as classISM = ISM_HIGH_IMP, no bitrate adaptation is used, i.e.

brate'totaln=bratetotal[n]. 	(5.6-12)

Step 5: Finally, the saved bit-budget (a sum of differences between the initial (bratetotal) and new (brate'total) total bitrates) is redistributed equally between the streams with active content in the frame. The following bit-budget redistribution logic is used:

First, the saved bit-budget is computed using the following relation:

bitsdiff=n=0NISM-1bitsCoreCoder,newn-bitsCoreCoder,new[n]   	(5.6-13)

where

bitsCoreCoder,new[n]=brate'total[n]50  .	(5.6-14)

CHANGE 19

#### 5.7.4.6	ParamMC analysis windowing and MDFT

Every frame of the multi-channel input signal si(n) is divided into 8 time slots and each time slot is windowed with a sinusoidal window. Figure 5.7-4 shows the time slot windowing and MDFT. The input to MDFT is a 5 ms long time domain signal si,mPMCana, mdft n , where m is the subframe index with 0≤m<8 and n is the sample index with 0≤n<10 samplingfrequency2000 , that is formed by windowing a time slot of si(n) and then applying the sin window function as shown in Equations (5.7-14), (5.7-15), and (5.7-16).

si,mPMCana, mdft n= s'i,mPMCana nWinPMCana(n) ,

(5.7-14)

s'i,mPMCana n=0,                                                1≤n≤(Lsf-Lfade)simLsf-Lprev+n,   Lsf-Lfade≤ n<(2Lsf)	(5.7-15)

WinPMCanan= 0,                            0≤n<(Lsf-Lfade) sin2n-Lsf-LfadeLfade-12Lfade ,Lsf-Lfade≤n<(Lsf)   1,              Lsf-Lprev+ Loffset+  Lfade<n≤2Lsf- Lfade1-sin2n-2Lsf- LfadeLfade-12Lfade ,                        2Lsf- Lfade≤n<2Lsf

(5.7-16)

where Lframe=20Lms , Lsf=2.5Lms, Lprev=8.5Lms  , Lfade=Lms ,   Lms= samplingfrequency1000,  m is the subframe index with 0≤m<8 and  sin where n<0 are samples from the previous frame.

Figure 5.7-4: Subframe windowing and MDFT

For each m, si,mPMCana, mdft n is converted to frequency domain signal Yk,s(b)  by applying the MDFT (as described in subclause 5.2.5.2) to si,mPMCana, mdft n, where m is the subframe index and b is the frequency bin index.

CHANGE 20

### 5.7.5	Parametric upmix MC coding mode

#### 5.7.5.1	General

For 7.1.4 MC layout operation at 160 kbps, a parametric upmix MC coding mode is used offering highly efficient coding for this configuration.

CHANGE 21

### 5.8.1	OSBA format overview

The encoder supports combined input with 1 – 4 ISMs and an SBA signal of order 1 – 3. Depending on the IVAS total bitrate and number of ISMs, different OSBA coding modes summarized in Table 5.8-1 are employed to combine these input signals.

Input to IVAS, consisting of audio signals in SBA and ISM formats and the associated metadata, is processed through a simplification stage and an encoding stage. At the simplification stage, the SBA and ISM signals are converted into a mezzanine format, as described in clauses 5.8.2 and 5.8.3. In the pre-rendering OSBA mode, the mezzanine format is the First order Ambisonics (FOA) format. In the discrete OSBA mode, the mezzanine format includes an Ambisonics signal and all discrete ISM objects. At the encoding stage, the audio output of the simplification stage is encoded into the IVAS bitstream which is then transmitted to the decoder.

CHANGE 22  (note: apply correct font color for paragraph: “In the instance that…”)

#### 5.9.4.2 	Low-bitrate pre-rendering coding method

Once the MASA and audio object content have been merged to the MASA format, consisting of the 2 transport audio signal channels and MASA metadata, encoding of the aforementioned “merged” MASA format is performed on the basis that the “merged” MASA format is treated as a stereo-MASA format content at the corresponding overall bitrate. The coding format is signalled as a MASA format at the beginning of the bitstream. In addition, the number of input audio objects is encoded into the bitstream using two bits reserved from the encoding of the MASA metadata, which is as follows:

- ‘01’ if there are 4 objects,

- ‘10’ if there are 3 objects,

- ‘11’ if there are 1 or 2 objects.

In the instance that the number of input audio objects is 1 or 2, i.e., encoded as ‘11’ according to the list above, the bit used to signal the number of transport channels for the MASA format (the MASA number of transport channel signal bit) is repurposed for use in distinguishing between 1 and 2 audio objects. Thus, when the MASA number of transport channel signal bit is ‘0’ this indicates the case of 1 input audio object, and when the MASA number of transport channel signal bit is ‘1’ this indicates the case of 2 input audio objects. In the instance that the number of objects is 3 or 4, the MASA number of transport channel signal bit is used for the encoding of the combined MASA format audio signal (formed by combining the input audio objects converted into the MASA format with the input MASA audio signal comprising the transport audio signals and MASA metadata).

CHANGE 23

### 5.9.5 	One object with MASA representation (One OMASA) coding mode

CHANGE 24  (style for readability and ToC)

#### 5.11.3.2	Inter spectral difference

CHANGE 25  (style for readability and ToC)

#### 5.11.3.3	Inter-channel phase difference (IPD) and realigned inter-channel correlation (ICCr)

CHANGE 26  (style for readability and ToC)

#### 5.11.3.4	Selection rule for PHA sub-mode

CHANGE 27  (style for readability and ToC)

#### 5.11.3.5	Determination of filter taps

CHANGE 28  (style for readability and ToC)

##### 5.11.3.5.1	General

CHANGE 29  (style for readability and ToC)

##### 5.11.3.5.2	Filter taps for IPD

##### 5.11.3.5.3	Filter taps for IPD2

CHANGE 30  (style for readability and ToC)

##### 5.11.3.5.4	Truncation and normalization

CHANGE 31  (style for readability and ToC)

#### 5.11.3.6	Downmix process

CHANGE 32

#### 6.2.3.3 	Channel Pair Element (CPE) decoder

The Channel Pair Element (CPE) decoder is a decoding tool that decodes two transport channels. It is based on one or two core-decoder module(s) supplemented by stereo coding tools. Its block diagram is shown in Figure 6.2-17.

Figure 6.2-17: Block diagram of the Channel Pair Element (CPE) decoder

As seen from Figure 6.2-17, the CPE decoder consists of the following modules:

(a) The stereo mode and audio bandwidth information are read from the bitstream.

(b) Memory handling and stereo switching (clause 6.3.4) controls the switching between DFT stereo, TD stereo and MDCT stereo coding modes and comprises dynamic allocation/deallocation of static memory of stereo modes data structures depending on the current stereo mode. Also, the TD stereo mode is set in this module.

(c) Core-decoder configuration, one per transport channel: high-level parameters like core-decoder internal sampling-rate, core-decoder nominal bitrate etc. are set. Note that it depends on the element bitrate, brateelement, which is constant at one IVAS total bitrate, brateIVAS, and ensures that there is no frequent switching between different core-decoder set-ups.

(d) One or two variable bitrate core-decoder(s) (clause 6.2.2) performed sequentially on one or two downmixed channels.

CHANGE 33

##### 6.2.7.3.2	Energy estimation

For IVAS the energy estimation (clause 5.4.3.4 of [5]) is replaced by the following procedure.

Low-level signals are detected by analysing 1 ms subsegments h of the interleaved input signal x, including the previous frame. If all subsegment energies Eh of the subsegments to be merged are below a threshold Emax, with Emax=-65 dB , then the frame is scaled as much as possible and in a manner that there are no residual samples left after the scaling, i.e. Lres=0:

A 20 ms frame is shortened by setting s as:

s=send+modLTS,TSM-modsend,LTS,TSM,LTS,TSM-mod(LTS,TSM-Lres,TSM,LTS,TSM)	(6.2-88)

s=s+-sLts,TSMLts,TSM,  if s<0s-s-(send-nch,TSM)Lts,TSMLts,TSM,   if s>sends, else	(6.2-89)

where	send is send for time shrinking according to Table 2 of [5] with the parameters adapted to the actual number of channels to process.

A 20 ms is extended by setting s as:

s=sstart+modLTS,TSM-Lres,TSM,LTS,TSM	(6.2-90)

where	sstart is sstart for time stretching according to Table 3 of [5] with the parameters adapted to the actual number of channels to process.

The generation of the output frame is as in clause 5.4.3.7 of [5] with the adaption to the number of channels to process.

Note that for low-level signal the output frame is generated without similarity estimation or quality estimation.

CHANGE 34

##### 6.3.4.5.1 	Switching from DFT stereo to MDCT stereo

A mechanism similar to the decoder-side switching from the DFT stereo mode to the TD stereo mode is used in this scenario, where the primary yn and secondary xn channels of the TD stereo mode are replaced by the left ln and right rn channels of the MDCT stereo mode.

Since the signal stored in the DFT synthesis memory (see clause 6.3.2.3.9) has the DFT window applied to it, an undressing of the window is done in any MDCT stereo frame following a previous DFT stereo frame. This is done by multiplying the memory with the inverse DFT synthesis window.

Additionally, all required core-decoder state variables and buffers are copied over to the second channel in the transition frame.

CHANGE 35

##### 6.3.4.5.2 	Switching from MDCT stereo to DFT stereo

A mechanism similar to the decoder-side switching from the TD stereo mode to the DFT stereo mode is used in this scenario, where the primary yn and secondary xn channels of the TD stereo mode are replaced by the left ln and right rn channels of the MDCT stereo mode.

When switching to DFT stereo after an MDCT stereo frame, a passive downmix is applied to the core-decoder output memories for both full-band and low-band core-decoder output signals. These downmixed memories are used in the core decoding of the single DFT stereo core channel. Furthermore, they are used for a linear cross-fade between the memories and the core-decoder outputs (LB and FB) of the single decoded downmix channel before the DFT stereo upmix. The fade durations are chosen as 2.5 ms for the FB signals and 7.5 ms for the LB signals.

CHANGE 36

#### 6.3.5.1 	DTX in Unified stereo

##### 6.3.5.1.1 	General

If a Unified stereo CNG frame is received, the decoder generally operates as in DFT-based stereo mode as described in clause 6.3.2.3. The main difference compared to active frame DFT-based stereo decoding is that the stereo CNG decoder extracts the spectral shape represented by the decoded CNG frame and uses this for stereo CNG generation, as explained in clause 6.3.5.1.2. In addition, there are two differences compared to the EVS CNG decoding, matching the differences as described in clause 5.3.5.1.2.

- Core codec reset in active frame onset

- Energy scaling of CNG

The DFT-based stereo parameters ITDm, gIPDm, gSm,b are decoded from the bitstream as described in clause6.3.5.1.3 with the band resolution set by the encoder. In addition, the stereo coherence is decoded as described in clause 6.3.5.1.4 and a slightly modified DFT-based stereo synthesis is done as described in clause 6.3.5.1.5.

CHANGE 37

### 6.3.6 	Stereo output format conversion

#### 6.3.6.1 	Overview

The stereo format supports the following output channel configurations: mono, stereo (same as external output), multichannel. The output to stereo (or external) does not require any further processing while a conversion to other output configurations is described in following clauses.

CHANGE 38

#### 6.4.1.4 	HOA2 and HOA3 signal decoding at 384 kbps

CHANGE 39

#### 6.4.1.5 	HOA2 and HOA3 signal decoding at 512 kbps

CHANGE 40

### 6.4.9 	SBA bitrate switching

When the received bitstream signals a change in bitrate, the SBA decoder is reconfigured. This involves multiple steps.

First, the number of channels in the signal vectors Sdmx, Sumx,  Ndmx and Nspar_ana, and the SBA analysis order are determined according to Table 5.4-1 and Table 5.4-2. Then the CLDFB memory is allocated and initialized accordingly.

If PCA is used with the current bitrate but not the previous, the corresponding structures are set up according to clause 6.4.6.3. If it is used in the previous but not the current bitrate, the corresponding buffers are freed.

If the number of transport channels changes to 1 with bitrate switching, then AGC is initialized otherwise AGC is deallocated if the number of transport channels changes from 1 to a value greater than 1.

If the DirAC metadata time resolution is 20 ms or if the DirAC to SPAR conversion is disabled with the new IVAS bitrate, then SPAR parameter buffers and matrices are reallocated with 1 frame instead of 4 subframes to save memory.

The table row index for SPAR bitrate distribution table is computed as per the new IVAS bitrate and SBA analysis order. If Ndmx changes, the SPAR decoder is re-configured. In this step, the necessary number of TD decorrelators is set up, the SPAR MD decoder is reconfigured, the buffer for the upmix matrix is allocated with the correct dimensions, and the necessary number of JBM transport-channel buffers is allocated.

The renderer is selected, configured, and initialized according to the current bitrate.

The number DirAC parameters band and DoAs are re-computed and the buffers allocated accordingly. The flag for the higher-order mode is activated is the current bitrate is 384 kbps or 512 kbps. The sector decoding paths are configured according to clause 6.4.6.5.5.

The core-decoder is re-configured to take into account the updated number of channels in the first set of transport channels, Ndmx, and the bitrate itself.

If the number of transport channels differs between the current and the previous bitrate, then HP20 filter buffers are allocated or deallocated to match the new number of transport channels. If the granularity or Ndmx changes, the JBM parameters are adjusted accordingly. The renderer buffers are flushed.

The JBM re-configuration upon a change of the IVAS bitrate is described in clause 6.2.7.5.

CHANGE 41

### 6.4.10 	SBA output format conversion

The decoded audio scene can be rendered to different formats directly by the DirAC decoder. Hence a format conversion is not required. The output is generated using the methods according to the clauses listed in Table 6.4-7.

Table

6.4-7

: Rendering method from SBA format to different output formats

| Output format | clause |
| --- | --- |
| FOA | 6.4.6.4 |
| HOA (high-order mode) | 6.4.6.5.3 |
| HOA (low-order mode) | 6.4.6.5.4 |
| BINAURAL | 6.4.6.5.6 |
| BINAURAL_ROOM_[IR, REVERB] | 6.4.6.5.6 |
| Multi-channel | 6.4.6.5.7 |
| Stereo | 6.4.6.5.8 |
| Mono | 6.4.6.5.8.5 |
| EXT | direct output |

CHANGE 42

###### 6.7.1.7.3.7	Generation of substitution signal

With the coefficients of the all-pole resonator filter available, the samples of the substitution frame in subsampled time domain are generated by IIR filtering according to the following filter recursion:

sn=k=120akγkŝ(n-k) ,  n=0…L-1   .	 (6.7-16)

Note that prior to the filtering, the filter memories are initialized with the most recent samples of the buffered previously synthesized LFE channel signal in down-sampled domain:

sn=s240-20+n ,  n=-20…-1    .	 (6.7-17)

The filtering is carried out for L=58 samples in down sampled domain, which accommodates for 32 samples of a frame, 16 samples needed for overlap-add processing in the LFE MDCT decoding/reconstruction framework and 10 samples delay of the lowpass filter used in up-sampling to a sampling rate of 48000 Hz.

CHANGE 43

### 6.7.6 	MC bitrate switching

When switching to McMASA mode (see clause 6.7.2), the number of transport channels and the separate channel parameters are determined based on the new bitrate. Then the McMASA decoder is reconfigured by re-initializing all buffers and variables.

When switching to parametric MC mode (see clause 6.7.3) from a different MC mode, the correct number of transport channels is set and the necessary initializations of ParamMC buffers and variables are done. If the previous mode was also ParamMC the decoder is reconfigured according to clause 6.7.3.2.

Finally, a core-decoder reconfiguration is performed to reflect possible changes in number of transport channels and perchannel bitrate.

CHANGE 44

### 6.7.7 	MC output format conversion

#### 6.7.7.1 	Overview

For discrete multichannel mode, in case the output multichannel format is different from the input layout, the loudspeaker renderer performs a format conversion by applying a conversion matrix to generate the output channels. This conversion matrix can either be an upmix or downmix matrix depending on the input and output loudspeaker layouts. If the output layout is one of the supported IVAS formats (see clause 4.3.2) a conversion matrix Mconv is constructed using coefficients stored in ROM. If the output layout is a custom loudspeaker layout (see clause 7.4.9) then EFAP is used to construct the conversion matrix Mconv. This matrix is of dimensions Nin×Nout where Nin is the number of input channels and Nout is the number of output channels.

Once this conversion matrix has been constructed, the matrix is used for time domain rendering by multiplication on the input channels to render the output channels. In case of ParamMC mode this matrix is used either in the covariance or CLDFB domain (see clause 6.7.3.6.3).

For Parametric Upmix mode, except for mono, stereo and binaural output formats, the 7.1.4 MC layout format of that coding mode is converted as described under clauses 6.7.7.4 - 6.7.7.5 to the respective target output format. For binaural output format the decoding is modified to binaural output format, as described in clause 6.7.4.2.1. For mono and stereo output formats the decoding is modified to output 5.1.2 MC layout format, as described in clause 6.7.4.2.2, and then further processing is applied as described under 6.7.7.4.1.

When the IVAS decoder is configured for external processing (EXT) output, the output audio contains audio channels with their number and layout corresponding to the input MC input configuration.

CHANGE 45

### 6.8.4	OSBA bitrate switching

CHANGE 46

### 6.8.5	OSBA output format conversion

CHANGE 47

### 6.8.6 	OSBA decoding with TSM

The processing of the time-scale modified frames in the OSBA decoder follows from the processing in the ISM and SBA format.

In the low-bitrate OSBA decoding mode, only the SBA decoder runs, and the audio contents of the object are pre-rendered into the SBA signal on the encoder side. Hence, the same processing is performed as in clause 6.4.11.

In the high-bitrate OSBA decoding mode, the additional transport channels containing the object audio are decoded by MCT. These channels are additionally written to the transport channel buffer. Additionally, the ISM and SBA metadata are decoded. Specifically, the high-bitrate OSBA transport-channel decoding stage comprises the following steps:

1.	ISM metadata decoding (see clause 6.6.3)

2.	SPAR MD decoding (see clause 6.4.3)

3.	DirAC MD decoding (see clause 6.4.2)

4.	Audio-channel decoding using SCE, CPE, or MCT

5.	If STEREO output is configured

a.	AGC decoding (see clause 5.4.7)

b.	PCA decoding (see clause 5.4.6)

6.	Calculation of the SPAR upmix matrix according to clause 6.4.5

a.	HP20 filtering of the transport channels

7.	If parametric binaural or binaural room rendering is configured

a.	SBA to binaural mixing matrix is calculated (see clause 7.2.2.3)

b.	Loudness correction is applied to the SBA TCs

8.	If rendering is not disabled or configured to stereo

a.	AGC and PCA are decoded (see clauses 5.4.7 and 5.4.6)

9.	If mono output is configured

a.	The W channel is copied to the transport-channel buffer

b.	ISMs are downmixed into a mono channel (see clause 6.6.7.2)

After this decoding, the transport channels are processed by the TSM and the metadata are interpolated. The time-scale modified audio channels are stored in the transport channel buffer according to according to clause 6.2.7.2. Local subframes are calculated according to clause 6.2.7.4.3.1 and fed to the rendering stage.

The metadata mapping mts,SPAR for the SPAR upmix parameters is determined according to clause 6.4.11. Again the mapping is different for the high- and low-order DirAC modes. The ISM metadata are interpolated according to clause 6.6.7.

Unlike in SBA format, the granularity in OSBA format is set to 5 ms as the ISMs are always processed with this time resolution and the lower resolution of both modes must be used.

The rendering stage is again equivalent to that of SBA in OSBA pre-rendering mode. In OSBA discrete mode, ISM rendering and SPAR and DirAC decoding run concurrently. This case involves the following steps for each 5 ms subframe.

1. If BINAURAL output is configured

a. DirAC decoding is applied according to clause 6.4.6.5.3 or 6.4.6.5.4

b. Decoded HOA3 output is rendered binaurally using FastConv (see clause 7.2.2.4)

c. ISM objects are rendered to binaural using the TD renderer (see clause 7.2.2.2)

d. Rendered outputs are multiplied by 0.5 and added up

2. If STEREO output is configured

a. ISM objects are rendered to stereo (see clause 6.6.7)

b. The SBA stereo stream generated in the TC decoding phase is loaded from the TC buffer

c. These two outputs are added up

3. If MONO is configured

a. No further processing is required as the output is already in the TC buffer

4. If FOA/HOA or MC output is configured

a. SBA and ISM are rendered to the configured output format MC (see clauses 6.4.10 and 6.6.7)

5. If BINAURAL_ROOM_IR/BINAURAL_ROOM_REVERB output is configured

a. SBA and ISM are rendered to MC (see clauses 6.4.10 and 6.6.7)

b. MC is binauralized using the FastConv renderer (see clause 7.2.2.4)

6. If EXT output is configured

a. SBA is rendered to SBA output with the ambisonics order corresponding to the input ambisonics order present at the encoder input (see clauses 6.4.6.5.3 or 6.4.6.5.4)

b. Object audio channels are copied from the transport-channel buffer to the output buffer.

CHANGE 48

#### 6.9.7.5 	Output for external processing

See clause 6.9.11.

CHANGE 49

### 6.9.11		OMASA decoding to original combined input format (EXT output)

CHANGE 50

## 7.1 	Rendering overview

The general rendering overview is provided in clause 4.5. This clause discusses on rendering modes for loudspeakers and headphones reproduction (clause 7.2), room acoustics rendering (clause 7.3), rendering control (clause 7.4), and pre-rendering (clause 7.5). Finally, binaural split rendering feature is discussed in clause 7.6.

The IVAS renderer (both the internal one and the external one) processes audio signals in time segments of 5 ms. The renderer frame size can be set to 5, 10, or 20 (default) ms.

CHANGE 51

### 7.5.3 	Pre-rendering into MASA format

#### 7.5.3.1 	Overview

When at least one of the audio inputs to the pre-rendering is a MASA format input, IVAS pre-rendering is always performed into MASA format. This is because rendering from MASA to another format generally results in a rendering delay of 5 ms. This rendering delay can also be seen in Table 4.4-1. On the other hand, generating a MASA format signal from another spatial audio format signal does not require any additional delay.

Pre-rendering into MASA format is performed as detailed in the following.

CHANGE 52

## 7.6	Split rendering

### 7.6.1	Overview

IVAS supports split rendering wherein the process of binaural rendering and headtracking is split between a main device (pre-renderer) and a light-weight head-worn device (post-renderer). The split-rendering architecture in IVAS is such that the complexity at the post-renderer is substantially less than the complexity of the IVAS decoder and renderer.

There are two architectures of split rendering supported in IVAS. The first architecture, described in Figure 7.6-1, extends IVAS decoder and internal renderer to perform the pre-rendering part of split renderer, whereas the post rendering is done using a separate post-renderer. In the second architecture, described in Figure 7.6-2, the IVAS decoder runs in pass-through mode and the IVAS external renderer is extended to perform the pre-rendering part of split renderer.

Split rendering in IVAS is supported with all immersive input formats and 48 kHz sampling rate. For configuration of split rendering, a degree of freedom (DOF) may be specified at the main device (pre-renderer), which determines the rotation axes for which pose correction can be performed at the light-weight device (post-renderer). The DOF value specifies the number of axes compensated, and a special value of 0 DOF means no pose correction capability is available at the post-renderer. The pre-rendering configuration is further described in clause 7.6.2.

Figure 7.6-1: Split rendering architecture with IVAS decoder

Figure 7.6-2: Split rendering architecture with IVAS external renderer

CHANGE 53

###### 7.6.4.2.3.3	Parameter Computation and Quantization

The phase alignment and prediction parameters are computed based on signal energies and (complex-valued) left/right covariance. Phase alignment is only considered if the left/right cross-correlation coefficient exceeds a threshold of 0.5, otherwise the phase parameter it is set to 0.

The left and right signal energies are computed as

EkL=n=0NFrame-1b∈k Xb,nL2 ,

EkR=n=0NFrame-1b∈k Xb,nR2 ,

Where Xb,nL and Xb,nRis the complex left and right input sample respectively and time index n, b is the LCLD band belonging to the perceptual band k and NFrame is the number of samples per frame and LCLD band. The left/right covariance is computed as

CkLR=n=0NFrame-1b∈k Xb,nLXb,n*R ,

where the * indicates complex conjugate. The inter-channel phase difference is computed from the covariance as

φk=arctan2realCkLRimagCkLR if CkLR2>0.5⋅EkLEkR .

Otherwise φk=0.

The phase quantization index is given as φkidx=roundφk⋅12π. If φkidxequals 12 then φkidxis set to 0.

And the quantized phase is

φkQ=φkidx⋅π12 .

In the case of left/right phase alignment using the quantized phase, the modified covariance is given as

Ck,φLR=CkLR⋅ejφkQ* .

The mid and side signal energies are computed from original or phase-modified covariance as

Ek,φM=0.25⋅EkL+EkR+2⋅realCk,φLR ,

Ek,φS=0.25⋅EkL+EkR-2⋅realCk,φLR .

The real-valued side prediction parameter which minimizes the side signal energy is computed as

pk=0.25⋅EkL-EkREk,φM  .

The quantized prediction coefficient is computed as

pkQ=maxminroundpk⋅12,12,-1212 .

Finally, the side signal (residual) energy, dependent on application of phase alignment and/or side prediction is computed as

Ek,φ,pS=Ek,φS+pkQ2Ek,φM-0.5⋅pkQ⋅EkL-EkR .

The mid and side signal energies can also be expressed in terms of the stereo coding type t defined in Table 7.6-10. They are provided below in Table 7.6-10a where k is the perceptual band index:

Table 7.6-10a: Mid and side signal energies for different joint stereo coding types

| Coding Type t | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Parameters | φkQ=0, pkQ=0 | φkQ, pkQ=0 | φkQ=0, pkQ | φkQ, pkQ |

CHANGE 54

##### 7.6.4.2.6	Perceptual Model

The perceptual model is designed to operate in both the encoder and decoder based only on transmitted information. Specifically, only the RMS envelope (see clause 7.6.4.2.5) is used to compute the target SNR requirements, for each channel, for each perceptual band, and each group segment of a frame. As the target SNR calculated by the perceptual model in both the encoder and decoder must match exactly, all arithmetic calculations described here are integer calculations with a maximum precision of 24 bits.

The perceptual model computes the target SNR for just noticeable distortion JND (the signal to mask ratio) for each group in a frame and for each perceptual band. As the RMS envelope is transmitted to the decoder in a log space the perceptual model is also computed in a log space. The following steps are used to compute the signal to mask ratio (SMRgm) for each group and perceptual band:

Step 1: Scale the RMS envelope and add the log domain width of the perceptual band as shown in equation (7.6-35):

Egm=64∙Rgm+64∙log2bm ,	(7.6-35)

where:

Egm is the scaled energy for the gth group and mth perceptual band,

Rgm is the RMS envelope for the gth group and mth perceptual band,

64∙log2bm is the scaled log domain bandwidth of the mth perceptual band, and

∙ is a floor operation.

The scaled log domain bandwidth (64∙log2bm ) values are constant so can be pre-calculated as shown in Table 7.6-13. In Table 7.6-13, 64∙log2bm is expressed as BWAm.

Step 2: Compute the sensation level for each perceptual band as the energy level of the band relative to the absolute threshold of hearing as shown in equation (7.6-36):

SVgm=Egm-TQm ,	(7.6-36)

where:

SVgm is the sensation level for the gth group in a frame and mth perceptual band, and

TQm is the hearing threshold in quiet. The values for  TQm are provided in Table 7.6-14.

Step 3: Compute the mask offset due to the sensation level as shown in equation (7.6-37):

SVmOffset= DmSVm256 ,	(7.6-37)

where:

SVmOffset is the mask offset due to sensation level, and

Dm is the sensation level offset gradient for each perceptual band. The values for Dm are provided in Table 7.6-15.

Step 4: Offset the energy in each perceptual band by the sensation level offset as shown in equation (7.6-38):

EgmOffset=Egm-SVmOffset ,	(7.6-38)

where:

EgmOffset is the energy for the gth group and mth perceptual band offset by the sensation level offset.

Step 5: Convert the sensation offset energy in each perceptual band to the loudness domain as shown in 
equation (7.6-39):

EgmLoudnes=614∙EgmOffset2048 .	(7.6-39)

Step 6: Spread the loudness energy in each perceptual band with a spreading function as shown in equation (7.6-40):

MgmLoudness=LogAddEgmLoudnes,EglLoudnes+SFml  ∀l= 0,1,…M-1 ,	(7.6-40)

where:

MgmLoudness is the loudness domain masking level,

SFml is the spreading function applied to the mth perceptual band due to the energy in the lth perceptual band. The values for SFml are provided in table 7.6-16.

LogAdda,b is a table look-up approximation of a multiply and add in the linear domain as shown in equation 
(7.6-41):

LogAdda,b=a+La-ba>bb+Lb-ab≥a  ,	(7.6-41)

where:

La is a table look up, the values of La are provided in Table 7.6-17.

Step 7: Convert mask back from the loudness domain as shown in equation (7.6-42):

Mgm=6827∙MgmLoudness2048	  	(7.6-42)

Step 8: Compute the signal to mask ratio (target SNR at JND) for the gth group and mth perceptual band as shown in equation (7.6-43):

SMRgm=Egm-Mgm .	(7.6-43)

The above steps are for mono or independent channel coding, however, if 2 channels are present in the signal and joint channel coding is in use, then there is an additional step after step 7. The additional step sets the masking level in both channels to be the greater of the two masking levels as shown in equation (7.6-44). This additional step is only used if there are 2 channels present, the mth band is jointly coded, and the band m is greater than 0.

Mgm1=Mgm1Mgm1≥Mgm2Mgm2Mgm2>Mgm1  ,	(7.6-44)

Mgm2=Mgm1  .

Table 7.6-13: Bandwidth adjust values (BWAm) for each perceptual band

| Band (m) | BWAm | Band (m) | BWAm |
| --- | --- | --- | --- |
| 0 | 0 | 12 | 64 |
| 1 | 0 | 13 | 64 |
| 2 | 0 | 14 | 64 |
| 3 | 0 | 15 | 64 |
| 4 | 0 | 16 | 101 |
| 5 | 0 | 17 | 101 |
| 6 | 0 | 18 | 128 |
| 7 | 0 | 19 | 165 |
| 8 | 0 | 20 | 165 |
| 9 | 0 | 21 | 180 |
| 10 | 0 | 22 | 213 |
| 11 | 64 |  |  |

Table 7.6-14: Threshold in quiet (TQm)for each perceptual band

| Band (m) | TQm | Band (m) | TQm |
| --- | --- | --- | --- |
| 0 | 133 | 12 | 282 |
| 1 | 133 | 13 | 307 |
| 2 | 133 | 14 | 330 |
| 3 | 133 | 15 | 352 |
| 4 | 133 | 16 | 404 |
| 5 | 133 | 17 | 461 |
| 6 | 133 | 18 | 525 |
| 7 | 133 | 19 | 631 |
| 8 | 138 | 20 | 1249 |
| 9 | 159 | 21 | 1511 |
| 10 | 183 | 22 | 1519 |
| 11 | 241 |  |  |

Table 7.6-15: Sensation level offset gradient (Dm) for each perceptual band

| Band (m) | Dm | Band (m) | Dm |
| --- | --- | --- | --- |
| 0 | 112 | 12 | 64 |
| 1 | 112 | 13 | 64 |
| 2 | 96 | 14 | 64 |
| 3 | 80 | 15 | 64 |
| 4 | 80 | 16 | 64 |
| 5 | 64 | 17 | 64 |
| 6 | 64 | 18 | 64 |
| 7 | 64 | 19 | 64 |
| 8 | 64 | 20 | 64 |
| 9 | 64 | 21 | 64 |
| 10 | 64 | 22 | 64 |
| 11 | 64 |  |  |

Table 7.6-16: Spreading function (SFml)

| m | SFml |
| --- | --- |
| 0 | 0, -1561, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552 |
| 1 | -289, -4, -1234, -2295, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552 |
| 2 | -569, -229, -8, -905, -1705, -2324, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552 |
| 3 | -789, -445, -173, -16, -656, -1271, -1765, -2172, -2520, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552 |
| 4 | -961, -616, -340, -136, -28, -488, -976, -1382, -1729, -2032, -2305, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552 |
| 5 | -1088, -743, -465, -257, -148, -31, -371, -769, -1114, -1417, -1689, -2054, -2483, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552 |
| 6 | -1198, -852, -574, -364, -209, -148, -42, -300, -635, -936, -1207, -1572, -2000, -2376, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552 |
| 7 | -1293, -948, -669, -458, -301, -183, -145, -56, -258, -547, -816, -1179, -1606, -1982, -2311, -2552, -2552, -2552, -2552, -2552, -2552, -2552, -2552 |
| 8 | -1375, -1029, -750, -539, -381, -260, -180, -142, -68, -231, -487, -846, -1272, -1647, -1976, -2261, -2552, -2552, -2552, -2552, -2552, -2552, -2552 |
| 9 | -1444, -1099, -820, -608, -449, -328, -233, -194, -138, -77, -213, -555, -978, -1352, -1681, -1966, -2268, -2552, -2552, -2552, -2552, -2552, -2552 |
| 10 | -1501, -1155, -876, -665, -505, -383, -287, -210, -193, -130, -79, -298, -711, -1083, -1411, -1696, -1997, -2288, -2550, -2552, -2552, -2552, -2552 |
| 11 | -1567, -1221, -942, -730, -570, -448, -351, -272, -206, -189, -151, -72, -349, -713, -1039, -1324, -1625, -1915, -2177, -2448, -2552, -2552, -2552 |
| 12 | -1650, -1304, -1025, -813, -653, -530, -432, -352, -285, -227, -177, -163, -69, -297, -613, -895, -1195, -1485, -1746, -2017, -2238, -2401, -2545 |
| 13 | -1727, -1381, -1102, -890, -730, -607, -509, -428, -360, -301, -249, -180, -153, -72, -257, -527, -824, -1112, -1373, -1643, -1865, -2028, -2171 |
| 14 | -1798, -1452, -1173, -960, -800, -677, -579, -498, -430, -370, -317, -246, -192, -145, -76, -224, -505, -790, -1050, -1320, -1540, -1703, -1847 |
| 15 | -1860, -1514, -1234, -1022, -862, -738, -640, -559, -490, -430, -377, -306, -224, -197, -136, -81, -242, -515, -771, -1040, -1260, -1422, -1566 |
| 16 | -1923, -1577, -1297, -1085, -925, -801, -703, -621, -553, -492, -439, -367, -284, -213, -198, -144, -83, -235, -479, -744, -963, -1125, -1268 |
| 17 | -1986, -1640, -1360, -1148, -988, -864, -766, -684, -615, -555, -501, -429, -345, -273, -211, -204, -146, -89, -216, -465, -680, -841, -984 |
| 18 | -2043, -1697, -1417, -1205, -1044, -921, -822, -741, -672, -611, -557, -485, -401, -328, -264, -211, -205, -140, -93, -227, -430, -588, -729 |
| 19 | -2104, -1758, -1479, -1266, -1106, -982, -884, -802, -733, -673, -619, -546, -461, -388, -324, -269, -212, -211, -151, -100, -195, -336, -472 |
| 20 | -2163, -1817, -1537, -1324, -1164, -1040, -942, -860, -791, -731, -676, -604, -519, -445, -380, -325, -268, -226, -219, -147, -114, -167, -280 |
| 21 | -2203, -1857, -1577, -1365, -1205, -1081, -982, -901, -831, -771, -717, -644, -559, -485, -420, -364, -306, -252, -239, -206, -132, -122, -163 |
| 22 | -2224, -1878, -1598, -1386, -1225, -1102, -1003, -921, -852, -792, -737, -665, -580, -505, -441, -385, -326, -271, -222, -224, -176, -121, -114 |

Table 7.6-17: Log Add look up table values (La)

| a | L[a] | a | L[a] | a | L[a] | a | L[a] | a | L[a] | a | L[a] | a | L[a] | a | L[a] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 64 | 64 | 37 | 128 | 21 | 192 | 11 | 256 | 6 | 320 | 3 | 384 | 1 | 448 | 1 |
| 1 | 64 | 65 | 37 | 129 | 20 | 193 | 11 | 257 | 6 | 321 | 3 | 385 | 1 | 449 | 1 |
| 2 | 63 | 66 | 37 | 130 | 20 | 194 | 11 | 258 | 5 | 322 | 3 | 386 | 1 | 450 | 1 |
| 3 | 63 | 67 | 36 | 131 | 20 | 195 | 11 | 259 | 5 | 323 | 3 | 387 | 1 | 451 | 1 |
| 4 | 62 | 68 | 36 | 132 | 20 | 196 | 10 | 260 | 5 | 324 | 3 | 388 | 1 | 452 | 1 |
| 5 | 62 | 69 | 36 | 133 | 20 | 197 | 10 | 261 | 5 | 325 | 3 | 389 | 1 | 453 | 1 |
| 6 | 61 | 70 | 35 | 134 | 19 | 198 | 10 | 262 | 5 | 326 | 3 | 390 | 1 | 454 | 1 |
| 7 | 61 | 71 | 35 | 135 | 19 | 199 | 10 | 263 | 5 | 327 | 3 | 391 | 1 | 455 | 1 |
| 8 | 60 | 72 | 35 | 136 | 19 | 200 | 10 | 264 | 5 | 328 | 3 | 392 | 1 | 456 | 1 |
| 9 | 60 | 73 | 35 | 137 | 19 | 201 | 10 | 265 | 5 | 329 | 3 | 393 | 1 | 457 | 1 |
| 10 | 59 | 74 | 34 | 138 | 19 | 202 | 10 | 266 | 5 | 330 | 3 | 394 | 1 | 458 | 1 |
| 11 | 59 | 75 | 34 | 139 | 19 | 203 | 10 | 267 | 5 | 331 | 3 | 395 | 1 | 459 | 1 |
| 12 | 58 | 76 | 34 | 140 | 18 | 204 | 10 | 268 | 5 | 332 | 2 | 396 | 1 | 460 | 1 |
| 13 | 58 | 77 | 33 | 141 | 18 | 205 | 10 | 269 | 5 | 333 | 2 | 397 | 1 | 461 | 1 |
| 14 | 57 | 78 | 33 | 142 | 18 | 206 | 9 | 270 | 5 | 334 | 2 | 398 | 1 | 462 | 1 |
| 15 | 57 | 79 | 33 | 143 | 18 | 207 | 9 | 271 | 5 | 335 | 2 | 399 | 1 | 463 | 1 |
| 16 | 56 | 80 | 32 | 144 | 18 | 208 | 9 | 272 | 5 | 336 | 2 | 400 | 1 | 464 | 1 |
| 17 | 56 | 81 | 32 | 145 | 17 | 209 | 9 | 273 | 5 | 337 | 2 | 401 | 1 | 465 | 1 |
| 18 | 55 | 82 | 32 | 146 | 17 | 210 | 9 | 274 | 5 | 338 | 2 | 402 | 1 | 466 | 1 |
| 19 | 55 | 83 | 32 | 147 | 17 | 211 | 9 | 275 | 5 | 339 | 2 | 403 | 1 | 467 | 1 |
| 20 | 55 | 84 | 31 | 148 | 17 | 212 | 9 | 276 | 5 | 340 | 2 | 404 | 1 | 468 | 1 |
| 21 | 54 | 85 | 31 | 149 | 17 | 213 | 9 | 277 | 4 | 341 | 2 | 405 | 1 | 469 | 1 |
| 22 | 54 | 86 | 31 | 150 | 17 | 214 | 9 | 278 | 4 | 342 | 2 | 406 | 1 | 470 | 1 |
| 23 | 53 | 87 | 30 | 151 | 16 | 215 | 9 | 279 | 4 | 343 | 2 | 407 | 1 | 471 | 1 |
| 24 | 53 | 88 | 30 | 152 | 16 | 216 | 8 | 280 | 4 | 344 | 2 | 408 | 1 | 472 | 1 |
| 25 | 52 | 89 | 30 | 153 | 16 | 217 | 8 | 281 | 4 | 345 | 2 | 409 | 1 | 473 | 1 |
| 26 | 52 | 90 | 30 | 154 | 16 | 218 | 8 | 282 | 4 | 346 | 2 | 410 | 1 | 474 | 1 |
| 27 | 51 | 91 | 29 | 155 | 16 | 219 | 8 | 283 | 4 | 347 | 2 | 411 | 1 | 475 | 1 |
| 28 | 51 | 92 | 29 | 156 | 16 | 220 | 8 | 284 | 4 | 348 | 2 | 412 | 1 | 476 | 1 |
| 29 | 51 | 93 | 29 | 157 | 15 | 221 | 8 | 285 | 4 | 349 | 2 | 413 | 1 | 477 | 1 |
| 30 | 50 | 94 | 28 | 158 | 15 | 222 | 8 | 286 | 4 | 350 | 2 | 414 | 1 | 478 | 1 |
| 31 | 50 | 95 | 28 | 159 | 15 | 223 | 8 | 287 | 4 | 351 | 2 | 415 | 1 | 479 | 1 |
| 32 | 49 | 96 | 28 | 160 | 15 | 224 | 8 | 288 | 4 | 352 | 2 | 416 | 1 | 480 | 1 |
| 33 | 49 | 97 | 28 | 161 | 15 | 225 | 8 | 289 | 4 | 353 | 2 | 417 | 1 | 481 | 1 |
| 34 | 49 | 98 | 27 | 162 | 15 | 226 | 8 | 290 | 4 | 354 | 2 | 418 | 1 | 482 | 0 |
| 35 | 48 | 99 | 27 | 163 | 15 | 227 | 8 | 291 | 4 | 355 | 2 | 419 | 1 | 483 | 0 |
| 36 | 48 | 100 | 27 | 164 | 14 | 228 | 8 | 292 | 4 | 356 | 2 | 420 | 1 | 484 | 0 |
| 37 | 47 | 101 | 27 | 165 | 14 | 229 | 7 | 293 | 4 | 357 | 2 | 421 | 1 | 485 | 0 |
| 38 | 47 | 102 | 26 | 166 | 14 | 230 | 7 | 294 | 4 | 358 | 2 | 422 | 1 | 486 | 0 |
| 39 | 47 | 103 | 26 | 167 | 14 | 231 | 7 | 295 | 4 | 359 | 2 | 423 | 1 | 487 | 0 |
| 40 | 46 | 104 | 26 | 168 | 14 | 232 | 7 | 296 | 4 | 360 | 2 | 424 | 1 | 488 | 0 |
| 41 | 46 | 105 | 26 | 169 | 14 | 233 | 7 | 297 | 4 | 361 | 2 | 425 | 1 | 489 | 0 |
| 42 | 45 | 106 | 25 | 170 | 14 | 234 | 7 | 298 | 4 | 362 | 2 | 426 | 1 | 490 | 0 |
| 43 | 45 | 107 | 25 | 171 | 13 | 235 | 7 | 299 | 4 | 363 | 2 | 427 | 1 | 491 | 0 |
| 44 | 45 | 108 | 25 | 172 | 13 | 236 | 7 | 300 | 4 | 364 | 2 | 428 | 1 | 492 | 0 |
| 45 | 44 | 109 | 25 | 173 | 13 | 237 | 7 | 301 | 3 | 365 | 2 | 429 | 1 | 493 | 0 |
| 46 | 44 | 110 | 24 | 174 | 13 | 238 | 7 | 302 | 3 | 366 | 2 | 430 | 1 | 494 | 0 |
| 47 | 43 | 111 | 24 | 175 | 13 | 239 | 7 | 303 | 3 | 367 | 2 | 431 | 1 | 495 | 0 |
| 48 | 43 | 112 | 24 | 176 | 13 | 240 | 7 | 304 | 3 | 368 | 2 | 432 | 1 | 496 | 0 |
| 49 | 43 | 113 | 24 | 177 | 13 | 241 | 7 | 305 | 3 | 369 | 2 | 433 | 1 | 497 | 0 |
| 50 | 42 | 114 | 24 | 178 | 13 | 242 | 6 | 306 | 3 | 370 | 2 | 434 | 1 | 498 | 0 |
| 51 | 42 | 115 | 23 | 179 | 12 | 243 | 6 | 307 | 3 | 371 | 2 | 435 | 1 | 499 | 0 |
| 52 | 42 | 116 | 23 | 180 | 12 | 244 | 6 | 308 | 3 | 372 | 2 | 436 | 1 | 500 | 0 |
| 53 | 41 | 117 | 23 | 181 | 12 | 245 | 6 | 309 | 3 | 373 | 2 | 437 | 1 | 501 | 0 |
| 54 | 41 | 118 | 23 | 182 | 12 | 246 | 6 | 310 | 3 | 374 | 2 | 438 | 1 | 502 | 0 |
| 55 | 41 | 119 | 22 | 183 | 12 | 247 | 6 | 311 | 3 | 375 | 2 | 439 | 1 | 503 | 0 |
| 56 | 40 | 120 | 22 | 184 | 12 | 248 | 6 | 312 | 3 | 376 | 2 | 440 | 1 | 504 | 0 |
| 57 | 40 | 121 | 22 | 185 | 12 | 249 | 6 | 313 | 3 | 377 | 2 | 441 | 1 | 505 | 0 |
| 58 | 39 | 122 | 22 | 186 | 12 | 250 | 6 | 314 | 3 | 378 | 2 | 442 | 1 | 506 | 0 |
| 59 | 39 | 123 | 22 | 187 | 11 | 251 | 6 | 315 | 3 | 379 | 2 | 443 | 1 | 507 | 0 |
| 60 | 39 | 124 | 21 | 188 | 11 | 252 | 6 | 316 | 3 | 380 | 1 | 444 | 1 | 508 | 0 |
| 61 | 38 | 125 | 21 | 189 | 11 | 253 | 6 | 317 | 3 | 381 | 1 | 445 | 1 | 509 | 0 |
| 62 | 38 | 126 | 21 | 190 | 11 | 254 | 6 | 318 | 3 | 382 | 1 | 446 | 1 | 510 | 0 |
| 63 | 38 | 127 | 21 | 191 | 11 | 255 | 6 | 319 | 3 | 383 | 1 | 447 | 1 | 511 | 0 |

CHANGE 55

###### 7.6.4.2.7.5	Prediction Signalling

Prediction processing is determined by the following bitstream elements:

Table 7.6-18: Prediction Syntax Elements

| Syntax Element | Comment | Number of Bits |
| --- | --- | --- |
| NumSubSets | Possible number of Subsets K | 3 |
| SubSetId | Current subset scur | if K > 4 3 Elseif K > 1 2 Else 0 |
| PredChanEnable | Prediction active/passive flags, one bit for each possible subset | K (per channel) |
| NumPredBands | The maximum LCLD band with active prediction for the current subset | if K >= 4 4 elseif K > 1 5 Else 6 (per channel) |
| PredBandEnable | Array of active/passive flags for all LCLD bands up to NumPredBands for each audio channel. | Int(NumPredBands / K)+1 (per channel) |
| PredictionMagnitude | For every active LCLD band belonging to the current subset for each audio channel | 3 bits per active LCLD band |
| Prediction Phase | For every active LCLD band belonging to the current subset for each audio channel | 5 bits per active LCLD band |

CHANGE 56

###### 7.6.4.2.9.3	Quantization of Normalized CLDFB coefficients and Prediction Residuals

The quantization of both normalized CLDFB coefficients and predictor residuals are the same and controlled by an allocation level generated by the bit allocation algorithm (see clause 7.6.4.2.8). The allocation level (Agm) is calculated for groups and perceptual bands, so the following equations assume the nth slot is a member of the gth group and the kth CLDFB band is a member of the mth perceptual band. The quantization of either the normalized CLDFB coefficients or the prediction residual is shown in equation (7.6-48):

Ink=ROUNDgAgm∙xnk  ,	(7.6-48)

where:

Qnk is a complex signed integer that will be Huffman coded and transmitted to the decoder,

Agm is the allocation level for the gth group and mth perceptual band,

gAgm is the allocation level scaling factor provided in table 7.6-19, and

xnk is either the normalized CLDFB coefficient (SnkCLDFB) or the prediction residual (enk) for the nth slot and kth CLDFB band. The nth slot is assumed to be an element of the gth group and the kth CLDFB band is assumed to be an element of the mth perceptual band.

As the quantized values for both the normalized CLDFB and the residual are needed when the prediction is enabled, equation 7.6-49 shows the inverse quantization from the quantization index (Ink):

xnk= g-1Agm∙Qnk  ,	(7.6-49)

where:

xnk is either the quantized CLDFB coefficient or the quantized residual and

g-1Agm is the inverse quantization scale factor provided in Table 7.6-19.

Table 7.6-19: Quantizer scaling factors () and inverse quantizer scaling factors () for each allocation level (Agm)

| Agm | gAgm | g-1Agm | Agm | gAgm | g-1Agm |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 16 | 4.7568 | 0.2098 |
| 1 | 0.3536 | 2.3675 | 17 | 5.6569 | 0.1765 |
| 2 | 0.4204 | 2.0464 | 18 | 6.7272 | 0.1485 |
| 3 | 0.5 | 1.7759 | 19 | 8 | 0.1249 |
| 4 | 0.5946 | 1.5364 | 20 | 9.5137 | 0.1051 |
| 5 | 0.7071 | 1.3231 | 21 | 11.3137 | 0.0884 |
| 6 | 0.8409 | 1.1329 | 22 | 13.4543 | 0.0743 |
| 7 | 1 | 0.9658 | 23 | 16 | 0.0625 |
| 8 | 1.1892 | 0.8213 | 24 | 19.0273 | 0.0526 |
| 9 | 1.4142 | 0.6951 | 25 | 22.6274 | 0.0442 |
| 10 | 1.6818 | 0.5878 | 26 | 26.9087 | 0.0372 |
| 11 | 2 | 0.4958 | 27 | 32 | 0.0312 |
| 12 | 2.3784 | 0.4181 | 28 | 38.0546 | 0.0263 |
| 13 | 2.8284 | 0.3522 | 29 | 45.2548 | 0.0221 |
| 14 | 3.3636 | 0.2962 | 30 | 53.8174 | 0.0186 |
| 15 | 4 | 0.2494 | 31 | 64 | 0.0156 |

###### 7.6.4.2.9.4	Huffman Coding of Quantized Normalized CLDFB coefficients and Quantized Prediction Residuals

As discussed in the previous clause 7.6.4.2.9.3, the output of the quantizer is a complex signed integer (Qnk) that will Huffman coded and transmitted to the decoder. Prior to Huffman coding, the complex signed integer is first separated into real and imaginary as shown in equation (7.6-50) and then further separated into a magnitude (Qnkrand Qnki) and a sign bit as shown in equation (7.6-51) for both real and imaginary parts:

Qnkr= RQnkQnki= IQnk	(7.6-50)

signnkr=1,  &Qnkr<00,  &Qnkr≥0signnki=1,  &Qnki<00,  &Qnki≥0  .	(7.6-51)

The magnitude component is then limited to a max value that is dependent on the allocation level (Agm), as shown in equation (7.6-52):

Qnkr=Qnkr,  &Qnkr<maxAgmmaxAgm,  &Qnkr≥maxAgmQnki=Qnki,  &Qnki<maxAgmmaxAgm,  &Qnki≥maxAgm   , 	(7.6-52)

where maxAgm is the maximum supported value depending on the allocation level Agm, the maximum values are provided in Table 7.6-20.

The Huffman codebook used to code the real and imaginary components is dependent on the allocation level (Agm,) and dependent on whether the underlying quantized values are prediction residuals or normalized CLDFB coefficients. Furthermore, for low values of the allocation level, the real and imaginary quantized magnitudes are packed into a single codeword. The Huffman packaging depends on the following logic:

- If Agm is equal to 0, no information is transmitted, and the decoder generates 0’s for all CLDFB coefficients in the gth group and mth perceptual band.

- If Agm is greater than 0 and less than 12, then the real and imaginary parts of the quantized magnitude are packed into a single code word. After the Huffman code the sign bit for Qnkr if Qnkr is greater than 0 then the sign bit for Qnki if Qnki is greater than 0. The code and number of bits written can be expressed as a table look up as follows:

code,length=HuffLookupresnkAgmQnkr∙max⁡[Agm+1+Qnki ,

where:

code is the codeword written to stream,

length is the number bits to write the code, and

resnk is a binary value that is 1 if the underlying quantized values are prediction residuals or 0 if the underlying quantized values are normalized CLDFB coefficients.

- If Agm is greater than or equal to 12, then the real and imaginary parts of the quantized magnitude are packed into separate codewords. After the Huffman code the sign bit for Qnkr if Qnkr is greater than 0 then the sign bit for Qnki if Qnki is greater than 0. The code and number of bits written can be expressed as a table look up as follows:

coder,lengthr=HuffLookupresnkAgmQnkr  ,

codei,lengthi=HuffLookupresnkAgmQnki  .

Due to the size of the Huffman lookup tables, the tables can be found in the reference code.

Table 7.6-20: Maximum values maxAgm per allocation level Agm

| Agm | maxAgm | Agm | maxAgm |
| --- | --- | --- | --- |
| 0 | 0 | 16 | 26 |
| 1 | 3 | 17 | 26 |
| 2 | 3 | 18 | 27 |
| 3 | 4 | 19 | 28 |
| 4 | 5 | 20 | 31 |
| 5 | 5 | 21 | 36 |
| 6 | 6 | 22 | 38 |
| 7 | 7 | 23 | 45 |
| 8 | 8 | 24 | 54 |
| 9 | 9 | 25 | 64 |
| 10 | 12 | 26 | 76 |
| 11 | 13 | 27 | 90 |
| 12 | 16 | 28 | 108 |
| 13 | 17 | 29 | 128 |
| 14 | 19 | 30 | 152 |
| 15 | 23 | 31 | 180 |

CHANGE 57

### 8.4.2	Bit allocation for MASA in SID frames

SID frame signalling for IVAS MASA operation (MASA) is summarized in Table 8.4-2 and the corresponding bitstream description in Table 8.4-3.

Table 8.4-2: MASA frame signalling, SID frames

| IVAS format | configuration | number of bits | Value |
| --- | --- | --- | --- |
| MASA | MASA1 | 2 | 0x3 |
|  | MASA2 | 2 | 0x7 |

Table 8.4-3: Bit allocation for MASA in SID frames

| Description | Ordering of bits | Number of bits |
| --- | --- | --- |
| total bits | Forward ordering of bits | 104 |
| SID format bits |  | 2 |
| No. of transport channels |  | 1 |
| Core-coder – SCE/CPE |  | 48 |
| MASA metadata | Reverse ordering of bits | 53 |

END OF CHANGES