3GPP TSG SA Meeting #105	SP-241125

# September 10 – 13, 2024, Melbourne, AU	(revision of SP-24xxxx)

Source: 	MediaTek Inc.

Title: 	Overview of AI/ML related activities in all Working Groups

Document for: 	Approval

Agenda Item: 	7

Work Item / Release:	FS_AIML_CAL/ Rel-19

Abstract of the contribution: This contribution overviews AI/ML related activities in all Working Groups since Rel-18 and proposes them to be adopted as part of FS_AIML_CAL.

# 1	Introduction

Work related to AI/ML has been ongoing across 3GPP since Rel-18 (Rel-17 when considering enhancements to data collection in RAN3 and management data analytics in SA5).

This contribution proposes to adopt the overview of AI/ML related activities in all Working Groups into 3GPP TR 22.850.

**** FIRST CHANGE ****

# 5	AI/ML related activities in all Working Groups

## 5.1 General

This clause will investigate and identify AI/ML related activities of all working groups of Rel-18 features and Rel-19 studies and work items in TSG CT, TSG RAN and TSG SA Working Groups.

Editor's Note: The AI/ML related content captured in TR 21.918 ("Release 18 Description; Summary of Rel-18 Work Items") can be considered as a starting point.

Work related to AI/ML has been ongoing across 3GPP since Rel-18 (Rel-17 when considering enhancements to data collection in RAN3 and management data analytics in SA5).

1)	[RAN1-led] [Radio interface] Rel-18 TR38.843 + Rel-19: AI/ML for NR air interface

-	Beam Management (DL Tx beam prediction)

-	Positioning accuracy (direct AI/ML, AI/ML-assisted)

-	CSI prediction (study continues in Rel-19 / 09’24 check)

-	CSI compression (study continues in Rel-19 / 09’24 check)

2)	[RAN2-led] [Radio interface] Rel-19 TR38.nnn: Study on AI/ML for mobility in NR

-	RRM measurement and event prediction

-	Cell-level measurement (intra and inter-freq) incl. inter-cell beam-level measurement for L3 mobility, HO failure detection, RLF detection, measurement events prediction)

3)	[RAN3-led] [NG-RAN] Rel-17 TR37.817 + Rel-18 AI/ML for NG-RAN

-	Data collection enh. and signaling support for AI/ML-based Network energy saving, Load balancing, Mobility optimization

4)	[RAN3-led] [NG-RAN] Rel-19 TR38.nnn: Study on enh. for AI/ML for NG-RAN

-	New use cases: slicing, coverage and capacity optimization

5)	[SA2-led] [AI/ML operation itself not in scope of this work] Rel-18 TR 23.700-80 + Rel-18 normative support for misc. system improvements to support AI/ML based application, namely

-	AI/ML operation splitting between AI/ML endpoints

-	AI/ML model/data distribution and sharing

-	Distributed/Federated Learning

through assisting UE selection and providing Network Data Analytics

6)	[SA2-led] Rel-19 TR 23.700-84 CN-enhanced support for AI/ML

-	Misc. improvements to the above

-	[not started] AI/ML cross-domain coordination

-	Whether & How: Data collection, Model transfer/delivery, Alignment w/RAN on model identification and management

7)	[SA3-led] Rel-18 TR33.898 Study on security and privacy of Artificial Intelligence/Machine Learning (AI/ML)-based services and applications in 5G; Rel-18 TR33.877 Study on the security aspects of Artificial Intelligence (AI)/Machine Learning (ML) for the Next Generation Radio Access Network (NG-RAN) + Rel-19 TR 33.784 Study on security aspects of core network enhanced support for Artificial Intelligence Machine Learning (AIML)

8) [SA4-led] Rel-19 26.927 Study on Artificial Intelligence and Machine learning in 5G media services + Rel-19 TR26.847 Evaluation of Artificial Intelligence and Machine learning in 5G media services

9)	[SA5-led] [Network management] Rel-17 TS28.104/.105 Management and orchestration: Management Data Analytics/AI/ML Management; Rel-18 TR28.908 Study on AI/ML Management + Rel-19 TS28.105 AI/ML Management and orchestration

**** END OF CHANGES ****