| 3GPP TR 28.875 V1.0.0 (2024-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on management of IAB nodes (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4 	Introduction	8

5 	Potential use cases and requirements to support IAB node management	8

5.1 	Use case #1 IAB node powers up and connects to OAM system	8

5.1.1	Description	8

5.1.2	Potential requirements	9

5.2 	Use case #2 IAB node connects to different OAM system due to mobility	10

5.2.1	Description	10

5.2.2	Potential requirements	11

5.3	Use case #3 Mobile IAB node needs new configuration in IAB-DU inter-CU migration	11

5.3.1	Description	11

5.3.2	Potential requirements	12

5.4	Use case #4 Mobile IAB node needs new configuration in IAB-MT HO F1 changed	12

5.4.1	Description	12

5.4.2	Potential requirements	13

5.5	Use case #5 Mobile IAB node needs new configuration in IAB-MT HO F1 unchanged	13

5.5.1	Description	13

5.5.2	Potential requirements	14

6	Potential solutions	14

6.1	Potential solutions #1 Configuration based solution to support IAB node connectivity to OAM system	14

6.2	Potential solutions #2 PDU session based solution to support IAB node connectivity to OAM system.	15

6.3	Potential solutions #3 Support of configuration based on location	15

6.4	Potential solutions #4 Mobile IAB node configuration update based on location	16

7 	Conclusion and recommendations	16

Annex <X> (informative): Change history	17

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document studies the use cases and identifies the potential requirements and solutions for the management of IAB nodes for 5G.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: “System architecture for the 5G System (5GS)”.

[3]	3GPP TS 38.401: “NG-RAN; Architecture description”.

[4]	3GPP TS 28.315: “Management and orchestration; Plug and Connect; Procedure flows”.

[5]	3GPP TS 28.541: “Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3”

[6]	3GPP TS 28.532: “Management and orchestration; Generic management services”

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CM	Configuration management

IAB	Integrated access and backhaul

MBSR	Mobile Base Station relay

NE	Network element

PM	Performance management

PnC	Plug and Connect

TAC	Tracking area code

TAI	Tracking area identity

HO	Handover

# 4 	Introduction

The support of IAB was introduced to 3GPP since Rel-16 with basic IAB framework and support of Intra-Donor migration of IAB nodes. The enhancement was added in Rel-17 to enable inter-donor F1 traffic migration, while in Rel-18 IAB node mobility and MBSR (Mobile Base Station Relay) are supported. As specified in TS 23.501 [2] and TS 38.401 [3], the IAB architecture and its mobility behaviour distinguish it from static gNodeB’s, this may require additional support from OAM system.

The present document describes the use cases when IAB node powers up and during its mobility how it can connect to the OAM system, and further identifies the potential requirements and solutions to support IAB node connectivity to OAM system in various scenarios.

Furthermore, the potential impact to CM, PM and FM is studied, and potential requirements and solutions are described to support IAB OAM.

# 5 	Potential use cases and requirements to support IAB node management

## 5.1 	Use case #1 IAB node powers up and connects to OAM system

### 5.1.1	Description

Figure 5.1.1-1 below describes overall architecture of IAB as specified in TS 38.401[3]. The IAB-node connects to an upstream IAB-node or an IAB-donor-DU via a subset of the UE functionalities of the NR Uu interface (named IAB-MT function of IAB-node).

Figure 5.1.1-1 Overall architecture of IAB

When IAB-node powers up, it needs to connect to OAM system. Figure 5.1.1-2 below describes simplified view of OAM connectivity.

Figure 5.1.1-2 IAB node connects to OAM system at power up

Today NE connects to OAM system when powers up via Plug and Connect (PnC) procedures as described in TS 28.315[4]. NE needs to obtain initial IP configuration from IP autoconfiguration service (e.g., DHCP server), satisfy security aspect including CA/RA enrolment and connectivity to SeGW before connecting to SCS. The initial IP configuration for OAM connectivity includes basic IP configuration information (e.g., IP address, netmask, default gateway, domain name, IP address(es) of DNS servers, time servers) or together with information such as IP address or FQDN of CA/RA server, IP address or FQDN of SeGW, IP address or FQDN of SCS, etc depends on operator’s deployment scenarios.

IAB node differs from NE as static node in term of its wireless connection with IAB-donor-node and UE, it can’t reach to DHCP server in the same way as NE like gNB. This use case studies the potential requirements and solutions how the IAB node obtains the IP configuration for OAM connectivity, with that IAB node performs the subsequent PnC procedures as same as specified today in the PnC specifications. This may involve connecting to multiple OAM systems in sequence as described in TS 28.315 [4] clause 5.1.

### 5.1.2	Potential requirements

REQ_IAB_CON_1: 3GPP management system should have the capability to support IP configuration for OAM connectivity based on IAB node location.

REQ_IAB_CON_2: 3GPP management system should have the capability to enable IAB node to obtain the IP configuration for OAM connectivity based on location during the integration procedure.

REQ_IAB_CON_3: IAB node should meet the security requirement of PnC procedures to connect to OAM system at power up.

## 5.2 	Use case #2 IAB node connects to different OAM system due to mobility

### 5.2.1	Description

Since Rel-18 IAB node mobility is supported, the mobility scenarios focus on mobile IAB node mounted on vehicles providing 5G coverage/capacity enhancement to onboard and/or surrounding UEs. The procedures of IAB-MT inter-donor handover and mobile IAB migration are specified in TS 38.401 [3]. In TS 23.501 [2], roaming of the MBSR is supported, MBSR (IAB-DU) can use IAB-node integration procedure or inter-IAB-donor gNB mobility procedure to integrate into the serving PLMN to provide service.

In some mobility scenarios mobile IAB node may need to connect to a different OAM system within same PLMN in non-roaming scenarios or different serving PLMN (VPLMN) in roaming scenarios.

This use case takes mobile IAB-DU migration scenario to study the potential requirement and solution to support mobile IAB node connectivity to a different OAM system due to mobility.

Referring to TS 38.401 [3] section 8.23.3, Figure 5.2.1-1 below describes the mobile IAB-DU migration procedure. To support this procedure, the mobile IAB node concurrently supports two logical mobile IAB-DUs. At the end of the procedure, the F1 link between source logical DU and source IAB-donor-CU is removed.

Figure 5.2.1-1 Mobile IAB-DU inter-CU migration procedure

Figure 5.2.1-2 below describes simplified view of mobile IAB node connect to OAM system during mobility. Before mobile IAB-DU migration, mobile IAB node is connected to source OAM system, mobile IAB node may need to connect to target OAM system at the end of this procedure.

Figure 5.2.1-2 Mobile IAB node connect to OAM system during mobility

Today PnC procedures specified in TS 28.315 [4] enable NE to connect the OAM system while satisfy the security aspect. To allow mobile IAB node to use PnC procedures to connect to the OAM system in mobility scenarios, mobile IAB node needs to obtain IP configuration for OAM connectivity (TS 28.315 [4] clause 5.2) based on its location.

### 5.2.2	Potential requirements

REQ_IAB_MOB_CON_1: 3GPP management system should have the capability to support IP configuration for OAM connectivity based on mobile IAB node location.

REQ_IAB_MOB_CON_2: 3GPP management system should have the capability to enable mobile IAB node to obtain IP configuration for OAM connectivity based on mobile IAB node location.

REQ_IAB_MOB_CON_3: Mobile IAB node should meet the security requirement of PnC procedures to connect to OAM system during mobility.

## 5.3	Use case #3 Mobile IAB node needs new configuration in IAB-DU inter-CU migration

### 5.3.1	Description

Mobile IAB node may need to use new configuration in IAB-DU inter-CU migration. For example, mobile IAB node needs new NR cell configuration based on its location while moving.

This use case takes mobile IAB-DU inter-CU migration scenario to study the CM impact. In this scenario, both OAM connectivity and F1 link are changed. Before IAB-DU migration, IAB-node1 is in geographical area1 and IAB-node2 is in geographical area2 and each is configured with set of NR cells. When IAB-node1 moves to a new geographical area2, it connects to new OAM management system and set up a new F1 link towards target IAB-donor-CU, such procedure refers to TS 38.401 [3] clause 8.23.3. IAB-node1 needs new NR cell configuration when move to geographical area2. See Figure 5.3.1-1 below for the simplified view of this scenario:

Figure 5.3.1-1 Mobile IAB-DU inter-CU migration

### 5.3.2	Potential requirements

REQ_IAB_MOB_CM_1: 3GPP management system should have the capability to support configuration based on mobile IAB node location.

REQ_IAB_MOB_CM_2: 3GPP management system should have the capability to receive notification from IAB node about its location.

REQ_IAB_MOB_CM_3: 3GPP management system should have the capability to update mobile IAB node configuration based on mobile IAB node location.

## 5.4	Use case #4 Mobile IAB node needs new configuration in IAB-MT HO F1 changed

### 5.4.1	Description

Mobile IAB node may need to use new configuration in mobile IAB-MT HO between different IAB-donor-CU. For example, mobile IAB node needs new NR cell configuration based on its location while moving.

This use case takes mobile IAB-MT HO scenario to study the CM impact. In this scenario, mobile IAB-MT HO between different IAB-donor-CU, F1 link is changed. Before IAB-MT HO, IAB-node1 is in geographical area1 and IAB-node2 is in geographical area2 and each is configured with set of NR cells. When IAB-node1 moves to a new geographical area2, IAB-node1 OAM connectivity is unchanged, it sets up a new F1 link towards target IAB-donor-CU, such procedure refers to TS 38.401 [3] clause 8.23.1. IAB-node1 needs new NR cell configuration when move to geographical area2. See Figure 5.4.1-1 below for the simplified view of this scenario:

Figure 5.4.1-1 Mobile IAB-MT HO between different IAB-donor-CU, F1 changed

### 5.4.2	Potential requirements

The requirements are same as use case 5.3 in 5.3.2.

## 5.5	Use case #5 Mobile IAB node needs new configuration in IAB-MT HO F1 unchanged

### 5.5.1	Description

Mobile IAB node may need to use new configuration in mobile IAB-MT HO between different IAB-donor-CU. For example, mobile IAB node needs new NR cell configuration based on its location while moving.

This use case takes mobile IAB-MT HO scenario to study the CM impact. In this scenario, mobile IAB-MT HO between different IAB-donor-CU but IAB-DU remain same F1 link with source IAB-donor-CU. Before IAB-MT HO, IAB-node1 in geographical area1 and IAB-node2 is in geographical area2 and each is configured with set of NR cells. When IAB-node1 moves to a new geographical area2, IAB-node1 OAM connectivity is unchanged, F1 link with source IAB-donor-CU is unchanged, such procedure refers to TS 38.401 [3] clause 8.23.1. IAB-node1 needs new NR cell configuration when move to geographical area2. See Figure 5.5.1-1 below for the simplified view of this scenario:

Figure 5.5.1-1 Mobile IAB-MT HO between different IAB-donor-CU, F1 unchanged

### 5.5.2	Potential requirements

The requirements are same as use case 5.3 in 5.3.2.

# 6	Potential solutions

## 6.1	Potential solutions #1 Configuration based solution to support IAB node connectivity to OAM system

IAB node needs to obtain the IP configuration for OAM connectivity before communication with the OAM. According to TS 28.315 [4], IP configuration for OAM connectivity may include IP address or FQDN of CA/RA server, SeGW and SCS, IP address of DNS servers depends on operator’s deployment.

This solution requires 3GPP management system to support the mapping of IP configuration for OAM connectivity with location. Such mapping is provisioned to IAB node after IAB node connects to OAM system at power up or prior to mobility. Alternatively, such mapping can be manually configured in IAB node.

- The IP configuration for OAM connectivity can be mapped to location information such as TAC, TAI, gNBId of IAB-donor node and PLMN info. The table 6.1-1 illustrates an example of mapping:

Table 6.1-1 example IP configuration mapping to location

| IP configuration for OAM connectivity | Location |
| --- | --- |
| IP configuration 1 | TAC1/TAI1, or gNBId of IAB-donor node and PLMN1 |
| IP configuration 2 | TAC2/TAI2 or gNBId of IAB-donor and PLMN2 |
| … | … |

- IAB node can look up this mapping table to get IP configuration for OAM connectivity with one or more of the following location information depends on its physical location or the mobility scenarios as specified in TS 38.401 [3] clause 8.12 and clause 8.23:

- TAC or TAI pertaining to the cells where IAB-MT is connected.

- The gNBId of the IAB-donor-CU and PLMN info that target IAB-DU connects to.

- The gNBId of the IAB-donor-CU and PLMN info that serves the IAB-MT.

- With IP configuration for OAM connectivity, IAB node can proceed Plug and Connect procedures to connect to OAM system.

## 6.2	Potential solutions #2 PDU session based solution to support IAB node connectivity to OAM system.

According to TS 28.315 [4], the initial IP configuration for OAM connectivity includes basic IP configuration information (e.g., IP address, netmask, default gateway, domain name, IP address(es) of DNS servers, time servers) or together with information such as IP address or FQDN of CA/RA server, IP address or FQDN of SeGW, IP address or FQDN of SCS, etc.

IAB node can get the initial IP configuration via PDU session that setup between IAB-MT and 5GC, using this connectivity IAB node can obtain the initial IP configuration provided by IP autoconfiguration service (e.g., DHCP server), and additionally may perform DNS query depends on the operator’s deployment scenario.

With initial IP configuration obtained as above, IAB node can proceed Plug and Connect procedures to connect to OAM system.

## 6.3	Potential solutions #3 Support of configuration based on location

Mobile IAB node may need new configuration due to mobility. This solution requires 3GPP management system to support the mapping of IAB-DU configuration (e.g., NRcellDU in TS 28.541[5]) with locations and IAB-donor-CU it connects to. Such mapping can be provisioned to IAB node after IAB node connects to management system at power up or prior to mobility. IAB node can look up this table to get its configuration based on its location.

The location information can be one or more of following depends on mobility scenarios, such as:

- TAC or TAI pertaining to the cells where IAB-MT is connected.

- The gNBId of the IAB-donor-CU and PLMN info that target IAB-DU connects to.

- The gNBId of the IAB-donor-CU and PLMN info that serves the IAB-MT.

The table 6.3-1 illustrates an example mapping of IAB-DU configuration:

Table 6.3-1 example of configuration mapping to location

| Configuration | Location |
| --- | --- |
| IAB-DU configuration 1 | TAC1/TAI1, gNBId of IAB-donor node and PLMN1 |
| IAB-DU configuration 2 | TAC2/TAI2, gNBId of IAB-donor node and PLMN2 |
| … | … |

Note: During F1 link setup procedure, the IAB-donor-CU may discover the NCI collision of the cells served by a mobile IAB-DU with cells of other gNB-DU. When such NCI collision occurs, the NCI can be reconfigured by the F1-terminating IAB-donor-CU serving the mobile IAB-DU as specified in TS 38.401 [3] clause 8.9.15, while the NCI that IAB-donor-CU selects for reconfiguration may be obtained from a list of provisioned NCIs or determined by IAB-donor-CU. The mobile IAB-DU can notify management system about the reconfigured cellLocalId(s) pertaining to NCI using notifications specified in TS 28.532 [6].

## 6.4	Potential solutions #4 Mobile IAB node configuration update based on location

Mobile IAB node may need new configuration while it moves, for example, new configuration of NR cells or IP configuration for OAM connectivity. This solution enables IAB node to update the configuration based on its location:

- 3GPP management system supports the mapping of configuration with locations of the IAB node. The location information can be one or more of location information such as TAC, TAI, PLMN info and gNBId of IAB-donor-CU depends on mobility scenarios.

- When IAB node powers up and connects to 3GPP management system for the 1st time, the IAB node can provide location information with IAB node indication via notifications to 3GPP management system as specified in TS 28.532 [6] (e.g., notifyMOIChanges) to indicate its current location. Upon the reception of notifications, 3GPP management system can provision the initial configuration (e.g.via createMOI operation) as specified TS 28.532 [6] based on its current location.

- When IAB node moves to a new location which may need new configuration, the IAB node can provide new location information via “notifyMOIChanges” or “notifyMOIAttributeValueChanges” or other notifications as specified in TS 28.532 [6] to 3GPP management system. Upon the reception of notifications of new location, 3GPP management system can provision the new configuration via “modifyMOIAttributes” or other operations as specified in TS 28.532 [6] to the IAB node based on its new location.

# 7 	Conclusion and recommendations

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-04 | SA5#154 |  |  |  |  | Initial skeleton. | 0.0.0 |
| 2024-04 | SA5#154 | S5-241572 | - | - | - | pCR TR 28.875 Add scope | 0.1.0 |
| 2024-04 | SA5#154 | S5-241574 | - | - | - | pCR TR 28.875 Add structure | 0.1.0 |
| 2024-04 | SA5#154 | S5-242116 | - | - | - | pCR TR 28.875 Add introduction | 0.1.0 |
| 2024-04 | SA5#154 | S5-242117 | - |  | - | pCR TR 28.875 new use case IAB node powers up and connects to OAM system | 0.1.0 |
| 2024-04 | S5#154 | S5-242118 | - | - | - | pCR TR 28.875 new use case IAB node connects to different OAM system due to mobility | 0.1.0 |
| 2024-05 | SA5#155 | S5-243299 | - | - | - | pCR TR 28.875 configuration based solution to support IAB node connectivity to OAM system | 0.2.0 |
| 2024-05 | SA5#155 | S5-243300 | - | - | - | pCR TR 28.875 PDU session based solution to support IAB node connectivity to OAM system | 0.2.0 |
| 2024-08 | SA5#156 | S5-244703 | - | - | - | S5-244703 pCR TR 28.875 new use cases mobile IAB node needs new configuration in mobility scenarios | 0.3.0 |
| 2024-08 | SA5#156 | S5-244704 | - | - | - | S5-244704 pCR TR 28.875 new solution support of configuration based on location | 0.3.0 |
| 2024-08 | SA5#156 | S5-244705 | - | - | - | S5-244705 pCR TR 28.875 new solution mobile IAB node configuration update based on location | 0.3.0 |
| 2024-09 | SA#105 | SP-241140 |  |  |  | Presented at SA#105 for Information | 1.0.0 |