3GPP TSG-SA5 Meeting #156	S5-244098

Maastricht, Netherlands, 19 - 23 August 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.535 | CR | 0075 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR TS 28.535 Update description of control loops |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the open control loop, the control entity can be human operator or other management entity. In the text, there are a few places only have human operator or operator without other management entity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change human operator to human operator or other management entity. Fix error in figure number. Add missing referred specification name. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will lead to misunderstanding and wrong implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3, 4.2.4, 4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

4.2.3	Open control loops

In an open control loop, the human operator or other management entity intervenes in one or more of the process steps inside the loop, see Figure 4.2.3.1. The human operator or other management entity is in control of the steps in the control loop, including decisions taken in the loop. The management system collects, analyses and presents the data to the human operator or other management entity, but the human operator or other management entity decides which action to take. In this case, the completion time for control loop is dependent on availability and reaction time of a human operator or other management entity.

Figure 4.2.3.1: Open control loop entities

2nd Change

4.2.4	Closed control loops

4.2.4.1	Description

In a closed control loop, there is no direct involvement of a human operator or other management entity in the control loop, the control loop is fully automated. As shown in Figure 4.2.4.1.1 the human operator or management entity is not directly controlling the details inside the process steps but provides control outside the loop. For example, configuring goals for the control loop to make autonomous decisions within the boundaries of the set goal. Once the control loop is configured with the goal, the controlled entity is adjusted according to the set goals.

In a closed control loop the input to the control loop provided by human operator or other management entity may include the goal or policies. The output of the closed control loop may include closed control loop status to a human operator or other management entity.

Typically, the goal is set within certain parameter boundaries, the closed control loop can automatically adjust the output based on the input within the parameter boundaries. Once a control loop cannot automatically adjust, the human operator or other management entity needs to be informed. The human operator or other management entity may decide to change the management of closed control loop so that it becomes an open control loop, where decisions are made by the human operator or other management entity and not by the closed control loop.

Figure 4.2.4.1.1: Closed control loop entities

3rd Change

4.3	Communication service assurance service

Communication service assurance relies on a set of management services that together provide the CSP with the capability to assure the communication service as per agreement (for example an SLS) with a CSC (e.g. enterprise). The overall solution and information flows between management services and the closed control loop steps described in Annex C of ETSI GS ZSM 002 [5] are shown in Figure 4.3.1.

Figure 4.3.1: Overview of closed control loop information flows

In Figure 4.3.1 the controlled entity represents the resources used by a communication service and the assurance of this communication service is provided by the closed control loop between the different management services provided by the management system.

The input to the closed control loop is the data concerning the resources used by the communication service and corrresponding service KPIs which is monitored by the closed control loop and step "Monitor", analyzed by the closed control loop step “Analyze”, a decision on potential solution by the closed control loop step "Decide" which may be a possible action for the closed control loop step "Execute", The role of the decision support services is to provide variable degrees of automated decision making and human oversight support. The following two examples demonstrate how a closed control loop can be used:

- when a service experience degradation is detected (for example due to resource shortage or faults in the network), the resources used by a communication service may be adjusted automatically to improve the service experience

- the data associated with the communication service is monitored by the management services for data collection, this management service provides information to an assurance root cause analysis management service (example of an analytics service) and based on that information the assurance root cause analysis takes place, followed by proposing activities, mitigation or suggestions to solve the problem. The proposed activities, for example mitigation or problem-solving suggestion(s) are executed through provisioning services to bring the behaviour of the communication service within the requested boundaries of the metrics (SLS goals) that are controlled by the closed control loop.

The management services available for the closed control loop steps for "Monitor", "Analyze" and "Decide" are based on file transfer described in TS 28.550 [3], or data streaming described in TS 28.550 [3] and notifications described in TS 28.545 [6].

The information provided from the "Monitor" step to the "Analyze" step includes performance measurements (see TS 28.552 [7]), KPIs (see TS 28.554 [8]), performance threshold monitoring events and fault supervision events (see TS 28.532 [9]).

The insights provided from the "Analyze" step to the "Decide" step includes analytics outcomes that are not specified in the present document.

The decision support services provided from the "Decide" step to the "Execute" step are not specified in the present document.

End of Changes