SA Meeting #105		SP-241372

Melbourne, Australia,  10 – 13 September, 2024

Title:	 LS on alignment of eCall over IMS with CEN

Release:	Rel-19

Work Item:

Source:	SA

To:	CEN TC 278 WG 15

Cc:

Contact person:	Xiaobao Chen

xiaobao.chen AT orange.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: 	C1-245026, C1-245025, C1-244731

# 1	Overall description

3GPP TSG SA would like to inform  CEN TC 278 WG 15  of the progress of the work  in CT1 Working Group in relation to  the LS from CEN on Next Generation eCall. 3GPP TSG CT has approved  a work item  and  the necessary changes (see the attachments)  on the relevant  specifications but have the following findings which need inputs from CEN

Finding#1:	CT1 considers that all 9 CEN cases can be aligned, but also considers that CEN case #1 on test call is of utmost urgency.

Finding#2:	On CEN Case #1: test call is a normal IMS call based on CEN requirements. CT1 considered that using dedicated URNs can work. However, as the test call is executed as a normal IMS call, then in a home routed scenario, the home P-CSCF has to be profiled to recognise that the normal call is actually to be treated as a test emergency call else, for example, the MSD transfer would not be possible or allowed.

Finding#3	On CEN case #5, CEN wishes to make the support of in-band modem optional for PSAP. In the current architecture, in-band modem support in PSAP is needed for requesting and providing MSD in the cases:

1)	an IMS emergency call of the automatically or manually initiated eCall type of emergency service, which was established in PS and later on handed over to CS;

2)	PSAP callback, which was established in PS and later on handed over to CS;

3)	PSAP callback, which was accepted by the UE in CS; or

4)	IMS non-emergency call of the test eCall type, which was established in PS and later on handed over to CS.

A way forward is to make the PSAP requirements of support of in-band modem conditional to support requesting and providing MSD in the cases 1), 2), 3), 4) above.i.e full optionality is not possible but optionality except for requesting and providing MSD in cases 1), 2) 3) and 4) above can be a way forward. This  is reflected in the attached CR in C1-245025.

# 2	Actions

To CEN TC 278 WG 15

ACTION:  3GPP TSG SA asks CEN TC 278 WG 15 to take the above information into account and confirm if finding#1, #2 and #3 are agreeable to CEN.

# 3	Dates of next TSG SA and CT meetings

SA#106				10-13 December 2024		Madrid, Spain

CT#106				9-10 December 2024		Mardrid, Spain