3GPP TSG-SA3 Meeting #117															           S3-242952

# Maastricht, Netherlands  19 - 23 August 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.535 | CR | 0217 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AKMA API Name correction along with other editorial corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA_Ph2 |  |  |  |  |  | Date: |  |  | 2024-07-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Unclear specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | API name correction. Editorial corrections in AKMA specs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.4, 6.1, 6.4.2, 6.4.3, 6.7, 7.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************ START OF CHANGES***********

### 4.2.4	AUSF

The AUSF is defined in TS 23.501 [3] with additional functions:

-	AUSF provides the SUPI and AKMA key material (A-KID, KAKMA) of the UE to the AAnF.

-	AUSF performs the AAnF selection.

******************NEXT CHANGES*************

## 6.1	Deriving AKMA key after primary authentication

There is no separate authentication of the UE to support AKMA functionality. Instead, AKMA reuses the 5G primary authentication procedure executed e.g. during the UE Registration to authenticate the UE. A successful 5G primary authentication results in KAUSF being stored at the AUSF and the UE. Figure 6.1-1 shows the procedure to derive KAKMA after a successful primary authentication.

Figure 6.1-1: Deriving KAKMA after primary authentication

1)	During the primary authentication procedure, the AUSF interacts with the UDM in order to fetch authentication information such as subscription credentials (e.g. AKA Authentication vectors) and the authentication method using the Nudm_UEAuthentication_Get Request service operation.

2)	In the response, the UDM may also indicate to the AUSF whether the AKMA Anchor key needs to be generated for the UE. If the AKMA indication is included, the UDM shall also include the RID of the UE.

3)	If the AUSF receives the AKMA indication from the UDM, the AUSF shall store the KAUSF and generate the AKMA Anchor Key (KAKMA) and the A-KID from KAUSF after the primary authentication procedure is successfully completed.

The UE shall generate the AKMA Anchor Key (KAKMA) and the A-KID from the KAUSF before initiating communication with an AKMA Application Function.

4)	After AKMA key material is generated, the AUSF selects the AAnF as defined in clause 6.7, and shall send the generated A-KID and KAKMA to the AAnF together with the SUPI of the UE using the Naanf_AKMA_AnchorKey_Register Request service operation. The AAnF shall store the latest information sent by the AUSF.

NOTE 1:	The AUSF need not store any AKMA key material after delivery to the AAnF.

NOTE 1a: When re-authentication runs, the AUSF generates a new A-KID, and a new KAKMA and sends the new generated A-KID and KAKMA to the AAnF. After receiving the new generated A-KID and KAKMA, the AAnF deletes the old A-KID and KAKMA and stores the new generated A-KID and KAKMA.

5)	The AAnF sends the response to the AUSF using the Naanf_AKMA_AnchorKey_Register Response service operation.

A-KID identifies the KAKMA key of the UE.

A-KID shall be in NAI format as specified in clause 2.2 of IETF RFC 7542 [6], i.e. username@realm. The username part shall include the RID and the A-TID (AKMA Temporary UE Identifier), and the realm part shall include Home Network Identifier.

The A-TID shall be derived from KAUSF as specified in Annex A.3.

The AUSF shall use the RID received from the UDM as described in step 2 to derive A-KID.

NOTE 2:	The chance of A-TID collision is not zero but practically low as the A-TID derivation is based on KDF specified in Annex B of TS 33.220 [4]. The detection of A-TID collision as well as potential handling of collision is not addressed in the present document.

KAKMA shall be derived from KAUSF as specified in Annex A.2. Since KAKMA and A-TID in A-KID are both derived from KAUSF based on primary authentication run, the KAKMA and A-KID can only be refreshed by a new successful primary authentication.

******************NEXT CHANGES*************

### 6.4.2	KAF re-keying

The KAF re-keying depends on the lifetime of the KAF and may be triggered by the AF, which means that when a new KAKMA is derived, the KAF will not be re-keyed automatically.

When the lifetime of KAF expires, the AF may reject UE’s access to the AF or refresh the KAF as described in clause 6.4.3 based on its policy. If the AF chooses to reject UE’s access, the AF may provide a cause indicating that the KAF has expired via Ua* protocol specific means so that the UE can take appropriate action. If there has been a change of KAUSF (e.g., due to a successful run of primary authentication), the UE may re-try accessing the AF by using the A-KID derived from the new KAUSF.

******************NEXT CHANGES*************

### 6.4.3	KAF refresh

There is no support for an explicit KAF refresh procedure in this document. If a primary authentication does not take place, the KAUSF, KAKMA and KAF remain unchanged since the latest primary authentication.

The KAF may be refreshed by the KAKMA refresh defined in clause 6.4.4 as decided by AAnF.

NOTE 1: The AAnF can decide KAKMA refresh based on local policy.

Ua* protocol may support refresh of derived session keys from KAF. If the Ua* protocol supports the refresh of derived session keys from KAF, the AF may refresh the KAF at any time using the Ua* protocol.

NOTE 2:	How a fresh key is derived for AKMA is up to Ua* protocol implementation.

NOTE 3:	A session key based on KAF refreshed using the Ua* protocol is only known by UE and AF.

******************NEXT CHANGES*************

## 6.7	AAnF Discovery and Selection

The NF consumer or the SCP performs AAnF discovery to discover an AAnF instance.

In the case of NF consumer-based discovery and selection, the following applies:

-	Internal AFs and the NEF performs AAnF instance selection that handles the AKMA request. The AF/NEF shall utilize the NRF to discover the AAnF instance(s) unless AAnF information is available by other means, e.g. locally configured on the AF/NEF.

-	The AUSF performs AAnF selection to allocate an AAnF Instance to send the AKMA key material related to the UE. The AUSF shall utilize the NRF to discover the AAnF instance(s) unless AAnF information is available by other means, e.g. locally configured on the AUSF.

-	The NF specified in clause 6.6 performs AAnF instance selection that handles the AKMA request. The NF shall utilize the NRF to discover the AAnF instance(s) unless AAnF information is available by other means, e.g. locally configured on the NF specified in clause 6.6.

The AAnF selection functionality in NF consumer or in SCP should consider the following factor:

-	the UE's Routing Indicator.

NOTE 1:	The AF/NEF obtains the Routing Indicator as part of the A-KID in the AKMA request. The AUSF obtains the Routing Indicator within the Nudm_UEAuthentication_Get Response from the UDM.

Internal AFs, the NEF and the AUSF shall select the same AAnF set based on the UE’s Routing Indicator.

When the UE's Routing Indicator is set to its default value as defined in TS 23.003 [9], the AAnF NF consumer can select any AAnF instance within the home network of the UE.

NOTE 2:	In scenarios where multiple sets of AAnFs are deployed, it is left up to implementation how to ensure that the AAnF NF consumers select an AAnF instance within the AAnF set the UE belongs to when the UE's Routing Indicator is set to its default value.

In the case of delegated discovery and selection in SCP, the AAnF NF consumer shall send all available factors to the SCP.

******************NEXT CHANGES*************

### 7.1.5	Naanf_AKMA_ApplicationKey_AnonUser_Getservice operation

Service operation name: Naanf_AKMA_ApplicationKey_AnonUser_Get.

Description: The NF consumer requests only the AKMA Application Key from the AAnF. This service is for allowing anonymous user access to the AF based on A-KID (i.e., UE identification is not required at the AF). The A-KID functions as a temporary user identifier.

Input, Required: A-KID, AF_ID

Input, Optional: Service Disable URI.

Output, Required: KAF, KAF expiration time.

Output, Optional: None.

************ END OF CHANGES**********