3GPP TSG-SA3 Meeting #117	S3-243159

Maastricht, Netherlands  19 - 23 August 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.535 | CR | 0220 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction for AKMA procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In some procedures and its description, wrong step number and wrong clause exist. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction wrong step number, wrong clause number, and typo. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It could mislead specification readers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 and 6.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** 1st CHANGE start ***

## 6.2	Deriving AKMA Application Key for a specific AF

### 6.2.1	AAnF response with UE Identity

Figure 6.2-1 shows the procedure used by the AF to request application function specific AKMA keys from the AAnF, when the AF is located inside the operator's network.

Figure 6.2-1: KAF generation from KAKMA

Before communication between the UE and the AKMA AF can start, the UE and the AKMA AF need to know whether to use AKMA. This knowledge is implicit to the specific application on the UE and the AKMA AF or indicated by the AKMA AF to the UE (see clause 6.5).

1.	The UE shall generate the AKMA Anchor Key (KAKMA) and the A-KID from the KAUSF before initiating communication with an AKMA Application Function. When the UE initiates communication with the AKMA AF, it shall include the derived A-KID (see clause 6.1) in the Application Session Establishment Request message. The UE may derive KAF before sending the message or afterwards.

2.	If the AF does not have an active context associated with the A-KID, then the AF selects the AAnF as defined in clause 6.7, and sends a Naanf_AKMA_ApplicationKey_Get request to AAnF with the A-KID to request the KAF for the UE. The AF also includes its identity (AF_ID) in the request. If AF wants to receive a notification for AKMA service disabling, the AF shall include AKMA service disable URI in the Naanf_AKMA_ApplicationKey_Get request. Based on the AKMA service disable URI, the AAnF shall create an implicit subscription for the AF for the AAnF to later notify the AF about AKMA service disable as defined in clause 6.8. Implicit subscription has an expiration time set by operator policy.

AF_ID consists of the FQDN of the AF and the Ua* security protocol identifier (see Annex A.4). The latter parameter identifies the security protocol that the AF will use with the UE.

The AAnF shall check whether the AAnF can provide the service to the AF based on the configured local policy or based on the authorization information available in the signalling (i.e., Oauth2.0 token). If it succeeds, the following procedures are executed. Otherwise, the AAnF shall reject the procedure.

The AAnF shall verify whether the subscriber is authorized to use AKMA based on the presence of the UE specific KAKMA key identified by the A-KID.

If KAKMA is present in AAnF, the AAnF shall continue with step 3.

If KAKMA is not present in the AAnF, the AAnF shall continue with step 6 with an error response.

3.	Once receiving the request from the AF, if the AAnF determines this specific AF needs GPSI, according to its local policy, the AAnF sends a Nudm_SDM_Get Request to the UDM to fetch the GPSI of the UE. If the specific AF does not need GPSI, the AAnF shall continue with step 5.

4.	The UDM responds with the GPSI of the UE. The AAnF shall store the received GPSI as part of UE’s AKMA context.

5.	Once receiving the request from the AF, the AAnF shall send a Nudm_EventExposure_Subscribe request to UDM with SUPI/GPSI to request the RoamingStatusReport from the UDM.

6.	The UDM shall send the Nudm_EventExposure_Subscribe response to the AAnF with the information of roaming status.

NOTE: Later on, when the roaming status changes, the UDM also sends a notification to the AAnF about the updated roaming information.

7.	Once the AAnF receives the roaming status from the UDM, it checks the local policy and determines whether to provide service to the UE. If yes, the AAnF derives the AKMA Application Key (KAF) from KAKMA if it does not already have KAF. The AAnF shall store the KAF expiration time as part of UE’s AKMA context.

When UE is dual registered, the UE is treated as roaming if at least one of the serving PLMNs indicates the UE is roaming.

The key derivation of KAF shall be performed as specified in Annex A.4.

8.	If the AAnF determines to provide AKMA service to the UE, the AAnF sends Naanf_AKMA_ApplicationKey_Get response to the AF with SUPI/GPSI, KAF and the KAF expiration time. Whether to send SUPI or GPSI is determined by AAnF based on the local policy. If the AAnF finds that roaming is not allowed, it shall respond the AF containing a failure indication that roaming is not allowed.

9.	The AF sends the Application Session Establishment Response to the UE. If the information in step 8 indicates failure of AKMA key request, the AF shall reject the Application Session Establishment by including a failure cause. Afterwards, UE may trigger a new Application Session Establishment request with the latest A-KID to the AKMA AF.

*** 1st CHANGE end ***

*** 2nd CHANGE start ***

## 6.8	Notification about AKMA service disabling

This procedure is used when the AKMA sessions have already been started (before roaming was detected), and as soon as PLMN change is detected at the AAnF, the AAnF may execute this procedure based on the roaming policy.

Figure 6.8.1-1: AAnF notification to AF about AKMA service disable

1.	 UE registers with a (H)PLMN.

2.	 UE is accessing the AF and key material is provided to AF as described in 6.2.1. While accessing the AAnF, AF may also provide the Notification URI.

3. UE is getting registered in a VPLMN and AAnF detects the PLMN change via the Nudm_EventExposure_Notification received from UDM.

4. AAnF determines if AF(s) have subscribed to receive notifications for AKMA service disabling and roaming policy is configured and restrict the AKMA access in the VPLMN; if yes, steps 5 and 6 are executed. Otherwise, steps 5 and 6 are skipped.

5. If AF(s) are determined at step 5, the AAnF shall send notifications to the subscribed AF(s) about AKMA roaming via Naanf_AKMA_ServiceDisableNotification with A-KID.

6. The AF shall send the response and based on the notification and internal policy, the AF may stop the UE service, may stop the encryption.

NOTE: By stopping the encryption (e.g., TLS 1.2 NULL cypher negotiation), LI interception could work in the VPLMN.

*** 2nd CHANGE end ***