3GPP TSG-SA WG1 Meeting #107	S1-242411

Maastricht, The Netherlands, 19-23 August 2024						(revision of S1-242282)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.989 | CR | 0033 | rev | 1 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New use cases: Merging of two multi-train voice communications by Train Controller (Ground FRMCS user) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_FRMCS_Ph6 |  |  |  |  |  | Date: |  |  | 2024-08-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of merging use cases for multi-train voice communications, as per UIC functional FRMCS specifications (FU-7120). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Description of two new use cases for merging: a) Multi-train voice communications initiated by drivers and merged by controller b) One multi-train voice communication initiated by driver and another by controller, both merged by controller. Initial and merged communications are using MCX Ad hoc Group Communications feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing requirements for downstream groups to specify ‘merging’ feature with MCX Ad hoc Group Communications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.5a (new), 6.2.5a.1 (new), 6.2.5a.2 (new), 6.2.5a.3 (new), 6.2.5a.4 (new), 6.2.5a.5 (new), 6.2.5b.1 (new), 6.2.5b.2 (new), 6.2.5b.3 (new), 6.2.5b.4 (new), 6.2.5b.5 (new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

## 6.2	Multi-train voice communication for Drivers and Ground FRMCS User(s)

### 6.2.1	Introduction

In this chapter the use cases related to the function of Multi-train voice communication from the Drivers and/or a Ground FRMCS User towards the Driver and/or Ground FRMCS Users are defined. Ground FRMCS Users may include Controllers. The following use cases are defined:

- Initiation of Multi-train voice communication for Drivers and Ground FRMCS User(s) communication

- Join a Multi-train voice communication for Drivers and Ground FRMCS User(s) communication

- Termination of Multi-train voice communication for Drivers and Ground FRMCS User(s) communication

- Interworking GSM-R and FRMCS for Multi-train voice communication for Drivers and Ground FRMCS User(s) communication

- Merging of two multi-train voice Ad hoc Group communications. There are different use cases when merging two multi-train voice communications depending on the ‘merging authority’ and the initiators of the two separate calls, as presented in the following:

- a) Two multi-train voice communications initiated by train drivers and merged by train controller

- b) A multi-train voice communication initiated by train driver and another initiated by train controller, merged by the train controller

- c) Two multi-train voice communications initiated by train drivers and merged by the train driver

- d) A multi-train voice communication initiated by train driver and another initiated by train controller, merged by the train driver.

Editor’s note:	Merging use cases c) and d) are FFS.

Next change

### 6.2.5a	Use case: Merging of two multi-train voice communications initiated by Train Drivers and merged by Train Controller (Ground FRMCS user)

#### 6.2.5a.1	Description

The use case is derived from train Controller’s operational activities. A Controller, as responsible for the movement of the trains in a certain area, may be involved in more than one multi-train communications, impacting neighbouring areas. In that context, he/she needs to be able to merge the two multi-train voice communications because there is interesting information to be shared by the two groups participating in the two multi-train communications. Moreover, for the controller it is finally more convenient to handle only one communication.

#### 6.2.5a.2	Pre-conditions

All involved FRMCS users (Controller A, Driver 1, Driver 2, Driver 3, Driver 4, Driver 5, Driver 6) have been successfully registered to the FRMCS Service Server and selected their functional roles.

Driver 1 is authorized to initiate a multi-train voice communication.

Driver 4 is authorized to initiate a multi-train voice communication.

Controller A is authorized to initiate a multi-train voice communication.

Controller A is authorized to merge two on going multi-train communications.

Controller A is authorized to terminate a multi-train voice communication.

There are two on-going multi-train voice communications. The participants of each one are selected by the FRMCS Service Server, based on the initiator’s location.

The participants of the two multi-train voice communications belong to the same FRMCS Service Server (same FRMCS domain).

Controller A benefits of the floor control priority (multi-user talker control of the FRMCS system) when involved in a multi-train communication.

Location information of all participants of multi-train communications can be provided by the FRMCS system.

The FRMCS system is preconfigured to define which controller is responsible for each addressed area.

#### 6.2.5a.3	Service flows

- 1st multi-train voice communication initiated by Driver 1 (GC1: Group Communication1)

1) Driver 1 initiates a multi-train voice communication, with specific criterion of initiator’s location.

2) The FRMCS Service Server verifies if driver 1 is authorized to initiate a multi-train voice communication. If not authorized, the FRMCS Service Server rejects the multi-train voice communication initiated by driver 1.

3) Based on driver’s 1 location, the FRMCS Service Server determines the addressed area (GC1_AdA) using the initiating area.

4) Driver 2 and driver 3 are in the addressed area. Controller A is responsible for this addressed area.

5) The FRMCS Service server invites the participants fulfilling the criterion of GC1 (GC1_AdA), driver 2, driver 3 and controller A. Finally, the multi-train voice communication is established with set-up time specified as NORMAL (see 12.10).

6) All participants’ respective floor priority is considered when multi-user talker control is used during the multi-train voice communication of this group.

- 2nd multi-train voice communication initiated by Driver 4 (GC2: Group Communication2)

1) Driver 4 is also initiating a multi-train voice communication with the specific criterion of initiator’s location.

2) Driver 4 is moving to the responsibility area of controller A

3) The FRMCS Service Server verifies if driver 4 is authorized to initiate a multi-train voice communication. If not authorized, the FRMCS Service Server rejects the multi-train voice communication initiated by driver 4.

4) Based on driver’s 4 location, the FRMCS Service Server determines the addressed area (GC2_AdA) using the initiating area.

5) Driver 5 and driver 6 are in the addressed area and controller A is responsible for this addressed area, as well.

6) The FRMCS Service server invites the participants fulfilling the criterion of GC2 (GC2_AdA), driver 5, driver 6 and controller A. Finally, the multi-train voice communication is established with set-up time specified as NORMAL (see 12.10).

7) All participants’ respective floor priority is considered when multi-user talker control is used during the multi-train voice communication of this group.

- Controller A requests the FRMCS system for merging on-going GC1 and GC2

1) Controller A in now participating in two multi-train communications, GC1 and GC2. From the exchanged information, controller A realises that there is interesting information for both groups. Moreover, the addressed areas of both communications are located close to each other. Therefore, controller A decides to merge GC1 and GC2.

2) Controller A initiates a new multi-train (merged) voice communication, named GC3, based on a new criterion, (GC3_AdA), which is the union of addressed areas of GC1 (GC1_AdA) and GC2 (GC2_AdA), with the updated participants of both addressed areas, at the merging time. The participants of the merged multi-train communications are notified.

3) The FRMCS Service Server verifies if controller A is authorized to merge a multi-train voice communication. If not authorized, the FRMCS Service Server rejects the request for merging of multi-train voice communication initiated by controller A.

4) The FRMCS Service server invites the participants fulfilling the criterion of (GC3_AdA), driver 1, driver 2, driver 3, driver 4, driver 5, driver 6 and controller A. Finally, the multi-train voice communication is established with set-up time specified as NORMAL (see 12.10).

5) The FRMCS Service server continues to check the participants that satisfy the new criterion, corresponding to the addressed area of (GC3_AdA) and invites or removes FRMCS users, as usually in a multi-train communication based on criterion.

6) Participants of GC1 or GC2 continue to use their respective groups when they need to exchange information without involving the merged group (GC3).

#### 6.2.5a.4 	Post-conditions

A merged multi-train voice communication is established, including all participants of both multi-train voice communications GC1 and GC2, at the time the ‘merging’ was performed. Once the merged communication is established, the relevant participants keep their previous floor priorities and authorisations.

#### 6.2.5a.5 	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.2.5a-001] | The FRMCS System shall enable an authorised FRMCS User to merge two multi-train voice communications, based on the same criterion (e.g., location of the initiator). | A | 22.280 | Not covered. CR is needed for an authorised user to merge Ad hoc group communications, using a new criterion, based on the union of Ad hoc Group IDs of both merged communications. |
| [R-6.2.5a-002] | The FRMCS system shall record, as metadata, the timestamp of merging of the two multi-train communications | A | 22.280 | Partially covered in [R-6.15.4-004]. CR is needed to add the relevant metadata for merged communications. |
| [R-6.2.5a-003] | The FRMCS shall notify the participants of the two initial multi-train communications for the new merged voice communication. | A | 22.280 | Not covered. CR is needed. Merging notification is missing for Ad hoc Group Calls |
| [R-6.2.5a-004] | The FRMCS system shall enable any participant of a merged multi-train voice communication to keep the authorisations of the initial multi-train voice communications | A | 22.280 | Not covered. CR is needed. Authorisations for merged communications based on Ad hoc Group calls are kept as prior to merging. |
| [R-6.2.5a-005] | The FRMCS system shall enable a participant who is authorised to terminate at least one of the initial multi-train communications, to also terminate the merged multi-train voice communication. | A | 22.280 | Not covered. CR is needed. Termination of merged multi-train communications for Ad hoc Group calls. |
| [R-6.2.5a-006] | The FRMCS system shall enable any participant of a merged multi-train voice communication, to keep the floor control policy for multi-user talker control of the initial multi-train voice communications | A | 22.179 | Not covered. CR is needed. Transfer of floor control policies to the merged multi-train communication for Ad hoc Group Calls. |
| [R-6.2.5a-007] | The FRMCS system shall set the number of simultaneous talkers of the merged multi-train communication to the sum of the number of simultaneous talkers of the original communications, with multi-user talker control. | A | 22.179 | Not covered. CR is needed. Addition of simultaneous talkers for Multi-user talker control for merged multi-train communication with Ad hoc Group Calls. |
| [R-6.2.5a-008] | The FRMCS system shall enable the establishment of a merged multi-train voice communication with a set-up time, specified as NORMAL | A/T | N/A | See §12.10 |
| [R-6.2.5a-009] | The FRMCS system shall enable the participants of a merged multi-train voice communication to address only the participants of their initial group, prior to merging, whenever needed. | A | 22.280 | Not covered. CR is needed for addressing each separate group, without involving all the participants of the merged group. |

### 6.2.5b	Use case: Merging of one multi-train voice communications initiated by train driver and another initiated by the train controller, merged by the train Controller (Ground FRMCS user)

#### 6.2.5b.1	Description

The use case is similar to the one described in chapter §6.2.5a with the main difference that one of the two initial multi-train communications is initiated by the train controller based on different criterion (e.g., addressed area).

#### 6.2.5b.2	Pre-conditions

All involved FRMCS users (Controller A, Driver 1, Driver 2, Driver 3, Driver 4, Driver 5, Driver 6) have been successfully registered to the FRMCS Service server and selected their functional roles.

Controller A is authorized to initiate a multi-train voice communication.

Controller A is authorized to terminate a multi-train voice communication.

Driver 1 is authorized to initiate a multi-train voice communication.

Controller A is authorized to merge two on going multi-train voice communications.

There are two on-going multi-train voice communication. The participants of each one are selected by the FRMCS Service server, based on different criteria (e.g., initiator’s location for the driver-initiated call and addressed area for the controller-initiated call).

The participants of the two multi-train voice communications belong to the same FRMCS Service Server (same FRMCS domain).

Controller A benefits of the floor control priority (multi-user talker control of the FRMCS system) when involved in a multi-train communication.

Location information of all participants of multi-train communications can be provided by the FRMCS system.

The FRMCS system is preconfigured to define which controller is responsible for each addressed area.

#### 6.2.5b.3	Service flows

- 1st multi-train voice communication initiated by Driver 1 (GC1): The service flow is the same as described in 6.2.5a.3 for the driver-initiated call.

- 2nd multi-train voice communication initiated by Controller A (GC2)

1) Controller A is also initiating a multi-train voice communication with the specific criterion of selected addressed area (GC2_SAdA).

2) The FRMCS Service Server verifies if controller A is authorized to initiate a multi-train voice communication. If not authorized, the FRMCS Service Server rejects the multi-train voice communication initiated by controller A.

3) Driver 4, driver 5 and driver 6 are moving inside the selected addressed area

4) The FRMCS Service server invites the participants fulfilling the criterion of (GC2_SAdA), driver 4, driver 5, driver 6 and controller A. Finally, the multi-train voice communication is established with set-up time specified as NORMAL (see 12.10).

6) All participants’ respective floor priority is considered when multi-user talker control is used during the multi-train voice communication of this group.

- Controller A requests the FRMCS system for merging on-going GC1 and GC2

1) Controller A in now participating in two multi-train communications, GC1 and GC2. Controller A considers that it is useful to merge GC1 and GC2, also impacting neighbouring addressed areas.

2) Controller A initiates a new multi-train (merged) voice communication, named GC3, based on a new criterion, (GC3_AdA), which is the union of addressed areas of GC1 (GC1_AdA) and GC2 (GC2_SAdA), with the updated participants at the merging time. The participants of the two merged communications are informed about the merging.

NOTE:	The service flow for the controller requesting the merging of the two communications is the same as described in chapter §6.2.5a.3, but the initial communications were based on two different criteria, initiator’s location for the driver-initiated call and selected addressed area for the controller-initiated call.

#### 6.2.5b.4 	Post-conditions

A merged multi-train voice communication is established, including all participants of both multi-train voice communications GC1 and GC2, at the time the ‘merging’ was performed. Once the merged communication is established, all the relevant participants keep their previous floor priorities and authorisations.

#### 6.2.5b.5 	Potential requirements and gap analysis

Potential requirements derived from the gap analysis of §6.2.5a.5 are also applicable for this use case. An additional one is:

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.2.5b-001] | The FRMCS System shall enable an authorised FRMCS User to merge two multi-train voice communications, based on different criteria (e.g., initiator’s location, selected addressed area). | A | 22.280 | Not covered. CR is needed for an authorised user to merge Ad hoc group communications, using different criteria (e.g., initiator’s location for one communication and selected addressed area for the other communication) |

End of changes