3GPP TSG-SA WG6 Meeting #62	S6-233879

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-233567)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0261 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the callback URL usage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The callback URL is created by the NMS for receiving the notification from the VAL server that is passed to SEAL NMC via notification channel. However, the notification message from VAL server to NMS doesn’t include any information about the target entity to whom the notification message is for. This needs to be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to add more clarification regarding the callback URL to say that the URL generated by the NMS is unique to NMC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Target notification channel information is unclear for implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.3.3, 17.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S6-233567, S6-233879 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 17.3.3	Procedure for creating notification channel to receive notifications

Figure 17.3.3-1 below illustrates the procedure for receiving notifications from the notification management server by the notification management client.

Pre-conditions:

1.	The notification management client does not have any notification channel open with the notification management server.

Figure 17.3.3-1: Receiving notifications from notification management server

1.	The VAL client requests the notification management client to subscribe to and receive notifications from the VAL server.

2.	The notification management client creates notification channels (i.e. endpoint URLs), if not created already, to be used by the VAL server to send notification messages. To create notification channel, the notification management client sends a create notification channel request to the notification management server. The desired validity duration for the channels to be used and the notification channel type (PUSH or PULL) are included in the request. The notification management client decides which type of the notification channel to be used based on the UE capability.

3.	The notification management server authenticates the notification management client and authorizes its request.

4.	The notification management server sends the notification management client the Create notification channel response with the endpoint URLs that will be used by the VAL server to send the notification messages and the notification management client to receive the notification messages. The notification management server also includes what is the valid duration for these endpoint URLs to be used and the channel ID to uniquely identify the channel created in the response.

5.	If the notification type is PULL method, the notification management client sends the Open notification channel to the notification management server to start receiving the notification message. For certain PUSH method notification type (such as WebSockets) the notification management client requests the notification management server to start the PUSH notification service with its specific protocol that is outside the scope of this specification.

NOTE 1:	This step is not required if the notification client wants to receive the notification messages via PUSH notification server.

6.	The notification management client responds to the VAL client the status of its request as in step 1 to receive notifications from the VAL server. It also includes the callback URL which needs to be used by the VAL client while subscribing to the VAL server for receiving notifications. The callback URL is unique to SEAL NMC.

7.	The VAL client subscribes to the VAL server for receiving notifications. This request includes the subscription data and the callback URL returned from the notification management server in step 4.

NOTE 2:	This step is outside the scope of this specification.

8.	The VAL server generates the notification message for the subscription request from the VAL client.

9.	The VAL server sends the notification message to the notification management server containing the VAL UE or VAL user ID, VAL service ID, VAL application ID and the notification data utilizing the callback URL provided in step 7.

10.	If the delivery method is PULL, the notification management server sends the notification message to the notification management client over the opened notification channel.

NOTE 3:	If the delivery method is PUSH via PUSH notification server, the notification management server sends the notification message to the OEM Push server (not shown in the figure and is outside the scope of this specification) to deliver to the notification management client.

11.	The notification management client delivers the received notification message to the appropriate VAL client based on the details as received in the notification message from the notification management server.

Editor's note: How to support different types of PULL and PUSH mechanisms for delivery of notification data is FFS.

* * * Next Change * * * *

#### 17.3.2.2	Create notification channel response

Table 17.3.2.2-1 describes the information flow create notification channel response from the notification management server to the notification management client.

Table 17.3.2.2-1: Create notification channel response

| Information element | Status | Description |
| --- | --- | --- |
| Notification URL | O | The URL to receive the notification message if a PULL method is requested. For some PUSH method implementation (such as WebSockets) this URL is used to start the PUSH notification service from the notification management server |
| Callback URL | M | The URL to be shared to the VAL server by the VAL client for sending the notifications to the notification management server. The callback URL is unique to the notification management client. |
| Validity duration | O | How long the notification channel will last (i.e. channel lifetime) as granted by the notification management server. |
| Channel ID | M | Unique identifier for the created notification channel |

* * * End of Change * * * *