3GPP TSG-SA2 Meeting #164	S2-2408998

# Maastricht, Netherlands, August 19 – 23, 2024							   (revision of S2-2407861)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0199 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on V2X layer operation regarding sidelink carrier failure report from AS layer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, MediaTek Inc., ZTE, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2 |  |  |  |  |  | Date: |  |  | 2024-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | R2-2405900 that is TS 38.321 CR approved at RAN2#126 includes the changes regarding sidelink carrier failure report from AS layer to V2X layer in clause 5.22.1.3.3 "HARQ-based Sidelink RLF detection": For each carrier associated with a PC5-RRC connection, the Sidelink HARQ Entity shall for each PSFCH reception occasion associated to the PSSCH transmission: 1> if PSFCH reception is absent on the PSFCH reception occasion: 2> increment numConsecutiveDTX by 1; NOTE 1: For SL operation with shared spectrum channel access, UE increases the numConsecutiveDTX by 1 when the UE fails to detect the HARQ feedback on all the associated PSFCH resources according to clause 16.3.0 of TS 38.213 [6]. 2> if more than one carrier selected as defined in clause 5.22.1.11 is considered as the carriers for HARQ-based Sidelink RLF detection: 3> if numConsecutiveDTX reaches sl-maxNumConsecutiveDTX for all carriers applied for HARQ-based Sidelink RLF detection: 4> indicate HARQ-based Sidelink RLF detection to upper layers (i.e., RRC layer and V2X layer). 3> else if numConsecutiveDTX reaches sl-maxNumConsecutiveDTX for a carrier applied for HARQ-based Sidelink RLF detection: 4> trigger the TX carrier (re-)selection procedure as specified in clause 5.22.1.11; 4> indicate HARQ-based Sidelink carrier failure to upper layers (i.e., RRC layer and V2X layer). 3> if numConsecutiveDTX reaches sl-maxNumConsecutiveDTX for all carriers applied for HARQ-based Sidelink RLF detection: 4> indicate HARQ-based Sidelink RLF detection to upper layers. 2> else: 3> if numConsecutiveDTX reaches sl-maxNumConsecutiveDTX: 4> indicate HARQ-based Sidelink RLF detection to upper layers. 1> else: 2> re-initialize numConsecutiveDTX to zero. That is, according to this part, the AS layer indicates sidelink carrier failure (i.e. failure report per sidelink carrier) to the V2X layer in addition to PC5-RRC connection release due to RLF specified in Rel-16. (※ Description about V2X layer operation regarding indication on PC5-RRC connection release due to RLF from AS layer exists in §5.2.1.4) The sidelink carrier failure report can enable the V2X layer to aware the case, e.g., all sidelink carriers for PC5 QoS Flow#1 are not available due to failure while other sidelink carriers are available for other PC5 QoS Flow(s) which means PC5 QoS Flow#1 cannot be transmitted although available sidelink carriers exist for the PC5 unicast link. In this case, it is considered appropriate for the V2X layer to remove the PC5 QoS Flow from the PC5 unicast link when all carriers for the PC5 QoS Flow were failed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §5.2.1.4 - Add description about V2X layer operation regarding sidelink carrier failure report from AS layer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | V2X layer operation is not clear regarding sidelink carrier failure report from AS layer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.2.1.4	Unicast mode communication over PC5 reference point

Unicast mode of communication is only supported over NR based PC5 reference point. Figure 5.2.1.4-1 illustrates an example of PC5 unicast links.

Figure 5.2.1.4-1: Example of PC5 Unicast Links

The following principles apply when the V2X communication is carried over PC5 unicast link:

-	A PC5 unicast link between two UEs allows V2X communication between one or more pairs of peer V2X services in these UEs. All V2X services in the UE using the same PC5 unicast link use the same Application Layer ID.

NOTE 1:	An Application Layer ID can change in time as described in clauses 5.6.1.1 and 6.3.3.2, due to privacy. This does not cause a re-establishment of a PC5 unicast link. The UE triggers a Link Identifier Update procedure as specified in clause 6.3.3.2.

-	One PC5 unicast link supports one or more V2X service types) if these V2X service types are at least associated with the pair of peer Application Layer IDs for this PC5 unicast link. For example, as illustrated in Figure 5.2.1.4-1, UE A and UE B have two PC5 unicast links, one between peer Application Layer ID 1/UE A and Application Layer ID 2/UE B and one between peer Application Layer ID 3/UE A and Application Layer ID 4/UE B.

NOTE 2:	A source UE is not required to know whether different target Application Layer IDs over different PC5 unicast links belong to the same target UE.

-	A PC5 unicast link supports V2X communication using a single network layer protocol e.g. IP or non-IP.

-	A PC5 unicast link supports per-flow QoS model as specified in clause 5.4.1.

-	If multiple V2X service types use a PC5 unicast link, one PC5 QoS Flow identified by PFI may be associated with more than one V2X service types.

When the Application layer in the UE initiates data transfer for a V2X service type which requires unicast mode of communication over PC5 reference point:

-	the UE shall reuse an existing PC5 unicast link if the pair of peer Application Layer IDs and the network layer protocol of this PC5 unicast link are identical to those required by the application layer in the UE for this V2X service, and modify the existing PC5 unicast link to add this V2X service type as specified in clause 6.3.3.4; otherwise

-	the UE shall trigger the establishment of a new PC5 unicast link as specified in clause 6.3.3.1.

After successful PC5 unicast link establishment, UE A and UE B use the same pair of Layer-2 IDs for subsequent PC5-S signalling message exchange and V2X service data transmission as specified in clause 5.6.1.4. The V2X layer of the transmitting UE indicates to the AS layer whether a transmission is for a PC5-S signalling message (i.e. Direct Communication Request/Accept, Link Identifier Update Request/Response/Ack, Disconnect Request/Response, Link Modification Request/Accept, Keep-alive/Ack) or V2X service data.

For every PC5 unicast link, a UE self-assigns a distinct PC5 Link Identifier that uniquely identifies the PC5 unicast link in the UE for the lifetime of the PC5 unicast link. Each PC5 unicast link is associated with a Unicast Link Profile which includes:

-	Application Layer ID and Layer-2 ID of UE A; and

-	Application Layer ID and Layer-2 ID of UE B; and

-	network layer protocol used on the PC5 unicast link; and

-	the information about PC5 QoS Flow(s). For each PC5 QoS Flow, the PC5 QoS Context and the PC5 QoS Rule(s) as defined in clause 5.4.1.1.3.

For privacy reason, the Application Layer IDs and Layer-2 IDs may change as described in clauses 5.6.1.1 and 6.3.3.2 during the lifetime of the PC5 unicast link and, if so, shall be updated in the Unicast Link Profile accordingly. The UE uses PC5 Link Identifier to indicate the PC5 unicast link to V2X Application layer, therefore V2X Application layer identifies the corresponding PC5 unicast link even if there are more than one unicast link associated with one V2X service type (e.g. the UE establishes multiple unicast links with multiple UEs for a same V2X service type).

The Unicast Link Profile shall be updated accordingly after a Layer-2 link modification for an established PC5 unicast link as specified in clause 6.3.3.4 or Layer-2 link identifier update as specified in clause 6.3.3.2.

Upon receiving an indication from the AS layer that the PC5-RRC connection was released due to RLF, the V2X layer in the UE locally releases the PC5 unicast link associated with this PC5-RRC connection. The AS layer uses PC5 Link Identifier to indicate to the V2X layer the PC5 unicast link whose PC5-RRC connection was released.

When the PC5 unicast link has been released as specified in clause 6.3.3.3, the V2X layer of each UE for the PC5 unicast link informs the AS layer that the PC5 unicast link has been released. The V2X layer uses PC5 Link Identifier to indicate the released unicast link.

When the V2X layer determines that no radio frequency is available for any PC5 QoS Flow(s) over a PC5 unicast link based on indication about radio frequency failure from the AS layer, the V2X layer removes the PC5 QoS Flow(s) from the PC5 unicast link by performing Layer-2 link modification procedure as specified in clause 6.3.3.4. The AS layer uses PC5 Link Identifier to indicate to the V2X layer the PC5 unicast link whose radio frequency has failed for the PC5 QoS Flow(s).

* * * * End of Changes * * * *