| 3GPP TR 26.822 V1.0.0 (2024-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on 5G Real-time Transport Protocol Configurations, Phase 2; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	10

3.3	Abbreviations	10

4	Architectural Assumptions and Requirements	11

4.1	Architectural Assumptions	11

4.2	Architectural Requirements	11

5	Key Issues	12

5.1	Key Issue #1: Inaccuracy of the PDU Set Size (PSSize) information	12

5.1.1	Description	12

5.2	Key Issue #2: QoS handling requirements for lone PDU	12

5.2.1	Description	12

5.3	Key Issue #3: Enhancements for application-layer FEC support	13

5.3.1	Description	13

5.4	Key Issue #4: AL-FEC awareness for PDU Set handling	13

5.4.1	Description	13

5.5	Key Issue #5: RTP transport of XR metadata	14

5.5.1	Description	14

5.6	Key Issue #6: PDU Set marking for XR streams with RTP end-to-end encryption	14

5.6.1	Description	14

5.7	Key Issue #7: RTCP messages to better support XR services in 5G	15

5.7.1	Description	15

5.8	Key Issue #8: RTP retransmission in supporting XR services in 5G	15

5.8.1	Description	15

5.9	Key Issue #9: Feasibility of RTP multiplexing options for transport of XR media streams	15

5.9.1	Description	15

5.10	Key Issue #10: Use cases and intended deployment scenarios for enhancements of RTP header extension for PDU Set marking	16

5.10.1	Description	16

5.11	Key Issue #11: Enhancements of RTP header extension for PDU Set marking	16

5.11.1	Description	16

5.12	Key Issue #12: Enhancements of Data Burst Marking	16

5.12.1	Description	16

5.13	Key Issue #13: Applicability of the RTP header extension for PDU Set marking to different PDU Set types	17

5.13.1	Description	17

5.14	Key Issue #14: Traffic detection and QoS flow mapping for multiplexed media stream data flows	17

5.14.1	Description	17

5.15	Key Issue #15: Media and metadata delivery over multiple sessions	18

5.15.1	Description	18

6	Solutions	19

6.0	Mapping of Solutions to Key Issues	19

6.1	Solution #1: Different PDU Set types to support handling of immersive media	19

6.1.1	Key Issue mapping	19

6.1.2	Description	19

6.2	Solution #2: Gap analysis on the QoS requirements for lonely PDU	20

6.2.1	Key Issue mapping	20

6.2.2	Description	21

6.2.3	Conclusion	22

6.3	Solution #3: SRTP Usage for end-to-end encryption	22

6.3.1	Key Issue mapping	22

6.3.2	Description	22

6.4	Solution #4: The importance of accuracy in PSSize	22

6.4.1	Key Issue mapping	22

6.4.2	Description	23

6.5	Solution #5: Introduction of AL-FEC schemes defined in IETF	29

6.5.1	Key Issue mapping	29

6.5.2	Description	29

6.5.3	Categorization	31

6.6	Solution #6: Time to next burst extension for the RTP HE for PDU Set marking	32

6.6.1	Key Issue mapping	32

6.6.2	Background	32

6.6.3	Description	33

6.7	Solution #7: PDU Set Size information correction by indicating the remaining PDU Set Size in RTP header extension	33

6.7.1	Key Issue mapping	33

6.7.2	Description	33

6.8	Solution #8: Definition of the PDU Set for Application-Layer FEC	35

6.8.1	Key Issue mapping	35

6.8.2	Description	35

6.9	Solution #9: RTP retransmission aware PDU Set handling	36

6.9.1	Key Issue mapping	36

6.9.2	Background	36

6.9.2.1	RTP retransmission payload format defined in IETF	36

6.9.2.2	WebRTC usage	38

6.9.3	Description	38

6.10	Solution #10: AL-FEC awareness at RAN while considering upstream and downstream packet losses	39

6.10.1	Key Issue mapping	39

6.10.2	Description	39

6.10.2.1	Motivation	39

6.10.2.2	Solution	40

6.11	Solution #11: PSI indication to optimize RTP retransmission	41

6.11.1	Key Issue mapping	41

6.11.2	Description	41

6.12	Solution #12: MID packet filtering	42

6.12.1	Key Issue mapping	42

6.12.2	Description	42

6.13	Solution #13: Dynamic traffic characteristics of RTP sender implementations	45

6.13.1	Key Issue mapping	45

6.13.2	Description	45

6.13.2.1	General	45

6.13.2.2	WebRTC and paced sender implementation	45

6.13.2.3	GStreamer multimedia framework RTP implementation for P2P	48

6.13.2.4	Server-side senders	51

6.13.3	Discussion and conclusion	55

6.13.4	Proposal	56

6.14	Solution #14: Candidate RTCP messages and RTP header extensions to support XR services in 5G	56

6.14.1	Key Issue mapping	56

6.14.2	Description	56

6.14.2.1	RTCP messages	56

6.14.2.2	RTP header extensions	59

6.15	Solution #15: PSI signaling for lone PDUs	59

6.15.2	Description	59

6.15.2.1	Background	59

6.15.2.2	Solution description	61

6.15.2.3	Analysis of the solution	61

6.16	Solution #16: RTP header extension for dynamic traffic characteristics	62

6.16.1	Key Issue mapping	62

6.16.2	Description	62

6.16.2.1	Intended usage in 5GS	63

6.16.2.3	One-byte RTP header extension format	63

6.16.2.4	Two-byte RTP Header Extension Format	64

6.16.2.5	Semantics	64

6.16.2.6	SDP Signaling	65

6.16.2.7	Guidelines for dynamic traffic characteristics signaling	65

6.16.2.8	Proposed Annex D.3	66

6.16.2.9	Discussion of the solution	66

6.17	Solution #17: Analysis of AL-FEC awareness in 3GPP	66

6.17.1	Key Issue mapping	66

6.17.2	Description	67

6.17.2.1	Common attributes of AL-FEC deployments	67

6.17.2.2	End-to-end transport perspective	68

6.17.2.3	Overhead of AL-FEC	69

6.18	Solution #18: Real-time communication congestion control algorithms and AL-FEC	72

6.18.1	Key Issue mapping	72

6.18.2	Description	72

6.18.2.1	General	72

6.18.2.2	Google Congestion Control (GCC)	72

6.18.2.3	PCC	74

6.18.2.4	NADA	74

6.18.2.5	SCReAMv2	74

6.18.2.6	Summary of congestion control algorithms	75

6.18.2.7	Packet loss rate calculation for AL-FEC	75

6.19	Solution #19: Congestion control enhancement to support AL-FEC awareness handling	76

6.19.1	Key Issue mapping	76

6.19.2	Description	76

6.19.2.1	Background of using AL-FEC for real-time communication in cellular networks	76

6.19.2.2	Potential Benefits of Application-layer FEC awareness for PDU Set handling	77

6.19.2.3	Implications of Application-layer FEC awareness for PDU Set handling on congestion control	78

6.19.2.4	The Proposed Solution	78

7	Overall Analysis	79

7.x	Analysis for Key Issue #x	79

8	Conclusions	79

8.x	Conclusions for Key Issue #x	79

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The Technical Report presents …

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[4]	IETF RFC 8872: "Guidelines for Using the Multiplexing Features of RTP to Support Multiple Media Streams".

[5]	IETF RFC 5761: "Multiplexing RTP Data and Control Packets on a Single Port".

[6]	3GPP TR 23.700-70: "Study on architecture enhancement for Extended Reality and Media service (XRM); Phase 2".

[7]	IETF RFC 8285 (2017): "A General Mechanism for RTP Header Extensions", D. Singer, H. Desineni, R. Even.

[8]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)".

[9]	IETF RFC 9335: "Completely Encrypting RTP Header Extensions and Contributing Sources".

[10]	IETF RFC 6904 (2013): "Encryption of Header Extensions in the Secure Real-time Transport Protocol (SRTP)", J. Lennox.

[11]	IETF RFC 8402 (2018): "Segment Routing Architecture".

[12]	IETF RFC 791 (1981): "Internet Protocol".

[13]	IETF RFC 5109: "RTP Payload Format for Generic Forward Error Correction (ULP FEC): Uneven Level Protection, different redundancies for different packets with different importance".

[14]	IETF RFC 8627: "RTP Payload Format for Flexible Forward Error Correction (Flex FEC): flexible FEC".

[15]	IETF RFC 6681: "Raptor Forward Error Correction (FEC) Schemes for FECFRAME: FEC scheme based on the Raptor".

[16]	IETF RFC 6865: "Simple Reed-Solomon Forward Error Correction (FEC) Scheme for FECFRAME: FEC scheme based on Reed-Solomon".

[17]	IETF RFC 5053: "Raptor Forward Error Correction Scheme for Object Delivery".

[18]	IETF RFC 6330: "RaptorQ Forward Error Correction Scheme for Object Delivery".

[19]	IETF RFC 6363: “Forward Error Correction (FEC) Framework”.

[20]	IETF RFC 8854: “WebRTC Forward Error Correction Requirements”.

[21]	3GPP TR 38.340: "Study on User Equipment (UE) power saving in NR".

[22]	IETF RFC 8298: "Self-Clocked Rate Adaptation for Multimedia".

[23]	Enhancing Video Network Resiliency With LTR and RS Code | At Scale Conferences, available online: https://atscaleconference.com/enhancing-video-network-resiliency-with-ltr-and-rs-code/

[24]	P. Aggarwal et al., [2304.03732] Enabling immersive experiences in challenging network conditions (arxiv.org)

[25]	Nvidia GeForce Now, Video FEC for WebRTC presentation 17 Nov. 2022, available online: https://www.youtube.com/watch?v=igm7QkqxHqk&ab_channel=KrankyGeek

[26]	Holmer S., et al., Handling Packet Loss in WebRTC, 2013 IEEE International Conference on Image Processing, available online: https://static.googleusercontent.com/media/research.google.com/en//pubs/archive/41611.pdf.

[27]	A Google Congestion Control Algorithm for Real-Time Communication, draft-ietf-rmcat-gcc-02, 2016.

[28]	WebRTC source code: https://source.chromium.org/chromium/chromium/src/+/main:third_party/webrtc, retrieved May 1, 2024.

[29]	IETF RFC 8698: "Network-Assisted Dynamic Adaptation: A Unified Congestion Control Scheme for Real-Time Media", 2020.

[30]	Self-Clocked Rate Adaptation for Multimedia, draft-johansson-ccwg-rfc8298bis-screamv2-00, 2024.

[31]	IETF RFC 4588: "RTP Retransmission Payload Format".

[32]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[33]	IETF RFC 4585: " Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF)".

[34]	IETF RFC 8085: "UDP Usage Guidelines".

[35]	IETF RFC 8834: "Media Transport and Use of RTP in WebRTC".

[36]	IETF RFC 2250: "RTP Payload Format for MPEG1/MPEG2 Video".

[37]	IETF RFC 9143: "Negotiating Media Multiplexing Using the Session Description Protocol (SDP)".

[38]	IETF RFC 7941: "RTP Header Extension for the RTP Control Protocol (RTCP) Source Description Items".

[39]	GStreamer: open source multimedia framework:
https://gstreamer.freedesktop.org/.

[40]	Wowza Streaming Engine™:
https://www.wowza.com/.

[41]	Media MTX: Ready-to-use SRT / WebRTC / RTSP / RTMP / LL-HLS media server and media proxy that allows to read, publish, proxy, record and playback video and audio streams:
http://github.com/blueviron/mediamtx.

[42]	Adobe RTMP Specification (veriskope.com):
https://rtmp.veriskope.com/docs/spec/.

[43]	RTP Extensions for Transport-wide Congestion Control, draft-holmer-rmcat-transport-wide-cc-extensions-01
https://datatracker.ietf.org/doc/html/draft-holmer-rmcat-transport-wide-cc-extensions-01

[44]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

[45]	IETF RFC 5104: "Codec Control Messages in the RTP Audio-Visual Profile with Feedback (AVPF)".

[46]	IETF RFC 6679: "Explicit Congestion Notification (ECN) for RTP over UDP".

[47]	IETF RFC 8888: "RTP Control Protocol (RTCP) Feedback for Congestion Control".

[48]	IETF RFC 8382: "Shared Bottleneck Detection for Coupled Congestion Control for RTP Media".

[49]	IETF RFC 3611: "RTP Control Protocol Extended Reports (RTCP XR)".

[50]	3GPP TS 26.510: "Media delivery; interactions and APIs for provisioning and media session handling".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Data Burst:	A data burst is a set of multiple PDUs generated and sent by the application such that there is an idle period between two data bursts. A Data Burst can be composed of one or multiple PDU Sets.

Lone PDU:	A PDU that is not marked by the sender as part of a PDU Set.

Multimedia Session:	An association among a group of participants engaged in the communication via one or more RTP sessions, as defined in section 2.2.4 of IETF RFC 7656 [18].

PDU Set marking:	Marking the PDUs carrying a payload with the PDU Set Information.

PDU Set:	One or more PDUs carrying the payload of one unit of information generated at the application level (e.g. frame(s), video slice(s), metadata, etc.).

XR Tethered Device:	Device connected indirectly to 5G Network.

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AL-FEC	Application-Layer Forward Error Correction

AP	Aggregation Packet

AVC	Advanced Video Coding

BLA	Broken Link Access

CDRX	Connected mode discontinuous reception

CRA	Clean Random Access

GCC 	Google Congestion Control ()

H.266/VVC         ITU H.266/MPEG Versatile Video Coding

HE	(RTP) Header Extension

HEVC	High Efficiency Video Coding

IDR	Instantaneous Decoder Refresh

IRAP	Intra Random Access Picture

MTSI	Multimedia Telephony Service for IMS

NADA	Network-Assisted Dynamic Adaptation

NAL	Network Abstraction Layer

NG-RAN	Next Generation Radio Access Network

NPDS	Number of PDUs in a PDU Set

NRI	nal_ref_idc

NTP	Network Time Protocol

OS	Operating System

PACI	Payload Content Information

PCC	Performance-oriented Congestion Control

PPS	Picture Parameter Set

PSI	PDU Set Importance

PSN	PDU Sequence Number within a PDU Set (PSN)

PSSize	PDU Set Size

PSSN	PDU Set Sequence Number

PTP	Precision Time Protocol

RADL	Random Access Decodable Leading

RASL 	Random Access Skipped Leading

RLC	Radio Link Control

rPSSize	remaining PDU Set Size

RTC	Real Time Communication

RTCP XR	RTCP eXtended Report

RTCP	RTP Control Protocol

SCReAM	Self-Clocked Rate Adaptation for Multimedia

SPS	Sequence Parameter Set

SRS	Split Rendering Server

SRTP	Secure RTPTID	Temporal Identifier

UDP	User Datagram Protocol

UPF	User Plane Function

XR	eXtended Reality

# 4	Architectural Assumptions and Requirements

## 4.1	Architectural Assumptions

<void>

## 4.2	Architectural Requirements

<void>

# 5	Key Issues

## 5.1	Key Issue #1: Inaccuracy of the PDU Set Size (PSSize) information

### 5.1.1	Description

This Key Issue follows up on the work on signalling the PDU Set Size in the RTP Header Extension for PDU Set marking in [2]. There was a solution included in Release 18 in 5G RTP, i.e. TS 26.522 [2], to enable accurate setting of the PDU Set Size field. However, several issues related to the end-end transmission that were raised that may change the size of the PDU Set Size. These issues were not adequately addressed in Release 18 and to enable the practical use of the solution adopted in release 18, there are needs for clarifications and/or refinements.

This key issue studies the issue of PDU Set Size signalling and several end-end transmission aspects, the following objectives have been identified:

-	Study and document the causes of PDU Set Size deviations in end-to-end transmission, such as NAT46/64 traversal, TURN, STUN use of IP Header Options, and/or Internet Protocol fragmentation.

-	Study and quantify the deviation percentages in practical RTC scenarios resulting from the causes.

-	Study what type of accuracy is needed from a RAN scheduling perspective, as the PDU Set Size is mainly used to optimize the scheduling at the RAN, take their feedback into account.

NOTE 1:	This point may require coordination with RAN2.

-	Study and develop solutions to improve 5G RTP workflows using PDU Set Marking taking such feedback into account.

-	Discuss the overhead and added complexity that can be tolerated to support this feature.

NOTE 2:	Coordination with RAN2 and SA2 might be needed.

## 5.2	Key Issue #2: QoS handling requirements for lone PDU

### 5.2.1	Description

In the FS_5G_RTP_Ph2, one objective is to study ‘issues around "lone" PDU, as identified by SA2’.

Editor’s note: The below Tdoc reference must be replaced or removed before publication of this TR.In the LS from SA2, S2-2313691/S4-240168, SA4 was asked the following questions:

SA2 in Rel-18 has agreed that the PSA UPF marks, in the downlink, each N6-unmarked PDU (lone PDU) with PDU Set information into a PDU Set over N3/N9. As a consequence, RAN will apply the PDU Set QoS parameters, e.g. apply the PDU Set Delay Budget (which is assumed to be larger than the PDB, if applicable) for the lone PDU.

Questions: Will applying PDU Set QoS parameters to these lone PDUs pose any issue from application perspective? If yes, what is the issue?

SA2 will not change the agreement to map N6-unmarked PDUs to PDU Sets over N3/N9 in Rel-18. However, since this topic may be in the scope of the FS_XRM_Ph2 study, SA2 would like to get feedback from SA4 on the questions above.

For a single PDU which doesn't belong to any PDU Set, the 5GS handles such lone PDU as a single PDU Set following the PDU Set QoS parameters. Furthermore, a lone PDU does not carry the RTP header extension for PDU Set marking defined in TS 26.522 and thus cannot convey any PDU Set Information to the 5GS. It’s proposed to study:

-	Whether there is any issue when applying PDU Set QoS parameters to the lone PDUs from the application layer perspective?

-	Study and detail the scenarios when such lone PDU may arise, e.g RTP/RTCP multiplexing, unmarked packets, incomplete sender implementation, existing guidelines for PDU Set marking.

-	Develop solutions to handle the issue of missing PDU Set Information in case of lonely PDUs.

-	Study the impact on applications of marking versus not marking of lone PDUs into PDU Sets.

NOTE 1:	Both the marking in DL direction and at the UE for uplink direction should be considered.

NOTE 2:	An answer to questions from SA2 in liaisons on this matter should be provided when concluding this key issue. If a solution requires changes or clarifications to SA2 specifications, SA2 should be informed in time.

## 5.3	Key Issue #3: Enhancements for application-layer FEC support

### 5.3.1	Description

Commercial adoptions may use application layer FEC (AL-FEC) as documented in clause 5.7.4 of TR 26.926. In RTC AL-FEC may optionally be used, but the usage is currently not documented. The objective of this key issue is to:

-	study and summarize the AL-FEC schemes that may be used as available in IETF standards and also the status of identified commercial deployments. A summary and categorization based on different aspects of the implementation such as complexity, arbitrary loss resilience, keeping the source stream unaltered will be studied. In addition, other potential gaps may be identified.

-	recommend adoption of one or more FEC schemes in 3GPP specifications for specific use cases such as split rendering, in case a clear benefit and a path forward is identified by the group for these use cases.

NOTE:	The outcome of this key issue should be shared in communication with SA2 to inform them about potential usage of AL-FEC in the RTC solutions developed by SA4 (and referenced by SA2).

NOTE:	The outcome of this key issue should be the basis for developing solutions for FEC awareness for PDU Set handling in Key Issue #4.

## 5.4	Key Issue #4: AL-FEC awareness for PDU Set handling

### 5.4.1	Description

The application layer FEC mechanisms are widely used to improve packet transmission robustness in the presence of packet losses without going through packet retransmissions that can introduce delays that violate real-time constraints. In the draft TR 23.700-70 [6] of FS_XRM_Ph2, Key Issue #1 is proposed to study the enhancement of PDU Set handling including Application-Layer Forward Error Correction (AL-FEC) encoded PDU Sets, and specifically:

whether, what and how PDU Set based handling (e.g. new standardized 5QI, enhancements to Alternative QoS profiles, FEC, etc.) and PDU Set information (including Control Plane and/or User plane information) provided by the AF/AS are enhanced.

To provide some background information, the basic idea is to expose AL-FEC related information to the NG-RAN via the control plane or user plane. The AL-FEC related information could be redundancy information or markings to differentiate among source and repair PDUs of a PDU Set etc. Based on the AL-FEC awareness, the NG-RAN may optimize the PDU Set delivery over the air interface accordingly (e.g., by discarding redundant PDUs of AL-FEC encoded PDU Sets). In the context of this cross-layer design, it is important to understand how to expose the AL-FEC information to the communication network (UPF, RAN) to enable intelligent resource allocation. Furthermore, there are intricate interactions to consider between the application and the network. In particular, the dropping extra PDUs of a PDU Set encoded with AL-FEC in a network, if any, may send a false signal to the application on the packet loss rate and the congestion level in the network. This may lead to undesired adaptation from the application such as increased redundancy ratio and reduced sending rate that can negatively impact end-to-end performance and user experience. It is thus important to understand the interactions between the application and the network in the case of AL-FEC and intentional packet dropping by the network and the impact on the media performance.

Therefore, it’s proposed to study:

-	Benefits of AL-FEC awareness for PDU Set handling given application and network interactions in the context of 3GPP, if any;

-	Whether and how to assist the 5GS to get aware of the AL-FEC;

-	For AL-FEC awareness for PDU Set handling, how to avoid/minimize the impact to the application layer, if any;

-	Benefits to the user experience and application end-to-end performance when enabling AL-FEC-awareness.

Editor’s Note: Collaboration with SA2/RAN2 is needed.

Editor’s Note: This Key Issue is based on the study on the Key Issue #3: Enhancements for application-layer FEC support.

## 5.5	Key Issue #5: RTP transport of XR metadata

### 5.5.1	Description

Editor’s note:	This clause provides a description of the key issue.

## 5.6	Key Issue #6: PDU Set marking for XR streams with RTP end-to-end encryption

### 5.6.1	Description

The usage of end-to-end encryption is broadly deployed in current networks to provide security. Similarly, secured deployments are expected for 5G RTP applications.

In this study, end-to-end encryption is referred to encryption that is commonly used in the industry that aims at the situation where only the two end users can access the confidential information but parties in between cannot.

Confidentiality is defined in this case as all user-related information being kept secret. This means that user-related information from endpoint A to endpoint B is kept secret from other entities. A 5G RTP end-to-end encrypted data flow contains RTP PDUs whose SDUs are encrypted, and headers may be partly encrypted.

Certain metadata not related to the information exchanged between the two parties need not be encrypted in this case. This follows industry best practices. For this issue the focus should be on the aspects within the scope of the study relating to XR media delivery.

This key issue proposes to study the enhancement of PDU Set Identification in encrypted RTP streams, in particular when using the RTP Header Extension for PDU Set marking.

The key issue should study the following aspects:

-     Explore and document the different scenarios for providing end-to-end RTP encryption as targeted for 5G RTP

-	If and how PDU Set information Identification may happen in an end-to-end encryption scenario for 5G RTP.

-     If needed, develop methods for signalling PDU set Information for end-to-end encrypted RTP streams applicable to different methods of end-to-end encryption.

NOTE 1:	Solutions that rely on breaking end-to-end encryption are out of the scope of this key issue.

NOTE 2:	The work on this key issue may need coordination with SA WG2 and WG3.

NOTE 3:	The end-to-end encryption based on QUIC is out of scope of this study.

## 5.7	Key Issue #7: RTCP messages to better support XR services in 5G

### 5.7.1	Description

Editor’s note:	This clause provides a description of the key issue.

## 5.8	Key Issue #8: RTP retransmission in supporting XR services in 5G

### 5.8.1	Description

RTP retransmission defined by IETF in RFC 4588 [31] is one of the media resilience techniques adopted in TS 26.114 [32] used to compensate packet losses for real-time media. Since retransmissions result in additional delay, the feasibility of RTP retransmission in XR applications subject to tight delay bounds needs to be investigated.

While application-layer retransmissions may be necessary for ensuring reliable data delivery in some cases, their usage may violate assumptions at the network layer such as 5GS QoS handling. Therefore, the usage of RTP retransmission when using 5GS QoS handling needs to be studied. In addition, the 5GS QoS handling is currently unable to distinguish retransmissions from original transmissions and thus cannot handle retransmission PDUs in a differentiated way. It needs to be understood how 5GS QoS handling, including PDU set based QoS Handling is affected when RTP retransmission is used and if there are potential benefits of retransmission awareness to 5GS QoS Handling.

It is therefore proposed to study:

-	the feasibility of RTP retransmission for XR media services

-	whether and how RTP retransmission can be combined with 5GS QoS handling,

-	if and how awareness of RTP retransmission can benefit PDU Set based QoS handling in the network.

-	whether and how PDU Set related information can be used to improve RTP retransmission.

NOTE: Work on this key issue may need coordination with SA2.

## 5.9	Key Issue #9: Feasibility of RTP multiplexing options for transport of XR media streams

### 5.9.1	Description

RTP originally relied on UDP multiplexing for carriage of different media streams (using different UDP ports for each stream and RTCP).

However, in practice assigning additional UDP ports has been problematic, and RTP based multiplexing is used as an alternative.

For RTP multiplexing of streams in a single RTP session, the SSRC is generally used as described in RFC 8872 [4].

In addition, combining RTCP and RTP on the same port is referred to as RTP/RTCP multiplexing.

In this case, the RTP and RTCP traffic can be multiplexed and demultiplexed using the shared second Byte of the UDP payload (i.e., the RTCP packet type and the RTP M bit & RTP payload type) as described in RFC 5761 [5].

In WebRTC the same port may be used for RTP, RTCP and different streams.

In addition, other forms of multiplexing may be used to support carriage of different streams over RTP that have been popular in the media industry (e.g. MPEG-2 based multiplexing).

For example, in MPEG-2 TS over RTP [36] the small TS packets of 188 bytes can be interleaved in an RTP packet. So, in this case an RTP packet contains multiple small transport stream packets that could be audio, video or even metadata related to a television program. This would lead to RTP packets including multiple media types.

It is proposed to:

-	Study and document existing options for RTP multiplexing.

-	Identify the potential gaps on support of different use cases.

-	Identify and document other popular ways of supporting multiplexed content in RTP transmission if any.

-	Study and identify how multiplexed RTP can benefit from PDU Set marking header extensions.

-	Study the relevance of identifying multiplexed streams in 5GS and explore potential benefits of additional support in 5GS.

NOTE:	This issue may require coordination with SA2. Additional support in 5GS refers to the multiplexed traffic detection and QoS Flow mapping in FS_XRM_Ph2.

## 5.10	Key Issue #10: Use cases and intended deployment scenarios for enhancements of RTP header extension for PDU Set marking

### 5.10.1	Description

Editor’s note:	This clause provides a description of the key issue.

## 5.11	Key Issue #11: Enhancements of RTP header extension for PDU Set marking

### 5.11.1	Description

Editor’s note:	This clause provides a description of the key issue.

## 5.12	Key Issue #12: Enhancements of Data Burst Marking

### 5.12.1	Description

Editor’s note:	This clause provides a description of the key issue.

A data burst indicates a set of multiple PDUs generated and sent in a short period of time as defined in clause 3.1 of TS 23.501 [3]. Data burst is a common transmission characteristic in communication networks.

The traffic characteristics regarding the data burst transmission could be beneficial for the 5GS network, e.g., power saving and efficient radio resource management. In Release 18, the End of Data Burst indication has been introduced to enable the UE power saving in the 5GS, i.e., the NG-RAN node can configure to move a UE into CDRX for power saving after transmitting the end PDU of the data burst.

Similarly, as stated in the draft TR 23.700-70 [6] from SA2 Rel-19 FS_XRM_Ph2, it also aims to study the 5GS network enhancements to support the burst related traffic characteristics.

Therefore, the following enhancements to data bust marking are studied in this key issue.

-	Identify additional traffic characteristics beneficial to the 5GS network, for example, time to next burst, burst size and other potentially relevant characteristics

Editor’s Note:	This is to be collaborated with SA2 and RAN WG.

-    Identify and document the way RTP senders can generate data bursts e.g.

- WebRTC paced sending implementation, including the different configurations of WebRTC paced sending.

- Other common RTP implementations or libraries that are commonly used

-	Develop potentially additional signalling in the 5G RTP Header Extension to include additional traffic characteristics.

-	Develop guidelines and recommendations for the setting of traffic characteristics related parameters in the RTP Header Extension (if needed).

-	Potential backward compatibility issues should be considered.

## 5.13	Key Issue #13: Applicability of the RTP header extension for PDU Set marking to different PDU Set types

### 5.13.1	Description

In the Rel-18 work, it was mainly assumed in TS 26.522 [2] that the PDU Set framework is applied to PDU Sets comprising either video frames or slices. However, the PDU Set definition in TS 23.501 [3] does not limit a PDU Set to be a video frame or slice.

PDU Set: One or more PDUs carrying the payload of one unit of information generated at the application level (e.g., frame(s) or video slice(s) etc. for eXtended Reality (XR) Services). All the PDUs of a PDU set are transmitted within the same QoS Flow.

The objective of this key issue is to study the applicability of the PDU Set concept for the cases where the PDU Set is not a video frame or slice.

This key issue aims at addressing the following points:

-	Study and document applicability criteria of PDU Set marking to different media types and formats.

-	Whether and how to apply PDU Set marking to non-video data: metadata, audio, text, image.

-	Whether and how to apply PDU Set marking to picture partitioning constructs other than slices such as tiles.

## 5.14	Key Issue #14: Traffic detection and QoS flow mapping for multiplexed media stream data flows

### 5.14.1	Description

RTP allows different delivery options for multiple media streams. The media streams can be transmitted as multiple RTP streams in a single RTP session, in multiple RTP sessions, or in some cases, multiplexed media can be carried in a single RTP stream. Hence, in some cases, a media stream may be split into multiple QoS flows or multiple media streams may be multiplexed into a single QoS flow. It is therefore important to study how the UPF and RAN nodes can identify the PDU sets belonging to a specific media stream in a PDU session in the case of multiplexed media streams.

In RTP, different streams typically use different multiplexing methods for the delivery of the media streams.  Given that a QoS flow is composed of PDUs from multiple media streams, the traffic over one QoS flow will be a mix of traffic from different media streams. PDU sets arriving at the UPF and RAN nodes are from different streams, and the RAN nodes needs to identify the respective media streams to which they belong.

In addition, some mappings of streams to QoS flows, may result in media streams that are split across one or more QoS flows. When media stream data is split across multiple QoS flows, then some PDU sets of the stream may go over one QoS flow and some may go over other QoS flow. Therefore, the UPF and RAN nodes needs to handle PDUs arriving at the UPF and NG RAN with missing PDU sets in a specific QoS flow. For example, the RAN nodes need to deal with gaps in the PDU set sequence number (PSSN) for a stream in a QoS flow.

It is proposed to:

-	study and document the issues arising due to multiplexing multiple media streams into a single QoS flow or splitting a media stream across multiple QoS flows.

-	determine benefits for identifying the PDU sets belonging to a media stream split over multiple QoS flows.

-	provide solutions on how to identify different PDU sets from the individual streams at UPF and RAN nodes. and how to handle missing PDU sets in a QoS flow when stream splitting is performed.

Editor’s Note:	Splitting a media stream into multiple QoS flows is under SA2 study and a recommendation for normative work has not been agreed yet.

## 5.15	Key Issue #15: Media and metadata delivery over multiple sessions

### 5.15.1	Description

In XR communication, certain media and metadata types, e.g., avatar and associated animation data, can be transmitted over a WebRTC or IMS data channel.

At the same time, it may still be possible to have a UE-to-UE voice call, e.g., an MTSI call, as the required latency for voice may be lower.

SDP procedures take care of grouping appropriate media flows for synchronization and other functionalities within the same RTP session. However, the synchronization between and RTP-based and a non RTP-based media/metadata delivery requires further investigation. It needs to be studied how this can be achieved effectively.

In addition, the case when media streams and metadata are delivered over different RTP sessions needs study.

Another use case where associated media may be sent over different RTP sessions is teleconferencing applications. The voice in this case maybe over a direct UE-to-UE communication (MTSI call), while other media (e.g., presentations, video) are delivered via a network media function. A high-level illustration is shown in Figure 5.15.1-1 below. Here the voice is delivered UE-to-UE, and the associated RTP session is shown as Session 3. The video from UE A to UE B is delivered via a network media function over two RTP sessions, Session 1 and Session 2. Depending on the use case and application requirements, the network media function may apply operations such as upscaling, merging video streams, or animation in case of avatar data.

Figure 5.15.1-1: An example scenario with multiple media sessions.

In this key issue, it is proposed to study:

-	Whether it is feasible (in terms of typical RTC use cases) to have media/metadata components that are sent over different paths, e.g., a UE-to-UE voice channel and a UE-MF-UE or AS/MF-to-UE channel for avatar data (sans audio).

-	Identify synchronization issues, if any.

-	Identify session establishment issues, if any.

-	How to achieve cross-session referencing for XR media and metadata that are sent over different RTP sessions and data channels that don’t have common endpoints.

-	E.g. SDP signalling description examples.

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 | KI#7 | KI#8 | KI#9 | KI#10 | KI#11 | KI#12 | KI#13 | KI#14 | KI#15 |
| #1 |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| #2 |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #3 |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| #4 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #5 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| #6 |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| #7 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #8 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| #9 |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| #10 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| #11 |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| #12 |  |  |  |  |  |  |  |  | X |  |  |  |  | X |  |
| #13 |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| #14 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| #15 |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #16 |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| #17 |  |  | X | X |  |  |  |  |  |  |  |  |  |  |  |
| #18 |  |  | X | X |  |  |  |  |  |  |  |  |  |  |  |
| #19 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |

## 6.1	Solution #1: Different PDU Set types to support handling of immersive media

### 6.1.1	Key Issue mapping

This solution addresses key issue #13 on the Applicability of the RTP header extension for PDU Set marking to different PDU Set types.

### 6.1.2	Description

In addition to frames and slices, video PDU Sets can be defined as other sub-picture constructs such as H.265/HEVC tiles.

Such PDU Sets are particularly relevant for immersive media use cases, where it could be beneficial to assign different importance to different regions of a picture, for example depending on content saliency and user’s viewing direction, as in the example of 360-degree video streaming. However, as the usage of tiles is not limited to 360-degree video streaming, the solution may also apply to other kinds of immersive media, such as volumetric video.

This allows a dynamic spatial adaptation of the PSI based on user’s viewport and/or other content-related metadata. Depending on the selected criteria (e.g., the user’s actual viewport), the sender can dynamically modify the PSI for each tile at different time points. Alternatively, the sender may consider a group of tiles as a PDU Set to reduce the number of PDU Sets contained within a picture.

The sender may determine the PSI for the PDU Sets belonging to a region based on the likelihood of that region corresponding to the user’s viewport. For example, if a tile is likely to fall outside the user's viewport, it is assigned a higher PSI for that time point. Hence, it is more likely to be dropped in case of congestion. Later, if it becomes more likely to fall inside the user's viewport, it is assigned a lower PSI.

The figure below shows an example where the left picture is divided into 12x6 tiles and the right picture into 8x4. The blue colur shows the current viewport region, and the grey colour shows the background region. The PSI values assigned by the sender for a given time point are shown inside each tile. When the picture is split into a greater number of tiles (left), the sender may set the viewport center with higher importance and gradually reduce the importance as the distance increases from the viewport center. PSI values for the tiles on the boundaries may be decided depending on the portion of the tile that falls into the viewport. In the example shown on the left, the sender assigns PSI=5, if more than 50% of a tile falls inside the viewport, and PSI=6, if a tile overlaps with the viewport but less than 50% of it falls inside the viewport. Also, the sender may distinguish between the background tiles that are outside but close to the viewport (set to PSI=10 in the example) and the other background tiles that are far from the viewport (set to PSI=15 in the example). With a smaller number of tiles, 8x4 as in the example on the right, the sender can perform the PSI assignment in a coarser manner.

Figure 6.1.2-1. Example tiling and PSI assignment.

In the current PDU Set handling framework, the network is unaware of what application data unit is marked as a PDU Set by the sender. When sub-picture constructs such as tiles are marked as PDU Sets, it may be beneficial for the network to be aware of the PDU Set type/structure used by the application to adequately configure the scheduling or the PSI-based discard, if it is used. For example, if the network knows that PDU Sets are tiles, the RAN may decide to apply a more relaxed discard mechanism to PDU Sets, knowing that it is not as critical as discarding an entire frame. The PDU Set type can be indicated by the application in the RTP header extension for PDU Set marking or via control plane signalling, for example as an extension to the Protocol Description provided by the AF.

The solution is also suitable for the next generation codec (H.266/VVC) that makes use of a more flexible partitioning mechanism called sub-pictures.

## 6.2	Solution #2: Gap analysis on the QoS requirements for lonely PDU

### 6.2.1	Key Issue mapping

This solution intends to give gap analysis on the KI#2: QoS handling requirements for lonely PDU.

### 6.2.2	Description

According to TS 23.501 [3], in case a single PDU doesn't belong to a PDU Set based on the Protocol Description for PDU Set identification, the UPF still maps it to a PDU Set and determines the PDU Set Information accordingly. In this case, both the single PDU and the PDUs belonging to a PDU Set are in the same service data flow and the single PDU is delivered to the UE in the DL direction following the PDU Set QoS parameters.

There could be different scenarios where the application server may send the PDU Sets and single/lonely PDUs in the same service data flow which can be detected by the 5GS. For a single data flow in a service data flow, as described in Annex A.2.2.1 of TS 26.522 [2], it is generally recommended that the network function considers Non-VCL NAL units (e.g. SPS NAL unit) as part of the PDU Set of the associated VCL NALUs, e.g. identified by the same timestamp. When PDU Set marking is activated, there should be no lonely PDUs in the service data flow. There are other scenarios where lonely PDUs and PDUs belonging to a PDU Set are multiplexed in a single service data flow as following.

-	Scenario #A: RTP streams multiplexed in a single RTP session. In this scenario, multiple RTP streams are multiplexed in a single RTP session which is carried over a single service data flow. For example, the audio and video streams are multiplexed in a single RTP session, while the PDU Set feature is needed for the video streams. Similarly, when FEC or RTP retransmission feature is enabled, the corresponding repair packets or retransmission packets may also be multiplexed with the original video stream. The 5GS cannot distinguish different RTP streams multiplexed in a single service data flow and has to take the PDUs in other RTP streams as lonely PDUs.

-	Scenario #B: RTP data and control packets are multiplexed on a single port. In this scenario, the RTP and RTCP flows are carried over a single service data flow. When the PDU Set feature is needed for the RTP flow(s), the 5GS cannot distinguish the RTP and RTCP traffic and has to take the RTCP traffic as lonely PDUs.

As can be seen from the above, one key reason for the lonely PDU handling is that the PDUs belonging to a PDU Set and the lonely PDUs are carried over a single service data flow and the 5GS cannot differentiate the multiplexed data flows in a single service data flow.

Therefore, it is clear that

-	the co-existence of lonely PDUs and PDUs belonging to a PDU Set in a single service data flow can be due to the lack of the capability to differentiate multiplexed media flows for 5GS.

Editor’s Note:	Other scenarios for the co-existence of lonely PDUs and PDU Set is FFS.

However, the scenario where lone PDUs may exist, is still possible due to the multiplexed RTP and RTCP or RTP audio and video traffic flows are in a single QoS Flow as requested by the application layer, e.g., the QoS requirements for them are the same.

And the QoS requirements for multiplexed media streams could be different. For example, the QoS requirements for audio and video streams could be different.

For PDU Set based QoS handling, the PDU Set QoS parameters are introduced in TS 23.501 [3] as following:

-	PDU Set Delay Budget, which defines an upper bound for the delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF.

-	PDU Set Error Rate, which defines an upper bound for the rate of PDU Sets that have been processed by the sender of a link layer protocol (e.g., RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g., PDCP in RAN of a 3GPP access).

-	PDU Set Integrated Information, which indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side.

If the NG-RAN receives PDU Set QoS Parameters, it enables the PDU Set based QoS handling and applies PDU Set QoS Parameters. When the PDU Set QoS parameters are available, they will supersede the PDU QoS parameters (i.e. PSDB/PSER supersedes the PDB/PER).

For the corresponding PDU QoS parameters, they are at a per packet granularity including the per-packet latency requirement (i.e. packet delay budget), the per-packet loss rate requirement (i.e. packet loss rate), etc. From the application perspective, the PDU Set QoS parameters and the PDU QoS parameters should reflect the same network requirements while at different granularities. Therefore, QoS requirements for multiplexed media streams could be different and applying the PDU Set QoS parameters to a single PDU could be an issue.

In addition, as discussed in draft TR 23.700-70 [6], how to support the traffic detection and QoS mapping for multiplexed data flows is ongoing in SA2 Rel-19 FS_XRM_Ph2 as shown below:

This key issue proposes study traffic detection and QoS Flow mapping in 5GS for different media streams multiplexed within a single end-to-end transport connection.

-	How to identify multiplexed traffic flows with different QoS requirements within a single transport connection.

-	How to do QoS Flow mapping for traffic flows with different QoS requirements.

-	Whether and what information needs to be provided from AF for traffic detection.

-	Whether and how AF provides QoS requirements of different traffic flows to the 5GS.

Via the potential R19 enhancements in 5GS, it is possible to differentiate the multiplexed RTP streams or RTP/RTCP flows, which may avoid the co-existence of lonely PDUs and PDUs belonging to a PDU Set.

### 6.2.3	Conclusion

Based on the gap analysis in the above, it is proposed to make the following conclusions.

-	QoS requirements for multiplexed media streams could be different and applying the PDU Set QoS parameters to a single PDU could be an issue.

-	The co-existence of lonely PDUs and PDUs belonging to a PDU Set in a single service data flow may be due to the lack of the capability to differentiate multiplexed media flows for 5GS.

Editor’s Note: 	Whether multiplexing is the only reason for lonely PDUs and whether the handling of multiplexed data flows in R19 SA2 FS_XRM_Ph2 can avoid this issue are FFS.

## 6.3	Solution #3: SRTP Usage for end-to-end encryption

### 6.3.1	Key Issue mapping

Solution to key issue number #6 PDU Set marking for XR streams with RTP end-to-end encryption using SRTP.

### 6.3.2	Description

When end-to-end encryption is applied, methods of inspecting the video bitstream will not work for PDU Set detection and NAL syntax cannot be read, neither.

Besides, when the RTP is tunneled over an end-to-end encrypted channel, the method of RTP header extension for PDU Set Marking in TS 26.522 [2] will not work, neither.

As the RTP header extension (HE) for PDU Set marking uses the general mechanism for RTP Header Extensions from RFC 8285 [7], it is not encrypted in secure RTP solution RFC 3711 [8]. Therefore, the SRTP could potentially be used together with RTP HE for PDU Set marking.

In Release 18 of TS 26.522 [2] SRTP is supported, so this solution requires limited changes to TS 26.522 [2] but some explicit text.

NOTE: Some cases when the RTP HE is also encrypted, e.g., RFC 6904 [10], RFC 9335 [9], are FFS.

## 6.4	Solution #4: The importance of accuracy in PSSize

### 6.4.1	Key Issue mapping

[We here discuss Key issue #1, providing justification and a potential solution.]

### 6.4.2	Description

[The PDU Set Size (PSSize) is calculated at the RTP packet source, including the RTP/UDP/IP packet headers, and is indicated in the “PDU Set marking” RTP header extension. The PSSize seen by the UPF may be different from the indicated PSSize value in the RTP header extension due to various reasons, including, but not limited to:

-	NAT64 or NAT46, as noted in [2].

-	IP fragmentation, where each increment in the number of IP packets adds an additional size worth of an IP packet header to the PSSize

-	TURN, where the TURN server may add a STUN message header, a STUN attribute, and transport address, as noted in [S4aR230110].

Editor’s note: The above Tdoc reference must be replaced or removed before publication of this TR.

-	Segment Routing, where an ingress router adds to the IP packet a list of segment identifiers for the segments in the Segment Routing domain [11]. This has an impact on RAN only if UE is part of the Segment Routing domain.

These reasons may individually or jointly affect the accuracy of the PSSize, for example, both NAT46 and IP fragmentation may happen to the IP packets of a PDU Set.]

Editor’s note: The above bracketed text is tentatively agreed.

Although the impact of those reasons on the size of a single IP packet seems insignificant, a PDU Set may consist of many IP packets and the aggregate impact can still be significant. If the PSSize for which gNB schedules is less than the actual PSSize, when the last packets arrive it may take gNB one or more slots to schedule them, therefore delaying the delivery of the PDU Set, which is detrimental to low-latency applications such as XR applications. The opposite can also happen, which wastes resources.

Therefore, we have the following observation:

Observation 1: it is important to make the PSSize accurate for low-latency applications.

There are efforts to correct the impact of NAT46/64 on the PSSize [11][S4-231305]. However, these efforts are not able to tackle other causes. Moreover, the list of causes in the introduction clause is not complete – even if we list all possible causes today, novel network protocols that change the PSSize are likely to be deployed in the future. We have the following observation:

Editor’s note: The above Tdoc reference must be replaced or removed before publication of this TR.

Observation 2: A generic solution for correcting the PSSize is preferred.

The UPF is the gateway to the 5G core. A UPF may handle a very large amount of traffic. In fact, some network operators have very few UPFs. Therefore, any solution that requires UPF to take action, such as addition, subtraction and multiplication, is undesirable. We have the following observation:

Observation 3: To reduce the UPF complexity, it is preferred not to require UPF to correct the PSSize.

When we don’t know the causes, how do we correct the error? There is a similar problem in physical-layer communication, where the channel seen by a receiver is the result of reflection and refraction of many unknown objects in the radio propagation environment. The solution there is to measure the channel by the sender sending a pilot signal known to the receiver and the receiver comparing the pilot signal and the received signal. We borrow the measurement idea, and the counterpart of the ‘pilot signal’ is the indicated PSSize value in the RTP header extension and the counterpart of the ‘received signal’ is the observed PSSize. If a PDU Set is delivered successfully, the UE will observe the same PSSize (by summing the sizes of all PDUs of a PDU Set) as the gNB does.

Once the UE figures out the difference between the indicated PSSize and the observed PSSize, it can signal to the sender on how to pre-compensate for the PSSize.

Proposal 1: UE computes the difference between the actual PSSize and the indicated PSSize, and signals the difference to the RTP sender for PSSize pre-compensation.

Specifically, the UE calculates a correction ratio - the actual PSSize to the indicated PSSize ratio – and sends the correction ratio to the RTP sender. The RTP sender pre-compensates the PSSize by multiplying the PSSize and this correction ratio.

To validate the effectiveness of the proposed method, we tested the method on video sequences. The results below are for the Racehorse video sequence, and the video encoder is H.264 with a target NAL Unit size of 1400 bytes. The average video frame size is 22.377 kB. The cumulative distribution of the slice size (with each slice encapsulated into an IP packet) is shown in Figure 6.4.2-1. Note that there are a significant number of sizes uniformly distributed between 0 and around 1300 bytes.

NAT46 occurs in the network.

Figure 6.4.2-1: Slice size distribution

Scenario 1 (MTU=576 bytes):

The MTU size is set to 576 bytes (considered as a ‘safe’ MTU, because it is the IP packet size that all IPV4 nodes need to support [12]). This leads to fragmenting a packet size of 1400 bytes into three IP packets. As a result, the error in the PSSize comes from two sources: the presence of NAT46 and IP fragmentation. For the measurement-based correction method, the correction ratio is initialized to 1.

NOTE: The network configuration, e.g., NAT46 and MTU, typically changes much slower than the time scale of the feedback delay which is on the order of RTT. Thus, the feedback mechanism does not cause instability.

NOTE: As needed feedback - The UE can adapt the rate of feedback based on the observed error in the PSSize and a threshold on the tolerance of error.

The correction ratio (blue line) is shown in Figure 6.4.2-2.

Figure 6.4.2-2: Correction for the per-frame feedback (blue line) and one-time feedback (red line).

The various PSSize’s are shown in Figure 6.4.2-3. The errors in the PSSize are shown in Figure 6.4.2-4. Without PSSize correction, the mean absolute error of the PSSize is 1616 bytes; with PSSize correction assuming NAT46 only, the mean absolute error is 1283 bytes; with the proposed measurement based PSSize correction, the mean absolute error is 27.5 bytes.

Figure 6.4.2-3: The actual PSSize (cyan line) vs the PSSize without PSSize correction (blue line), the PSSize with measurement-based correction (red) and the PSSize with the ‘NAT46/64 only correction’ (green).

If the UE sends the feedback once during the transmission of the video sequence, without further correction ratio received, the sender will use a correction ratio of 1 for future frames. The correction ratio is shown by the red curve in Figure 6.4.2-2. The mean absolute error is 23.9 bytes, which is lower than the error when per-frame (or per PDU Set) feedback is used. This is because the first frame is an I-frame with a large size and the estimated ratio based on it tends to be more accurate than the estimates obtained in other smaller sized frames which are P frames.

Observation 4: For measurement-based PSSize correction, one-time feedback can be more accurate than more-frequent feedback.

Figure 6.4.2-4: PSSize error for the cases of without correction (blue), with correction assuming NAT46 only (green), and with the proposed measurement-based correction with per-frame feedback(red) for Scenario 1.Scenario 2 (MTU=1300 bytes):

This is to show the effect of MTU size. The MTU size is set to 1300 bytes. A typical packet of 1400 bytes is fragmented into 2 packets. The errors in the PSSize are shown in Figure 6.4.2-5. The mean absolute error in the PSSize are 963.3 bytes, 630.0 bytes and 24.7 bytes for the case of no PSSize correction, NAT46 only correction and measurement-based correction, respectively.

With one-time feedback, the error is 20.7 bytes, again lower than the error of per-frame feedback 24.7 bytes.

We see that the respective errors decrease compared to smaller MTU size. However, the errors for case of no PSSize correction and the case of NAT46 only correction are still significant.

Figure 6.4.2-5: PSSize error for the cases of without correction (blue), with correction assuming NAT46 only (green), and with the proposed measurement-based correction with per-frame feedback (red) for Scenario 2.

From the results, we observe that:

Observation 5: The proposed pre-compensation based PSSize correction method effectively reduces the PSSize error.

For the ‘NAT46/64 only correction’ method to work, the sender needs to be aware of the presence of NAT46/64. The awareness may be obtained by feedback from the receive on the type of the IP address type seen by the receiver. The feedback may be one time during a session, and can be done by SDP signaling in a similar way in [S4-231809].

Editor’s note: The above Tdoc reference must be replaced or removed before publication of this TR.

TS 26.522 [2] provides guidelines for preventing IP fragmentation, either through path MTU discovery or by assuming a conservative MTU size at the sender in generating IP packets. Path MTU discovery needs support from the routers on the end-to-end path and incurs communication overhead, and a conservative MTU size may lead to unnecessarily small IP packet sizes which come with a higher packet header cost (i.e., the ratio of the size of packet headers to the size of the media). We consider the guidelines as a solution for IP fragmentation prevention.

We compare the three solutions. The first criteria is whether the solution is generic, i.e., whether the solution can tackle multiple and even unknown causes to the error in the PSSize. The criteria ‘need support from the network?’ means whether the network needs to be configured (e.g., configured to support path MTU discovery) to enable a solution.

Observation 6: Pros and Cons of the three solutions

| Solution | Generic | Accuracy | Need support from the network? | Communication overhead? | Need spec change? |
| --- | --- | --- | --- | --- | --- |
| NAT46/64 only correction | No | Low | No | Low | Yes |
| IP fragmentation prevention guidelines | No | Low | Yes if use path MTU discovery; no if use conservative MTU size. | Moderate if use path MTU discovery; none if use conservative MTU size. | No |
| Measurement-based correction | Yes | High | No | From low (one-time feedback) to high (per PDU Set feedback) | Yes |

## 6.5	Solution #5: Introduction of AL-FEC schemes defined in IETF

### 6.5.1	Key Issue mapping

This maps to Key Issue #3.

### 6.5.2	Description

IETF defined a few AL-FEC schemes including the codes, packet formatting and transmission methods, as detailed below. Some of them are Maximum Distance Separable (MDS) codes, meaning that they enable a receiver to recover the k source symbols from any set of k received encoded symbols.

-	Non-MDS FEC schemes:

-	FlexFEC: or Flexible Forward Error Correction, as defined in RFC 8627 [14]:

-	FlexFEC relies on XOR operation in generating repair packets from source packets.

-	FlexFEC currently is supported in the WebRTC implementation (RFC 8854 [20]).

-	The encoding may be done in 1-dimensional or 2-dimensional fashion.

-	A repair packet may protect a limited number of source packets.

-	In the WebRTC implementation, the amount of redundancy depends on the packet loss rate, bitrate and RTT.

-	The source packets have the same RTP packet format as regular packets without FEC, and the repair packets carry encoding information in the FEC Header (shown below) indicating which of the source packets are protected by this repair packet.

-	Note that the FEC Header is part of the RTP payload and becomes invisible in the case of SRTP.

Figure 6.5.2-1: RTP packet format for the repair packet for FlexFEC.

-	ULPFEC: or Uneven Level Protection Forward Error Correction, as defined in RFC 5109 [13]:

-	ULPFEC is similar to FlexFEC in the encoding operation but has the additional feature of providing multiple FEC levels for different parts of an application data unit.

-	ULPFEC currently is supported in the WebRTC implementation.

-	The source packet (called media packet in RFC 5109 [13]) follows the same RTP packet format without FEC, and the repair packet (called FEC packet in RFC 5109 [13]) follow the format shown below. Note that multiple FEC levels (protection levels) are supported.

-	Again, the FEC Headers will be invisible in the case of SRTP.

Figure 6.5.2-2: RTP packet format for ULPFEC

-	MDS or near-MDS schemes:

-	Reed-Solomon (RS) FEC: defined in RFC 5510 and RFC 6865 [16].

-	RS FEC codes are MDS.

-	They are commercially deployed in for example Meta Messenger.

-	The source packet format and the repair packet format are shown in Figure 6.5.2-3.

Figure 6.5.2-3: Format of the source packet and repair packet for RS FEC

-	Raptor: defined in RFC 5053 [17].

-	Raptor is a fountain code, i.e., as many encoding symbols as needed can be generated by the encoder on-the-fly from the source symbols of a source block of data. The decoder can recover the source block from any set of encoding symbols only slightly more in number than the number of source symbols.

-	RaptorQ: defined in RFC 6330 [18].

-	RaptorQ is a fountain code.

-	RaptorQ codes provide superior flexibility, support for larger source block sizes, and better coding efficiency than Raptor codes. The RTP schemes for RaptorQ and Raptor are defined in RFC 6681 [15].

### 6.5.3	Categorization

Table 6.5.3-1 categorizes available standardized FEC schemes from IETF based on different criteria.

In addition, for RFC 6681 [15] and 6865 [16], and generally for the underlying FEC framework in RFC 6363 [19], the source data may be modified which may affect backwards compatibility of endpoints not supporting FEC and the application of encryption (i.e., if it happens before or after FEC).

For Raptor RaptorQ different schemes are defined in RFC 6681 [15].

-	arbitrary sequence/arbitrary packet flow this needs additional information in the source packets

-	single sequenced flow -> there is no change to the source packets

This is why in the fourth column both options yes and no are marked.

Performance is considered good if there is general repair capability for any loss without introducing too much latency. Performance is considered medium if there is general repair capability for any loss but introducing some latency and complexity. Performance is poor when reliability is still not guaranteed.

Table 6.5.3-1: Categorization of AL-FEC schemes for RTP in IETF

| Name | RFC | Type | Format of source packets unchanged/ backward compatible | Resilience to Arbitrary packet loss | Flexible redundancy | Overhead (bytes) | Performance (repair capability) | MDS (incl. approximate MDS) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ULP FEC | 5109 | Parity/ XoR | Yes | No | Yes | High | Low | No |
| FlexFec | 8627 | Parity/ XoR | Yes | No | Yes | High | Low | No |
| Raptor/ RaptorQ | 6681 | Fountain/ LT | Yes/No | Yes | Yes | Medium | Good | Yes |
| Reed Solomon | 6865 | Polynomial | No | Yes | Limited | Medium | Good | Yes |

## 6.6	Solution #6: Time to next burst extension for the RTP HE for PDU Set marking

### 6.6.1	Key Issue mapping

This solution addresses the key issue #12.

### 6.6.2	Background

TS 26.522 [2] defines a data burst as a set of multiple PDUs generated and sent by the application such that there is an idle period between two data bursts. A Data Burst can be composed of one or multiple PDU Sets.

TS 23.501 [3] enables an End of Data Burst (EoDB) indication to be added to the last PDU of each Data Burst in the GTP-U header to configure the UE power management schemes like Connected Mode Discontinuous Reception (CDRX). The procedure is as follows:

-	PCF may provision the Protocol Description within the PCC rules based on the information provided by the AF and/or the local operator policies.

-	SMF should request the UPF to detect the last PDU of the data burst and mark the EoDB in the GTP-U header of the last PDU in downlink, according to the PCC rule and/or the local operator policies.

-	UPF identifies the last PDU of a data burst in the downlink traffic based on the End indication according to the Protocol Description and provides an EoDB indication to the RAN in the GTP-U header of the last PDU of a data burst.

If packets are transmitted in a bursty fashion, the idle time between two bursts is largely determined by the video frame inter-arrival time (e.g., ~33 ms for 30 fps). However, it may vary from burst to burst depending on the instantaneous variations in frame rate and when the NAL units comprising a PDU Set is made available by the encoder to the RTP packetizer, which may depend on the scene complexity. Furthermore, encoders that enable frame reordering may pass multiple frames to the RTP sender at once which are transmitted in a single burst that continues over multiple frame intervals.

Paced sending is a technique used in WebRTC to smooth the flow of packets sent to the network by spreading transmission across the frame interval. If paced sending is used, the sending time of the next packet or group of packets is determined by the pacer which may also consider factors such as frame rate and total bytes currently in the queue. In this case, since the transmission of the packets of a video frame is spread over the entire frame, the data burst concept may not apply depending on the amount of packets grouped together.

The EoDB indication informs the UE that there is an opportunity to sleep until the beginning of the next burst and enables the usage of CDRX mechanisms. However, the optimal power state of the UE depends on the time to next burst (TTNB) since the UE requires different transition times to switch to different power/sleep states, as described in TR 38.840 [21]. Therefore, EoDB by itself does not provide enough information for the UE or RAN to determine the appropriate sleep state for maximal power saving.

Table 6.6.2-1 shows the relative power consumption and total transition time (ramping down and up) of each sleep state for FR1 (frequency range 1, up to 7 GHz). Time interval for the sleep should be larger than the total transition time entering and leaving a power state.

Table 6.6.2-1: Relative power and total transition time for the sleep states defined in TR 38.340 [21]

| Sleep State | Relative Power | Total transition time |
| --- | --- | --- |
| Deep sleep | 1 | 20 ms |
| Light sleep | 20 | 6 ms |
| Micro sleep | 45 | 0 ms* |
| * Immediate transition is assumed for power saving study purpose from or to a non-sleep state |  |  |

### 6.6.3	Description

In this solution, the RTP HE for PDU Set marking defined in TS 26.522 [2] clause 4.2 is extended to include a TTNB field. TTNB is the time interval between the transmission of the last packet in the current burst and first packet of the next burst, i.e., inter-burst time.

The TTNB field is 8 bits in length and is expressed in milliseconds. It can be an optional field added by the RTP sender only if the AS is able to obtain the TTNB information and it is subject to SDP negotiation between the sender and the receiver. TTNB is set to 0, if the current PDU is not the last PDU of the burst or if the sender cannot determine the TTNB.

Real-time congestion control algorithms like SCReAM [22] maintain an RTP queue at the sender side to temporarily store the RTP packets pending transmission. Thus, they can adapt the sending rate of packets depending on the congestion level and size of the frames produced by the video encoder. TTNB can be determined by RTP senders that implement such congestion control algorithms since they can choose when the next burst of packets will be sent.

An RTP sender determines the time to next burst based on its wall clock time. The timing is determined based on the sending time of the packets and not the time they are received by the RTP receiver. Optimization based on potential network jitter is not in the scope of this solution but can be implemented in the UPF or RAN.

TTNB can be used by the receiver UE to initiate the most adequate sleep state. For example, if TTNB is more than 20ms, the receiver UE could achieve the optimal power saving by going to deep sleep.

An example implementation of the TTNB field is shown below for the one-byte version of the RTP HE for PDU Set marking.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | len   |E| R |D|  PSI  |      PSSN         |     PSN   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                     PSSize                    |     NPDS

+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

|      TTNB     |

+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

TTNB can be signaled as an optional extension attribute in the SDP signaling. An example usage of extmap attribute for such signaling is shown below.

a=extmap:7 urn:3gpp:pdu-set-marking:rel-18 short ttnb

Editor’s Note: This solution requires coordination with SA2 and RAN2.

## 6.7	Solution #7: PDU Set Size information correction by indicating the remaining PDU Set Size in RTP header extension

### 6.7.1	Key Issue mapping

This maps to Key Issue #1.

### 6.7.2	Description

According to the current TS26.522, all PDUs of a PDU Set carry the same information in the PDU Set Size (PSSize) field in the RTP header extension for PDU Set marking. Repeating the same information in general is a waste of resource.

We propose to reuse the PSSize field, giving it a new interpretation or a new name, to indicate the remaining PDU Set Size (rPSSize), which includes the size of the PDU carrying the PSSize field, i.e., how many bytes the PDU Set has from this PDU to the last PDU of the PDU Set. For the first PDU of a PDU Set, the rPSSize and the PSSize fields indicate the same value and have the same meaning. As an example, if the PSSize is 4000 bytes consisting of 4 PDUs with 1000 bytes each. The rPSSize field of the first PDU in the PDU Set will indicate 4000 bytes, the same as the PSSize does. The rPSSize field of the second PDU in the PDU Set will indicate 3000 bytes instead of 4000 bytes that would be indicated by PSSize.

This proposal allows a router to compare the indicated size (i.e., the size from the perspective of the application sender) of a PDU (by taking the difference in the rPSSize between two adjacent PDUs) and the observed size of the PDU and derive the PSSize error due to network operations such as NAT46/64 that alter the PSSize. Using the same example, we assume that there is NAT46 in the network unknown to the packet source. The router can derive the indicated size of the first PDU (PDU Sequence Number or PSN=0) by taking the difference between the rPSsize carried in the 1st PDU (PSN=0) and the rPssize carried in the 2nd PDU, as shown in Figure 6.7.2.-1. The difference will be 4000 – 3000 = 1000 bytes. On the other hand, the router observes that the 1st PDU has an actual size of 1020 bytes. Then, the router knows that the network has added 20 bytes to the packet, and it needs to add 20 bytes for each PDU (this means that the number of PDUs in the PDU Set (NPDS) needs to be signaled in the PDUs) in the PDU Set to get the actual PSSize.

Figure 6.7.2-1 Deriving the indicated size of the 1st PDU by taking the difference in the rPSSize between the 1st PDU and the 2nd PDU

One may argue that in the event of out-of-order delivery, with the current specification TS 26.522, if every PDU carries the PSSize, the first received PDU (whose PSN may not be equal to 0) will provide the PSSize information needed by a router. This is not necessary. First, for low-latency applications, a reasonable design should not lead to severe out-of-roder delivery.

Second, if every PDU carries its respective rPSSize, the router can use the PSN filed in the RTP header extension together with the rPSSize to estimate the PSSize. As more packets arrive, the estimate will get more accurate.

Third, even if the packets arrive at a router out-of-order, it may not have a problem. To see this, consider two cases. Case (1) all packets arrive in an ideal burst (i.e., all PDUs arrive within a time slot or a transmission time interval (TTI) equal 1ms): in this case, the router performs the PSSize correction described earlier, but in a very short duration of an ideal burst, offering even more lead time for scheduling. Case (2) all PDUs are evenly distributed in time until the next PDU Set: in this case, not being able to get the PSSize in the first time slot does not necessarily prevent scheduling the PDUs arriving in the first time slot. Under rare conditions (e.g., the first arrived PDU has the largest PSN among all PDUs of the PDU Set), the rPSSize obtained in the first time slot is less than the total size of the PDUs in the first time slot, the unscheduled PDUs can still be scheduled in the next time slot. What really matters is to timely deliver the PDU Set as a whole, an observation that motivated the notion of Nominal PDU Set Delay Budget (NPSDB) (see clause 6.20 of [6]). As long as the last few PDUs are scheduled on time, which is guaranteed, the timely delivery of the whole PDU set is not affected.

To implement this solution, we can replace the PSSize in the RTP header extension for PDU Set marking with rPSSize, updating the semantics of the field without changing the format.

Alternatively, without changing the name, the PSSize field can be re-interpreted as the remaining PSSize during session setup.

Pros: compared to other solutions, it doesn’t incur additional signaling in the user plane or the control plane beyond what is needed for supporting the PSSize in the current TS 26.522.

Cons: it assumes that the intermediate routers (e.g., UPF, gNB) use the rPSsize value to correct the PSSize, but when a router serves a large number of traffic flows

-	such computation may not be scalable,

-	the router needs to maintain a state variable (to store the rPSSize in the most recently received PDU) in the memory,

-	it needs the inclusion of the optional Number of PDUs in a PDU Set (NPDS) field in the RTP header extension for PDU Set marking,

-	this method cannot correct the PSSize when the first PDU is received, and it needs to wait for at least another PDU before it can correct the PSSize

-	this solution would not work with current Stage 2 work when the first PDU of the PDU set is not delivered first (i.e., in order) and requires stage 2 update to address this issue.

-	it also requires changing the semantics of PSSize as defined in TS 26.522

## 6.8	Solution #8: Definition of the PDU Set for Application-Layer FEC

### 6.8.1	Key Issue mapping

This maps to Key Issue #4.

### 6.8.2	Description

In Rel-18, the PDU Set was defined without the consideration of AL-FEC. When AL-FEC is used, the RTP source typically generates both source packets and repair packets. A natural question is whether we need to conglomerate the source packets and the repair packets of an ADU into a single PDU Set or into two PDU Sets. To answer this question, we need to consider how the source packets and the repair packets are multiplexed because the multiplexing has an impact on the QoS provisioning.

Regarding the definition of the PDU Set in the case of AL-FEC, there are two options:

-	Option 1 (separate PDU Sets): A PDU Set includes only the source packets of an ADU and another PDU Set includes only the repair packets of the same ADU

-	Option 2 (the same PDU Set): A PDU Set includes both the source packets and the repair packets of an ADU

There are three ways to multiplex the source packets and the repair packets:

-	Scheme 1 (in a single RTP stream): The source packets and the repair packets of an ADU are sent in the same RTP stream, which is identified by an SSRC. This multiplexing scheme is used for ULPFEC in the WebRTC implementation [28].

-	Scheme 2 (in different RTP streams of an RTP Session): The source packets and the repair packets of an ADU are sent in two separate RTP streams of the same RTP session, and the streams are identified by two different SSRC’s. This multiplexing scheme is used for FlexFEC in the WebRTC implementation [28].

-	Scheme 3 (in different RTP sessions (IP 5-tuples)): The source packets and the repair packets of an ADU are sent in two separate RTP sessions, which are identified by two different IP 5-tuples. This is recommended in RFC 5109 [13], although we are not aware of any such commercial implementation.

In TS 23.501, the QoS for PDU Sets is provisioned on a per QoS flow basis. A QoS flow is typically identified by an IP 5-tuple. The network identifies which IP 5-tuple a PDU Set is associated with and then provisions QoS. With schemes 1 and 2, the source packets and the repair packets of an ADU are still associated with the same IP 5-tuple, allowing for both options for the definition of the PDU Set. However, if option 1 is used, the network needs to correlate the two PDU Sets, and this incurs additional complexity. Therefore, option 2 is preferred.

In contrast, with scheme 3, the source packets and the repair packets of an ADU are associated with different IP 5-tuples. For option 1 of the PDU Set definition, i.e., the source packets and repair packets forming two PDU Sets, the network needs to correlate the two PDU Sets for QoS provisioning. For option 2 of the PDU Set definition, a PDU Set is split into two QoS flows, and it will be difficult for the network to provision QoS to the two QoS flows jointly to meet the QoS for a single PDU Set.

The complexity for PDU Set QoS provisioning is summarized in the table below:

Table 6.8.2-1: Complexity for PDU Set QoS Provisioning

|  | Complexity for PDU Set QoS provisioning |  |  |
| --- | --- | --- | --- |
|  | Multiplexing Scheme 1 (in a single RTP stream) | Multiplexing Scheme 2 (in two RTP streams of an RTP session) | Multiplexing Scheme 3 (in two RTP sessions or with two IP 5-tuples) |
| Option 1: separate PDU Sets | High | High | High |
| Option 2: the same PDU Set | Low | Low | High |

NOTE:	Scheme 1 and Scheme 2 are deployed commercially, while commercial deployment of Scheme 3 has not been found.

Based on the summary, we see that option 2 of the PDU Set definition has the advantage of having lower complexity for PDU Set QoS provisioning. Therefore, we arrive at the following conclusion:

Observation 1: To minimize the complexity for PDU Set QoS provisioning in the case of AL-FEC, a PDU Set is defined to include both the source packets (PDUs) and the repair packets (PDUs) of an ADU.

## 6.9	Solution #9: RTP retransmission aware PDU Set handling

### 6.9.1	Key Issue mapping

This solution addresses the key issue #8.

### 6.9.2	Background

#### 6.9.2.1	RTP retransmission payload format defined in IETF

IETF defined an RTP retransmission payload format in RFC 4588 [31]. The payload format was designed for use with the extended RTP profile for RTCP-based feedback, the RTP/AVPF defined in RFC 4585 [33].

Retransmission packets carry copies of lost packets along with sequence numbers and timestamps to facilitate accurate reconstruction at the receiver. The timing and frequency of retransmission packets are controlled by the sender based on network conditions and feedback from the receiver. This allows a trade-off between reliability and delay; the endpoint may give up on retransmitting after a given buffering time.

RTP retransmission can be performed selectively, meaning that only some of the lost packets are retransmitted, rather than entire data blocks. This selective approach can minimize the overhead associated with retransmissions, as only the packets deemed necessary by the sender may be retransmitted. TS 26.114 [32] recommends senders to retransmit packets which they deem beneficial for timely recovery.

Senders are not required to retransmit and exact copy of the lost source packet. For example, they may retransmit the same encoded data at a lower rate to avoid overloading the network. However, senders must ensure that the receiver will still be able to decode the payload sent in the retransmission packet. Senders can determine the acceptable bit rate and packet rate according to the congestion control mechanism defined in the RTP/AVPF profile.

The format for retransmission packets is shown in Figure 6.9.2.1-1. The sequence number of the source RTP packet, i.e., the Original Sequence Number (OSN), is inserted into the first two octets of the RTP payload as the payload header. The remaining payload corresponds to the original RTP packet payload.

Figure 6.9.2.1-1: Retransmission packet format defined in RFC 4588

Upon detection of a lost packet, the receiver decides whether to request a retransmission or not. The decision may depend on e.g. the media type, tolerable application delay and network conditions. Receivers should use the RTCP NACK feedback message format defined in the RTP/AVPF profile to send retransmission requests. Before sending another NACK to request a new retransmission of a packet, receivers should detect that the previous retransmission failed based on an estimate of the round-trip time (RTT). NACKs can be sent in regular compound RTCP packets or early RTCP packets (as per RTP/AVPF). Format of the generic NACK message is shown in Figure 6.9.2.1-2.

Figure 6.9.2.1-2: Syntax for the Generic NACK message defined in RFC 4585

Semantics of the fields are:

-	Packet ID (PID): RTP sequence number of the lost packet.

-	Bitmask of following lost packets (BLP): Allows for reporting losses of any of the 16 RTP packets immediately following the RTP packet indicated by the PID.

When SDP is used to indicate the use of retransmission for an RTP stream, the mapping is done using the fmtp attribute as follows:

a=fmtp:<number> apt=<apt-value>;rtx-time=<rtx-time-val>

-	<number>: payload type of the retransmission stream

-	<apt-value>: payload type of the original/source stream

-	<rtx-time-val>: time in milliseconds (measured from the time a packet was first sent) that a sender keeps an RTP packet in its buffers available for retarnsmission. If this parameter is absent, max retransmission time is undefined and but may be negotiated by other means.

TS 26.114 [32] recommends that the minimum “rtx-time” value should be equal to the RTT and the maximum value should be 400ms.

RFC 4588 [31] requires the original and retransmission packets to be sent in two separate streams. Two options are given.

1)	Session-multiplexing: The streams are multiplexed by sending them in two different sessions. In this case, the original and retransmission streams are sent to different network addresses or port numbers.

2)	SSRC-multiplexing: The streams are sent in the same session using different SSRC values. This allows minimizing the port usage since the same port can be used for both streams.

According to TS 26.114 [32], MTSI senders and receivers shall support handling of RTP retransmission packets using SSRC-multiplexing.

WebRTC requires, in RFC 8834 [35], that the endpoints support handling of RTP retransmission packets using SSRC multiplexing and leaves the support of session-multiplexing optional.

#### 6.9.2.2	WebRTC usage

The libwebrtc implementation of WebRTC uses an adaptive NACK/FEC scheme where the configuration is adapted depending on the RTT. If the RTT is below a threshold, the NACK mode is used, i.e., lost packets are retransmitted, and FEC is not used. If the RTT is above another threshold, only FEC is used since the delay incurred by retransmissions may be prohibitive for the application. Another hybrid mode utilizes both FEC and retransmission in an adaptive manner when the RTT lies between the two thresholds.

When retransmissions are used, WebRTC applies a selective retransmission scheme. When a retransmission request (NACK) is received, the RTP sender ignores the request if the packet has been retransmitted in the last RTT msecs. Otherwise, it retransmits the packet if a copy of the packet is still found in its buffer. Retransmission rate is limited according to a bandwidth estimate to avoid sending too many retransmissions and aggravating the congestion.

More information on the adaptive NACK/FEC mechanism can be found in the paper: "Handling packet loss in WebRTC" [26].

### 6.9.3	Description

RTP retransmission is negotiated and configured end-to-end between the sender and the receiver. However, currently, there is no mechanism to indicate to the 5G network whether RTP retransmission is performed and, if yes, how it is configured.

When PDU Set based handling is used, this may lead to suboptimal operation since the 5G network cannot configure the network operations like buffering, scheduling, packet discarding in a way that would benefit from awareness of RTP retransmission. For example, in case of momentary congestion, the RAN could have a higher preference for discarding packets from applications that use retransmission considering that the discarded packets will be retransmitted by the application hopefully within a non-congested period (and assuming that the packet is still found in the sender application buffer).

In this solution, the sender indicates to the network that it has successfully negotiated the use of RTP retransmission with the receiver, and thus retransmissions can take place during the session. The indication can be sent via control plane signalling e.g. in the Protocol Description signalled by the AF.

### Differentiated configuration of PDU Set QoS parameters

According to RFC 4588 [31], original packets and retransmission packets are carried in different RTP streams, either in the same RTP session or in different RTP sessions. When PDU Set handling is used, PDU Set QoS parameters can be set for each RTP stream by the AF.

PDU Set QoS parameters applied to retransmission streams can benefit from differentiated configuration. For example, the retransmission stream may be assigned a shorter PDU Set Delay Budget (PSDB) so that it becomes more likely that the retransmitted packets reach the receiver before the playout deadline of the media unit they are associated to.

NOTE:	It is assumed that the retransmitted PDUs and original PDUs are placed in PDU Sets mapped to different QoS flows such that different PDU Set QoS parameters can be applied.

### Modified PDU Set marking for retransmitted PDUs

When PDU Set handling is used, RTP senders can insert the RTP HE for PDU Set marking (defined in TS 26.552 clause 4.2) to outgoing RTP packets in order to add the PDU Set Information. However, for retransmitted PDUs, some of the data fields present in the RTP HE for PDU Set marking may not be necessary.

The two optional fields in the RTP HE for PDU Set marking are the PDU Set Size (PSSize) and the Number of PDUs in the PDU Set (NPDS).

PSSize is intended to be used by the RAN for allocation of scheduling resources efficiently to PDU Sets. Until a retransmitted PDU is delivered, most or all of the other original PDUs in a PDU Set will have been transmitted. Therefore, a retransmitted PDU does not need to be marked with PSSize, since this information would no longer provide any benefit to the network.

NPDS is intended to be used by the UPF to correct the PSSize calculation, in case a NAT64/NAT46 conversion has occurred in the network path changing the IP header size and thus invalidated the PSSize calculated at the sender. Thus, NPDS is similarly not necessary once all or most of the original PDUs in a PDU Set have been transmitted.

Among the mandatory fields, the PDU Sequence Number within a PDU Set (PSN) is not necessary for a retransmitted PDU since this information would not provide a correct ordering information in case of a retransmitted PDU.

The End of Data Burst (D) field is maintained since a data burst may contain PDUs from both the original and the retransmission stream and the last PDU of a data burst may correspond to a retransmitted PDU.

The PDU Set Sequence Number (PSSN) is maintained since this field allows the network to identify to which PDU Set a retransmitted PDU belongs and thus allows the network to determine whether the entire PDU Set (including the retransmitted PDU) can be delivered on time. For example, the network may estimate the delivery time for the retransmitted PDU and determine whether the transmission time for the entire PDU Set is still within the PSDB. If not, there might be no point of delivering the retransmitted PDU since the playout deadline will likely be missed.

The PDU Set Importance (PSI) field is maintained since retransmitted PDUs may also be subjected to PSI-based packet discarding in case of congestion. However, for differentiated handling of retransmitted PDUs, it could be beneficial to also indicate in the RTP HE whether the marked PDU is a retransmitted PDU. For example, if the network receives two PDUs with the same PSI value and one of them is a retransmitted PDU, that one may be considered to have higher importance and be treated more favorably in terms of resource allocation and scheduling.

NOTE:	Lonely PDUs may also benefit from a more compact RTP HE for PDU Set marking since parts of the RTP HE for PDU Set marking defined in TS 26.522 such as PSSN, PSN, PSSize and NPDS may not be useful for the network in case of lonely PDUs.

An example implementation of the modified RTP HE for PDU Set marking for retransmitted PDUs is shown in Figure 6.9.3-1. The flag indicating that the marked PDU is a retransmitted one is denoted by “X”.

Figure 6.9.3-1: Modified RTP HE for PDU Set marking for retransmitted PDUs.

Editor’s Note: This solution requires coordination with SA2 and RAN2.

## 6.10	Solution #10: AL-FEC awareness at RAN while considering upstream and downstream packet losses

### 6.10.1	Key Issue mapping

This maps to Key Issue #4.

### 6.10.2	Description

#### 6.10.2.1	Motivation

As explained in clause 5.4.2.2, there is an upstream network segment before the RAN and there may be a downstream network segment after the RAN (e.g., a tethered link). When the RAN drops extra PDUs of a PDU Set encoded with AL-FEC, it needs to consider how many of the PDUs have already been lost in the upstream network segment. The RAN also needs to consider how many packets may get lost in the downstream network segment. Otherwise, the application receiver may not receive enough PDUs for reconstructing all the source PDUs.

NOTE 1:	Although the solution below is on how the RAN drops extra PDUs, it applies to a generic network device if the network device receives the downstream packet loss probability.

#### 6.10.2.2	Solution

Figure 6.10.2.2-1: The RAN drops extra PDUs by considering packet losses in the upstream network segment and downstream network segment.

The RAN derives the PDUs that have been lost in the upstream network segment, e.g., by taking the difference between the number of PDUs in the PDU set (NPDS) in the RTP header extension for PDU Set marking for a PDU Set and the number of PDUs of the PDU Set that it has received.

The downlink network segment informs the RAN of the packet loss rate over the tethered link. This is shown in Figure 6.10.2.2-1.

NOTE 2:	Although in Figure 6.10.2.2-1 it is shown that the UE provides the packet loss feedback, the XR Tethered Endpoint can also provide the packet loss feedback.

The RAN then decides on how many PDUs are extra PDUs given (1) the upstream packet losses, (2) the packet losses at the RAN (not dropped as extra PDUs), and (3) the downstream packet loss rate.

The following example shows how the RAN decides on the number of extra PDUs that it can drop while still ensuring that the receiver can reconstruct all the source packets with a high probability. Denote the following:

(1)	the upstream packet losses: N1

(2)	the packet losses at the RAN (not dropped as extra PDUs): N2

(3)	the downstream packet loss rate: p

It is assumed that the packet losses in the downsteam are independent. Consider a PDU Set with be K=30 source PDUs, and 15 repair PDUs, hence a total of N=45 PDUs. Let the packet losses at the RAN due to wireless link error (not dropped as extra PDUs) be N2. Let the AL-FEC code be a MDS code. For simplicity each PDU is assumed to be sent in a separate transport block over the RAN air interface. Let N1=2, p=2%. Let the minimum reconstruction success probability be 0.995. Then the allowed values for the number of extra PDUs N3 that the RAN can drop must satisfy the following inequality:

n=Km-N3m-N3n(1-p)n pm-N3-n> 0.995, where m=N-N1-N2.

In the above, the summand gives the probability that n packets out of m-N3 packets are a success and the remaining m-N3-n packets are a failure. The probabilities are summed across all values of n which allow the application receiver to reconstruct all the source packets.

Figure 6.10.2.2-2: The RAN can drop up to N3=6 extra PDUs while still meeting the reconstruction probability threshold 0.995 with the packet losses in the upsteam, at the RAN (due to wireless link error, not dropped as extra PDUs), and in the downstream.

NOTE 3:	Although the solution presented above is for DL traffic, a similar solution can be devised for UL traffic, in which case the tethered link becomes upstream and the gNB to the AS segment (5G core network and N6 segment) becomes downstream.

The above assumes that the extra PDUs dropped at the RAN are not taken as signals of network congestion. Network congestion can occur in the upstream network segment, at the RAN or in the downstream network segment.

NOTE 4:	It is FFS under what conditions the RAN can drop extra PDUs considering network congestion in the upstream network segment, at the RAN or in the downstream network segment.

## 6.11	Solution #11: PSI indication to optimize RTP retransmission

### 6.11.1	Key Issue mapping

This solution addresses the key issue #8.

### 6.11.2	Description

Senders have the best view on which packets are sufficiently important to be retransmitted. Selective RTP retransmission prioritizes the packets that contain crucial data, such as keyframes in video streaming or important audio segments in voice calls.

In this solution, the sender signals a threshold/upper bound to the receiver indicating the range of PSI values assigned to the PDU Sets that are deemed critical for the application. The PSI threshold can be sent by the sender in an SDP negotiation, e.g., by means of a new SDP attribute “psi-thr” that can be set a value between 0 and 15 (inclusive).

By this signaling, the sender indicates that such PDU Sets are critical for the session and need to be retransmitted in case they are lost. For example, a PSI threshold of 8 means that the sender asks the receiver to consider requests retransmissions for the PDU Sets with PSI between 0 and 8 (inclusive).

The receiver uses the indicated PSI threshold to determine for which lost packets it will send retransmission requests (NACKs). Upon detection of a lost packet, the receiver inspects the PSI values of the received packets within the same PDU Set (i.e., have the same PSSN) and have RTP sequence numbers (SN) adjacent to the lost packet. From these PSI values, it can derive the PSI value of the lost packet. The receiver then sends a NACK, if the PSI value of the lost packet is lower than or equal to the PSI threshold.

NOTE 1:	While making retransmission requests, receivers may also consider other factors such as the possibility of timely arrival of requested packets, as described in TS 26.114, clause 9.3.2.

NOTE 2:	In case a PDU Set consists of a single PDU Set, the solution does not apply, since PSI cannot be inferred from the adjacent PDUs in the same PDU Set. However, for typical video applications and bitrates, it is expected that a PDU Set comprises multiple PDUs.

The sender may also consider other factors (e.g. network conditions, number of lost packets) to decide whether to retransmit packets falling into the indicated PSI range during the session.

NOTE 3:	This solution assumes that the PDU Set integrated handling is not used, i.e., the network does not discard the whole PDU Set when one PDU of a PDU Set is lost.

Figure 6.11.2-1 illustrates the solution with two example cases. The labels in the boxes show the PSN values of the PDUs with subscripts showing the PSI values for the respective PDU Sets.

In example 1, the PDU with RTP SN=14 (PSN=2, PSSN=3) is lost. When the receiver detects that loss, it can look at either the previous or the next PDU to infer the PSI since they are both in the PDU Set 3.

In example 2, the PDU with the RTP SN=15 (PSN=3, PSSN=3) is lost. In this case, the next PDU would not provide the correct PSI since it belongs to PDU Set 4, which has a different PSI value 9. The receiver can first inspect the PSSN value to check whether the next PDU is in a different PDU Set. If that is the case, it can instead use the value from the previous PDU with RTP SN=14 to obtain the correct PSI value 7 for the lost PDU.

Figure 6.11.2-1: Example cases illustrating the solution. Subscripts denote the PSI values.

Benefit of the solution: With guidance from the sender on the PSI range that is assigned to critical PDU Sets, the receiver can make more informed retransmission requests and improve bandwidth usage.

## 6.12	Solution #12: MID packet filtering

### 6.12.1	Key Issue mapping

This maps to Key Issue #9 and Key Issue #14.

### 6.12.2	Description

When multiple RTP streams are associated and multiplexed into a single RTP session, they can be grouped together called BUNDLE. IETF RFC 9143 [37] provides SDP BUNDLE framework using SDP offer/answer mechanism to negotiate which "m=" sections will become part of a BUNDLE group. In this framework, each "m=" section is associated with its identification-tag (the values of "mid" attribute) and a BUNDLE group is defined as the SDP 'group:BUNDLE' attribute having identification-tag list. All RTP-based media within a single BUNDLE group belong to a single RTP session. It means that all "m=" sections representing RTP-based media within a BUNDLE group share a single synchronization source (SSRC) numbering space. Additional rules and restrictions to be applied on that single RTP session are given in the section 9.1 of RFC 9143 [37].

IETF RFC 7941 [38] provides the RTP header extension method for the RTCP Source Description (SDES) items. The Figures 6.12.2-1 and 6.2.12-2 show the format of 1-byte and 2-byte extended headers, respectively. IETF RFC 9143 [37] defines the MID RTP HE registered by IANA in the "RTP SDES Compact Header Extensions" subregistry and carrying the identification-tag of the associated "m=" section. The MID RTP HE enables a receiver to associate each RTP stream with a specific "m=" section.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   |  len  | SDES item text value ...                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 6.12.2-1: One-byte Header extension format for SDES items

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      ID       |      len      |  SDES item text value ...     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 6.12.2-2: Two-byte Header extension format for SDES items

Figure 6.12.2-3 shows an example of SDP answer when the answerer accepts the creation of the bundle group. In this example, three RTP media streams are bundled together by announcing from "a=group:BUNDLE" and each of stream is identified by using "a=mid:XXX".

Figure 6.12.2-3: Example of SDP for bundled media streams

As addressed above, MID value which was exchanged in SDP negotiation can be inserted into the extended header of each RTP packet, it can be used to identify how each of RTP packets are associated with the RTP media stream. Figure 6.12.2-4 illustrates the QoS mapping structure using MID packet filter which maps and video streams (RTP stream 1 & 2) to QoS Flow 1 and an audio stream (RTP stream 3) to QoS Flow 2, respectively.

Figure 6.12.2-4: QoS flow mapping by MID packet filtering

Editor’s Note: TR 23.700-70 [6] has the interim agreements on the additional packet filter to identify each media flow. Therefore, it is desired to have a coordination with SA2 for normative work.

## 6.13	Solution #13: Dynamic traffic characteristics of RTP sender implementations

### 6.13.1	Key Issue mapping

This is a solution to KI #12 Enhancements of Data Burst Marking.

### 6.13.2	Description

#### 6.13.2.1	General

A data burst indicates a set of multiple PDUs generated and sent in a short period of time as defined in clause 3.1 of TS 23.501 [3]. Data burst is a common transmission characteristic in communication networks.

The source of traffic in this study is Real Time Transport Protocol as used for conversational or Extended Reality applications.

The data burst transmission could be beneficial for the 5GS network, e.g., power saving and efficient radio resource management. In Release 18, the End of Data Burst indication has been introduced to enable the UE power saving in the 5GS, i.e., the NG-RAN node can configure to move a UE into CDRX for power saving after transmitting the end PDU of the data burst. In Release 19, the data burst size has been concluded to enable the RAN radio resource management as described in clause 8.5 of TR 23.700-70 [6].

This contribution aims to observe dynamic traffic characteristics from popular RTP senders and possible implications for low latency and real-time communication for usage and support in 5G RTP.

In this study we explore:

-	The open source WebRTC implementation of RTP Sender in a browser implementation.

-	The cross platform GStreamer [39] implementation of RTP senders for peer-to-peer (P2P) transmission.

-	The sending and receiving measured of two popular real-time streaming servers.

#### 6.13.2.2	WebRTC and paced sender implementation

##### 6.13.2.2.1	WebRTC and paced sender implementation

WebRTC (Web Real-Time Communication) [28] is an open-source project providing web browsers and mobile applications with real-time communication (RTC) via application programming interfaces (APIs). It allows audio and video communication and streaming to work inside web pages by direct peer-to-peer communication, eliminating the need to install plugins.

It can be implemented in web browsers and is a likely source of real-time conversational RTP traffic in practice.

Based on the documentation available [28] from November 2021 in the WebRTC source code this clause describes the way packets may be generated and transmitted in a WebRTC environment.

WebRTC does not send out data packets generated by an encoder directly, instead a pacing module is used that limits and smoothens the packet train that is sent out to the network.

The idea is that, say for example, a 30 fps stream of 6 MBps is sent to the network, in the ideal case this would result in equally sized frames of around 25 kiloBytes in around 21 equally sized packets. While in practice, over a 1 second sliding window the average bit-rate of 6 Mbps might be measured, on a shorter timescale there may be periodic bursts that overshoot this average bit-rate with a much larger instantaneous bit-rate. One of the causes could be a sudden movement in a frame.

To avoid potential impact of these cases on the network and application, the WebRTC solution introduces the paced sender. The paced sender introduces a buffer in which the media packets are queued before being sent out. This avoids, for example, that large video frames are split into a large burst of packets that will disrupt other streams such as audio streams that are critical for intelligibility. WebRTC uses multiplexed RTP transport with different streams sent on the same port and IP tuple, thus potentially such an overshoot in video packets could stop other packets from being sent out in time as the socket may be blocked when transmitting the video packets.

NOTE:     The standard approach in RTP is to use different ports for different media streams, but in practice this could have some drawbacks as this increases resource usage and may complicate NAT traversal. In this case, the prioritization would depend on the operating system implementation. WebRTC allows multiplexing audio and video on the same port by establishing a relationship between the media lines included in the SDP using the attribute a=group:BUNDLE.

For sending out the media, a leaky bucket algorithm is used for pacing them on to the network.  In this implementation packets are inserted into a buffer first before being transmitted. The buffer contains separate first in first out (FIFO) queues for each media type/track, such that audio can for example be prioritized over video. Equal priority streams can be sent in round-robin fashion, avoiding these streams from blocking each another.

In a leaky bucket the rate of packets that is sent out is limited, potentially leading to a buffering of the input in case the input is larger than the output rate. A leaky bucket is one way to limit the burst sizes in traffic sent to the network.

In WebRTC this output rate is called the pacing_rate.

The typical way a packet is generated and paced on to the network in WebRTC is as follows:

1.	Media frame is packetized into RTP packets

2.	The packets are sent for scheduling (transmission)

3.	The pacer is called to enqueue the packet

4.	The packets stay in the queue until the pacer finds a scheduled moment to transmit them out based on the leaky bucket algorithm

5.	At the time that the packet is scheduled, the packet is forwarded to the RTP module for final timestamping

6.	The packet is sent on the low level interface (i.e. the UDP socket), and it is now out of scope of the WebRTC implementation

Asynchronous to this, the WebRTC implementation tries to estimate the send bandwidth in order to set the pacing_rate, i.e. the rate at which packets are sent to the network and secondly the padding_rate, the bit rate (if any) used to send padding in case no packets are available to be sent out. Figure 6.13.2.2.1-1 illustrates the WebRTC transmission flow.

Figure 6.13.2.2.1-1: Packet processing in WebRTC paced sender of an RTP packet

The pacer of WebRTC prioritizes based on different criteria:

a)	Packet Type, with most to least prioritized:

1.	Audio

2.	Retransmission

3.	Video and FEC

4.	Padding

b)	Based on the enqueuing order.

When the queues are empty the implementation will aim to generate padding frames and send them out at the padding_rate. In some cases the pacer_rate may be overridden and ignored, when there is a significant encoding overshoot for example. In addition WebRTC implementation enables setting the maximum time a packet can stay in a queue using a function (setQueueTimeLimit(TimeDelta limit)).

Bandwidth estimation in the WebRTC implementation is done by bandwidth probing, i.e. a cluster of packets is requested to be transmitted over the network to gauge if this will lead to increased delay and or loss. The implementation provides a function to do this measurement, enabling one to use this information to update the pacing_rate.

The WebRTC implementation has defined additional API function to monitor the states and statistics of the pacer.

##### 6.13.2.2.2	WebRTC and paced sender evaluation

To collect traffic characteristics, the following setup is deployed:

-	In this case a call is started on a Microsoft edge browser and Wireshark is used to collect the traffic trace. The machine initiating the call is a Lenovo laptop with 16 GB of RAM running windows 11 operating system.  The network is a wifi connection on 2.4 GHz band setup by a Samsung Galaxy A54 5G connected to a 5G network using wifi hotspot feature.

Figure 6.13.2.2.2-1 illustrates the traffic pattern from a WebRTC sender that joined a google hangout call on Machine A. This stream has a lower bit-rate as compared to the next experiment.  The traffic is clearly separated in burst that corresponds to the frame transmission.

Figure 6.13.2.2.2-1: Sending pattern from WebRTC in a Google hangout call with video only

In a second experiment, we use a machine running Ubuntu 24.4 with 8 GB or RAM and Intel core i5 processor. The Ubuntu machine/linux kernel enables throttling of the bandwidth and latency. The communication is via a wifi and a 5G hotspot.

Linux traffic control was used using a token bucket filter (tbf) to slow down the data rate to 200 kbit and only enable 16 kbit bursts. The results are shown in Figure 6.13.2.2.2-2. We see the magnitude of the peaks is reduced as the hangout video quality drops after the traffic control filter is enabled. However the general traffic characteristics look the same, there is no smoothening effect observed that may result from the paced sender. So, it seems in this case the general rate control of hangout for the video encoding reacted quick enough to avoid critical network operation.

Figure 6.13.2.2.2-2: Resulting traffic characteristics at sender when the network bandwidth is throttled to 200 kbit audio + video

#### 6.13.2.3	GStreamer multimedia framework RTP implementation for P2P

##### 6.13.2.3.1	GStreamer multimedia framework RTP implementation details

GStreamer [39] is a library for constructing graphs of media-handling components. It supports a range of applications from simple playback to streaming up to complex mixing and editing workflows.

GStreamer works on all major operating systems such as Solaris, Unix etc.

It enables graph-based pipeline construction and has a broad coverage of different multimedia technologies for codecs, protocols, file format encapsulations etc.

The GStreamer 1.x API has been stable since 2012 and the library is quite likely to be found as a sub-component in many practical media systems implementations, especially open-source implementations.

In addition, the design of light weight data passing implies high performance and low latency, very applicable to real-time communications.

Due to the stability and long track record and wide deployment of this library, some sample pipelines utilizing the RTP features are developed in this study for collecting details on the traffic characteristics of this framework in simple RTP workflows.

The setup uses a camera followed by an H.264 encoder tuned for ultra low latency and optimal speed settings that is fed to an RTP pay-loader linked to a network sink. The receiver receives the RTP packets, decodes the streams and renders it to screen enabling a 1-way conversational application.

##### 6.13.2.3.2	GStreamer RTP sender evaluation video

In this section the RTP video sender and receiver are evaluated.

To collect traffic characteristics, the following setup is used:

-	Two Machines A and B (Lenovo and HP, both AMD Ryzen 5 processor, 16GB RAM are connected over wifi network (2.4 GhZ) setup by a Galaxy A65 5G phone using hotspot function. Machine A runs the real time-frame capture, encoding packetization and UDP transmission (sender pipeline from the previous clause). Machine B receives the packets and depacketizes, decodes and renders the frame (receiver pipeline from the previous clause). Wireshark is used to collect the traffic data. In this setup a relatively high bit-rate of 16 MBps is used. GStreamer 1.24.5 is used for the experiment.

Figure 6.13.2.3.2-1 outlines the experimental setup.

Figure 6.13.2.3.2-1: Experimental setup flow

Figure 6.13.2.3.2-2 illustrates the sending data traffic pattern in bytes per millisecond on a logarithmic scale. It is clear that the traffic is bursty and that the bursts occur on the timespan of 1-3 milliseconds.

Figure 6.13.2.3.2-3 illustrates the receiving data traffic pattern in bytes per millisecond on a logarithmic scale. It is clear that the traffic bursty and that the burst occur in the timespan of 1-3 milliseconds, however compared to yy already a little bit of dispersion can be seen due to transmission and delays experienced in the transmission.

Table 6.13.2.3.2-4 demonstrates a snapshot from the data collected from the sender A, one interesting observation is that the inter burst time is not constant.  Between some burst there is approximately 20 milliseconds of idle period, while for other bursts the inter burst time is about 40-45 milliseconds.

Figure 6.13.2.3.2-2: Traffic pattern in GStreamer RTP sender on machine A

Figure 6.13.2.3.2-3: Traffic on machine B at the RTP receiver

Table 6.13.2.3.2-4: Snapshot of data from the sender, illustrating the inter-burst times

| packet number | time | source ip | receiver ip |  | length |
| --- | --- | --- | --- | --- | --- |
| 108 | 0.192814 | 192.168.178.229 | 192.168.178.110 | UDP | 766 |
| 109 | 0.192830 | 192.168.178.229 | 192.168.178.110 | UDP | 767 |
| 110 | 0.192846 | 192.168.178.229 | 192.168.178.110 | UDP | 845 |
| 111 | 0.192859 | 192.168.178.229 | 192.168.178.110 | UDP | 520 |
| 112 | 0.192872 | 192.168.178.229 | 192.168.178.110 | UDP | 448 |
| 113 | 0.210483 | 192.168.178.229 | 192.168.178.110 | UDP | 56 |
| 114 | 0.210584 | 192.168.178.229 | 192.168.178.110 | UDP | 565 |
| 115 | 0.210627 | 192.168.178.229 | 192.168.178.110 | UDP | 985 |
| 116 | 0.210655 | 192.168.178.229 | 192.168.178.110 | UDP | 944 |
| 117 | 0.210687 | 192.168.178.229 | 192.168.178.110 | UDP | 1110 |
| 118 | 0.210716 | 192.168.178.229 | 192.168.178.110 | UDP | 1027 |
| 119 | 0.210742 | 192.168.178.229 | 192.168.178.110 | UDP | 758 |
| 120 | 0.210770 | 192.168.178.229 | 192.168.178.110 | UDP | 722 |
| 121 | 0.256040 | 192.168.178.229 | 192.168.178.110 | UDP | 56 |
| 122 | 0.256087 | 192.168.178.229 | 192.168.178.110 | UDP | 317 |
| 123 | 0.256104 | 192.168.178.229 | 192.168.178.110 | UDP | 416 |
| 124 | 0.256125 | 192.168.178.229 | 192.168.178.110 | UDP | 584 |
| 125 | 0.256143 | 192.168.178.229 | 192.168.178.110 | UDP | 812 |

##### 6.13.2.3.3	Gstreamer RTSP sender/receiver evaluation PCM audio + video

In this section the RTP video + audio sender and receiver are evaluated.

The video is as in previous section, while the audio is transmitted as PCM audio.

To collect traffic characteristics, the following setup is used:

-	Two Machines A and B (Lenovo and Asus, first with AMD Ryzen 5 processor and second with Intel i5, 16 GB RAM/8 GB RAM are connected over wifi network (2.4 GHz) setup by a Galaxy A65 5G phone using hotspot function. Machine A runs the real time-frame capture, encoding packetization and UDP transmission (sender pipeline from the previous clause). Machine B receives the packets and depacketizes, decodes and renders the frame (receiver pipeline from the previous clause). Wireshark is used to collect the traffic data.

The results are shown in the figures below, the dynamic traffic characteristics bear similarity with those in the previous section, but the audio packets are much more frequent peaks with a smaller size.

Figure 6.13.2.3.3-1: Receiver audio + video

Figure 6.13.2.3.3-2: Sender audio + video

#### 6.13.2.4	Server-side senders

##### 6.13.2.4.1	Server sender implementation details

In this clause we consider server implementations of RTP senders using RTSP or RTP.

Both a popular commercial server and a popular open-source server implementation are evaluated.

The commercial server used for evaluation is Wowza Streaming Engine™ [40] (before Wowza Media Server™), one of the popular streaming servers that can support a wide range of transmission protocols. Wowza Streaming Engine™ is a Java application that runs as a service. The server can be used for a variety of live and on-demand streaming applications and it supports RTP/RTSP and low latency delivery.

The open-source server used in this evaluation is the popular Media MTX (before called simple RTSP server) [41], Media MTX is a real time media server/proxy that can support different protocols, including RTSP and RTP, but also WebRTC and RTMP [42].  The GitHub statistics on [41] show this as one of the most popular open source RTP/RTSP server implementations. It is possible, similar as in Wowza to ingest camera and other streams into the server for redistribution.

##### 6.13.2.4.2	Server side implementation evaluation

Wowza Streaming Engine™ version 4.8.27 was used deployed on a Lenovo laptop with 16 GB or RAM with AMD Ryzen 5. Another laptop with Intel i7-6600 2.6 GHz CPU and 16 GB of RAM is used to receive the stream from the server.  A camera stream is ingested over RTP to the Wowza Streaming Engine™ using a GStreamer pipeline similar as the P2P sender in the previous section. In Wowza Streaming Engine™ a predefined session description file is used to ingest the stream. The stream is distributed using RTP/RTSP. The machines are connected over a 2.4 GHz wifi network setup by a Galaxy A54 5G phone using the hotspot function. The receiver laptop uses VLC media player 3.0.8 for receiving the stream using RTSP and RTP.

Media MTX version 1.8.24 was used deployed on a Lenovo laptop with 16GB or RAM with AMD Ryzen 5. Another laptop Microsoft surface with Intel i7-6600 2.6 GHz CPU and 16GB of RAM was used to receive the stream.  A camera stream is ingested over RTP to the wowza streaming engine using gstreamer, instead of RTP + SDP in this case WebRTC + WHIP was used for the ingest, which automizes the session description exchange needed for the ingest. The stream is distributed by Media MTX engine using RTP/RTSP. The machines are connected over a 2.4 GHz wifi network setup by a Galaxy A54 5G phone using the hotspot function. The receiver laptop uses VLC media player 3.0.8 for receiving the stream over RTSP.

The flow of the experiment can be shown in Figure 6.13.2.4.2-1, GStreamer is used for the camera ingest to MTX Media or Wowza Streaming Engine™, then the data is transmitted to the client by the server, and Video Lan Client (VLC) is used to receive the stream and play it back. In this scenario RTSP + RTP is used.

Figure 6.13.2.4.2-1: Setup of the experiment using server

The results for the different configurations are plotted in Figures 6.13.2.4.2-2 - 6.13.2.4.2-6 collecting traffic measurements. Only short intervals of time are measured, and the measurements from sender and receiver do not correspond timewise. The main point of investigation are the dynamic traffic characteristics relating to the burst size and periodicity, in each of the setups the burstiness corresponding to frame transmissions can be observed. It seems the MTX media the times between the bursts vary a bit more compared to the Wowza setup.

From the experiments it seems the lower framerate mostly results from the camera capture, we have also run tests with an artificial test source generating content at higher framerate, this results in the higher burst frequency.

In Figure 6.13.2.4.2-6 an artificial video source is used to ingest to MTX with a higher framerate and quality. The results show that in this case the burst frequency of traffic transmitted by MTX media is also increased.

In addition, in 6.13.2.4.2-7 also shows when both audio (PCM) and video (H.264) are transmitted from MTX media server, in this case we see the audio bursts are more frequent and smaller in size compared to the video burst.

Figure 6.13.2.4.2-2: Wowza server side traffic sending to a VLC receiver using RTSP/RTP

Figure 6.13.2.4.2-3: VLC receiver side traffic from received RTP packets from Wowza Streaming Engine™

Figure 6.13.2.4.2-4: Camera RTP Traffic monitored from sender from MTX Media

Figure 6.13.2.4.2-5: Camera RTP traffic monitored at VLC receiver from MTX Media

Figure 6.13.2.4.2-6: Synthetic test source RTP Traffic monitored from sender from MTX Media server

Figure 6.13.2.4.2-7: Traffic pattern from MTX sender server audio + video

### 6.13.3	Discussion and conclusion

Observation 1:

The data from the GStreamer RTP and from WebRTC browser implementation show bursts of data being transmitted in short time intervals. There is no evidence that the encoder/packager gradually produces and transmits packets, as in the experiments only short bursts were observed. It could happen on a per frame(s) basis in a short time interval. It may also be that some frames are combined in a single burst, but this cannot be observed from the current results.

Similar observations were made when an intermediate server was used for redistributing the streams.

It should therefore be possible for these applications to calculate the burst size a priori before sending it out. The added latency should be limited to around a few milliseconds even in the worst case.

Bursts related to audio are more frequent than bursts related to video.

NOTE 1: 	The latency is related to sending the packets on the network, not to the encoder or RTP packager generating the packets. Detailed study of the cause of the delay is FFS. The preliminary conclusion is that a few milliseconds of delay may be introduced is only a worst-case estimate.

Observation 2:

The inter-burst time interval seems regular, but not constant. Therefore, signalling of the time to next burst, if known by the application, may be suitable for signalling as a dynamic traffic characteristic.

NOTE 2:	This requires more study of different patterns and situations.

Observation 3:

It can be derived that when the network is not overloaded the WebRTC and GStreamer implementations do not differ too much and the influence of the paced sender module is limited.

In addition, when the network bandwidth changes all of a sudden, the rate control may adapt so well that the effect of the paced sender is not significant.

Observation 4 (extra on P2P versus Server based):

When comparing the end-end latency of the peer-to-peer setups to the server based setups, for the peer to peer setups latencies < 1 seconds are achieved while in the server routing setup latencies of around 3 seconds are observed.

Conclusion:

Short periodic traffic bursts in short intervals occur in typical real time conversational applications using real-time video + audio.

Given the observed traffic behaviour and the observed application behaviour it seems achievable to include information about a burst size before it is being sent out as the durations of the burst are in the order of 1-2 milliseconds, but it may require some changes in the sender implementation to achieve this.

### 6.13.4	Proposal

Given the observed behaviour of common RTP Sender implementations, it is recommended to extend the signalling of such dynamic traffic characteristics in Release 19 in TS 26.522 [2], also considering the option to signal the data burst size and or time to next burst in the first packet as requested by RAN. The exact signalling or additions are complementary to this solution, the main goal of this solution was to address the questions raised in the KI description:

-	Identify additional traffic characteristics beneficial to the 5GS network, for example, time to next burst, burst size and other potentially relevant characteristics.

-    Identify and document the way RTP senders can generate data bursts e.g.

- WebRTC paced sending implementation, including the different configurations of WebRTC paced sending.

- Other common RTP implementations or libraries that are commonly used.

## 6.14	Solution #14: Candidate RTCP messages and RTP header extensions to support XR services in 5G

### 6.14.1	Key Issue mapping

This maps to Key Issue #7.

### 6.14.2	Description

The following RTCP messages and RTP header extensions are relevant for XR services.

#### 6.14.2.1	RTCP messages

To understand the RTCP messages that may be used for supporting XR applications, we need to know what RTCP messages have been defined in standard specifications such as IETF RFCs and what RTCP messages have been used in commercial systems. For this purpose, we look at the WebRTC implementation [28] and IETF RFCs.

RTCP messages defined in RFCs:

-	Receiver report (RR) (RFC 3550 [44]): The packet type (PT) is 201. It provides reception quality feedback to the other RTP endpoint on a per source SSRC basis. Among the reported information is

1)	the fraction lost: The fraction of RTP data packets from a source SSRC lost since the previous SR or RR packet was sent

2)	cumulative number of packets lost: The total number of RTP data packets from a source SSRC that have been lost since the beginning of reception

3)	interarrival jitter: An estimate of the statistical variance of the RTP data packet interarrival time

-	Sender report (SR) (RFC 3550 [44]): The packet type (PT) is 200. It is the same as the RR except that it carries additional 20 bytes of information about the RTP endpoint that originates this report.

-	Application-Defined Packet (APP) (RFC 3550 [44]): The PT is 204. It is is intended for experimental use as new applications and new features are developed, without requiring packet type value registration.

-	Feedback messages:

1)	Transport layer FeedBack messages (RTPFB): The PT value is 205.

a)	Generic NACK (RFC 4585 [33]): The PT value is 205 and the format type (FMT) value is 1. The Generic NACK is used to indicate RTP packet losses, identified by the means of a packet identifier and a bit mask.

b)	TMMBR (RFC 5104 [45]): The PT value is 205 and the FMT value is 3. The Temporary Maximum Media Stream Bit Rate Request message is used to notify the media sender about the changes in downlink bandwidth allocation with a new current maximum bitrate.

c)	TMMBN (RFC 5104 [45]): The PT value is 205 and the FMT value is 4. The Temporary Maximum Media Stream Bit Rate Notification message is used to notify the media receiver about the adjusted new media bitrate by a media sender. This message is sent in response to a received TMMBR message.

d)	RTP-ECN-FB (RFC 6679 [46]): The PT value is 205 and the FMT value is 8. The RTP Explicit Congestion Notification (ECN) feedback message reports reception of an ECN-CE-marked RTP packet so that the RTP sender may perform congestion control.

e)	RTP Congestion Control Feedback (CCFB) (RFC 8888 [47]): The PT value is 205 and the FMT value is 11. The congestion control feedback message is used to transmit the status of each RTP packet received at the RTP receiver along with the RTP sequence number, packet arrival time and packet Explicit Congestion Notification (ECN) Marking. This message specifies a common RTCP feedback packet format that can be used by Network-Assisted Dynamic Adaptation (NADA) (RFC 8698 [29]), Self-Clocked Rate Adaptation for Multimedia (SCReAM) (RFC 8298 [22]), Google Congestion Control [27], and Shared Bottleneck Detection (RFC 8382 [48]) congestion control algorithms.
The format and semantics of the RTCP CCFB packet is as shown below.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|V=2|P| FMT=11  |   PT = 205    |          length               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                 SSRC of RTCP packet sender                    |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   SSRC of 1st RTP Stream                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|          begin_seq            |          num_reports          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|R|ECN|  Arrival time offset    | ...                           .

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

.                                                               .

.                                                               .

.                                                               .

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   SSRC of nth RTP Stream                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|          begin_seq            |          num_reports          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|R|ECN|  Arrival time offset    | ...                           |

.                                                               .

.                                                               .

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                 Report Timestamp (32 bits)                    |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

-	begin_seq [16 bits]: The first sequence number that this feedback message reports on.

-	Received (R) [1 bit]:  A boolean that indicates whether the packet was received.  0 indicates that the packet was not yet received and the subsequent 15 bits (ECN and ATO) in this 16-bit packet metric block are also set to 0 and MUST be ignored. 1 indicates that the packet was received and the subsequent bits in the block need to be parsed.

-	num_reports [16 bits]: The number of RTP packets reported in this feedback message. The report block contains a 16-bit packet metric block for each RTP packet that has a sequence number in the range begin_seq to begin_seq+num_reports inclusive.

-	ECN [2 bits]: The echoed ECN mark of the packet. These bits are set to 00 if not received or if ECN is not used.

-	Arrival time offset (ATO) [13 bits]: The arrival time of the RTP packet at the receiver, as an offset before the time represented by the Report Timestamp (RTS) field of this RTCP congestion control feedback report. The ATO field is in units of 1/1024 seconds (this unit is chosen to give exact offsets from the RTS field) so, for example, an ATO value of 512 indicates that the corresponding RTP packet arrived exactly half a second before the time instant represented by the RTS field.

-	Report Timestamp (RTS) [32 bits]: This value denotes the time instant on which this packet is reporting and is the instant from which the arrival time offset values are calculated. The value of the RTS field is derived from the same clock used to generate the NTP timestamp field in RTCP Sender Report (SR) packets.

f)	Transport-wide feedback [43]: The PT value is 205 and the FMT value is 15. This feeds back information about each packet received with a transport-wide packet sequence number.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|V=2|P|  FMT=15 |    PT=205     |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

0 |                     SSRC of packet sender                     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

4 |                      SSRC of media source                     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8 |      base sequence number     |      packet status count      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

12 |                 reference time                | fb pkt. count |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

16 |          packet chunk         |         packet chunk          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

.                                                               .

.                                                               .

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|         packet chunk          |  recv delta   |  recv delta   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

.                                                               .

.                                                               .

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|           recv delta          |  recv delta   | zero padding  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

-	base sequence number:  The transport-wide sequence number of the first packet in this feedback.

-	reference time: it indicates an absolute reference time in some (unknown) time base chosen by the sender of the feedback packets. The first recv delta in this packet is relative to the reference time.

-	packet chunk: A list of packet status chunks, indicating the status of one or more packets starting with the one identified by base sequence number.

-	recv delta: it represents a time interval in units of 0.25ms for a packet indicated in the packet chunk relative to the reference time.

2)	Payload-specific FeedBack Messages (PSFB): The PT value is 206.

a)	Picture Loss Indication (PLI) (RFC 4585 [33]): The PT is 206 and the FMT is 1. It indicates the loss of an undefined amount of coded video data belonging to one or more pictures.

b)	Full Intra Request (FIR) (RFC 5104 [45]): The PT value is 206 and the FMT value is 4. The FIR indicates the request of a full intra or Instantaneous Decoder Refresh picture from the media sender.

c)	Region of Interest (ROI) (TS 26.114 [32]): The PT value is 206 and the FMT value is 9. The ROI message indicates the request by the media receiver for an interested region of the media.

d)	Viewport (VP) (TS 26.114 [32]): The PT value is 206 and the FMT value is 11. The ROI message indicates the request by the media receiver for a region of the media in an interested viewport.

-	RTCP XR Reports (RFC 3611 [49]): The RTCP XR report is identified by PT equal to 207, which refers to an extended report block message. The block type (BT) field defined in RFC 3611 is used to identify the block format.

#### 6.14.2.2	RTP header extensions

For application bitrate adaptation and congestion control, it is important for the network to understand the state of the network, i.e., whether the network is in congestion or not. Many congestion control algorithms, e.g., Google congestion control algorithms [28], NADA [29] and SCReAMv2 [30], use the queueing delay as a signal of network congestion. Therefore, it is important to measure the delays and make the measurements available to the RTP sender in an efficient manner.

TS 26.522 [2] defined two RTP header extensions for in-band end-to-end delay measurement. The first RTP header extension that carries only one timestamp, also known as the "Absolute Sender Time" RTP header extension, is already implemented in WebRTC [28]. The second RTP header extension that carries three timestamps returns the measured one-way delay in the direction from the sender to the receiver back to the sender. The current implementation in WebRTC uses RTCP messages to carry the one-way delay back to the sender and that may introduce large delay due to the RTCP bandwidth limitation or large overhead due to the additional IP/UDP packet headers for a separate packet.

TS 26.522 [2] defines an RTP header extension for XR Pose which can be used for signaling either a 6DoF or 3DoF XR pose. The HE can be used either by an RTP receiver to indicate the XR pose used for rendering the media (rendered pose) or by an RTP sender to indicate to an RTP receiver the XR pose to be rendered.

NOTE:	Other RTCP messages and RTCP header extensions in 3GPP TS 26.522 and IETF RFCs may be also considered.

## 6.15	Solution #15: PSI signaling for lone PDUs

### 6.15.1	Key Issue mapping

This solution addresses the key issue #2.

### 6.15.2	Description

#### 6.15.2.1	Background

As of Rel-18, there is no mechanism to mark PDUs carrying protocol data other than RTP. Thus, PDUs belonging to protocols such as RTCP, STUN, etc. cannot be marked.

In Rel-18, SA2 has agreed that the PSA UPF marks, in the downlink, each N6-unmarked PDU ("lone PDU") with PDU Set Information into a PDU Set. If the UPF receives a PDU that does not belong to a PDU Set based on Protocol Description for PDU Set identification, the UPF still maps it to a PDU Set and determines the PDU Set Information by implementation-specific means.

NOTE:	The solution to KI#4 in TR 23.700-70 [6] enables the network to differentiate multiplexed streams sent in the same media transport such that they can be mapped into distinct QoS flows. However, in some cases this may result in unintended behavior, e.g. RTCP packets mapped to a different QoS flow would no longer measure the RTP media QoS flow characteristics which may result in errors e.g. in measuring the roundtrip delay using RTCP sender/receiver reports. On the other hand, it could be problematic to apply the PDU Set QoS to lone PDUs, as described in the solution #2 in clause 6.2. How to balance this trade-off is FFS.

Observation 1: PDUs of non-RTP protocols (e.g. RTCP) are mapped by the UPF into PDU Sets. The associated PDU Set Information is determined by the UPF.

When the PDU Set Information is not provided by the sender in an RTP HE, the UPF may be able to reliably obtain some parts of the PDU Set Information based on the UPF implementation. Annex A of TS 26.522 [2] describes how a Network Function can obtain the PDU Set Information from the RTP header and RTP payload, respectively.

When the RTP HE for PDU Set marking is not available, the UPF may derive some parts of the PDU Set Information from the (S)RTP header, as described in clause A.2.1 of TS 26.522. However, PDU Set Importance (PSI) cannot be obtained and PDU Set Size (PSSize) can only be obtained after the reception of the last PDU of the PDU Set. Also, the derivation in clause A.2.1 only applies for the case when a PDU Set is a video frame. It is not applicable to e.g. audio PDU Sets or PDU Sets that are video slices.

When the RTP payload is not encrypted (i.e., SRTP is not used), the UPF may derive some parts of the PDU Set Information from the RTP payload. Clause A.2.2 of TS 26.522 describes how this can be done for RTP payloads carrying H.264/AVC and H.265/HEVC coded bitstreams. In summary, the UPF needs to parse the NAL unit header, which is the first one and two byte(s) of the RTP payload for H.264/AVC and H.265/HEVC, respectively. Similar to the derivation from the RTP header, the PSSize can only be obtained after the reception of all PDUs of the PDU Set. PSI can be obtained by using the same logic the RTP sender uses to populate the RTP HE for PDU Set marking (i.e., parsing the NAL unit header), as described in the relevant guidelines in clause 4.2.6.2 of TS 26.522. However, this means more operations and thus more processing load for the UPF since it would need to check one or more of the NAL unit header fields. This is not feasible considering that the UPF processes data from several endpoints simultaneously under tight latency constraints.

Observation 2: For RTP PDUs, if the RTP HE for PDU Set marking is not present, the UPF may derive some parts of the PDU Set Information from the RTP header or RTP payload (if unencrypted), albeit with some restrictions. In particular, PSI cannot be obtained from the RTP header, and deriving PSI from the RTP payload imposes a significant processing overhead for the UPF given its high processing load and tight latency constraints.

Signaling the PDU Set Information in an RTP HE or deriving it from the RTP header/payload is not possible for PDUs carrying protocol data other than RTP (e.g. RTCP). Such PDUs are considered as lone PDUs by the UPF and placed into a PDU Set in a way that is determined by the UPF implementation. The UPF also has to define the PDU Set Information for the PDU Sets containing the lone PDUs. For some parts of the PDU Set Information (PSN, PSSN, End PDU), this operation is straightforward.

For example, consider an RTCP PDU that is placed by the UPF into its own PDU Set (i.e., the PDU Set contains only that PDU). It is described below whether/how the PDU Set Information can be obtained for that PDU Set.

-	Number of PDUs in the PDU Set (NPDS) is set to 1.

-	PSN is set to 0 and the End PDU flag is set to 1 (since NPDS=1).

-	PSSN is trickier since it depends on the transmission order of the PDU Set by the sender and is monotonically increased by the sender by 1 for each subsequent PDU Set. Since there is no gap between the PSSNs assigned by the sender for the RTP PDUs, the UPF would have to either assign an existing PSSN (i.e., a PSSN that is already used by another PDU Set) to the new PDU Set that contains the lone PDU, or a predetermined value that indicates that PSSN is undefined for that PDU Set.

-	PSSize is equal to the size of the RTCP packet since that packet would be the only PDU in the PDU Set.

-	To determine the PSI, the UPF must resort to preconfiguration (i.e. use a pre-defined value) since it has no means to obtain the PSI from the packet header or payload for non-RTP PDUs.

Any default PSI setting at the UPF may not be accurate since the importance of different PDU Sets is application- and codec-dependent. For example, some RTCP message types may be considered more important for low latency applications. In another example, RTCP feedback messages for viewport signaling may be crucial for the functionality of an immersive application, and thus it would be beneficial to be able to indicate a low PSI value for the RTCP packets carrying those.

Editor’s note:	Other potential benefits and limitations of PDU Set handling for non-RTP protocol types (e.g. RTCP, STUN) is FFS.

Observation 3: For lone PDUs, some parts of the PDU Set Information must be determined by the UPF. However, the UPF cannot reliably determine the PSI and may only assign a pre-defined PSI value (e.g. by the network operator). Sender applications are in the best position to determine the PSI.

Editor’s note:	Whether it is questionable that the UPF can reliably determine PDU Set Information for lone PDUs based on local configuration needs to be verified with SA2.

#### 6.15.2.2	Solution description

In this solution, the Media Application Provider defines a mapping between a set of PSI values and non-RTP protocols that are used in the media delivery session.

The mapping can be provided using the RTC provisioning feature (TS 26.510 [50], clause 5.2.10) of the media delivery session. In an example implementation, the Media Application Provider adds a property lonePduInfoList to the RTCConfiguration resource. The property lonePduInfoList contains an array of lonePduInfo objects as defined below.

Table 6.15.2.2-1: Definition of lonePduInfo object

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| protocol | string | 1..1 | Protocol information such as RTCP, STUN, etc. |
| packetType | integer | 0..1 | Packet type specific to the protocol. |
| feedbackMsgType | integer | 0..1 | RTCP feedback messages type [RFC 4585]. Can only be present if the protocol is RTCP. |
| pduSetImportance | integer | 1..1 | PSI value between 0 and 15 (inclusive). |

If a lonePduInfoList is provided, the Media AF extends the mediaTransport‌Parameters property of the Application Flow Description that it has received from the Media Session Handler with the information in the lonePduInfoList. The Media AF then sends the Application Flow Description to the 5G Core, where the mediaTransport‌Parameters (as Protocol Description) is passed to the UPF. For example, the Protocol Description may then have the following structure after addition of the property lonePduInfoList:

{ "transportProto": "RTP",

"rtpPayloadInfoList": [{"rtpPayloadFormat": "H265", "rtpPayloadTypeList": [96]}],

“lonePduInfoList:” [{"protocol": “RTCP”, "packetType": 206, “feedbackMsgType”: X, psi: 2},

{"protocol": “RTCP”, "packetType": 207, psi: 10}],

}

In the example, the first lonePduInfo object provides the PSI mapping for RTCP packets that contain viewport feedback messages TS 26.114 [32], clause Y.7.2. The RTCP feedback message is identified by Packet Type = 206, which refers to payload-specific feedback message (RFC 4585 [33]). FMT (feedback message type) is set to the value ‘11’ for viewport feedback messages. The second lonePduInfo object provides the PSI mapping for RTCP Extended Reports (XR) messages identified by Packet Type = 207 (RFC 3611 [49]). In this example, the Media Application Provider chooses to assign PSI=2 to viewport feedback messages and PSI=10 to RTCP XR messages.

NOTE:	There may be PDUs carrying other protocol data such as STUN in the media delivery session. Whether/how a PSI mapping should be provided for such PDUs is FFS.

After receiving the Protocol Description, the UPF can determine the PSI for lone PDUs based on the information provided in each lonePduInfo object.

Editor’s note:	Potential benefits and limitations of PDU Set handling for media streams that do not require both high bitrate and low latency (e.g., audio, haptics) are FFS and will be documented under KI#13.

#### 6.15.2.3	Analysis of the solution

The benefit of the proposed solution is that the UPF does not have to rely on a pre-defined value (e.g. provided by the network operator) to determine the PSI for lone PDUs and can make a more reliable decision based on a PSI mapping provided by the Media Application Provider.

In terms of UPF processing, complexity is not increased because the UPF only needs to inspect the packet headers (e.g. the RTCP header) to check for packet type.

Benefits of the solution are summarized below.

-	More dynamic solution as it allows applications to set PSI value for specific unmarked traffic instead of using a fixed value for all applications pre-configured at the UPF.

-	More reliable as applications are in a better position to determine the importance of PDU Sets.

-	Reuses the RTCConfiguration mechanism. Requires the UPF to determine the PSI value for lone PDUs at the start of session based on RTCConfiguration.

-	Extensible to other types of unmarked packets in the future.

The impacted entities in the 5G System are:

Real-time Media Provisioning API:

-	RTCConfiguration provided by the Media Application Provider is extended with the lone PDU information.

Media AF:

-	Receives the extended RTCConfiguration and adds it to the Application Flow Description.

UPF:

-	Receives the extended Protocol Description and parses the lonePduInfoList property to retrieve the PSI mapping for lone PDUs.

Editor’s Note:	This solution requires coordination with SA2.

## 6.16	Solution #16: RTP header extension for dynamic traffic characteristics

### 6.16.1	Key Issue mapping

This is a solution to KI #12 Enhancements of Data Burst Marking.

### 6.16.2	Description

A data burst indicates a set of multiple PDUs generated and sent in a short period of time as defined in clause 3.1 of TS 23.501 [3]. Data burst is a common transmission characteristic in communication networks.

The traffic characteristics regarding the data burst transmission could be beneficial for the 5GS network, e.g., power saving and efficient radio resource management. In Release 18, the End of Data Burst indication has been introduced to enable the UE power saving in the 5GS, i.e., the NG-RAN node can configure to move a UE into CDRX for power saving after transmitting the end PDU of the data burst. In Release 19, the data burst size has been concluded to enable the RAN radio resource management as described in clause 8.5 of TR 23.700-70 [6].

For marking dynamic traffic characteristics, the RTP HE for Dynamic Traffic Characteristics is defined in this clause.

A new RTP HE is proposed for the following reasons:

1)	Avoiding compatibility issues with release 18 Header Extension for PDU Set marking.

2)	Avoiding overhead, RTP HE need not be present in each RTP packet, but for Release 18 Header Extension it is common understanding that usually each RTP packet is marked. The information for dynamic traffic characteristics on the other hand is specifically useful in specific packets at the beginning of a traffic pattern.

Dynamic Traffic Characteristics marking can be performed by an RTP sender, such as an Application Server (e.g., MRF), a sender UE that sends media to an RTP receiver, such as a UE, or other 5G network components.

Endpoints that support the RTP HE for Dynamic Traffic Characteristics can support both RTP HE formats (i.e., the one-byte and the two-byte formats) according to RFC 8285 [7].

If the RTP HE for Dynamic Traffic Characteristics is the only RTP HE used, the endpoints can use the 1-byte header format. If other 2-byte RTP HE elements are used in the same RTP stream, then the 2-byte header can be used, unless the "a=extmap-allow-mixed" is successfully negotiated through SDP offer/answer, as described by RFC 8285 [7].

NOTE:	The headers are not shown with padding as this depends on other prospective extension elements in use, as per RFC 8285 [7] alignment specifications.

The IANA registration information for the RTP HE for RTP HE for Dynamic Traffic Characteristics is in 6.16.8.

#### 6.16.2.1	Intended usage in 5GS

The solution of adding dynamic traffic characteristics serves the following key use case:

1)	Based on the SDP negotiation, the RTP HE for Dynamic Traffic Characteristics is enabled. The RTP Sender or Application Server adds header extension of a dynamic traffic characteristic in the first few packets of a data burst.

2)	The dynamic traffic characteristics header is added by the packet sender potentially for groups of packets to be sent, this may include multiplexed RTP or Multiplexed RTP/RTCP traffic. The sender, may add information such as the Burst Size of the group of packets to be transmitted, or based on its own internal scheduling the time until the next burst can be sent.

3)	The UPF detects packets that include the Header Extension for Dynamic traffic characteristics and marks the dynamic traffic characteristics into the GTP-U header of downlink packets, including the End of Data Burst indication and data burst size.

4)	As concluded in clause 8.5 of TR 23.700-70 [6], the data burst size carried in the RTP HE can be identified by the UPF and then further sent to the NG-RAN via the GTP-U header to assist the radio resource management. The procedure is as follows:

a)	The (RTC-)AF may provision the Protocol Description to PCF directly over N5 interface or via NEF over N33. The Protocol Description indicates that the RTP HE for Dynamic Traffic Characteristics is enabled.

b)	PCF may provision the Protocol Description within the PCC rules based on the information provided by the AF and/or the local operator policies.

c)	SMF requests the UPF to detect and mark the burst size of the data burst and mark itin the GTP-U header of the first few PDUs in downlink, according to the PCC rule and/or the local operator policies.

d)	UPF identifies the burst size of a data burst in the downlink traffic based on the RTP HE according to the Protocol Description and provides the data burst size to the RAN in the GTP-U header of the first few PDUs of a data burst to assist radio resource management.

e)	RAN efficiently optimizes the radio resource for the timely data burst transmission based on the data burst size in the GTP-U header.

Editor’s Note:	Whether other dynamic traffic characteristics are needed depends on further conclusion of SA2 and RAN WGs.

#### 6.16.2.3	One-byte RTP header extension format

The one-byte RTP HE for the marking of PDU Sets and End of Bursts is defined as follows:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | len   | R   |D|  RR   |           [TCIN]             |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                     BSSize                    |     TTNB

+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

|

+.+.+.+.+.+.+.+.+

#### 6.16.2.4	Two-byte RTP Header Extension Format

The two-byte RTP HE for the marking of PDU Sets and End of Bursts is defined as follows:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|         0x100         |appbits|           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      ID       |      len      |  R  |D|  RR   |        [TCIN]

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   BSSize                      |

+-+-+-+-+-+-+-+-+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

|             TTNB              |

+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+

#### 6.16.2.5	Semantics

The semantics of the fields of the RTP HE for PDU Set marking are defined as follows:

-	End of Data Burst [D] (1 bit): This field is a flag that can be set to 1 for the last PDU of a Data Burst. It can be set to 0 for all other PDUs. A Data Burst may consist of one or more PDU Sets.

-	Reserved [R] 3 bits): This field is reserved for future usage (e.g., dynamic burst indication). It can be set to 0 by the RTP sender and can be ignored by the RTP receiver.

-	Reserved [RR] 4 bits): This field is reserved for future usage (e.g., dynamic burst indication). It can be set to 0 by the RTP sender and can be ignored by the RTP receiver.

-	[Traffic Characteristics Identifier Number [TCIN] (16 bits): A pseudo random number or a monotonously increasing number to indicate the Traffic characteristics signalling, this enables the receiver to distinguish different traffic characteristics signalling and identify repeated dynamic traffic identification signalling (if present). The main goal of this identifier is to enable identification of different traffic characteristics signalings.

NOTE 1:	TCIN may be helpful to enable identification of packets belonging to a burst in case of out of order delivery on N6. Its utility and applicability are FFS.]

Editor’s note: The above bracketed text is tentatively agreed.

-	Burst Size [BSSize] (24 bits): The Burst Size indicates the total size of the burst to be transmitted. The burst size corresponds to the size of the data burst [corresponding to the TCIN] If the burst size is not known it is set to 0.

NOTE 2:	If a packager generates all packets of the burst at once, no additional delay is introduced when setting the burst size, as the packets can be marked with the complete burst size. If this is not the case a delay as large as the burst duration could be introduced by marking the entire burst. Therefore, this approach may not be suitable for all types of packagers/encoders, especially those that gradually produce packets additional latency may be introduced if the size is not known in advance.

-	Time To Next Burst [TTNB] (16 bits): Indicates the approximate time (within a 1-5 millisecond range) to the next burst in milliseconds. If the time to next burst is not known it is set to 0. This time is relative to the time of the current burst that is the send time in milliseconds of the current burst, taking the packet in the middle of the burst as reference.

NOTE 3:	Inaccuracy on the TTNB may occur due to different reasons such as re-ordering or unknown, this number is indicative and can be accurate within 1-5 ms range. An addition solution will be provided to show some typical XR traffic and related burst showing that for Real Time A/V traffic bursts of 1-2 milliseconds can be common.

NOTE 4:	This solution has some overlap with the solution for Data burst marking in R18 and PDU Set marking, more discussion is needed on the benefits. As PDU Set marking requires marking each packet while traffic characteristics marking does not, this separate solution is proposed.

NOTE 5:	The introduction of this header extension may need some alignment with other working groups such as SA2 and/or RAN2. Specifically, it should be checked with SA2 and RAN2 if the timing information should be in the first or last packet.

NOTE 6:	The anchor time of the Time to Next burst is expected to be for further study in case the solution is adopted as a basis for normative work.

NOTE 7:	Additional optional fields of this Header Extension are for further study.

NOTE 8:	The layout of the dynamic traffic characteristics header extension mimics the RTP Header for PDU set marking enabling re-using of parsing mechanisms.

#### 6.16.2.6	SDP Signaling

An RTP sender capable of sending RTP HE for Dynamic Traffic Characteristics can use the SDP extmap attribute for RTP HE for RTP HE for Dynamic Traffic Characteristics in the media description of the RTP stream(s) carrying the RTP HE for RTP HE for Dynamic Traffic Characteristics. An RTP receiver that does not support RTP HE for Dynamic Traffic Characteristics can ignore that RTP HE when included. The signaling of the Dynamic Traffic Characteristics RTP HE can follow the SDP signaling design and the syntax and semantics of the "extmap" attribute as outlined in RFC 8285.The URN for the PDU Set marking can be set to "urn:3gpp:dynamic-traffic-characteristics:rel-19".

The ABNF syntax for the extmap attribute for the signaling of RTP HE for PDU Set marking is defined as follows, extending the ABNF in RFC 8285:

extensionname = "urn:3gpp:dynamic-traffic-characteristics:rel-19"

format = "short" / "long"

The extension attributes have the following semantics:

-	format: indicates if the RTP HE for Dynamic Traffic Characteristics uses the 1-byte (short) or the 2-byte (long) format. This extension attribute can not be included more than once.

NOTE:	Regardless if this extension attribute is present or not, the use of long or short format is determined as described by section 4.1.2 of RFC 8285, i.e., based on what format other RTP HEs use in the same RTP session, unless both endpoints announced support for handling mixed format with "a=extmap-allow-mixed" as described by section 6 of RFC 8285 [7].

Below is an example:

a=extmap:7 dynamic-traffic-characteristics:rel-19 long

#### 6.16.2.7	Guidelines for dynamic traffic characteristics signaling

It is recommended that the first several RTP packets and the last packets contain the dynamic traffic characteristics traffic signalling. In addition, some additional RTP packets may contain the RTP Header Extension for dynamic traffic characteristics.

It is recommended that the application signals the presence of RTP HE for dynamic traffic characteristics out of band using SDP signalling as defined in 6.16.2.6.

In addition, dynamic traffic characteristics can only be used if the generated data can be marked for such characteristics, i.e. it contains burst and potentially some periodicity information or it knows the timing to a next data burst.

A sender, that is scheduling to send out a group of packets, may calculate the size of the group of the packets, and then add the overhead of adding the RTP Header and then update the packets to include the RTP Header Extension for dynamic traffic characteristics.

#### 6.16.2.8	Proposed Annex D.3

The desired extension naming URI:

urn:3gpp:dynamic-traffic-characteristics:rel-19

A formal reference to the publicly available specification:

[TS 26.522]

A short phrase describing the function of the extension:

Marking of dynamic traffic characteristics such as burst size and time to next burst

Contact information for the organization or person making the registration:

3GPP Specifications Manager

3gppContact@etsi.org

+33 (0)492944200

#### 6.16.2.9	Discussion of the solution

This solution presents a way forward to enable dynamic traffic characteristics signalling in release 19. The main advantages are that:

1)	Backward compatibility is achieved by not changing the Release 18 HE for PDU Set marking. The default behaviour is to ignore unknown RTP headers. By not changing the release 18 RTP HE for PDU Set marking this can still be used the same way. In case efficient data burst marking is needed in the first packet as requested by RAN2 the current solution can be used.

2)	Sparsity and reduced overhead are achieved as recommended in RFC 8285. Not every RTP packet has to carry the Header Extension for Dynamic Traffic Characteristics.

3)	Can be implemented separately and independently from the PDU Set marking Header Extension, a sender or scheduler that sends out a group of packets in a burst such as multiple PDU Sets, can apply marking for the traffic characteristics.

4)	Well defined usage within the 5G System context is documented.

NOTE:	How to combine bursts sent over different IP Tuples is for Further Study.

## 6.17	Solution #17: Analysis of AL-FEC awareness in 3GPP

### 6.17.1	Key Issue mapping

This solution addresses key issue #4 on AL-FEC awareness for PDU Set handling and Key Issue #3: Enhancements for application-layer FEC support.

### 6.17.2	Description

#### 6.17.2.1	Common attributes of AL-FEC deployments

AL-FEC codecs, as the schemes documented in Solution #5, are used in ensuring low latency media delivery for networks with bursty losses and RTT delays comparable or higher than the jitter buffer delay constraints of an application. AL-FEC is not an exclusive packet loss mechanism and may be used interchangeably with retransmissions (e.g., as documented in Solution #8 in section 6.8 as media source bit rate adaptation or other flow control mechanisms. AL-FEC encoding is in fact utilized in practice in supplement to RTCP NACK indication and the AL-FEC redundancy level is usually dynamically adapted [25], [26] to network conditions.

The advantage of AL-FEC over other schemes is that error recovery is proactively realized by redundant packets and a decoder is able to recover from any packet losses without any additional transport-related delays (e.g., as for retransmissions-based mechanisms). However, the cost to pay for using AL-FEC is additional bandwidth utilization. A dynamic AL-FEC controller (or media optimizer [26]) ensures usually in practice an optimal trade-off between AL-FEC bandwidth utilization and QoS is achieved by balancing the bandwidth split between the source media content and the AL-FEC redundancy added. Figure 6.17.2.1-1 outlines potential operation points for an AL-FEC encoded media stream (e.g., a video stream) as adapted by a dynamic AL-FEC controller based on network conditions. Operation point (1) illustrates an unprotected media stream at 30 Mbps, operation point (2) illustrates an AL-FEC protected media stream with a high AL-FEC redundancy rate of 100% (i.e., equal bandwidth split between media and repair packets), and operation point (3) illustrates an AL-FEC protected media stream with a low AL-FEC redundancy rate of 25%.

Figure 6.17.2.1-1: Example operation points of AL-FEC encoded media

The network metrics and statistics that influence the operation of AL-FEC controller are usually diverse and may involve at least [23], [24], [25], [26]:

-	available bandwidth estimation/information

-	packet loss statistics

-	packet loss feedback

-	RTT delay estimation/information.

Congestion events impact therefore the operation of the AL-FEC dynamic behavior. At a high-level in low packet loss conditions the AL-FEC redundancy rate is reduced considerably (e.g., operation point (3) in Figure 6.17.2.1-1), or even eliminated (e.g., operation point (1) in Figure 6.17.2.1-1) as per application configuration and preferences. On the other hand, for some higher packet loss values, given that the link has necessary bandwidth, the AL-FEC redundancy rate is increased to provide more redundancy and protection against network losses. However, in case congestion events persist and available bandwidth degrades the AL-FEC redundancy rate is usually backed-off and the media source rate is adapted to a lower source rate to account for the lower bandwidth available. The high-level control loop detailed above implies frequent network conditions monitoring (e.g., every second, [26]) and corresponding reactive AL-FEC redundancy rate and media source rate adaptation. This achieves an elastic and robust transport mechanism for low latency media delivery even over bursty lossy networks.

NOTE:	Typical bursty losses are usually comparable to the network conditions monitoring times (e.g., couple of seconds resolution) and may affect multiple frames in a row.

#### 6.17.2.2	End-to-end transport perspective

Figure 6.17.2.2-1 illustrates the end-to-end perspective of AL-FEC encoded media streams over a 3GPP network. The Application Server (AS) is situated into a data network with potentially no QoS guarantees. It serves DL AL-FEC encoded XR media traffic to a UE connected to a 3GPP network. The DL traffic is transported over the data network to the UPF (ingested at reference point N6), then over the core network to the NG-RAN (ingested at reference point N3) and finally reaches the UE over the UE air-interface. Optionally, in case the UE is not the XR endpoint, it may relay over a tethered connection the media content to an XR tethered device.

Figure 6.17.2.2-1: End-to-end transport path for AL-FEC encoded PDU Sets.

The following points are worth remarking if AL-FEC awareness is enabled for NG-RAN to actively discard obsolete PDUs out of AL-FEC encoded PDU Sets:

-	the AL-FEC obsolete PDU discarding at NG-RAN may be perceived by applications as congestion events, unless applications are fully aware of the NG-RAN behaviour (e.g., based on configuration, e.g., QoS configuration, or other feedback mechanisms);

-	a fixed operation point at a static AL-FEC redundancy rate is not advisable from an information rate optimization perspective and media delivery standpoint since the AL-FEC redundancy will inefficiently utilize available bandwidth in detriment of the media source, as described above;

-	a dynamic AL-FEC behavior is preferrable;

NOTE 1:	Dynamic AL-FEC awareness signalling should be supported in supplement to PDU Set awareness, yet how to support it is FFS and may further involve SA2 and RAN2 coordination.

-	the AL-FEC obsolete PDUs discarding at RAN will in effect reduce the 5GS operating bandwidth over the Uu interface to the media source rate;

-	the AL-FEC encoding may protect against bursty packet losses over any of the data network, core network and air-interface link segments, yet the core network and air-interface link segments are part of the QoS flow architecture ensuring QoS guarantees (e.g., PSDB, PSER, or alternatively, PDB, PER);

-	it seems that AL-FEC is mostly effective against the data network link segment, or alternatively, any link segment in the path to N6 that is out of the scope of 3GPP QoS flow architecture;

NOTE 2:	How the AL-FEC encoding will impact the QoS configuration if NG-RAN discarding of obsolete AL-FEC PDUs is enabled is FFS.

-	the AL-FEC obsolete PDU discarding at NG-RAN cannot protect against losses on tethered links, if present;

The analysis and remarks above implicitly assume NG-RAN feasibility (e.g., fast determination of PDUs ACK/NACK in various RLC modes to enable obsolete AL-FEC encoded PDU discard) and net benefits (e.g., capacity gains/bandwidth savings vs. added complexity) in discarding obsolete AL-FEC PDUs. However, the latter assumptions should be further verified in coordination with RAN2.

#### 6.17.2.3	Overhead of AL-FEC

To all the application receiver to reconstruct the source packets, the application sender sets the redundancy for AL-FEC based on the end-to-end packet loss rate in the network. It is important to understand the overhead, i.e., the ratio of the number of repair packets to the number of source packets needed to meet a probability that the application receiver can reconstruct all the source packets.

For a small number of source packets, the overhead can be significantly higher than the theoretical limit for the case of an infinite number of source packets (which is equal to p/(1-p), where p is the end-to-end packet loss rate). This is because in a realization of the random packet losses, the packet loss rate may be higher than p and this effect is more prominent when the number of source packets is smaller. To illustrate, consider an example where p=0.1%, 1% and 10%, and the probability of reconstructing all the source packets, denoted preconstruct, is set to 99.9%. The 10% represents the BLER without HARQ retransmission in typical implementations, and the 1% and 0.1% may represent the BLERs with HARQ retransmissions. Furthermore, it is assumed that the packet losses are independent, and each packet is sent in a separate transport block. The overhead (in percent) as a function of K is shown in Figure 6.17.2.3-1. For K=20, the overhead is 45%, and even as K increases to 100 the overhead still stays at 24%. In contrast, the theoretical limit is 11.1% (the red dashed line). The theoretical limit can be considered as the overhead needed to let the application receiver receive K packets on average.

(a)

(b)

Figure 6.17.2.3-1: The overhead of AL-FEC as a function of the number of source packets K for 99.9% probability of reconstructing all the source packets at the application receiver: (a) linear scale; (b) logarithmic scale.

The steps for calculating the overhead are as follows:

1)	Find the minimum value (denoted m*) of m that satisfies n=Kmmn(1-p)n pm-n> preconstruct, where preconstruct=0.999

2)	The overhead is (m*-K)/K

Note that in the RAN, a transport block may carry multiple packets. Thus, even if K is large, the effective value for the purpose of reconstructing the source packets may be small. For example, if K=100 and each transport block carries 4 packets, then effectively we are dealing with 25 transmissions and the overhead would be corresponding to the overhead for K=25 rather than the overhead for K=100 in Figure 6.17.2.3-1.

Observation 1: the overhead of AL-FEC may be much higher than the overhead needed to let the application receiver receive K packets on average, where K is the number of source packets.

If the RAN transmits every packet (or PDU) of a PDU Set with AL-FEC encoding, then the overhead at the RAN (i.e., on average how many packets beyond the number of source packets the RAN needs to transmit normalized by the number of source packets) can be high.

In contrast, if the RAN can drop obsolete packets (or PDUs), the overhead at the RAN can be reduced. With ideal assumptions, e.g., the base station knows immediately and reliabily which PDUs are delivered successfully, the overhead at the RAN can drop to the theoretical limit. With practical assumptions, the overhead will be higher than but still can still be close to the theoretical limit.

Observation 2: if the RAN can drop obsolete PDUs of a PDU Set with AL-FEC encoding, the overhead at the RAN can be dropped to be close to the theoretical limit.

## 6.18	Solution #18: Real-time communication congestion control algorithms and AL-FEC

### 6.18.1	Key Issue mapping

This solution addresses key issue #4 on AL-FEC awareness for PDU Set handling and Key Issue #3: Enhancements for application-layer FEC support.

### 6.18.2	Description

#### 6.18.2.1	General

This clause introduces the congestion control algorithms for real-time communication, including those currently implemented in WebRTC [28] and two algorithms described in two IETF RFCs [29] [30].

Observations are made on these algorithms and AL-FEC.

#### 6.18.2.2	Google Congestion Control (GCC)

Google Congestion Control (GCC) is one of the two congestion control algorithms supported in the WebRTC implementation. The algorithm determines a sending bit rate, which limits the total bit rate for RTP packets, RTCP packets and RTP retransmissions (if any) and drives the bit rate adaptation of the media codecs. It uses packet losses and delays as signals of network congestion in adjusting the sending rate. The packet loss feedback is based on RTCP reports. The delays are one-way delays measured by the “absolute send time” RTP header extension as described in TS 26.522 [2], and the change in the one-way delay is computed to detect network congestion.

GCC has not been standardized. Although the informational IETF document [27] describes GCC, the description is outdated. In what follows, we present GCC based on the current WebRTC implementation [28].

GCC sets the sending rate to the lesser of a loss-based bandwidth estimation and a delay-based bandwidth estimation. That is, the sending rate Rt  at time t is set to:

Rt=min⁡(RLtk,RDtn)

where RLtk is the loss-based bandwidth estimation made at time tk when the lastest RTCP loss report (the kth) is received, and RDtn is delay-based bandwidth estimation made at time tn when the lastest one-way delay measurement is taken or reported.

Delay-based bandwidth estimation:

This is described in aimd_rate_control.cc, based on the detection of overuse. The one-way delay

RDtn=βRDtn-1,                                in 'decrease' state RDtn-1,                                           in 'hold' state RDtn-1+ (tn-tn-1)δ,      in 'increase' state

where δ is a constant equal to 1200 bytes/second/RTT, β<1 is a backoff factor. The state ‘decrease’, ‘hold’ and ‘increase’ are based on the slope of the change in the one-way delay as a function of time. The change in the one-way delay from packet group i-1 to packet group i is defined as

di= ti-Ti-(t(i-1)- T(i-1))

where ti is the arrival time of packet group i, and Ti is the departuture time of packet group i, ti-1 is the arrival time of packet group i-1, and Ti-1 is the departuture time of packet group i-1. The slope (termed trendline in the WebRTC source code) is estimated and compared with thresholds to determine the state.

Loss-based bandwidth estimation:

There are three versions of loss-based bandwidth estimation algorithms.

Version 0 (static thresholds):

The loss-based bandwidth estimation is calculated as:

RLtk=RLtk-11-0.5ptk,                        ptk>0.1 RLtk-1,                                       0.02≤ptk≤0.11.08RLtk-1+1000 bits/sec,           ptk<0.02

where tk-1 is the time when the previous RTCP loss report is received, ptk is the loss (fration of loss) received in the most recent RTCP loss report, and RLtk-1 is the loss-based bandwidth estimation at the time when the previous RTCP loss report is received.

Version 1 (dynamic thresholds):

This version is similar to version 0, except that it uses (1) dynamic thresholds are used, and (2) the factor for increasing the estimation is adaptive to RTT, instead of being a constant 1.08 in Version 1. The loss-based bandwidth estimation is:

RLtk=0.99RAtk,                                             pdectk>γdec αRLtk-1+1000 bits/sec,               pinctk<γinc RLtk-1,                                                   otherwise

where γdec and γinc are dynamic thresholds, γdec=(4 kpbsRLtk-1)1/2, γinc=(0.5 kpbsRLtk-1)1/2

pdectk is the probability for decreasing the bandwidth estimation, equal to the minimum of the average loss probability and the most recent loss probability,

pinctk is the probability for increasing the bandwidth estimation, equal to the average loss probability pavgtk if pinctk-1<pavgtk and equal to pinctk-1+(1-exp⁡(∆tw-1))(pavgtk-pinctk-1),

where  ∆t is time elaspsed, w is a time window,

0.99RA is the bit rate of the acknowledged transmissions,

α=1.02+0.061-minmax(minτ, τmax,τmin )-τmin τmax-τmin , 1

where τ is the RTT, τmin is a pre-configured minimum RTT and τmax is a pre-configured maximum RTT. With this dynamic threshold α, the increase in loss-based bandwidth estimation will be slower when the RTT gets higher.

Version 2 (maximum likelihood):

To calculate the loss-limited bandwidth, in a nutshell, the sender chooses a tuple of an inherent loss probability Pinherent_loss (loss probability induced by the channel error rather than network congestion) and a loss limited bandwidth  RL that maximizes the following objective function which is based on the logarithmic maximum likelihood of observing the number of packet losses:

U=i=0K-1wi(NllogPl(i)+Nrlog(1-Pl(i))+B(Nl+Nr))

where:

Kis the number of observations,

w=0.9 is a weight,

Nl is the number of bytes lost,

Nr is the number of bytes received,

Pli=Pinherent_loss+(1-Pinherent_loss)(Rti-RL)/Rti,

where Rti is the sending rate at the time ti of the ith observation,

B is a high bandwidth bais (that depends on RL),

i=0 is the index of the most recent observation and Kis the index of the oldest observation.

#### 6.18.2.3	PCC

Performance-oriented Congestion Control (PCC) is the other congestion control algorithm supported in the current WebRTC implementation [28]. The sender adjusts the sending rate and observe the performance metrics including the delay and packet loss, and pick the sending rate that maximizes a utility function:

Ux,dτdt, L=xt-bxmdτdt-cxmL

where x is the sending rate, dτdt is the gradient of RTT τ, L is the loss rate, m=1 or 2, and t, b, c are coefficients.

#### 6.18.2.4	NADA

Network-Assisted Dynamic Adaptation (NADA) is specified in RFC 8698 [29]. This algorithm considers delay, packet loss, and ECN marking as signals of network congestion. Furthermore, it converts packet loss and ECN marking to some equivalent penalty in terms of delay and forms an aggregate congestion signal. Specifically, the receiver calculates:

xcurr=d+Dmark(pmarkpmark_ref )2+Dloss(plossploss_ref )2

where d is the equivalent delay after non-linear warping,

pmark is the estimated packet ECN marking ratio, pmark_ref is the reference packet ECN marking ratio,

Dmark is the reference delay penalty for ECN marking when packet marking is at pmark_ref,

ploss is the estimated packet loss ratio, ploss_ref is the reference packet loss ratio,

Dloss is the reference delay penalty for packet loss when packet loss is at ploss_ref.

The receiver decides whether the sender is to be in the accelerated ramp-up mode (rate update mode rmode=0) or in the gradual update mode (rmode=1). The sender should be in the accelerated ramp-up mode if there are no recent packet losses in an observation window (of 500ms) and there is no build-up of queueing delay. Otherwise, the sender should be in the gradual update mode.

The receiver also calculates the receiving bitrate brecv.

The receiver sends xcurr, rmode, and brecv to the sender.

The sender performs multiplicative increase in the accelerated ramp-up mode using the RTT and brecv, and performs additive decrease in the gradual update mode using xcurr.

#### 6.18.2.5	SCReAMv2

Self-Clocked Rate Adaptation for Multimedia 2 (SCReAMv2) [30] uses packet losses, delay, and ECN marking as signals of network congestion.

A congestion window size limits the sending rate and is adjusted based on:

-	Packet losses: A packet loss causes the congestion window size to decrease, but not as much as a packet loss does in the case of TCP Reno

-	Queueing delay: The congestion window size decreases linearly when the average queueing delay exceeds a threshold.

-	ECN marking: When a classic ECN marking is received, the congestion window size decreases by a factor. When L4S ECN marking is received, the congestion window size decreases in proportion to the fraction of packets that are L4S ECN marked.

#### 6.18.2.6	Summary of congestion control algorithms

The interactions between the application and the network are through signaling manifested by packet losses, queueing delay and ECN marking, as summarized in the table below:

Table 6.18.2.6-1: Comparison of congestion control algorithms

| Congestion Control Algorithm | React to packet losses? | React to ECN marking? | React to queueing delay? |
| --- | --- | --- | --- |
| GCC (with loss based bandwidth estimation v0) | No, if the packet loss rate is between 2% and 10%; yes otherwise. | No | Yes |
| GCC (with loss based bandwidth estimation v1) | Yes | No | Yes |
| GCC (with loss based bandwidth estimation v2) | Yes | No | Yes |
| PCC | Yes | No | Yes |
| NADA | Yes | Yes | Yes |
| SCReAMv2 | Yes | Yes | Yes |

Observation 1: All congestion control algorithms for real-time communication in Table 5.43.5-1 use queuing delay (among other metrics) as a signal of network congestion.

Observation 2: Although all congestion control algorithms for real-time communication in Table 5.4.3.5-1 use packet losses as a signal of network congestion, one algorithm is not sensitive to packet losses when the packet loss rate is within the range [2%, 10%].

Observation 3: Two of the six congestion control algorithms for real-time communication in Table 5.4.3.5-1 support ECN marking.

Although WebRTC currently implements only GCC (with three versions) and PCC, it does not prevent one from adding other congestion control algorithms such as NADA and SCReAMv2. Since these algorithms support ECN marking, we have the following observation.

Observation 4: It is possible to add ECN marking support to WebRTC for congestion control. Updates of RFCs is required to ensure interoperability.

#### 6.18.2.7	Packet loss rate calculation for AL-FEC

When AL-FEC is used, the source packets and the repair packets may be sent in a single RTP stream (identified by an SSRC). This is the case for ULPFEC in the WebRTC implementation [28].

Alternatively, the source packets and the repair packets may be sent in different RTP streams (identified by different SSRC’s) within the same RTP session. This is the case for FlexFEC in the WebRTC implementation [28]. The packet loss rate is calculated individually. The packet loss rates are then combined to form a single packet loss rate as an input to the congestion control algorithms.

Observation 5: For AL-FEC, the packet loss rate on the source packets and that on the repair packets can be calculated separately. RFC 5109 [13] (Clause 12) give congestion considerations. However, there are no considerations to handle repair losses differently than source packet losses.

Although RFC 5109 [13] recommended that the source packets and the repair packets may be sent in different sessions (identified by different IP 5-tuples), we are unaware of any commercial implementation of such scheme.

Note that RFC 8085 [34] (UDP usage Guidelines) recommend that an "application SHOULD perform congestion control over all UDP traffic it sends to a destination, independently from how it generates this traffic".

Recommendation: SA4 to study how to handle the packet losses for congestion control when the application traffic is encoded with AL-FEC.

## 6.19	Solution #19: Congestion control enhancement to support AL-FEC awareness handling

### 6.19.1	Key Issue mapping

This maps to Key Issue #4.

### 6.19.2	Description

#### 6.19.2.1	Background of using AL-FEC for real-time communication in cellular networks

There are inherent losses in the over-the-air transmission in cellular networks. To recover from the losses, retransmission in PDCP, RLC and MAC may be used. However, the low-latency requirements for XR applications put  constraints on the use of PDCP and RLC layer retransmissions.

If retransmission is needed, MAC layer HARQ retransmission is preferred. However, RAN implementations typically have an instantaneous BLER (iBLER) of 10% for high spectral efficiency. That requires a large number of HARQ retransmissions, resulting in large delays. If AL-FEC is used, the need for HARQ retransmission is greatly reduced. This is illustrated in the simulation study below.

Scenario: TDD with subframe format DDDSU, 30 kHz SCS, HARQ turnaround time about 5 ms, 100 MHz bandwidth, 60 fps, video frame size following a truncated Gaussian (STD, Max, Min) distribution: (10.5%, 150%, 50%) of average frame size, the average frame size 0.5 Mbits, the average SNR 5 dB, iBLER 10% and the subsequent BLERs for HARQ retransmissions following a BLER correlation model on the successes/failures of the HARQ transmissions based on field data (which gives the probability pm,nthat the current TB is successfully transmitted in the nth attempt conditioned on that the previous TB is successfully transmitted in the mth attempt, where n=1, …, 5, m=1,…5), RLC acknowledged mode (AM) (t-reassembly 25 ms, t-StatusProhibit 10 ms), and MDS AL-FEC code each time applied to the PDUs of a single video frame. RLC AM is used to handle the 0.22% residual BLER resulting from the BLER correlation model given that we evaluate 99.9 percentile latency (otherwise with RLC UM, 0.22% of the PDUs will never be delivered successfully and the 99.9 percentile latency for the “No AL-FEC case” will be infinity).

Table 6.19.2.1-1: Delay without and with AL-FEC

| Scheme | Redundancy ratio | Latency (ms) |  |
| --- | --- | --- | --- |
|  |  | 99.9 percentile | 99 percentile |
| No AL-FEC | 0 | 55 | 43.5 |
| With AL-FEC | 30% | 15 | 14 |

We see from the table that AL-FEC reduces the 99-percentile delay from 43.5 ms to 14 ms and reduces the 99.9-percentile delay from 55 ms to 15 ms.

Observation 1: AL-FEC can reduce the delay for practical RAN implementations.

#### 6.19.2.2	Potential Benefits of Application-layer FEC awareness for PDU Set handling

When the RTP source adds redundant PDUs for an ADU, the redundancy is over budgeted to account for error in the estimation of the packet loss rate in the network. That is, there are more packets than needed for reconstructing the ADU. At the last hop of the PDU Set delivery, if the base station is aware of AL-FEC, it can drop PDUs that are no longer needed for reconstructing the ADU. This has two benefits:

-	Reducing the usage of resources and hence improving the spectral efficiency (the amount of resources per PDU Set)

-	Lowering the power consumption of the UE because the network can let the UE go to the sleep mode earlier.

This AL-FEC aware PDU Set handling is illustrated in the Figure 6.19.2.2-1. Packets 0 and 1 are served in the first time slot (which is a ‘D’ slot), and packets 2 and 3 in the second slot, and so on. In Case-1, without AL-FEC awareness, the redundant packets are still transmitted, which wastes network resources and keeps the UE awake longer before the network lets the UE go to the sleep mode. In contrast, in Case-2, with AL-FEC awareness, the use of network resources becomes more efficient, and the UE goes to the sleep mode earlier.

For ease of exposition, we assume that the number of source packets is 20 in Figure 6.19.2.2-1.

Figure 6.19.2.2-1: Potential Benefits of AL-FEC awareness at RAN

A Simulation study was carried out for the following scenario to quantify the benefits:

The video frame size being fixed at 1Mbits, TDD with slot pattern DDDSU, 30 kHz SCS, HARQ retransmission (initial transmission and up to 2 retransmissions), 100 MHz bandwidth, iBLER = 10%, the BLER after the 2nd HARQ transmission being 5%, the BLER after the 3rd HARQ transmission being 1%, the minimum value for the PDSCH to HARQ feedback timing indicator (i.e., K1) being 2 time slots (allowing the NACK for packets 18 and 19 and the ACK for packets 20 and 21 to be multiplexed in the same U slot in Figure 6.19.2.2-1), the power saving feature being Rel-18 eCDRX + PDCCH skipping, 30 fps, MDS AL-FEC code each time applied to the PDUs of a single video frame, redundancy ratio 30% (FEC code rate = 22%). The power consumption values are presented by relative power as defined in TS 38.840, and it accounts for the power consumed by the modem for both uplink and downlink.

Table 6.19.2.2-2: Potential Benefits of AL-FEC aware PDU Set handling at RAN

|  | 99-Percentile Latency (ms) | Power Consumption | Network loading |
| --- | --- | --- | --- |
| Case 1 – without AL-FEC awareness at RAN | 27 | 312 | 91.08% |
| Case 2 – with AL-FEC awareness at RAN | 27 (0%) | 270 (-13%) | 74.03% (-19%) |

We see from the table that AL-FEC aware handling reduces power consumption of the UE by 13%, which is significant for the UE. It also reduces the network loading by 19%, and this allows the network to accommodate more users.

Observation 2: AL-FEC aware PDU Set handling can potentially reduce the UE power consumption and network loading.

To perform intentional discard, the NG-RAN needs to retain state variables (e.g., how many PDUs have been delivered successfully for each PDU Set). The associated complexity may be acceptable given that a base station typically serves a much smaller number of UEs than the UEs served by a UPF.

#### 6.19.2.3	Implications of Application-layer FEC awareness for PDU Set handling on congestion control

Many congestion control algorithms, such as Google congestion control algorithms [28], NADA [29] and SCReAMv2 [30], use packet losses as a signal of network congestion. Therefore, it is important for the congestion control algorithms to correctly interpret packet losses in the case of AL-FEC awareness handling of the PDU Set.

#### 6.19.2.4	The Proposed Solution

##### 6.19.2.4.1	Case 1: In congestion

When there is congestion, as long as the AL-FEC awareness handling of the PDU Set does not alter the packet loss statistics, there is no impact on congestion control. Examples are given below.

Example 1: the network can discard repair packets rather than source packets in the case of FlexFEC without changing the overall packet loss rate, which will not lead to over reduction of the sending rate.

Example 2: the network can discard redundant packets across different PDU Sets while still meeting the required redundancy ratios for reconstructing the respective ADUs.

##### 6.19.2.4.2	Case 2: Not in congestion

When there is no congestion, the network can discard obsolete packets (by obsolete, it means that the packets are no longer needed for reconstructing the ADU at the receiver), which will increase the packet loss rate observed by the RTP sender, and to avoid the RTP sender mis-interprets the packet losses as signals of congestion, the network can indicate to the RTP sender that there is no network congestion and such packet losses should not be taken into account by the congestion control algorithm for determining the sending rate.

NOTE 1:	It is FFS how the network provides feedback to the application on obsolete packets dropped in the network.

NOTE 2:	Although congestion control is currently not in the scope of TS 26.522, it could be studied in TR 26.822.

Conclusion: The behavior of the application at the sender may be impacted because congestion control that may be triggered by RAN discard needs to be handled differently than in current implementations (i.e., not see RAN intentional discard as an indication of congestion).

## 6.X	Solution #X: <Solution Title>

### 6.X.1	Key Issue mapping

Editor's note:	This clause lists the key issue(s) addressed by this solution.

### 6.X.2	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Sub-clause(s) may be added to capture details.

# 7	Overall Analysis

Editor's note:	This clause provides analysis of different solutions.

## 7.x	Analysis for Key Issue #x

# 8	Conclusions

Editor's note:	This clause will list the conclusions that have been agreed during the study item activities.

## 8.x	Conclusions for Key Issue #x

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-04 | SA4#127-bis-e | S4-240795 | - | - | - | Implementing S4aR240017, S4-240604, S4-240657, S4-240771, S4-240772, S4-240773, S4-240777, S4-240769, S4-240829, S4-240830 | 0.0.1 |
| 2024-05 | SA4#128 | S4-241293 | - | - | - | Implementing S4-240968, S4-241012, S4-241018, S4-241096, S4-241191, S4-241260, S4-241271, S4-241275, S4-241283, S4-241285, S4-241312, S4-241314 | 0.1.0 |
| 2024-08 | SA4#129-e | S4-241486 | - | - | - | Implementing S4aR240032 | 0.1.1 |
| 2024-08 | SA4#129-e | S4-241672 | - | - | - | Implementing S4-241444, S4-241497, S4-241503, S4-241538, S4-241649, S4-241650, S4-241651, S4-241657, S4-241658, S4-241659, S4-241660, S4-241663, S4-241747, S4-241749, S4-241751, S4-241763 | 0.2.0 |
| 2024-09 |  |  |  |  |  | Version 1.0.0 created by MCC for presentation to TSG SA | 1.0.0 |