3GPP TSG-SA WG6 Meeting #63	S6-244403

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244056)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0593 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to client-triggered one-time location information report for shared functional alias procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Netherlands Police |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Release 19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A redundant information flow exists for location reports. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of redundant information flow and update of procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclarity in stage 3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9.2.10, 10.9.3.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.9.2.10	Void

* * * Second Change * * * *

##### 10.9.3.8.1	Client-triggered one-time location information report

Figure 10.9.3.8.1-1 illustrates the procedure when a location management client requests one-time location information from other location management clients for location information reporting using functional alias, which can be shared between several MC service users. Under this condition, the actual location of all MC service users sharing the same functional alias are reported.

Pre-conditions:

1.	MC service client 2 and MC service client 3 share the same functional alias.

2.	MC service client 2 and MC service client 3 activated the functional alias.

3.	MC service client 1 may have an activated functional alias.

4.	The location management server has subscribed to the functional alias controlling MC service server within the MC system for functional alias activation/de-activation updates.

Figure 10.9.3.8.1-1: One-time location information report for shared functional alias

1.	Location management client 1 sends a location reporting trigger, limited to one MC service at the time, to the location management server to activate a one-time location report procedure which shall retrieve the location information of the MC service users that may share the contained functional alias. Location management client 1 may include its own activated functional alias.

2.	Location management server resolves the target functional alias and checks whether location management client 1 is authorized to send a location reporting trigger for location information of location management clients 2 and 3.

3.	The location management server uses on-demand location reporting procedures. The location information request contains the functional alias provided by the location management client 1 to address location management client 2 and location management client 3.

4.	Upon receiving the reports, the location management server updates location of the reporting location management clients. If the location management server does not have location information of a reporting location management client before, then just stores the reporting location information for that location management client.

5.	The location management server sends location information reports to location management client 1 containing the provided location information for each location management client using the given functional alias. If the optional MC service UE label is present, the location management server sends it, as part of the location report response to location management client 1. If not all location management clients immediately respond to the location management server, i.e. other reports some time later, subsequent location reporting responses may be sent.

* * * End of changes * * * *