3GPP TSG-SA3 Meeting #119	S3-244807

Orlando, US, 11 - 15 November 2024														  (revision of S3-yyxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.328 | CR | 0080 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing MRF from IMS data channel architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_SEC |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA2 has removed MRF as a supporting node for DC in S2-2408776. In order to be aligned with the changes in SA2, we remove MRF from IMS data channel architecture. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing MRF from IMS data channel architecture. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with TS 23.228. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2.4, 4.2.1, 4.2.5, 5.4.4, 5.5.6, 6.2.4, 7.2.4, 7.3.4, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN of 1st CHANGE ***

#### 4.1.2.4	Certificate fingerprints based solution for e2DCe DTLS

The key management solution for e2DCe protection of IMS data channel-based media is based on the cipher suites and session keys negotiated via the DTLS handshake between the UE and MF. The DTLS record protocol secures the actual media. Mutual authentication during the DTLS handshake is achieved using certificates, with the certificate fingerprints being transmitted using the SDP fingerprint attribute in the SDP offer-answer exchange between the UE and the MF via the P-CSCF, S-CSCF, IMS AS.

This approach is specified in RFC 8841 [48] and RFC 8842 [49], where "UDP/DTLS/SCTP" is used as the protocol identifier in the m-line of the SDP, "a=fingerprint" attribute is used to provide the fingerprint of the certificate, and "a=tls-id" provides the DTLS instance identification to handle DTLS restart scenarios.

DTLS profile considerations discussed in annex M of this specification shall be followed to support IMS media plane security.

*** END of 1st CHANGE ***

*** BEGIN of 2nd CHANGE ***

### 4.2.1	General

This clause describes the impact of IMS media plane security on the IMS architecture.  Five cases need to be distinguished. The IMS UEs are impacted in all five cases. The network impact varies with the cases.

1. 	E2ae security: here the P-CSCF (IMS-ALG), the IMS Access GW, and the Iq interface between them are impacted.

2. 	E2e security using SDES: minor impact on the network infrastructure (see TS 29.162 [20] for details).

3.	E2e security using KMS: here, the network infrastructure needs to be enhanced with a Key Management Server, which, in turn, relies on a GBA [6] infrastructure, or an infrastructure to provide corresponding services, to be in place. Otherwise, there is minor impact on the network infrastructure (see TS 29.162 [20] for details).

4.	E2DCe IMS Data Channel media plane security: The P-CSCF as well as other network functions such as S-CSCF, IMS AS, and the related interfaces Mw, ISC, DC2 are impacted. The IMS Data Channel media plane security is established between the UE and MF.

5.	E2e IMS Data Channel media plane security: The IMS Data Channel media plane security is established between two UEs or between a UE and a Data Channel Application Server.

A pre-requisite for support of e2e security is that media packets are forwarded transparently by any nodes present in the media path (SRTP packets in case of secure RTP, TLS packets in case of secure MSRP, and DTLS packets carrying SCTP in case of IMS Data Channel). This implies that transcoding of RTP streams is no longer possible.

These prerequisites apply irrespective of whether the SRTP session was established by means of SDES or KMS.

NOTE:	The lawful interception architecture is outside the scope of this TS.

*** END of 2nd CHANGE ***

*** BEGIN of 3rd CHANGE ***

### 4.2.5	E2DCe security

For e2DCe security, and for IMS Data Channels that terminate in the MF, the IMS Access GW is not needed in the media path for security purposes.

The interface between IMS AS and the Data Channel Media Function (DC2) needs to be able to transport parameters related to the management of DTLS cryptographic contexts. There is no impact on other parts of the network infrastructure. This is depicted in Figure 4.2.5-1. Details can be found in clauses 6.2.4, 7.2.4 and 7.3.3.

Figure 4.2.5-1: IMS signalling and media plane entities relevant to e2DCe security

*** END of 3rd CHANGE ***

*** BEGIN of 4th CHANGE ***

### 5.4.4	Authentication and authorization for e2DCe protection

E2DCe security implies that the UE and Data Channel Media Function terminate IMS Data Channel media security.

In the DTLS solutions for IMS data channels, mutual authentication between the IMS UE and the Data Channel Media Function relies on secure transport of certificate fingerprints using SIP signalling integrity protection. If the fingerprints of the certificates used for the DTLS handshake match the fingerprints transmitted via SIP signalling, then the DTLS endpoints can be sure that DTLS is really established between the nodes that exchanged the SIP signalling.

The IMS UE implicitly authorizes the P-CSCF (IMS-ALG) and the Data Channel Media Function to perform e2DCe security by indicating support for e2DCe security during the registration in line with the IMS UE’s policy, cf. clause 7.1.

Conversely, an IMS UE is always authorized to participate in e2DCe security if the network policy allows e2DCe security, cf. clause 7.1.

*** END of 4th CHANGE ***

*** BEGIN of 5th CHANGE ***

### 5.5.6	Security properties for e2DCe protection using DTLS

Based on secure mutual authentication leveraged by the integrity protection of the SIP signalling messages (see clause 5.4.4), DTLS provides secure derivation of session keys to protect the media.

In addition to SIP signalling security, also the Mw, ISC interfaces for signalling between the P-CSCF (IMS-ALG), and the media node terminating the IMS Data Channel towards the UE, i.e. the MF, needs to be secured. Also, the DC2 interface for signalling between the IMS AS and the Data Channel Media Function needs to be secured, see clause 6.2.4.

DTLS profile considerations discussed in annex M of this specification may be followed to support IMS media plane security.

*** END of 5th CHANGE ***

*** BEGIN of 6th CHANGE ***

### 6.2.4	Key management mechanisms for e2DCe protection

#### 6.2.4.1	Endpoints for e2DCe protection

For IMS Data Channel traffic terminated by the MF, the MF shall send DTLS-protected SCTP packets to and accept DTLS-protected SCTP packets from the served UE as requested by the IMS AS via the P-CSCF.

For the definition of the Data Channel Media Function cf. TS 23.228 [3].

#### 6.2.4.2	Key management protocol for e2DCe protection

The key management mechanism for e2DCe protection of IMS data channel traffic shall be based on certificates and the transmission of certificate fingerprints as defined in RFC 8841 [48] and RFC 8842 [49].

#### 6.2.4.3	Functional extension of the Mw, ISC, and DC2 interfaces for e2DCe protection

##### 6.2.4.3.1	Functional extension of the Mw, ISC, and DC2 interfaces for e2Dce protection for IMS data channel

For each IMS data channel media stream to be set-up with e2DCe security the P-CSCF shall send the certificate fingerprint received from the IMS UE over the Mw interface to the S-CSCF in a way that the S-CSCF is able to uniquely associate the fingerprint with a media stream. For IMS data channel media streams to be set-up with e2DCe security, the S-CSCF and IMS AS shall send the certificate fingerprint received from the IMS UE over the DC2 interface, respectively, to the MF in a way that the MF is able to uniquely associate the fingerprint with a media stream.

For each IMS data channel media stream to be set-up with e2DCe security the MF shall send the fingerprint of its certificate over the DC2 interface to the S-CSCF in a way that the S-CSCF is able to uniquely associate the fingerprint with a media stream. For each IMS data channel media stream to be set-up with e2DCe security S-CSCF shall send the fingerprint of the MF certificate over the Mw interface to the P-CSCF in a way that the P-CSCF  is able to uniquely associate the fingerprint with a media stream.

For protection of session-based messaging traffic and IMS data channel traffic, the MF shall, upon reception of a certificate fingerprint, use the certificate fingerprint (as described in RFC 8841 [48]) to verify the establishment of the DTLS session to belong to the served user. When the DTLS session has been established, the MF shall be prepared to convert unprotected SCTP packets to protected SCTP packets and vice versa and send the packets to the UE or receive them from the UE, as described in clause 7.

The integrity of the fingerprints sent over the Mw and DC2 interfaces is required. The Mw and DC2 interfaces shall be protected by NDS/IP [5]. If cryptographic protection is applied to the Mw and DC2 interfaces then integrity protection shall be used.

NOTE: 	If the P-CSCF (IMS-ALG), S-CSCF and MF are located in the same security domain then cryptographic protection is not mandated by NDS/IP. From TS 33.210 [5]: "The Zb-interface is located between SEGs and NEs and between NEs within the same security domain. The Zb-interface is optional for implementation."

*** END of 6th CHANGE ***

*** BEGIN of 7th CHANGE ***

### 7.2.4	IMS UE originating procedures for e2DCe

Figure 7.2.4-1 shows the originating session set-up procedures for one or more media stream(s) using e2DCe security.

Figure 7.2.4-1: Originating call flow for e2DCe using MF case

The IMS UE A performs an IMS originating session set-up according to 3GPP TS 23.228 [3], with modifications as described in the following. If both IMS UE and network indicated support for e2DCe security for IMS data channel during registration, then the IMS UE shall request e2DCe security for IMS data channel media streams to be established as described in this clause, unless the IMS UE initiates a procedure for e2e security for an IMS data channel media stream.

The originating procedures for establishing IMS data channels with e2e security are described in clause 7.2.5 of this specification. The IMS UE may learn of a preference for e2e security for a particular session or media stream by explicit user action via the user interface or by the security policy implemented on the IMS UE.

The procedure in the above figure for requesting e2DCe security for a media stream is now described step-by-step.

1.	IMS UE A sends an SDP Offer for a media stream containing cryptographic information, together with an indication "e2DCe-security for IMS data channel requested by UE", to the IMS AS via the P-CSCF .

For e2DCe protection of IMS data channel the cryptographic information contained in the SDP Offer consists of the fingerprint of the certificate of IMS UE and the tls-id attribute, in accordance with RFC 8841 [48].

2.	For each media stream that uses transport "UDP/DTLS/SCTP", the P-CSCF (IMS-ALG) checks for the presence of the indication "e2DCe-security for IMS data channel requested by UE".

If the indication is present and the P-CSCF (IMS-ALG) indicated support of e2DCe-security for IMS data channel during registration, the P-CSCF (IMS-ALG) allocates the required resources, includes the IMS Access GW and Data Channel Media Function in the media path and proceeds as specified in this clause. If the indication is not present for an IMS data channel media stream, the P-CSCF (IMS-ALG) proceeds for this media stream as described in clause 7.2.5 of the present specification.

NOTE 1: 	The inclusion of the IMS Access GW and Data Channel Media Function in the media path is needed for the purposes of e2DCe security even if it was not needed otherwise.

NOTE 2:	If an indication for e2DCe security for a media stream is present in an SDP offer but the support for e2DCe security for the respective protocol was not successfully established during registration then this is an error case.

3.	The IMS AS sends the SDP offer towards the S-CSCF.

4.	For e2DCe protection of IMS Data Channel, the S-CSCF stores the received fingerprint of the IMS UE A certificate, performs the required procedures according to TS 23.228 [3], and forwards the SDP Offer to the terminating network.

5.	The S-CSCF receives the SDP Answer from the terminating network.

6.	The S-CSCF sends the SDP answer towards the IMS AS.

7.	The IMS AS and the Data Channel Media Function exchange the cryptographic information.

For e2DCe protection of IMS Data Channel the cryptographic information communicated by the IMS AS to the Data Channel Media Function consists of the fingerprint of the UE's certificate and tls-id in accordance with RFC 8841 [48]. The IMS AS instructs the Data Channel Media Function to verify during the subsequent DTLS handshake with the IMS UE (see step 10) that the fingerprint of the certificate passed by the IMS UE during this DTLS handshake matches the fingerprint passed by the IMS AS to the Data Channel Media Function. In turn, the Data Channel Media Function communicates the fingerprint of the certificate it is going to use for setting up protection for this media stream to the IMS AS.

8.	The S-CSCF forwards the SDP Answer to the P-CSCF (IMS-ALG).

9.	The P-CSCF (IMS-ALG) sends the SDP Answer to the IMS UE A. After receiving this message, IMS UE A completes the media security setup.

NOTE 3:	The IMS UE can deduce that e2DCe security is used from two facts: first, that the P-CSCF (IMS-ALG) indicated its support for e2DCe security during registration, and second, that the IMS UE requested e2DCe-security in the SDP Offer.

10.	In case of IMS data channel, when the full session setup has been completed, the DTLS connection shall be established between the IMS UE and the Data Channel Media Function. When subsequently media are sent from or to the IMS UE, the UE and the Data Channel Media Function perform the required DTLS specific cryptographic operations on the media.

*** END of 7th CHANGE ***

*** BEGIN of 8th CHANGE ***

### 7.3.4	UE terminating procedures for e2DCe

Figure 7.3.4-1 shows the terminating session set-up procedures for one or more media streams using e2DCe security.

NOTE 1:	The procedures shown in the figure apply to users located in their home service area. The same concepts apply to roaming users.

Figure 7.3.4-1: Terminating call flow for e2DCe using MF case

The IMS UE performs an IMS terminating session set-up according to 3GPP TS 23.228 [3], with modifications as described in the following. If both IMS UE and network indicated support for e2DCe-security for IMS data channel media streams during registration and the P-CSCF receives an SDP Offer for an IMS data channel media stream using "UDP/DTLS/SCTP" transport from the S-CSCF, then the P-CSCF (IMS-ALG) shall establish e2DCe-security for the IMS data channel media stream as described in this clause.

NOTE 2: 	The P-CSCF (IMS-ALG) will not establish e2DCe security for IMS data channel media if the SDP offer received from the S-CSCF indicates that e2e security is being offered, cf. clauses 7.3.2, 7.3.3, and 7.3.5 for the establishment of e2e security on the terminating side.

The procedure in the above figure is now described step-by-step.

1.	The S-CSCF in the terminating network receives an SDP Offer for an IMS data channel stream with transport "UDP/DTLS/SCTP" from the originating network.

2.	The S-CSCF performs the required procedures according to TS 23.228 [3] and forwards the SDP Offer for the media stream to the IMS AS.

3.	The IMS-AS checks the media streams in the SDP Offer.

For each IMS data channel media stream offered with transport "UDP/DTLS/SCTP",

-	if both the IMS UE and P-CSCF (IMS-ALG) indicated support for e2DCe-security for IMS data channel during registration, the IMS-AS and S-CSCF proceeds for the respective media stream as described in this clause and allocates the required resources, includes the Media Function in the media path for establishing the DTLS towards the IMS UE and retrieves from the Media Function the fingerprint, tls-id, and setup role of the certificate the Media Function is going to use for setting up security for this media stream

-	if not both the IMS UE and P-CSCF (IMS-ALG) indicated support for e2DCe-security for IMS data channel during registration, the IMS-AS should continue using e2e security, as described in clause 7.3.5 of the present specification.

NOTE 3: 	The inclusion of the Media Function in the media path is required for the purposes of e2DCe security even if it was not required otherwise.

The IMS-AS modifies the SDP Offer before sending the SDP Offer back to S-CSCF.

For e2DCe protection of an IMS data channel media stream, the IMS-AS keeps the transport as "UDP/DTLS/SCTP" in the SDP Offer, conveys the fingerprint, tls-id, and setup attribute information of the MF in accordance with RFC 8841 [48] as well as an indication that e2DCe security is offered by the network to UE.

The IMS-AS then sends the updated SDP Offer to S-CSCF.

4.	The S-CSCF performs the required procedures according to TS 23.228 [3] and forwards the SDP Offer for the media stream to the P-CSCF(IMS-ALG).

5.	The P-CSCF (IMS-ALG) forwards the SDP Offer to the IMS UE B.

6.	IMS UE B replies with an SDP Answer for a secured media stream.

For e2DCe protection of IMS Data Channel, the IMS UE B includes in the SDP Answer the fingerprint of the UE´s certificate, tls-id, and setup attributes in accordance with RFC 8841 [48].

7.	The P-CSCF sends the SDP Answer to the S-CSCF.

8.	The S-CSCF performs the required procedures according to TS 23.228 [3] and sends the SDP Answer to the IMS AS.

9.	The IMS-AS communicates the cryptographic information contained in the SDP Answer to the Media Function, performs the required procedures according to TS 23.228 [3] and sends the SDP Answer to the S-CSCF.

For e2DCe protection of IMS Data Channel, the cryptographic information communicated to the Media Function consists of the fingerprint of the IMS UE B's certificate, tls-id, and setup attributes in accordance with RFC 8841 [48]. The S-CSCF instructs the Media Function to verify during the subsequent DTLS handshake with the IMS UE (see step 9) that the fingerprint of the certificate passed by the IMS UE during this DTLS handshake matches the fingerprint passed by the S-CSCF to the Media Function.

10.	The S-CSCF forwards the SDP Answer towards the originating network.

11.	In case of IMS Data Channel, when the full session setup has been completed, the DTLS connection shall be established between the IMS UE and the Data Channel Media Function. When subsequently media are sent from or to the IMS UE, the Data Channel Media Function performs the required DTLS specific cryptographic operations on the media.

NOTE 4:	 It is left to stage 3 specifications whether the IMS UE takes the role of DTLS client or DTLS server. These alternatives are equivalent from a security point of view.

*** END of 8th CHANGE ***