3GPP TSG-SA5 Meeting #157	S5-246091

Hyderabad, India, 14 - 28 October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.873 | CR | 0001 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.873_1900 Remove normative text from the TR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Data_SREP |  |  |  |  |  | Date: |  |  | 2024-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The TR contains a couple of statements with normative language, which is not allowed. Corrections due to comments from EditHelp. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The normative language is changed to non normative. A clause heading with wrong clause number is corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TR would contain normative language. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.2.1, 6.3.1, 6.4.1, 6.5.1 and 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 5.1.1	Description

In the current 3GPP system, the number of automation functions are increasing. There are domain specific entities and interdomain entities. Both entities are performing automated functionality. All of these require collecting measurement data from the NFs and/or 3GPP management system. A unique identity is required to identify the measurement collection requests to avoid collisions. The example of this unique measurement job identity, which is used for all management and orchestration jobs, are:

- traceReference in TraceJob, refer to 3GPP TS28.622 [2]

- qoEReference in QMCJob, refer to 3GPP TS28.622 [2]

- jobId in PerfMetricJob, TraceJob, and QMCJob, refer to 3GPP TS28.622 [2]

With the large number of different consumers, the uniqueness of the measurement job identities cannot be guaranteed. One example is:

Consumer A (e.g. NWDAF) request MDT data "1", "2", "5", and "22" in Cell "a" and want the collected data to be sent to itself.

Consumer B (e.g. a customer care central) request MDT data "1", "5" and "22" in Cell "a" and want the collected data to be sent to second line support.

These two requests are received within a short time difference.

As the two consumers are different and do not have any possibility to correlate the value for the Trace Reference, both use the value "123", which will lead to that the base station supporting Cell "a" cannot differ on which collected data is sent where.

The examples of consumer are: SLS assurance function, Centralized coverage and capacity optimization function, Distributed load balancing function, Operator technician and Customer care center.

The examples of producer are: Base station, AMF-node, UDR-node and Element manager.

2nd Change

### 5.2.1		Description

In the current 3GPP system, the number of automation functions are increasing. There are domain specific entities and interdomain entities. Both entities are performing automated functionality. All of these require collecting measurement data from the NFs and/or 3GPP management system. There are large possibility that the measurement collection requests are redundant. The followings are some examples on how the redundant PM/Trace/MDT/QoE subscriptions are created:

- Multiple Trace/MDT/QoE activation requests from different consumers with different measurements on the same target, e.g. same UE or same area scope.

- Multiple Trace/MDT/QoE activation requests from different consumers with overlapped measurements on the same target, e.g. same UE or same area scope.

- Multiple Trace/MDT/QoE activation requests from different consumers with different measurements and overlapped target, e.g. overlapped area scope.

- Multiple Trace/MDT/QoE activation requests from different consumers with overlapped measurements and overlapped target, e.g. overlapped area scope.

When the number of redundant subscriptions increased, there are many potential issues:

- RRC interface may be overloaded due to too many MDT/QoE activations and reporting.

- Inefficient due to duplicated measurements and reports which may require too many resources on the same task.

- Exhausted network signaling in order to support multiple Trace/MDT at mobility.

3rd Change

### 6.3.1 Description

As discussed in subclause 5.2, the traffic node needs to support a limitation of a maximum number of a specific PM/Trace/MDT/QoE measurement.

One solution, for use case 2 defined in clause 5.2, which could reduce the number of subscription requests is to introduce a new aggregation function by enhancement of the existing subscription service.

At PM/Trace/MDT/QoE activation, all requests are going to be sent to this subscription aggregation function. Upon receiving the activation request, it checks if this new subscription request can be aggregated into existing subscription(s), or a new activation is needed. If aggregation is possible, a subscription updating request will be created and sent to the data producer. Otherwise, a new subscription activation request is created.

In both above use cases, the reporting address may be changed if report data privacy is a concern. When report is received, the subscription aggregation function needs to split the report information based on the original request and distribute the split PM/Trace/MDT/QoE report to multiple consumers. The details of reporting needs FFS.

4th Change

### 6.4.1 Description

As discussed in subclause 5.2, the traffic node needs to support a limitation of a maximum number of a specific PM/Trace/MDT/QoE measurement.

One solution, for use case 2 defined in clause 5.2, is to reduce the number of subscriptions at measurement activations is that the consumer could provide a measurement scope indicator which specifies how the measurements are expected to be done.

When more than one measurement is configured in one activation request, the consumer may inform the producer of the expectation of the requested measurements, with following measurement scope indicator in the Activation Request message:

- Best effort: the producer should provide the requested measurements as much as it can.

- AllNeeded: the producer should provide all requested measurements configured in the activation request.

This indication provides more clear requirements in the subscription that the producer could make a choice to activate a minimum set of the requested measurements based on the resource conditions. If the producer is not able to produce the mandatory measurements, the job can be rejected.

This indication avoids the situation that a producer only can provide part of the requested measurements due to a reason, but the consumer is expecting the full set of measurements for data analytic.

Examples:

Example A: a consumer needs to perform a data analytic which is based on 5 measurements result. If only part of the measurements result is provided, the data analytic cannot be completed. Meaning providing part of the requested measurements result will only waste the producer resource. In this case, the consumer can provide a measurement scope indicator with value 'AllNeeded' in the activation request message. Upon receiving the activation request, if the producer cannot provide all 5 measurements due to a reason, e.g. resource limitation, it may reject the activation request.

5th Change

### 6.5.1 Description

As discussed in subclause 5.2, the traffic node needs to support a limitation of a maximum number of a specific PM/Trace/MDT/QoE measurement.

One solution, for use case 2 defined in clause 5.2, is to allow the producer to report a measurement failure and notify the management system or consumer with a reason. In TS 32.422 [3], a Trace failure notification and administrative messages are specified for that purpose. Both the message and procedure are limited to Trace. It can be specified for MDT/QoE as well. It needs to be enhanced and enabled for Trace/MDT/QoE.

6th Change

## 7.1	Conclusions

The following issues are identified in the present document so far:

-	The identity used in a measurement job needs to be globally unique between consumers and producers. The following potential solutions are studied.

-	Defining a globally unique Collection Id, refer to subclause 6.1

-	Defining a Job identity generator, refer to subclause 6.2

-	The traffic node needs to support a limitation of a maximum number of a specific PM/Trace/MDT/QoE measurement.

-	Defining a subscription aggregation function, refer to subclause 6.3

-	Defining a measurement scope indicator, refer to subclause 6.4

-	Enhancement on trace failure notification and administrative messages, refer to subclause 6.5

End of Change