3GPP TSG-SA WG6 Meeting #63	S6-244357

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244245)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0094 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to APIs exposed by SEALDD client side |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD |  |  |  |  |  | Date: |  |  | 2024-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Sdd_RegularTransmissionConnection API operations can be invoked by the SEALDD server in the SEALDD client (SEALDD enabled regular data transmission connection establishment based on policy, clause 9.2.2.3). However, the API definition in clause 9.2.4 only describes the Sdd_RegularTransmissionConnection API exposure by the SEALDD server. The Sdd_TransmissionQualityMeasurement API can be invoked by the SEALDD server in the SEALDD client (Data transmission quality measurement reported by SEALDD client, clause 9.7.2.3). However, the SEALDD enabled data transmission quality measurement notification (clause 9.7.3.3) only refers to the SEALDD server and the API definition in clause 9.7.4 only describes the Sdd_TransmissionQualityMeasurement API exposure by the SEALDD server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of 9.2.4 to cover the Sdd_RegularTransmissionConnection API request received by the SEALDD client. Update of 9.7.3.3 to refer to the SEALDD server. Update of 9.7.4 to cover the Sdd_TransmissionQualityMeasurement API request received by the SEALDD client. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect TS, missing server side exposure information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.4.1, 9.7.3.3, 9.7.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.2.4.1	General

Table 9.2.4.1-1 illustrates the APIs exposed by SEALDD server for regular connection establishment.

Table 9.2.4.1-1: List of SEALDD server APIs for data distribution

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_RegularTransmission | Request | Request/Response | VAL server |
| Sdd_RegularTransmissionConnection | Establish | Request/Response | SEALDD client, SEALDD server |
|  | Release | Request/Response | SEALDD client, SEALD server |
| Sdd_ConnectionStatusEvent | Subscribe | Subscribe/Notify | VAL server |
|  | Notify | Subscribe/Notify | VAL server |
|  | Update | Subscribe/Notify | VAL server |
|  | Unsubscribe | Subscribe/Notify | VAL server |

* * * Next Change * * * *

#### 9.7.3.3	SEALDD enabled data transmission quality measurement notification

Table 9.7.3.3-1 describes the information flow from the SEALDD server to the VAL server for notifying the transmission quality measurement reports.

Table 9.7.3.3-1: SEALDD transmission quality measurement notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| Transmission quality measurement reports list | M | The generated transmission quality results in SEALDD server or SEALDD client, as specified in Table 9.7.3.3-2. |

Table 9.7.3.3-2 describes the information elements for the transmission quality measurement reports list, provided by the SEALDD server after performing transmission quality measurement.

Table 9.7.3.3-2: SEALDD transmission quality measurement reports list

| Information element | Status | Description |
| --- | --- | --- |
| > Measurement ID | M | Measurement identifiers, e.g. latency, bitrate, packet loss rate, jitter |
| > VAL UE/user ID(s) | O | It indicates the VAL UE(s) or VAL user(s) under SEALDD measurement. For a single VAL UE/user, it can be omitted and the associated measurement values are for the single VAL UE/user. For multiple VAL UEs/users with reporting granularity set to individual UE, the associated measurement values are for individual VAL UE/user as indicated in this IE. For multiple VAL UEs/users with reporting granularity set to VAL UE/user group/list or all VAL UEs/users, the associated measurement values are aggregation for all VAL UEs/users or the VAL UE/user group/list and this IE includes the measured VAL UEs/users. |
| > Average measurement value | O | The average measurement value of measurement results |
| > Minimum measurement value | O | The minimum measurement value of measurement results |
| > maximum measurement value | O | The maximum measurement value of measurement results |
| > Standard deviation measurement value | O | Standard deviation measurement value of measurement results |
| > kPercentile measurement value | O | Indicates the kpercentile measurement value of measurement results |
| > Measurement period | O | Indicates the measurement period |
| > Timestamp | O | Indicates the timestamp of measurement results |

* * * Next Change * * * *

#### 9.7.4.1	General

Table 9.7.4.1-1 illustrates the APIs exposed by SEALDD server for data transmission quality measurement.

Table 9.7.4.1-1: List of SEALDD server APIs for transmission quality measurement

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_TransmissionQualityMeasurement | Subscribe | Subscribe/Notify | VAL server, SEALDD server |
|  | Update | Subscribe/Notify | VAL server, SEALDD server |
|  | Unsubscribe | Subscribe/Notify | VAL server, SEALDD server |
|  | Notify | Subscribe/notify | VAL server, SEALDD server |
|  | Query | Request/Response | VAL server, SEALDD server, NSCE server, ADAE server |

* * * END of Change * * * *