3GPP TSG-SA WG6 Meeting #63	S6-244648

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244274)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0207 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | API discovery update and clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, eCAPIF |  |  |  |  |  | Date: |  |  | 2024-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are several API invokers which interact with the CCF to obtain the different information of service APIs. For example, the application developers, application service provider may interact with CCF via the application management client to discover the interested service APIs, and the hosted application may interact the CCF to obtain the full informaiton especially the interface address to initiate the service API invocation. Currently, the existing single service API discover procedure supports the discovery of both the interested service APIs and the detailed service API information. However, such procedure lacks the clarifying how it is used by different API invokers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the service API discovery procedure and information flow to reflect that it may be used by different API invokers for their individual purpose. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The readers may be confused about when and which API invoker can use the service API discovery procedure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3, 8.7.1, 8.7.2.1, 8.7.2.2, 8.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************ 1st changes *************************/

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

API: The means by which an API invoker can access the service.

API invoker: The entity which invokes the CAPIF or service APIs.

API invoker profile: The set of information associated to an API invoker that allows that API invoker to utilize CAPIF APIs and service APIs.

API exposing function:  The entity which provides the service communication entry point for the service APIs.

API exposing function location: The location information (e.g. civic address, GPS coordinates, data center ID) where the API exposing function providing the service API is located.

API category: A name used to group service APIs by the service domain in which they are used (e.g., IoT, V2X). The CAPIF core function maintains a list of API category names.

API management function: The entity which enables the API provider to perform administration of the service APIs.

API publishing function: The entity that enables the API provider to publish the Service APIs information in order to enable the discovery of APIs by the API invoker.

Authorization function: The entity which issues access tokens to the API invokers after successfully authenticating the resource owner and obtaining authorization.

CAPIF administrator: An authorized user with special permissions for CAPIF operations.

Common API framework: A framework comprising common API aspects that are required to support service APIs.

Designated CAPIF core function: The CAPIF core function which is configured as the serving CAPIF core function for interconnection.

Network Slice Info: Network slice information of service API which include a list of network slice related identifiers, as described in clause 8.4 of 3GPP TS 23.435 [13]).

Northbound API: A service API exposed to higher-layer API invokers.

Onboarding: One time registration process that enables the API invoker to subsequently access the CAPIF and the service APIs.

Resource: The object or component of the API on which the operations are acted upon.

Resource owner: A UE user or an MNO subscriber capable of granting access to a protected resource related to the invoked API via resource owner function.

Resource owner function: The entity that enables the authorization for resource access and managing and revoking authorization for resource access.

Resource owner-aware northbound API access: An API invocation scenario where the API invoker needs an authorization from the resource owner.

Service API: The interface through which a component of the system exposes its services to API invokers by abstracting the services from the underlying mechanisms.

Serving Area Information: The location information for which the service APIs are being offered to.

CAPIF provider domain: A domain that contains an instance of CAPIF core function and may contain API provider domains and API invokers. The CAPIF provider could be a PLMN, SNPN or 3rd party. Throughout this document, PLMN trust domain is often used as the typical deployment of a CAPIF provider domain however SNPN trust domain or 3rd party trust domain are applicable as well.

PLMN trust domain: The entities protected by adequate security and controlled by the PLMN operator or a trusted 3rd party of the PLMN.

SNPN trust domain: The entities protected by adequate security and controlled by the SNPN operator or a trusted 3rd party of the SNPN. 3rd party trust domain: The entities protected by adequate security and controlled by the 3rd party.

Application management client: The application developers utilize the CAPIF APIs using an application management client as an API invoker to obtain the service APIs information to implement the application program. Such application programs are hosted on cloud, edge or on a UE.

Hosted applications: The hosted applications (which are programmed to utilize the service APIs) as an API invoker invoke the CAPIF APIs and service APIs as per the application business logic.

Channel Aggregator Platform: The Channel Aggregator aggregates the APIs from one or more southbound CAPIF providers with the intention to re-expose such APIs or expose value-added APIs developed using the APIs from the southbound CAPIF providers to the northbound side Application management client or Hosted applications. The Channel Aggregator Platform as API invoker invokes the CAPIF APIs and service APIs as per its business logic.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 32.240 [6] apply:

Offline charging

Online charging

# /************************ Next changes *************************/

8.7.1	General

The following procedure in this subclause corresponds to the architectural requirements on discover service APIs. This procedure may be invoked by different types of API invokers (i.e., application management client, hosted application) at different phases (i.e., hosted application development, test phase and hosted application running phase). Different API invokers utlize this procedure for different purpose:

-	service API discovery for interested service APIs with service API informaiton including e.g., API name, API provider name, API category.

-	service API discovery for the interface information (e.g. IP address, port number, URI) of a particular service API in case of the interface information is not provided in the service API information during hosted application running phase.

8.7.2	Information flows

8.7.2.1	Service API discover request

Table 8.7.2.1-1 describes the information flow service API discover request from the API invoker to the CAPIF core function.

Table 8.7.2.1-1: Service API discover request

| Information element | Status | Description |
| --- | --- | --- |
| API invoker identity information | M | Identity information of the API invoker discovering service APIs |
| Query information | O (NOTE 2) | Criteria for discovering matching service APIs (e.g. service API category, Serving Area Information (optional), preferred AEF location (optional), required API provider name (optional), UE IP address (optional), interfaces, protocols, Service KPIs (optional), and Network Slice Info (optional)). (see NOTE 1) |
| Service API identification | O (NOTE 2) | The identification information of the service API(s) for which the request is targeting |
| NOTE 1: It should be possible to discover all the service APIs. NOTE 2: Either "Query information" IE or " Service API identification" IE shall be present. |  |  |

8.7.2.2	Service API discover response

Table 8.7.2.2-1 describes the information flow service API discover response from the CAPIF core function to the API invoker.

Table 8.7.2.2-1: Service API discover response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the discovery of the service API information |
| Service API information (see NOTE 2) | O (see NOTE 1, NOTE 3) | List of service APIs corresponding to the request, including service API information such as service API name, API provider name (optional), service API category, communication type, description, Serving Area Information (optional), interface details (e.g. IP address, port number, URI) (optional), protocols, version, data format, Service KPIs (optional), and Network Slice Info (optional). |
| Interface information | O (NOTE 3) | The interface details (e.g. IP address, port number, URI) of the requested service API(s) |
| CAPIF core function identity information | O (see NOTE 1, NOTE 3) | Indicates the CAPIF core function serving the service API category provided in the query criteria |
| NOTE 1: The service API information or the CAPIF core function identity information or both shall be present if the Result information element indicates that the service API discover operation is successful. Otherwise both shall not be present. NOTE 2: If topology hiding s enabled for the service API, the interface details shall be the interface details of AEF acting as service communication entry point for the service API. NTOE 3: "Interface information" IE shall not be present when "Service API information" or "CAPIF core function identity information" is present. |  |  |

8.7.3	Procedure

Figure 8.7.3-1 illustrates the procedure for discover service APIs.

The service API discovery mechanism is supported by the CAPIF core function.

Pre-conditions:

1.	The API invoker is onboarded and has received an API invoker identity.

2.	The CAPIF core function is configured with a discovery policy information (e.g. to restrict discovery to category of APIs) for API invoker(s).

Figure 8.7.3-1: Discover service APIs

1.	The API invoker sends a service API discover request to the CAPIF core function. It includes the API invoker identity , either query information or the service API identification.

For service API discovery for interested service APIs, the query information is included.

For service API discovery for interface information, service API identification is included.

2.	Upon receiving the service API discover request, the CAPIF core function verifies the identity of the API invoker (via authentication).

If the query information is included, the CAPIF core function retrieves the stored service API(s) information from the CAPIF core function (API registry) as per the query information in the service API discover request. Further, the CAPIF core function applies the discovery policy and performs filtering of service APIs information retrieved from the CAPIF core function.

If the service API identification is included, the CAPIF core function retrieves the interface information of the requested service API(s).

3.	The CAPIF core function sends a service API discover response to the API invoker.

If the query information is included, the list of service API information is returned.

If the service API identification is included, the interface information is returned.

# /************************ Next changes *************************/

Annex F (informative):
Examples of API invoker roles in CAPIF

The figure F-1 provides examples of API invoker roles in CAPIF and illustrates the usage of CAPIF capabilities.

Figure F-1: Examples of API invoker roles in CAPIF