3GPP TSG-SA WG#106	SP-241849

Madrid, Spain, 10-13 December 2024	(was S2-240xxxx)

Source:	NEC

Title:	Discussion paper on remaining issue of KI#2 5GSAT_Ph3_ARCH

Document for:	Discussion/Approval

Agenda Item:	19.2

Work Item / Release:	5GSAT_Ph3-ARCH/Rel.19

Abstract of the contribution: Discusses the pending issues of KI#2 support of store and Forward and proposes away forward to handle this scenario in the exception sheet.

# 1.	Background

SA2#166 has addressed and captured the normative work for KI#2: Support of Store and Forward Satellite operation for small data transmission(Please see Annex for the agreed CRs). However, two crucial scenarios remain unaddressed:

- ESM Procedures:

  - Attach with PDN Connection: Necessary for split MME architectures to establish user sessions.

  - Standalone PDN Connection Establishment: Essential for specific use cases in split MME environments.

  - Other ESM procedure

- SMS over SGs Interface:

  - Required to support SMS transmission over the SGs interface, a key functionality for CIoT, NB-IoT user communication via SMS.

Addressing these two scenarios is imperative to:

- Enable User Data Transmission: Facilitate the transfer of user data to AF or SCEF in split MME architectures.

- Support SMS Functionality: Ensure SMS services operate correctly, in split MME configurations.

Without these enhancements, the overall feature of Store and Forward Satellite operation for small data transmission will be limited in split MME environments or most likely user data (IP and non-IP ) will not be supported.

During the study phase of KI#2: Support of Store and Forward Satellite operation for small data transmission, following things have been concluded it was concluded to have following two models are captured in normative CRs (Please see Annex for list of agreed CRs in Q4, 2024).

1) Model A: Split MME architecture

1.	eNB is onboard the satellite.

2.	MME is split into two functions:

a.	MME-onboard:	the MME part which is onboard the satellite. MME-onboard is in charge of (1) handling the S1 interface with the onboard eNB and (2) handling the NAS protocol signalling from/to UEs via the onboard eNB.

b.	MME-ground:	the MME part which is on the ground network. MME-ground is in charge of handling the rest of interfaces towards other CN functions (e.g. S6a towards HSS, SGd towards SMS-GMSC/IWMSC /SMS Router, T6a towards SCEF, T6ai towards IWF-SCEF, S11 towards SGW).

Figure1: "Split-MME" architecture for supporting S&F satellite operation for SMS and CP CIoT services

2) Model B: Full EPC in each satellite:

## Architecture and Principles of Operation

An example architecture of Model B is shown in Figure Y.3.2-1. Each satellite contains the functionally of an eNB plus a full EPC that can include an MME, SGW, PGW, HSS, E-SMLC, SMSC etc. Each satellite further includes an endpoint proxy function that emulates the behaviour of a real endpoint (e.g. an AF) from the perspective of a UE. There is also store and forward functionality on the ground that can be part of, or connected to, an NTN Gateway and that contains the proxy functionality.

NOTE:   This architecture does not support roaming.

Figure 2: Example Architecture of Model B

3. Relevant part from Agreed CR S2-2412903.

When an MME is operating in S&F Mode:

- If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, if the UE supports Store and Forward Satellite operation, the MME shall include a reject cause indicating the NAS rejection is due to S&F operation.

- If a UE indicates Store and Forward capability, the MME may provide to the UE a S&F Wait Timer when accepting or rejecting a NAS procedure.

# 2. 	Discussion

Following are the facts and conclusions based on the CRs agreed in SA2#166 (See Annex).

Fact 1: The MME in the satellite rejects the NAS procedure if the NAS procedure can’t be completed e.g. due to non-availability of user subscription or authentication vectors or other UE parameters. When the UE receives the S&F wait time, the UE will start the S&F wait timer and restart the NAS procedure again when the service link is available.

To support the Store and Forward procedure for split MME architecture, following principle is agreed and captured (Please refer CR S2-2412903).

When an MME is unable to complete a NAS procedure due to network congestion or other constraints, it will:

- Reject the NAS Procedure: The MME will send a rejection message to the UE.

- Provide S&F Wait Timer: If the UE has indicated support for Store and Forward (S&F) functionality, the MME will provide an S&F wait timer.

- Fetch Required Information: Once the feeder link is available, the MME will retrieve necessary information from the ground network (e.g., subscription data, authentication vectors) to complete the pending NAS procedure.

- UE Retries NAS Procedure: After the S&F wait timer expires, the UE will reinitiate the NAS procedure.

- MME Completes NAS Procedure: When the service link is available, the MME will process the restarted NAS procedure and complete it.

The outline of this procedure is described below.

Figure 3: Describing general approach of the conclusion.

Fact 2: The Attach procedure, tracking area update procedure and service request procedure capturing above conclusion is captured in the normative text in TS 23.401. Annex captures the basic outline of the split MME architecture (e.g. part of MME will be on satellite and part in the ground and how they synchronize) and it doesn’t describe any procedure part.

Fact 3: Normative work for EPS session management procedure including attach with PDN connection, standalone PDN connection and other UE or network-initiated session management procedure are not captured yet in the split MME Architecture.

Fact 4: UE is agnostic of two architectures define to support S&F feature i.e. the UE can’t know if it is attaching a network based on split MME.

Problem statement 1: Lack of ESM Support in Split MME Architectures.

The absence of normative work for ESM procedures in split MME architectures, specifically for:

- Attach with PDN Connection: Establishing initial user sessions.

- Standalone PDN Connection Establishment: Supporting specific use cases without initial attach.

Prevents the successful transmission of user data (IP or non-IP) to AF or SCEF.

Proposal 1: It is captured in the exception sheet that the SA2 will work on normative part of the ESM procedure including attach with PDN connection and standalone PDN connection establishment for split MME architecture case in Q1, 2025.

Fact 4: In the EPS the SMS can be delivered using SMS in MME or SMS over SGs for split MME case.

Fact 5: In the MME split architecture mode, a procedure to establish an SGs interface association to send an SMS is not defined yet.

Fact 6: The IMS services (including IMS voice and SMS) can’t be supported using split MME architecture as the user plane data can’t be established for the split MME architecture.

Problem statement 2: SMS Limitations in Split MME Architectures.

Operators relying solely on the SGs interface for SMS transmission will face challenges in supporting SMS functionality in split MME architectures. This limitation hinders the delivery of SMS messages in these environments.

Proposal 2: It is captured in the exception sheet that the SA2 will work on normative part of to support transmission of the SMS over SGs interface.

# 3.	Conclusion

It is proposed to capture proposal 1 and proposal 2 in the exception sheet for 5GSAT_PH3-ARC. A contribution SP-241850 has been submitted capturing proposal 1 and proposal 2 in the exception sheet of 5GSAT_PH3-ARC.

# 1. Annex

2. References:

Agreed CRs in SA2#166.

| 19.1.2 | S2-2412903 | CR | 23.401 CR3801R18 (Rel-19, 'B'): Introduction of Store and Forward feature in EPC |
| --- | --- | --- | --- |
| 19.1.2 | S2-2413021 | CR | 23.401 CR3824R6 (Rel-19, 'B'): Procedures for Store and Forward Satellite Operations |
| 19.1.2 | S2-2412668 | CR | 23.682 CR0495R9 (Rel-19, 'B'): Monitoring event for S&F |
| 19.1.2 | S2-2411421 | CR | 23.401 CR3815R11 (Rel-19, 'B'): Annex update of Full EPC Store and Forward Satellite Operation |

3. Basic design principle Attach without PDN connection procedure for UE and network supporting S&F operation mode.

Figure 4: Attach procedure without PDN connection establishment

#### 4. Attach procedure with default PDN Connection establishment (Subclause 5.3.2.1	E-UTRAN Initial Attach of TS 23.401)

Figure 5:  Figure 5.3.2.1-1: Attach procedure of TS 23.401

## 5. Combined attach procedure (Attach procedure sub clause 5.2 of TS 23.272)

The attach procedure for the CS fallback, SMS over SGs or "SMS in MME" in EPS is realized based on the combined GPRS/IMSI Attach procedure specified in TS 23.060 [3].

Figure 6: Figure 5.2-1: Attach Procedure of TS 23.272