3GPP TSG-SA Meeting #106	SP-241970

# Madrid, Spain, December 10-13, 2024	 	(revision of SP-241838)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5148 | rev | 5 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Event exposure service enhancement to support signalling storm data collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Huawei, HiSilicon, Nokia, Oracle, SK Telecom, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon, SK Telecom, Verizon |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-12-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA2#165, S2-2410976 is agreed that NWDAF will collect input data from NFs for Signalling Storm Analytics as described in clause 6.x.2, but the NFs’ corresponding Nnf_EventExposure service does not contain some new content in input data. Thus, the event exposure service enhancement is needed to support NWDAF collecting new input data. In addition, for NRF, it proposes to reuse the Nnrf_NFManagement service for NWDAF collect NF/NF service load information, NF/NF service capability informartion, etc. To avoid frequent signalling, it proposes to notify in periodic mode. For SCP event exposude, see considerations in S2-2411388 SMF and AMF can take mitigation actions based on a subscription to signalling storm analytics. As agreed in the conclusions for key issue 4 in TR 23.700-84, the SCP should be enabled to take actions to mitigate an observed or predicted signaling load caused by some other NF when being informed via analytics about it Rev3 -> Rev4/5: 1, Align the name of events exposed by PCF/SMF/SCP. Different terms “Abnormal Signaling characteristics” “Signalling storm” were used in different places for same event, and should be aligned. Though the event is used for detecting anormaly, but it is not appropriate to call the event itself as an abnormal signalling characteristics, since the data source NF only provides the signaling related information to NWDAF, the information may or may not indicate signaling storm, it is NWDAF who detects if there will be signaling storm or not. it is better to use a generic event ID e.g. Service Signalling characteristics. 2, Alignment on “distribution of reasons for failed responses” In the 9th change, the “distribution of reasons for failed responses” is misaligned with the following NOTE 5 in the approved S2-2412939: NOTE 5: Distribution of the failed responses, e.g how many time-outs vs how many server errors vs how many consumer errors. It’s better to align the explanation with TS 23.288. 3, Remove duplicated sentence from NOTE X in the 9th change and othe editorial changes. |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add signalling storm related input data as event exposure in Namf_EventExposure service; - Add signalling storm related input data as evemt exposure in Npcf_EventExposure service. - Add signalling storm related input data as event exposure in Nsmf_EventExposure service; - Add signalling storm related input data as event exposure in Naf_EventExposure service; - Add signalling storm related input data as NF status in Nnrf_NFManagement service. - Define SCP Event Exposure service to enable SCP to help detecting a signaling storm - SMF and AMF can take mitigation actions based on a subscription to signalling storm analytics. - SCP can take mitigation actions based on a subscription to signalling storm analytics. Rev3 -> Rev4/5: 1, Align the name of the event of PCF/SMF/SCP. 2, Align the explanation of distribution of reasons with approved S2-2412939. 3, Remove duplicated sentence from NOTE X in the 9th and other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for collecting input data from AMF, PCF, SMF, AF, SCP and NRF. Mitigation actions are also not documented. |  |  |  |  |  |  |  |  |
|  |  | s |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1, 4.17.9, 5.2.2.3.1, 5.2.5.7.1, 5.2.7.2.5, 5.2.7.2.6, 5.2.8.3.1, 5.2.19.2.1, 5.2.19.2.2, 5.2.19.2.4, new 5.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

### 4.3.1	General

Clause 4.3 defines the Session Management related procedures. It refers to clause 4.4 for the N4 interactions.

As defined in clause 5.6.3 of TS 23.501 [2], considering the case of Home Routed PDU Session, the NAS SM information processing by SMF considers following kind of NAS SM information:

-	Information that both the V-SMF and H-SMF process: indication of the nature of the NAS SM signalling (e.g. PDU Session Establishment Request), PDU Session Type, Session-AMBR, UE addressing information (allocated IPv4 address, interface identifier).

-	Information that is not visible to the V-SMF, only processed by the H-SMF: SSC mode, Protocol Configuration Options, SM PDU DN Request Container, QoS Rule(s) and QoS Flow level QoS parameters if any for the QoS Flow(s) associated with the QoS rule(s).

NOTE 1:	"Information that is not visible to the V-SMF" refers to information that the V-SMF is to relay between the UE and the H-SMF (and that it can store in CDR) but that the V-SMF is not assumed to process otherwise.

The NAS SM information processing split between V-SMF and H-SMF is transparent to the UE.

Both V-SMF and H-SMF process information interpreted by the AMF as the PDU Session ID, the DNN, the S-NSSAI (with values for the Serving PLMN and HPLMN processed by the V-SMF and with a value for the HPLMN processed by the H-SMF).

In the case of Home Routed PDU Session the H-SMF provides also the V-SMF with the IPv6 Prefix allocated to the PDU Session.

NOTE 2:	IPv6 Prefix allocated to the PDU Session is provided to allow the V-SMF fulfilling regulatory requirements for data storage in the visited country.

In non roaming and LBO cases the SMF processes all NAS SM information.

In HR roaming scenarios, in order to support SM features only requiring support from the H-SMF without impacting the V-SMF, as specified in detail in TS 29.502 [36]:

-	The V-SMF transfers NAS SM information, which is not visible to the V-SMF, in a container towards the H-SMF;

-	The V-SMF transfers NAS SM information which it does not comprehend (unknown IEs or IEs with an unknown value not set to "reserved" according to the release to which the V-SMF complies), in a different container towards the H-SMF;

-	The H-SMF transfers NAS SM information which the V-SMF does not need to interpret, in one container towards the V-SMF;

-	The V-SMF appends unknown NAS SM information received in the N16 container at the end of the NAS SM message it sends to the UE.

AMF and SMFs may subscribe at the NWDAF for Signalling Storm Analytics to obtain information related to signalling storms caused by UE or NF signalling as specified in TS 23.288 [50] and may decide to take mitigations actions as described in the present Clause 5.19 of TS 23.501 [2], based on the received analytics information.

# * * * * 2nd change * * * *

### 4.17.9	Delegated service discovery when NF service consumer and NF service producer are in same PLMN

Figure 4.17.9-1: Delegated NF service discovery when NF service consumer and NF service producer are in same PLMN

1.	The NF service consumer intends to communicate with an NF service producer. The NF service consumer sends the service request to an SCP. The request may include discovery and selection parameters necessary to discover and select a NF service producer instance. The discovery and selection parameters are included in the request by the NF service consumer in a way that the SCP does not need to parse the request body.

2.	The SCP may perform discovery upon the request either by interacting with an NRF using Nnrf_NFDiscovery service NRF or may use information collected during the previous interactions with an NRF (by the Nnrf_NFDiscovery service or Nnrf_NFManagement_NFStatusNotify service operation). The SCP together with the NRF authorizes the request. The SCP may subscribe at the NWDAF for analytics related to abnormal network behaviour/signalling storm, and then obtain related information from the NWDAF as specified in TS 23.288 [50]. This subscription and related notifications may already have occurred before step 1, and the SCP may cache such information. The SCP selects the target NF service producer and may take the analytics information obtained from the NWDAF into consideration for that selection.

Editor’s Note: The following is FFS: the SCP can select target NF service producers displaying no abnormal behaviour or reject request targeting NF service producers displaying abnormal behaviour with appropriate error information about the abnormal behaviour to enable NF service consumers to take mitigation actions, and the SCP can also updates its profile at the NRF to indicate that certain NF sets or SCP domains are no longer reachable through the SCP.

NOTE 1:	If the discovery and selection parameters in the request include a UE identity, e.g. SUPI or IMPI/IMPU, the SCP can resolve the requested NF's Group ID corresponding to the UE identity and then invoke the Nnrf_NFDiscovery service, as defined in clause 6.3.1 of TS 23.501 [2].

3.	If the NF service consumer is authorized to communicate with the NF service producer, the SCP forwards the request to the selected NF service producer according to the configuration of the Network Slice, e.g. the expected NF instances are only reachable by NFs in the same network slice.

Editor´s Note: The following is FFS: The SCP may include information related to current or predicted abnormal network behavior of the NF service consumer to enable the NF service producer to take mitigation actions.

4.	The NF service producer sends a response to the SCP. If the request in step 3 creates a resource in the NF service producer, such as depicted in Figure 4.17.9-1, the NF service producer responds with resource information identifying the created resource.

5.	The SCP routes the response to the NF service consumer.

Editor´s Note: The following is FFS: The SCP may include information related to current or predicted abnormal network behavior of the NF service producer to enable the NF service consumer to take mitigation actions.

If the NF service consumer receives a resource address, it uses it for subsequent requests regarding the concerned resource. Otherwise, the procedure ends here.

6.	On a subsequent operation on the created resource, the NF service consumer addresses the resource via the resource address returned by the NF service producer at step 4.

7.	The SCP resolves the NF service producer address and selects a target NF service producer instance. The SCP then routes the request to the selected NF service producer instance. See the clause 6.3.1.0 of TS 23.501 [2] for the details of selection of a target NF service producer instance by SCP.

8.	The SCP delivers the request to the NF service producer.

9.	The NF service producer sends a response to the SCP. The NF service producer may respond with an updated resource information different to the one received in the previous response.

10.	The SCP sends a response to the NF service consumer. If the resource information was updated, the NF service consumer uses the received resource information for subsequent operations (requests) on the resource.

NOTE 2:	In the similar manner of handling the resource information, a binding indication may be also provided by the NF service producer and used for the subsequent requests of the NF service consumer. See more details in clause 4.17.12.

# * * * * 3rd change * * *

#### 5.2.2.3	Namf_EventExposure service

##### 5.2.2.3.1	General

Service description: This service enables an NF to subscribe and get notified about an Event ID.

Following UE access and mobility information event are considered (Event ID is defined in clause 4.15.1 and Table 4.15.3.1-1):

-	Location Report (TAI, Cell ID, N3IWF/TNGF node, UE local IP address and optionally UDP source port number);

-	UE moving in or out of a subscribed "Area Of Interest" as described in clauses 5.3.4.4 and 5.6.11 of TS 23.501 [2];

-	Number of UEs served by the AMF and located in "Area Of Interest";

-	Time zone changes (UE Time zone);

-	Access Type changes (3GPP access or non-3GPP access);

-	Registration state changes (Registered or Deregistered);

-	Connectivity state changes (IDLE or CONNECTED);

-	UE loss of communication;

-	UE reachability status;

-	UE indication of switching off SMS over NAS service;

-	Subscription Correlation ID change (implicit subscription);

-	UE Type Allocation code (TAC);

-	Frequent mobility re-registration;

-	Subscription Correlation ID addition (implicit subscription);

-	User State Information in 5GS, as described in clause 5.4.4 of TS 23.632 [68];

-	UE access behaviour trends (see clause 4.15.4.2);

-	UE location trends (see clause 4.15.4.2);

-	Assigned Trajectory (see clause 5.13 of TS 23.256 [80]);

-	Total number of Mobility Management transactions:

-	The Total number of Mobility Management transactions is used to collect the number of MM transactions of a SUPI or Internal Group ID, for example Dispersion Analytics as specified in TS 23.288 [50]. The Total number of transactions is incremented when the NAS signalling transactions from Authentication, Registration, De-Registration, Service Request and UE Configuration Update procedures is completed. Only the periodic reporting mode applies.

For support signalling storm analytics, total signalling measures is considered:

-	Total Signalling Measurements: The event is used to collect the Request type and the number of requests corresponding to the request type from UE/NG-RAN, Request type and number from NF, Public Warning information and Frequent Mobility Registration Update, State transition information, Timer information, etc. The detail of the total signalling measures refers to signalling storm analytics as specified in clause 6.x of TS 23.288 [50].

In addition to UE access and mobility information event, AMF exposes the "S-NSSAIs per TAI mapping" event providing, per TAI, the related access type and list of supported S-NSSAIs (including indication of S-NSSAIs restricted by AMF). The Event Consumer may use as target of event reporting a list of TAIs, or "any TAI" and may use event filter information including a list of "S-NSSAIs". Whenever there is a change in supported S-NSSAIs (including indication of S-NSSAIs restricted by AMF) for a TAI, the event notification is generated with the updated information.

Event Filters are used to specify the conditions to match for notifying the event (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides some examples on how the conditions to match for event reporting can be specified for various Event IDs for AMF exposure.

NOTE:	The conditions to match can be set based on AMF-associated expected UE Behaviour parameter(s) to only notify the event when the UE's behaviour deviates from its expected UE behaviour as described in TS 23.288 [50].

Table 5.2.2.3.1-1: Example of Event Filters for AMF exposure events

| Event ID | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| Location Report | <Parameter Type = LocationFilter, Value = TA1> |
| UE moving in or out of Area of Interest | <Parameter Type = TAI, Value = TA1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> <Parameter Type = NSI ID, Value = NSI ID1> <Parameter Type = PRA ID, Value = PRA ID value> <Parameter Type = RAN Node ID, Value = RAN Node ID value> <Parameter Type = Cell ID, Value = Cell ID value> <Parameter Type = RAN timing synchronization status change event > (NOTE 1) (NOTE 2) <Parameter Type = Adjust AoI based on RA, Value = Yes or No> to indicate that AMF may adjust the received AoI depending on UE's current Registration Area. Absence of this parameter in the request is interpreted as "AoI remains unchanged". <Parameter Type = Notify the consumer considering UE identity, Value = List of SUPIs or Internal Group ID > The parameter may be included when the request is targeted to Any UE. Absence of this parameter in the request is interpreted as "AMF reports an event regardless to UE's identity triggering the event". (NOTE 2) <Parameter Type = Notify the consumer considering DNN/S-NSSAI, Value = DNN and/or S-NSSAI> Absence of this parameter in the request is interpreted as "AMF reports the event without checking to which DNN/S-NSSAI the UE has PDU sessions established". (NOTE 2) |
| Access Type | <Parameter Type=AN Type, Value=3GPP Access"> |
| Location | <Parameter Type=TAI, Value=wildcard> (to report any TAI change) |
| Location | <Parameter Type=TAI Value=abnormal> (to report only when the TAI deviates from expected values based on Expected UE Moving Trajectory). |
| Assigned Trajectory | <Parameter Type = Notify when the UE deviates from the assigned trajectory, Value=TAI, Scheduled Time in a particular TA and/or Height> (to report only when the UE deviates from the assigned values of TAI, Scheduled Time in a particular TA and/or moves outside the altitude range based on the assigned flight plan). The assigned trajectory may consist of multiple segments identified by geographical areas that, for instance, are served by different UAV Service Suppliers. In this case, the information of each segment shall be provided. (NOTE 4) |
| Reachability Filter | Applicable to the event UE reachability. Value = UE reachability status change or UE reachable for DL traffic. Absence of this parameter in UE reachability event request is interpreted as "UE reachability status change". |
| Total number of Transactions | <Parameter Type = TAI, Value = TA1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| Number of UEs present in a geographical area | <Parameter Type = UE Type, Value = Aerial UE> <Parameter Type = PDU session status, Value = PDU session established for DNN subject to aerial services> <Parameter Type = LCS broadcasting assistance data type, Value = A bitmap indicates broadcast location assistance data types for which the UE is subscribed to receive ciphering keys to decipher broadcast assistance data (see Table 7.1-3 in clause 7.1 of TS 23.273 [51] and clause 6.1.6.2.72 of TS 29.503 [52])> (NOTE 3) |
| NOTE 1: The Parameter Type = RAN timing synchronization status change event can be present with Parameter Type defining a RAN Node ID. NOTE 2: The use of event filter parameters is described in clause 5.3.4.4 of TS 23.501 [2]. NOTE 3: The mapping relationship between the LCS broadcasting assistance data element and corresponding positioning SIB type is described in clause 7.2 of TS 37.355 [56]. NOTE 4: Filter information for the events is provided as a set of TAI, Scheduled Time and/or Height values for each of the trajectory segments assigned to the UE/UAV, see clause 5.13 of TS 23.256 [80]. Editor's note: How RAN or UAV/UE provides height information to AMF is FFS. |  |

The following service operations are defined for the Namf_EventExposure service:

-	Namf_EventExposure_Subscribe.

-	Namf_EventExposure_UnSubscribe.

-	Namf_EventExposure_Notify.

# * * * * 4th change * * * *

#### 5.2.5.7	Npcf_EventExposure service

##### 5.2.5.7.1	General

Service description: This service enables an NF to subscribe and get notified about PCF events for a group of UE(s) or any UE accessing a combination of (DNN, S-NSSAI).

The events can be subscribed by a NF consumer are described in clause 6.1.3.18 of TS 23.503 [20].

When the consumer NF is the NWDAF, the event ID “Service Signalling characteristics” including Request type and number of requests corresponding to the request type from NF, Unexpected operational status indicator, etc. is used to collect data for NF related information from PCF for Signalling Storm Analytics as specified in clause 6.x.2 of TS 23.288 [50].

The following service operations are defined for the Npcf_EventExposure service:

-	Npcf_EventExposure_Subscribe.

-	Npcf_EventExposure_UnSubscribe.

-	Npcf_EventExposure_Notify.

# * * * * 5th change * * * *

##### 5.2.7.2.5	Nnrf_NFManagement_NFStatusSubscribe service operation

Service Operation name: Nnrf_NFManagement_NFStatusSubscribe.

Description: Consumer can subscribe to be notified of the following:

-	Newly registered NF along with its NF services.

-	Updated NF profile.

-	Deregistered NF.

Inputs, Required: callback URI.

Inputs, Optional:

-	PLMN ID of the target NF/NF service, in the case of the subscription to the status of NF/NF service instance(s) in home PLMN from the serving PLMN, or PLMN ID and NID in the case of SNPN (see clause 5.30.2.9.3 of TS 23.501 [2]).

-	Home Network Identifier: PLMN ID in the case of PLMN, PLMN ID + NID in the case of SNPN. Optionally, some NFs may additionally include a Home Network Identifier (including the identification of the CH with AAA Server or DCS with AAA Server) in the form of a realm e.g. in the case of access to an SNPN using credentials owned by CH with AAA Server or in the case of SNPN Onboarding using a DCS with AAA Server.

-	Validity Time, in case to indicate the time instant after which the subscription becomes invalid.

-	For updated NF profile subscription, conditions that trigger a notification from NRF. Includes monitored attributes in the NF profile (changes trigger a notification) or unmonitored attributes in the NF profile (changes do not trigger a notification).

-	The following parameters are mutually exclusive:

-	NF Type (if NF status of a specific NF type is to be monitored).

-	NF Instance ID or NF Instance ID list (if NF status of a specific NF instance or a list of NF instance is to be monitored).

-	NF Service name (if NF status for NF which exposes a given NF service is to be monitored).

-	NF Set (if NF status of a set of NF Instances belonging to a certain NF Set is to be monitored).

-	NF Service Set (if the status of a set of NF Service Instances belonging to a certain NF Service Set is to be monitored).

-	NF Group (if the NF status of NF Instances identified by a NF (UDM, AUSF, PCF, BSF, CHF, HSS or UDR) Group Identity is to be monitored).

-  NF load status information, capacity and priority information of NF and NF Services, etc as described in Table 6.x.2-2 and Table 6.x.2-5 of TS 23.288 [50].

-	SCP Domain (if the status of NF or SCP instances belonging to a certain SCP domain is to be monitored).

-	For the UPF Management defined in clause 4.17.6: UPF Provisioning Information as defined in that clause.

-	For AMF, Consumer may include list of GUAMI(s), or AMF Set or AMF Region, or TAIs.

-	For SMF, Consumer may include list of TAIs.

-	S-NSSAI(s) and the associated NSI ID(s) (if available).

-	For NWDAF, Consumer may include:

-	Analytics ID(s) (possibly per service).

-	TAI(s).

-	NF Set ID, NF Type of NF data sources.

-	Analytics aggregation capability and/or Analytics metadata provisioning capability.

-	ML Model Accuracy checking capability.

-	Analytics Accuracy checking capability.

-	Roaming exchange capability.

-	For NWDAF containing MTLF, Consumer may also include the ML model Filter information parameters S-NSSAI(s) and Area(s) of Interest for the trained ML model(s) per Analytics ID(s) and ML Model Interoperability indicator per Analytics ID(s), if available (see clause 5.2 of TS 23.288 [50]).

-	For NWDAF containing MTLF with Federated Learning (FL) capability, Consumer may include FL capability type (i.e. FL client and/or FL server, if available), Time Period of Interest, if available (see clause 5.2 of TS 23.288 [50]).

Details about NWDAF discovery and selection are described in clause 6.3.13 of TS 23.501 [2].

-	For ADRF, Consumer may include:

-	Data and analytics storage and retrieval capability.

-	ML model storage and retrieval capability.

-	For NEF, Consumer may include Event ID(s) provided by AF.

-	For DCCF, Consumer may include:

-	TAI(s).

-	NF type of the NF data sources.

-	NF Set ID of the NF data sources.

-	Support for relocation of data subscription.

Details about NWDAF discovery and selection are described in clause 6.3.19 of TS 23.501 [2].

Outputs, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Validity Time.

Outputs, Optional: None.

NOTE:	Alternatively, other means such as OA&M can also be used to subscribe for NF status.

##### 5.2.7.2.6	Nnrf_NFManagement_NFStatusNotify service operation

Service Operation name: Nnrf_NFManagement_NFStatusNotify.

Description: NRF notifies subscribed consumers of the following:

-	Newly registered NF along with its NF services.

-	Updated NF profile.

-	Deregistered NF.

Inputs, Required: Notification Event type; NF instance ID; for newly registered NF, NF profile; for updated NF, NF profile or NF profile changes.

NOTE:	See clause 6.1.6.3.6 of TS 29.510 [37] for the detailed use of Notification Event type.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

# * * * * 6th change * * * *

#### 5.2.8.3	Nsmf_EventExposure Service

##### 5.2.8.3.1	General

Service description: This service provides events related to PDU Sessions towards consumer NF. The service operations exposed by this service allow other NFs to subscribe and get notified of events happening on PDU Sessions. The following are the key functionalities of this NF service.

-	Allow consumer NFs to Subscribe and unsubscribe for an Event ID on PDU Session(s);

-	Allow the NWDAF to collect data for network data analytics from SMF as specified in TS 23.288 [50] and from UPF as specified in clause 4.15.4.5;

-	Notifying events on the PDU Session to the subscribed NFs; and

-	Allow consumer NFs to acknowledge or respond to an event notification.

The following events can be subscribed by a NF consumer (Event ID is defined in clause 4.15.1):

-	UE IP address / Prefix allocation/change: The event notification may contain a new UE IP address / Prefix or an indication of which UE IP address / Prefix has been released.

-	PDU Session Establishment and/or PDU Session Release.

The event notification may contain following information:

-	PDU Session Type.

-	DNN.

-	UE IP address/Prefix.

-	UP path change: a notification corresponding to this event is sent when the UE IP address / Prefix and / or DNAI and /or the N6 traffic routing information has changed.

The event notification may contain following information:

-	the type of notification ("EARLY" or "LATE").

-	for both the source and target UP path between the UE and the DN, the corresponding information is provided when it has changed:

-	DNAI.

-	UE IP address / Prefix.

-	N6 traffic routing information.

-	Candidate DNAI(s) for the PDU Session.

-	Change of common EAS.

NOTE 1:	UP path change notification, DNAI and N6 traffic routing information are further described in clause 5.6.7 of TS 23.501 [2].

-	QoS Monitoring: the event notification may contain the QoS Monitoring report for the QoS parameter(s) to be measured defined in clause 5.45 of TS 23.501 [2]. Implicit subscription of the PCF on behalf of the NEF/AF as part of setting PCC rule(s) may trigger SMF to send this event notification.

-	Change of Access Type; The event notification contains the new Access Type for the PDU Session. For MA PDU Session the Change of Access Type may include two Access Type information that the user is currently using.

-	Change of RAT Type; the event notification contains the new RAT Type for the PDU Session.

-	PLMN change; The event notification contains the new PLMN Identifier for the PDU Session and may indicate:

-	whether local traffic offload is possible, i.e. mobility of the PDU session either towards HPLMN or towards a VPLMN where HR-SBO is supported and allowed; and

-	DNN and S-NSSAI of HPLMN.

-	Change of Satellite backhaul category; The event notification contains the new Satellite backhaul category for the PDU session.

-	Downlink data delivery status. The event notification contains the status of downlink data buffering in the core network including:

-	First downlink packet per source of the downlink IP traffic in extended buffering and Estimated maximum wait time.

-	First downlink packet per source of the downlink IP traffic discarded.

-	First downlink packet per source of the downlink IP traffic transmitted after previous buffering and/or discarding of corresponding packet(s).

-	QFI allocation: The event notification is sent whenever a new QoS Flow is established and contains:

-	If the Target of Event Reporting is a PDU Session and the QoS Flow is associated with this PDU Session, both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). The 5QI corresponding to the QoS Flow and the DNN, S-NSSAI corresponding to the PDU Session are also sent.

-	If the Target of Event Reporting is a SUPI and the PDU Session is associated with this SUPI, both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). The 5QI corresponding to the QoS Flow and the DNN, S-NSSAI corresponding to the PDU Session are also sent.

-	If the Target of Event Reporting is an Internal-Group-Id and the PDU Session is associated with this Internal-Group-Id (i.e. the PDU Session belongs to a UE belonging to this Internal-Group-Id), both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). The 5QI corresponding to the QoS Flow and the DNN, S-NSSAI, PDU Session ID, SUPI corresponding to the PDU Session are also sent.

-	If the Target of Event Reporting is any UE, both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). The 5QI corresponding to the QoS Flow and the DNN, S-NSSAI, PDU Session ID, SUPI corresponding to the PDU Session are also sent.

NOTE 2:	When the consumer NF is the NWDAF, the event QFI allocation is used to collect data for analytics as specified in TS 23.288 [50].

-	Total number of Session Management transactions:

-	The total number of Session Management transaction is used to collect the number of SM transactions of a SUPI or Internal Group ID, for example Dispersion Analytics as specified in TS 23.288 [50]. The transaction count is incremented when the NAS transactions from PDU Session Establishment, PDU Session Authentication, PDU Session Modification and PDU Session Release procedures is concluded. Only the periodic reporting mode applies.

-	Information on PDU Session for WLAN (i.e. Access Type is Non-3GPP and RAT Type is TRUSTED_WLAN).

-	User plane status information: The event notification contains:

-	PDU Session ID.

-	User Plane Inactivity Timer (as specified in TS 29.244 [69]).

-	PDU Session status (activated, deactivated).

NOTE 3:	When the consumer NF is the NWDAF, the event user plane status information is used to collect data for UE Communication analytics as specified in TS 23.288 [50].

-	Session Management Congestion Control Experience for PDU Session: The event notification contains the data related to Session Management Congestion Control experience per PDU Session as described in TS 23.288 [50].

-	UE session behaviour trends (see clause 4.15.4.3);

-	UE communications trends (see clause 4.15.4.3);

-	UP with redundant transmission: the event notification indicates if redundant transmission (see clause 5.33.2.2 of TS 23.501 [2]) has been activated or not for the PDU session;

-	User Data Usage Measures (see clause 4.15.4.5): SMF conveys the subscription to UPF on behalf of the consumer. Consumer receives the events directly from UPF. For certain UE(s), the SMF conveys the subscription to I-SMF on behalf of the consumer, and the I-SMF conveys the subscription to UPF on behalf of SMF. Consumer receives the events directly from UPF; and

-	User Data Usage Trends (see clause 4.15.4.5): SMF conveys the subscription to UPF on behalf of the consumer. Consumer receives the events directly from UPF. For certain UE(s), the SMF conveys the subscription to I-SMF on behalf of the consumer, and the I-SMF conveys the subscription to UPF on behalf of the SMF. Consumer receives the events directly from UPF.

-  Service Signalling characteristics: The event notification includes Request type and number from NF, Number of receiving Session Report from UPFs, State transition information, Timer information, Unexpected operational status indicator, etc. from SMF for Signalling Storm Analytics as specified in clause 6.x.2 of TS 23.288 [50].

When the consumer NF is the NWDAF, the event Information on PDU Session for WLAN is used to collect data for WLAN performance analytics as specified in TS 23.288 [50].

When the consumer NF is the NWDAF, the event Session Management Congestion Control Experience for PDU Session is used to collect data for Session Management Congestion Control Experience analytics as specified in TS 23.288 [50].

When the consumer NF is the NWDAF, the events QoS Monitoring, User Data Usage Measures and User Data Usage Trends are used to collect data from UPF for analytics as specified in clause 4.15.4.5 and in TS 23.288 [50]. SMF conveys the subscription to UPF on behalf of the NWDAF or the SMF conveys the subscription to I-SMF on behalf of the NWDAF and the I-SMF conveys the subscription to UPF on behalf of SMF.

The consumer NF may request to subscribe the UPF exposure events (including event ID of exposed UPF event of QoS monitoring, User Data Usage Measures and User Data Usage Trends) via SMF indirectly by Nsmf_EventExposure. After receiving this subscription request, the SMF does a third-party subscription onto UPF on behalf of this consumer. The consumer should also provide the subscribed UPF event to SMF.

Event Filters are used to specify the conditions to match for notifying the events (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides as an example how the conditions to match for event reporting can be specified for various Event IDs for SMF exposure.

Table 5.2.8.3.1-1: Example of Event Filters for SMF exposure events

| Event ID for SMF exposure | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| DNAI Change | None |
| Candidate DNAI(s) has changed | None |
| PDU Session Release | <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| PDU Session Establishment | <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| QoS Monitoring | <Parameter Type = S-NSSAI, Value = S-NSSAI1> <Parameter Type = DNN, Value = DNN1> <Parameter Type = Application Identifier, Value = Application Identifier1> <Parameter Type = AoI, value = AoI1> <Parameter Type = UPF Id, value = UPF Id1> <Parameter Type = DNAI, value = DNAI1> |
| QFI allocation | <Parameter Type = DNN, Value = DNN1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| QFI allocation | <Parameter Type = Application Identifier, Value = Application Identifier1> |
| Transaction Count | <Parameter Type = TAI, Value = TA1> (NOTE) <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| User plane status information | <Parameter Type = Application Identifier, Value = Application Identifier1> <Parameter Type = SUPI, Value = SUPI1> |
| Information on PDU Session for WLAN | <Parameter Type = Access Type, Value = Non-3GPP> && <Parameter Type = RAT Type, Value = TRUSTED_WLAN> |
| Session Management Congestion Control Experience for PDU Session | <Parameter Type = DNN, Value = DNN1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| UP with redundant transmission | <Parameter Type = DNN, Value = DNN1> |
| User Data Usage Measures | <Parameter Type = S-NSSAI, Value = S-NSSAI1> <Parameter Type = DNN, Value = DNN1> <Parameter Type = Application Identifier, Value = Application Identifier1> (NOTE 2) <Parameter Type = Flow Info, Value = Packet Filter Set1> (NOTE 2) <Parameter Type = AoI, value = AoI1> <Parameter Type = SSID/BSSID, Value = SSID/BSSID1> |
| User Data Usage Trends | <Parameter Type = S-NSSAI, Value = S-NSSAI1> <Parameter Type = DNN, Value = DNN1> <Parameter Type = Application Identifier, Value = Application Identifier1> (NOTE 2) <Parameter Type = Flow Info, Value = Packet Filter Set1> (NOTE 2) <Parameter Type = AoI, value = AoI1> |
| NOTE 1: Optionally the SMF can fetch the location information from the AMF but transaction information correlation at the location can also be achieved without it and through transaction information associated with the requested time period, which corresponds to the UE's time span at the location of interest. NOTE 2: These Parameters are exclusive and only one of them can be provided. |  |

The target of SMF event reporting may correspond to a PDU Session ID, an UE ID (SUPI), an Internal Group Identifier, an indication that any UE is targeted (e.g. on a specific DNN), or an indication that any PDU session is the target.

When acknowledgment is expected the SMF also provides Notification Correlation Information to the consumer NF in the event notification.

The consumer NF may provide the following event-specific information when acknowledging an event notification:

-	For UP path change event:

-	N6 traffic routing information related to the target DNAI.

NOTE 4:	Acknowledgement to a UP path change event notification is further described in clause 5.6.7 of TS 23.501 [2].

# * * * * 7th change * * * *

#### 5.2.19.2	Naf_EventExposure service

##### 5.2.19.2.1	General

Service description: This service enables consumer NF to subscribe and get notified of events.

The events can be subscribed by a NF consumer are described in TS 23.288 [50].

The following service operations are defined for the Naf_EventExposure service:

-	Naf_EventExposure_Subscribe.

-	Naf_EventExposure_Unsubscribe.

-	Naf_EventExposure_Notify.

The following events can be subscribed by a NF consumer (Event ID is defined in clause 4.15.1):

-	Service Experience information, as defined in clause 6.4.2 of TS 23.288 [50].

-	Performance Data information, as defined in clauses 6.4.2 and clause 6.14.2 of TS 23.288 [50].

-	Collective Behaviour information, as defined in clause 6.5.2 of TS 23.288 [50].

-	UE Mobility information, as defined in clause 6.7.2.2 of TS 23.288 [50].

-	UE Communication information, as defined in clause 6.7.3.2 of TS 23.288 [50].

-	Exceptions information, as defined in clause 6.7.5.2 of TS 23.288 [50].

-	User Data Congestion information, as defined in clause 6.8.2 of TS 23.288 [50].

-	UE Data volume Dispersion information, as defined in clause 6.10.2 of TS 23.288 [50].

-	DN Performance information, as defined in clause 6.14.2 of TS 23.288 [50].

-	Global Navigation Satellite System (GNSS) UE location as defined in clause 6.17.2 of TS 23.288 [50].

-	E2E data volume transfer time information, as defined in clause 6.18.2 of TS 23.288 [50].

-	Relative Proximity information, as defined in clause 6.19.2 of TS 23.288 [50].

-	Signalling Storm information, as defined in clause 6.x.2 of TS 23.288 [50].

Event Filters are used to specify the conditions to match for notifying the event (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides some examples on how the conditions to match for event reporting can be specified for various Event IDs for AF exposure.

Table 5.2.19.2.1-1: Example of Event Filters for AF exposure events

| Event ID for AF exposure | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| Exceptions information | <Parameter Type = Exception ID, Value = Exception ID1> |
| Service Experience information | <Parameter Type = TAI, Value = TAI1> |
| Service Experience information | <Parameter Type = geographical area, Value = civic address1 or shape1> |
| Collective Behaviour information | <Parameter Type = collective attribute, Value = collective value1> |
| Collective Behaviour information | <Parameter Type = data processing type, Value = data processing type1> |

##### 5.2.19.2.2	Naf_EventExposure_Subscribe service operation

Service operation name: Naf_EventExposure_Subscribe

Description: The consumer NF subscribes the event to collect AF data for UE(s), group of UEs, or any UE, or updates the subscription which is already defined in AF.

Input, Required: Target of Event Reporting (either UE ID(s), or UE IPv4 address(es), or UE IPv6 prefix(es), or Internal/External Group Identifier, or indication that any UE is targeted or GPSI range), (set of) Event ID(s), Notification Target Address (+ Notification Correlation ID) and Event Reporting Information as defined in Table 4.15.1-1.

NOTE 1:	UE ID includes GPSI or SUPI.

Input, Optional: NF ID, Event Filter(s) associated with each Event ID, (set of) External Application Identifier(s), Subscription Correlation ID (in the case of modification of the existing subscription), Expiry time.

NOTE 2:	In the case of untrusted AF, NEF ID is used as NF ID.

Output, Required: Operation execution result indication. When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the local policy).

Output, Optional: First corresponding event report is included, if corresponding information is available (see clause 4.15.1).

# * * * * 8th change * * * *

##### 5.2.19.2.4	Naf_EventExposure_Notify service operation

Service operation name: Naf_EventExposure_Notify

Description: The AF provides the previously subscribed event information to the consumer NF which has subscribed to that event before.

Input, Required: Notification Correlation Information, Event ID, corresponding UE ID(s) (either external UE ID(s), or Internal/External Group Identifier, or UE IP v4 address(es) or UE IP v6 prefix(es), or GPSI range, time stamp.

NOTE:	UE ID includes GPSI or SUPI.

Input, Optional: Event specific parameter list.

Output, Required: None.

Output, Optional: None.

# * * * * 9th change * * * *

5.2.X	SCP Services

5.2.X.1	General

The following table shows the SCP Services and SCP Service Operations.

Table 5.2.X.1-1: NF services provided by the SCP

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nscp_EventExposure | Notify | Subscribe/Notify | NWDAF, DCCF |
|  | Subscribe | Subscribe/Notify | NWDAF, DCCF |
|  | Unsubscribe | Subscribe/Notify | NWDAF, DCCF |

5.2.X.2	Nscp_EventExposure Service

5.2.X.2.1	General

Service description: This service can expose information collected by the SCP to other NFs..

The following events can be notified to a NF consumer:

-	Service Signalling characteristics: An NF, NF set, Service Type, may be provided as target. A reporting threshold (in terms of absolute transaction or deviation from average, or in terms of a minimum percentage failed transactions) and time window may be provided as filters. The SCP reports the observed rate (i.e. number per time interval) of different types of signalling messages received and sent by different NF instance(s), the percentage of failed transactions, and when an NF is observed to fail to reply to the received requests. In particular, the following information can be contained in this Event ID:

-	The number of requests of a Request type received from NF during a time interval.

-	Average Ingress - Time duration between the request from NF and response to NF. (see NOTE X).

-	Average Egress - Time duration between the routed/forwarded request to producer NF and response received from it. (see NOTE X).

NOTE X: e.g. average time duration of a transaction (averaged over all of the transactions handled by the SCP within the requested time interval).

-	Number of successful responses associated to their initial requests during a time interval.

-	Number of failed responses associated to their initial requests during a time interval.

-  Distribution of reasons for failed responses associated to their initial requests, e.g. how many time-outs vs. how many server errors vs. how many consumer errors.

-	Number of received redundant signalling during a time interval. The redundant signalling means the signalling which is transmitted multiple times.

-	Load information indicates the current load of the NF and NF Service(s).

-	Average NF-NRF heart-beat related information per peer NF, such as response time, Number of retransmissions, heart-beat intervals.

NOTE:	The “Service Signalling characteristics” event is expected to be subscribed by an NWDAF to generate analytics related to abnormal signalling (see TS 23.288 [50]).

5.2.X.2.2	Nscp_EventExposure_Notify service operation

Service operation name: Nscp_EventExposure_Notify

Description: This service operation reports the event and related information to the consumer that has subscribed.

Input Required: Event ID, Notification Correlation Information.

Input, Optional:, Event specific parameters as described in clause 5.2.X.2.1, time stamps for the measurements.

Output Required: Result Indication.

Output, Optional: None.

5.2.X.2.3	Nscp_EventExposure_Subscribe service operation

Service operation name: Nscp_EventExposure_Subscribe

Description: This service operation is used by an NF to subscribe or modify a subscription to SCP event exposure notifications.

Input, Required: Event ID(s) defined in clause 5.2.X.2.1, Notification Target Address (+ Notification Correlation ID).

Input, Optional: Subscription Correlation ID (in the case of modification of the event subscription), Expiry time, Event specific target and filtering information defined in clause 5.2.X.2.1

Output, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Output, Optional: First corresponding event report is included, if available (see clause 4.15.1). Notification Target Address (+ Notification Correlation ID) is used to correlate Notifications sent by SCP with this subscription.

5.2.X.2.4	Nscp_EventExposure_UnSubscribe service operation

Service operation name: Nscp_EventExposure_UnSubscribe

Description: The NF consumer uses this service operation to unsubscribe from an existing subscription.

Input, Required: Subscription Correlation ID.

Input, Optional: None.

Output, Required: Operation execution result indication.

Output, Optional: None.

# * * * * End of changes * * * *