3GPP TSG-SA3 Meeting #120	S3-251032

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | 0078 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security requirements and procedures for CAPIF-8 reference point |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3_sec |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It was agreed in the study of CAPIF phase 3 to specify the security requirements and procedures for the CAPIF-8 reference point, i.e., between the ROF and the CCF. This is in line with the conclusions for KI#1.1 in TR 33.700-22. |  |  |  |  |  |  |  |  |
| Summary of change: |  | normative text for the security procedures for the CAPIF-8 reference point. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.C (new) and 4.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Merging S3-250411 and S3-250475 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 4.8	Security requirements on the CAPIF-8 reference points

The CAPIF-8 reference points between the ROF and the CCF shall fulfil the following requirements:

-	 [CAPIF-SEC-4.8-a] The transport of messages over the CAPIF-8 reference points shall be integrity protected.

-	 [CAPIF-SEC-4.8-b] The transport of messages over the CAPIF-8 reference points shall be protected from replay attacks.

-	 [CAPIF-SEC-4.8-c] The transport of messages over the CAPIF-8 reference points shall be confidentiality protected.

* * * Next Change * * * *

## 6.C	Security procedures for CAPIF-8 reference point

TLS shall be used to provide integrity protection, replay protection and confidentiality protection.

* * * End of Changes * * * *