3GPP TSG-SA5 Meeting #159	S5-250964

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0655 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.552 Add a new measurement related to number of failed NWDAF roaming analytics service subscriptions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NWDAF_OAM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-2-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As described in TS 23.288 (SA2) subclause 4.3 and 6.1.5, analytics may be exchanged between PLMNs (i.e. HPLMN and VPLMN). Considering the scenario of analytics exposure from VPLMN to HPLMN, failed roaming analytics service subscriptions may be caused in two cases: - Upon receiving an analytics information request/subscription from a service consumer NF in HPLMN, the H-RE-NWDAF may reject the subscription. - If the subscription is accepted by the H-RE-NWDAF, the H-RE-NWDAF sends a roaming analytics subscribe to the V-RE-NWDAF in VPLMN. The V-RE-NWDAF may also reject the subscription. By measuring the number of failed roaming analytics service subscriptions in two cases, operators can monitor the performance of different RE-NWDAFs. With this measurement, operators can analyze and evaluate the performance of the RE-NWDAFs, such as conducting more precise root cause analysis of failures. The measurement on the number of failed roaming analytics service subscriptions in the second case is missing. Similarly, considering the scenario of analytics exposure from HPLMN to VPLMN, failed roaming analytics service subscriptions may be caused in two cases: - Upon receiving an analytics information request/subscription from a service consumer NF in VPLMN, the V-RE-NWDAF may reject the subscription. - If the subscription is accepted by the V-RE-NWDAF, the V-RE-NWDAF sends a roaming analytics subscribe to the H-RE-NWDAF in HPLMN. The H-RE-NWDAF may also reject the subscription. The measurement on the number of failed roaming analytics service subscriptions in the second case is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new measurement “Number of failed roaming analytics service subscriptions rejected by the RE-NWDAF in the PLMN that provides analysis services” is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The existing measurement of the number of failed roaming analytics service subscriptions is not sufficiently clear to define the cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-250964 was S5-250165 |  |  |  |  |  |  |  |  |

Start of First change

5.18	Performance measurements for NWDAF

5.18.9	Measurements related to RE-NWDAF roaming analytics

5.18.9.x	Number of failed roaming analytics service subscriptions caused by the rejection from the RE-NWDAF in the other PLMN

a)	This measurement provides the number of roaming analytics service subscriptions rejected by the RE-NWDAF in the other PLMN. The RE-NWDAF receives subscriptions for the roaming analytics service. Each rejected subscription is added to the corresponding counter to obtain the number of failed roaming analytics service subscriptions.

b)	CC

c) 	In the procedure for analytics exposure (See TS 23.288 [59]), the analytics service subscriptions may be recieved and accepted by the first RE-NWDAF. This RE-NWDAF and the consumer NF are within the same PLMN where the subscription is requested. If the subscription is accepted by the first RE-NWDAF, the first RE-NWDAF sends a roaming analytics subscribe to the second RE-NWDAF. This RE-NWDAF is in the other PLMN that provides the requested analytics. The roaming analytics service subscriptions may be rejected by the second RE-NWDAF.

In the scenario where analytics services are exposed from the VPLMN to the HPLMN, when the subscription for the roaming analytics service is accepted by the H-RE-NWDAF, this subscription may be rejected by the V-RE-NWDAF. In this case, each subscription rejected by the V-RE-NWDAF is added to the corresponding counter.

In the scenario where analytics services are exposed from the HPLMN to the VPLMN, when the subscription for the roaming analytics service is accepted by the V-RE-NWDAF, this subscription may be rejected by the H-RE-NWDAF. In this case, each subscription rejected by the H-RE-NWDAF is added to the corresponding counter.

The measurement can be split into sub-counters per Analytics ID, per S-NSSAI and per PLMN. Furthermore, the measurement can be split into sub-counter per rejection cause.

d)	It is an integer value

e)	DANS. RoamingCaseRoamingAnalyticsSubscriptionRejected

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

Start of Second change

5.18.9.5	Number of failed roaming analytics service subscriptions caused by the rejection from the RE-NWDAF in the same PLMN as the consumer NF

a)	This measurement provides the number of roaming analytics service subscriptions received and rejected by the RE-NWDAF in the same PLMN as the consumer NF.

b)	CC

c)	In the procedure for analytics exposure (See TS 23.288 [59]), the analytics service subscription is received by the first RE-NWDAF. This first RE-NWDAF is within the same PLMN as the consumer NF. On receiving the subscription, this RE-NWDAF may reject the subscription with a proper cause in the response to the consumer NF. Eeach rejected subscription is added to the corresponding counter.

In the scenario where analytics services are exposed from the VPLMN to the HPLMN, the subscription may be rejected by the H-RE-NWDAF. In this case, each subscription rejected by the H-RE-NWDAF is added to the corresponding counter.

In the scenario where analytics services are exposed from the HPLMN to the VPLMN, the subscription may be rejected by the V-RE-NWDAF. In this case, each subscription rejected by the V-RE-NWDAF is added to the corresponding counter.

The measurement can be split into sub-counters per Analytics ID, per S-NSSAI and per PLMN. Furthermore, the measurement can be split into sub-counter per rejection cause (e.g. due to user consent for UE(s) revoked, due to RE-NWDAF overloading, out-of-coverage of roaming UE(s) from the serving RE-NWDAF, unavailable data related to roaming UE(s), roaming analytics not ready before the deadline, etc.).

d)	It is an integer value

e)	DANS. RoamingCaseAnalyticsSubscriptionRejected

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS.

End of Changes